| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 16th Meeting: Poznan, Poland, July, 2005 | Document: JVT-P202 Filename: input.doc |
| --- | --- |

| Title: | Joint Scalable Video Model JSVM-3 |  |  |
| --- | --- | --- | --- |
| Status: | Draft Output Document from JVT |  |  |
| Purpose: | Text of the Joint Scalable Video Model |  |  |
| Editor(s) / Contact(s): | Julien Reichel VisioWave (GE Security), Rte. de la Pierre 22, CH-1024 Ecublens, Switzerland Heiko Schwarz Heinrich Hertz Institute (FhG), Einsteinufer 37, D-10587 Berlin, Germany Mathias Wien Institut für Nachrichtentechnik, RWTH Aachen University, D-52056 Aachen, Germany | Tel: Fax: Email: Tel: Fax: Email: Tel: Fax: Email: | +41 (21) 695-0041 +41 (21) 695-0001 reichel@visiowave.com +49 (30) 31002-226 +49 (30) 39272-00 heiko.schwarz@hhi.fhg.de +49 (241) 80-27681 +49 (241) 80-22196 wien@ient.rwth-aachen.de |
| Source: | JVT |  |  |

_____________________________

[Ed.Note(JR,MW) :

- The following non-normative items need to be added to the JSVM

  - [H. Schwarz] JVT-P059 Comparison of MCTF and closed-loop hierarch. B pics., non-normative - DONE

  - [N. Cammas] JVT-P029 (M12304) CE05: CE report on quality layers: Non-normative part adopted. - DONE

  - JVT-P028 [J. Reichel, D. Santa Cruz, F. Ziliani] CE01: Improved residual picture buffer management / JVT-P040 [M.-H. Lee] Progressive MCTF - DONE

- The following items will be included into the JSVM software; a short explanation of the tools will be provided and included into the JSVM for information.

  - [S. Sun] JVT-P012 CE10Pt1: Direct interpolation for upsampling: Adopt an additional option into software as at this meeting. Verify performance related to activities of resampling AHG. - DONE

  - [J. Ridge] JVT-P056-L Variable length codes for SVC: CAVLC for base layer. Implementation of existing tools. Adopt into JSVM software, New tools for scalable coding. - DONE

]

# Index

Index	2

Foreword	3

Introduction	3

1	Introduction to the Scalable Coder	4

1.1	General Coder Structure	4

1.1.1	Hierarchical B pictures	5

1.1.2	Motion Compensated Temporal Filtering (MCTF)	5

1.2	Scalability Dimensions	7

1.2.1	Temporal Scalability	9

1.2.2	Fine Grain SNR (Quality) Scalability (FGS)	10

1.2.3	Spatial and Coarse Grain SNR (Quality) Scalability	11

1.2.4	Combined Scalability	13

1.2.5	Extended Spatial Scalability	15

2	Encoder Description	19

2.1	Generation of the multi-resolution inputs	19

2.2	Temporal decomposition	19

2.2.1	Adaptive prediction/update steps for MCTF	20

2.2.2	Progressive MCTF	23

2.3	CAVLC encoding process for FGS (descriptive)	23

2.3.1	Introduction	23

2.3.2	Initialisation	23

2.3.3	Significance pass	24

2.3.4	Refinement pass	25

3	Operational Encoder Control	26

3.1	Scalability and Decomposition Structure	26

3.2	Closed-loop and open-loop encoder control for hierarchical B pictures	26

3.3	Open-loop encoder control for MCTF	27

3.4	Adaptive GOP structure	27

3.5	Motion estimation and mode decision process	31

3.6	Fine grain SNR scalability	32

3.6.1	Simple truncation of progressive refinement NAL units	32

3.6.2	Quality layers	33

3.7	Quantizer selection	34

3.8	Rate Control	35

4	Bitstream Extraction	35

4.1	Scalability Information SEI	36

4.1.1	Scalability Information SEI message syntax	37

4.1.2	Scalability Information SEI message semantics	37

4.2	Analysis of the input bitstream	38

4.3	Bistream Extraction	38

5	References	39

[Ed.Note(MW): Proposed changes to the WD are now kept in an Annex to this document which is hosted in a separate file]

# Foreword

Reference software, descriptions of reference encoding methods, description of  the scalability features and extraction methods are useful in aiding users of a video coding standard to establish and test conformance and interoperability, and to educate users and demonstrate the capabilities of the standard.  In this spirit, the methods described herein and the accompanying software modules are provided for implementers of the scalable extension of the ITU-T Recommendation H.264 | ISO/IEC International Standard ISO/IEC 14496-10 advanced video coding.

# Introduction

The scalable video coding (SVC) is currently being developed as an extension of the ITU-T Recommendation H.264 | ISO/IEC International Standard ISO/IEC 14496-10 advanced video.  In addition to implementing the normative decoding process specified in the text of the Working Draft (WD)[8], software is provided to demonstrate effective (non-normative) encoding techniques and (non-normative) extraction techniques for use with the standard.  These non-normative aspects are described in this document.

In addition this document also describes possible modification to the normative and non-normative description of the future version of the WD and the JSVM.

Joint MODEL FOR
NON-NORMATIVE ASPECTS OF Scalable VIDEO CODING

# Introduction to the Scalable Coder

## General Coder Structure

The applicable JSVM encoder structure depends on the scalability dimensions that are to be achieved. An example for an encoder using three spatial layers and motion compensated temporal filtering is presented in Figure 1.

For each spatial layer a motion compensated temporal decomposition is performed. This decomposition provides temporal scalability. Motion information from lower spatial layers can be used for prediction of motion on the higher layers. For texture encoding, spatial prediction between successive spatial layers can be applied to remove redundancy.

The residual signal resulting from intra prediction or motion compensated inter prediction is transform coded. A quality base layer residual provides minimum reconstruction quality at each spatial layer. This quality base layer can be encoded into an AVC compliant stream if no inter layer prediction is applied. For quality scalability, quality enhancement layers are additionally encoded. These enhancement layers can be chosen to either provide coarse or fine grain quality (SNR) scalability.

Figure 1: Scalable encoder using a multi-scale pyramid with 3 levels of spatial scalability.

In the JSVM temporal scalability can be achieved in two ways. Either hierarchical B pictures or motion compensated temporal filtering are employed. The concept of hierarchical B pictures provides a fully predictive structure that is already provided with AVC. Motion compensated temporal filtering introduces an additional temporal update-step that distributes the introduced quantization error more evenly over the pictures over a group of frames.

### Hierarchical B pictures

In AVC, any picture can be marked as reference picture and used for motion-compensated prediction of following pictures independent of the coding types of the corresponding slices. The behaviour of the decoded picture buffer (DPB), which can hold up to 16 pictures, can be adaptively controlled by memory management control operation (MMCO) commands, and the reference pictures of the DPB that are used for motion-compensated prediction of another picture can be arbitrarily selected via reference picture list re-ordering (RPLR) commands. These features of AVC allow the selection of arbitrary coding/prediction structures, which are not possible with older video coding standards.

A typical hierarchical prediction structure with 4 dyadic hierarchy stages is depicted in Figure 2. This structure provides 4 temporal scalability levels.The first picture of a video sequence is intra-coded as IDR picture; so-called key pictures (black in Figure 2) are coded in regular (or even irregular) intervals. At this, a picture is called a key picture when all previously coded pictures precede this picture in display order. A key picture and all pictures that are temporally located between the key picture and the previous key picture (the IDR picture at the beginning of a video sequence is also a key picture) are considered to build a group of pictures (GOP). The key pictures are either intra-coded (e.g. in order to enable random access) or inter-coded using previous (key) pictures as reference for motion compensated prediction. The remaining pictures of a GOP are hierarchically predicted as illustrated in Figure 2.

Figure 2: Dyadic hierarchical coding structure with 4 temporal levels and a GOP size of 8. Each B pictures is predicted using 2 reference pictures, which are the nearest pictures of the lower temporal level from the past and the future.

### Motion Compensated Temporal Filtering (MCTF)

The temporal decomposition of the input signal relies on Motion Compensated Temporal Filtering (MCTF). MCTF is based on the lifting scheme [3]. The lifting scheme insures perfect reconstruction of the input in the absence of quantization of the decomposed signal. This property is valid even if non-linear operations are used during the lifting operation.

The generic lifting scheme consists of three types of operations: polyphase decomposition, prediction(s), and update(s). In most cases the MCTF is restricted to a special case of lifting scheme with only one prediction and one update step. Figure 3 illustrates the lifting representation of an analysis-synthesis filter bank.

Figure 3: Lifting representation of an analysis-synthesis filter bank.

At the analysis side (a), the odd samples s[2k+1] of a given signal s are predicted by a linear combination of the even samples s[2k] using a prediction operator P(s[2k]) and a high pass signal h[k] is formed by the prediction residuals. A corresponding low-pass signal l[k] is obtained by adding a linear combination of the prediction residuals h[k] to the even samples s[2k] of the input signal s using the update operator U(h[k]):

Since both the prediction and the update step are fully invertible, the corresponding transform can be interpreted as critically sampled perfect reconstruction filter bank. The synthesis filter bank simply consists of the application of the prediction and update operators in reverse order with the inverted signs in the summation process followed by the reconstruction process using the even and odd polyphase components. For a normalization of the low- and high-pass components, appropriately chosen scaling factors Fl and Fh are applied, respectively. However, in practice, these scaling factors are included into the quantization step sizes as described in Section 3.7.

Let s[x, k] be a video signal with the spatial coordinate x = (x, y)T and the temporal coordinate k. The prediction and update operators for the temporal decomposition using the lifting representation of the Haar wavelet are given by

For the 5/3 transform, the prediction and update operators are given by

The extension to motion-compensated temporal filtering is realized by modifying the prediction and update operators as follows

where the reference indices r  0 allow a general frame-adaptive motion-compensated filtering. The motion vectors m are not restricted to sample-accurate displacements. In case of sub-sample accurate motion vectors, the term s[x+m, k] has to be interpreted as a spatially interpolated value.

As can be seen from the above equations, both the prediction and update operators for the motion-compensated filtering using the lifting representation of the Haar wavelet are equivalent to uni-directional motion-compensated prediction. For the 5/3 wavelet, the prediction and update operators specify bi-directional motion-compensated prediction.

Since bi-directional motion-compensated prediction generally reduces the energy of the prediction residual but increases the motion vector rate in comparison to uni-directional prediction, it is desirable to switch dynamically between uni- and bi-directional prediction, and thus between the lifting representation of the Haar and the 5/3 wavelet. The choice of the transform can be made on a macroblock or sub-macroblock basis and is signalled as part of the motion information.

The difference between MCTF and hierarchical B pictures is that with hierarchical B pictures, the temporal update step is omitted, leading to a purely predictive coding structure. An MCTF without update step is also referred to as unrestricted MCTF (UMCTF).

## Scalability Dimensions

There are two different ways of introducing scalability in a codec, either by using a technique that is intrinsically scalable (such as bitplane arithmetic coding) or by using a layered approach (same concept as the one that is used in many previous standards [7-9]). Here, a combination of the two approaches to enable a full spatio-temporal and quality scalable codec is used. Temporal scalability is enabled by hierarchical B pictures or MCTF, whereas spatial scalability is provided using a layered approach. For quality (SNR) scalability, two different possibilities are provided; an embedded quantization approach for coarse grain scalability and a fine grain scalability (FGS) approach based on the principle of sub-bitplane arithmetic coding.

A detailed block diagram of the encoder is presented in Figure 4. As in AVC, the encoding of the input video is performed on a macroblock basis. As the codec is based on a layer approach to enable spatial scalability, the encoder provides a down-sampling filter stage that generates the lower resolution signal for each spatial layer. In its basic version, the down-sampling ratio is equal to 2 (the base layer resolution is half the spatial resolution of its spatial enhancement layer). Extended Spatial Scalability, as described in section 1.2.5, generalizes this concept by enabling the base layer to be a cropped version of its enhancement layer with a down-sampling ratio different from 1 or 2. Depending on the application requirements in terms of spatio-temporal scalability, the inputs of the different layer may have different frame-rates.

The input video signal of each layer is temporally decomposed using either hierarchical B pictures or MCTF. The output of this process is a set of temporal low-pass {L} and high-pass {H} picture (or residual picture) with residual texture information and motion description information.

The encoding of the motion and the residual texture is based on AVC [1], with few modifications to handle the spatial and SNR scalability aspects.

Motion information from the lower layer may be used for prediction of the higher layer. The application of this prediction is switchable on a macroblock basis.

Each macroblock can be coded either intra or inter. Here, inter coding denotes the application of a motion prediction/compensation scheme to predict the current macroblock. In case of intra coding, a prediction from surrounding macroblocks or from other spatial layers is possible. These prediction techniques do not employ motion information and hence, are referred to as intra prediction techniques.

For each spatial resolution, the residual texture is encoded using a SNR base layer at a certain quantization level and enhancement layers that are provided in an embedded structure.

Figure 4: Example for the encoder structure providing two spatial resolution layers. For temporal decomposition, either hierarchical B pictures or MCTF can be employed.

### Temporal Scalability

The temporal decomposition framework of hierarchical B pictures and MCTF inherently provides temporal scalability. By using n decomposition stages, up to n levels of temporal scalability can be provided.

In Figure 5, an example for the temporal decomposition of a group of 12 pictures using 3 decomposition stages is illustrated. This structure provides a non-dyadic decomposition in the coarsest layer. Note that other decomposition structures are available providing dyadic decomposition for all layers. If only the low-pass pictures {L}3 that are obtained after the third (coarsest) decomposition stage are transmitted, the picture sequence {L}3* that can be reconstructed at the decoder side has 1/12 of the temporal resolution of the input sequence. The picture sequence {L}3 at the coarsest temporal layer is also referred to as temporal base layer. By additionally transmitting the high-pass pictures {H}3, the decoder can reconstruct an approximation of the picture sequence {L}2 that has 1/4 of the temporal resolution of the input sequence. The high-pass pictures {H}3 are also referred to as the first temporal enhancement layer. By further adding the high-pass pictures {H}2, a picture sequence {L}1* with half the temporal resolution can be reconstructed. And finally, if the remaining high-pass pictures {H}1 are transmitted, a reconstructed version of the original input sequence with the full temporal resolution is obtained.

Figure 5: Illustration of temporal scalability.

In general, by using n decomposition stages, the decomposition structure can be designed in a way that n levels of temporal scalability are provided with temporal resolution conversion factors of 1/m0, 1/(m0m1), …, 1/(m0m1…mn-1), where mi represents any integer number greater than 1. Therefore, a picture sequence has to be coded in groups of N0 = (jm0m1…mn-1) pictures with j being an integer number greater than 0. The GOP size does not need to be constant within a video sequence.

It should be mentioned that a similar degree of temporal scalability could be realized with standard AVC coding (or the presented coding scheme with GOP’s of only 1 picture) by using sub-sequences and/or regularly inserted non-reference pictures.

### Fine Grain SNR (Quality) Scalability (FGS)

Within each spatial resolution fine grain SNR scalability is achieved by encoding successive refinements of the transform coefficients, starting with the minimum quality provided by AVC compatible intra / residual coding. This is done by repeatedly decreasing the quantization step size and applying a modified CABAC entropy coding process akin to sub-bitplane coding. This coding mode is referred to as progressive refinement.

For each quality enhancement layer the coding process for the transform coefficient refinement levels is divided into two scans: significant scan, and the refinement scan.

In the first scan, the refinement levels of all transform coefficients with the following properties are coded:

- The transform coefficient levels that have been coded in the base layer representation and all subordinate enhancement layer representations are equal to zero, also called  non-significant transform coefficient.

In the second scan, the refinement levels of all remaining transform coefficients, i.e. those with the following properties, are coded:

- A transform coefficient level not equal to zero has been coded in the base layer or any previous enhancement layer representation, also called significant transform coefficients.

In each scan the corresponding transform coefficient are transmitted.

### Spatial and Coarse Grain SNR (Quality) Scalability

The presented subband approach provides the possibility to efficiently incorporate coarse grain SNR (CGS) scalability using quality layers. In fact the exact same mechanism is used both for the spatial and for the CGS scalability. The only difference is that motion and texture information are scaled in case of increase of resolution between layers. This is the same concept as the ones used in the video coding standards H.262/MPEG-2 Visual [4], H.263 [5], or MPEG-4 Visual [6] for the spatial scalability.

Across spatial resolution spatial scalability is achieved by conceptually using a pyramid of spatial resolutions. Each spatial resolution is coded using a new layer, which include refinement of the motion and texture information. Within each spatial resolution coarse grain SNR scalability is achieved by encoding successive quality layers, starting with the minimum quality provided either by AVC compatible texture encoding or by a resolution enhancement layer. This is done by decreasing the quantization step size and applying an entropy coding process for the differential information.

MCTF is independently applied in each spatial layer and a large degree of inter-layer prediction is incorporated. Intra and inter macroblocks can be predicted using the corresponding signals of previous layers. Moreover the motion description of each layer can be used for a prediction of the motion description for following enhancement layers.

#### Inter-layer intra texture prediction

Intra texture prediction using information from the previous layer is provided in the Intra_Base macroblock mode. Three possible configurations are can be applied for this mode.

- Unrestricted inter-layer intra texture prediction

The inter-layer intra texture prediction can be applied to any block without restrictions on the layer to be predicted from. In this configuration, the decoder has to decode all lower spatial resolutions that are provided in the bitstream for the reconstruction of the target resolution. In the JSVM software [7], this mode can be enabled by setting the following compiler flag:

#define UNRESTRICTED_INTER_LAYER_PREDICTION   1   // unrestricted inter-layer prediction (not WD / JSVM compatible)

- Constrained inter-layer intra texture prediction

The inter-layer intra texture prediction can be applied to macroblocks for which the corresponding blocks of the base layer are located inside intra-coded macroblocks. With this mode, the temporal reconstruction is only required for the spatial layer that is actually decoded. For key pictures, multiple decoding loops are required.

- Constrained inter-layer intra texture prediction for single-loop decoding

The inter-layer intra texture prediction can be applied to macroblocks for which the corresponding blocks of the base layer is located inside intra-coded macroblocks for the temporal decomposition as well as for key pictures. In this configuration, only a single decoding loop at the target spatial resolution is required.

The single/multiple-loop decoding for key pictures is controlled by the value of constrained_intra_pred_flag of PPS that is used by the base layer key pictures.

For generating the intra prediction signal for high-pass macroblocks coded in Intra_Base mode, the corresponding blocks of the base layer high-pass signal are directly de-blocked and eventually interpolated as illustrated in Figure 6. The interpolation depends on the actual spatial resolution ratio between the enhancement and base layers pictures. When this ratio is equal to 2, the interpolation is performed using the half-pel interpolation filter of AVC. For any other ratio different from 1 and 2, an interpolation process based on the quarter-pel AVC interpolation is used, as described in section 1.2.5.

Figure 6: Low-complexity inter-layer prediction of intra macroblocks

Before applying the interpolation filter, the intra macroblocks of the previous layer are extended by a 4-sample border in each direction using the border extension process specified in the WD. Before performing the border extension, the deblocking filter is applied to all boundaries inside intra macroblock or between intra macroblocks.

#### Inter-Layer Motion Prediction

For encoding the motion field of an enhancement layer, two macroblock modes are possible in addition to the modes applicable in the previous layer: a base layer mode and a quarter pel refinement mode.

If the base layer mode is used then no further motion information is transmitted for the corresponding macroblock. This macroblock mode indicates that the motion/prediction information including the macroblock partitioning of the corresponding macroblock of the previous layer is used. When the previous layer represents a layer with a reduced spatial resolution, the motion vector field including the macroblock partitioning is scaled as described in the WD. If the previous layer macroblock (or macroblocks in case of Extended Spatial Scalability) represents an intra macroblock, the current macroblock mode is set to Intra_Base (intra macroblock with prediction from previous layer). For the macroblock partitions of the current macroblock, the same reference indices as for the corresponding macroblocks/sub-macroblock partitions of the previous layer blocks are used. The associated motion vectors are multiplied by a factor corresponding to the resolution ratio between the enhancement layer and its base layer.

The quarter pel refinement mode is used only if the previous layer represents a layer with a reduced spatial resolution relative to the current layer. The quarter pel refinement mode is similar to the base layer mode. The macroblock partitioning as well as the reference indices and motion vectors are derived as for the base layer mode. However, for each motion vector a quarter-sample motion vector refinement (-1, 0, or +1 for each motion vector component) is additionally transmitted and added to the derived motion vectors.

If none of the techniques above are used, the macroblock mode as well as the corresponding reference indices and motion vector differences are encoded according to the AVC syntax.

#### Inter-Layer Residual Prediction

If the motion information from one layer to the next is changed, it may be or may be not beneficial to predict residual information of the enhancement layer from the previous layer. If the motion vector(s) for a block of the current layer is/are identical or nearly identical to the motion vector(s) of the corresponding blocks in the previous layer, it is likely that the coding efficiency can be increased when the coded previous layer residual (high-pass signal) is used as prediction for the current residual (high-pass signals) and thus only the difference between the current residual signal and the previous layer reconstruction is coded. However, when the motion vector(s) are not similar it is very unlikely that a prediction of the residual signal could improve the coding efficiency. Consequently, an adaptive approach for the prediction of the residual signal is used.

The application of inter-layer residual prediction is indicated by the syntax element residual_prediction_flag as specified in the WD [8].

If the previous layer represents a layer with half the spatial resolution of the current layer, the residual signal is upsampled using a separable bi-linear filter [1,1]/2 before it is used as prediction signal. For inter-layer spatial resolution ratio different from 1 and 2, the interpolation process is based on the quarter-pel interpolation process as specified in AVC. The interpolation filter is not applied across boundaries between transform blocks; the right and lower block boundaries are extended by constant border extension.

### Combined Scalability

The concepts of temporal, SNR, and spatial scalability presented in Sections 1.2.1 – 1.2.3, can easily be combined to a general scalable coding scheme, which can provide a wide range of temporal, SNR, and spatial scalability. It should be noted that when only one type of scalability (temporal, SNR or spatial resolution) is considered, there is an intrinsic notion of layering. For instance a CIF layer is used to increase to resolution of a base QCIF layer. When combined scalability is considered, the strict notion of layer might not apply anymore, as there are no intrinsic notions of layers in the three dimensional scalability spaces obtained by combining the three types of scalability. For instance it is perfectly possible to build a bitstream for a video in CIF@30 fps from which one can extract both a stream QCIF@30 fps and a stream CIF@15 fps. In this case the notion of layer at the bitstream level can not be applied.

Example of combined scalability

In Figure 7, an example for combined scalability is illustrated. In this example, the spatial base layer (QCIF) is coded at a frame rate of 15 Hz using standard AVC, where the structure with hierarchical B pictures depicted in section 1.1.1 is used to provide temporal scalability. The spatial enhancement layer in CIF resolution is coded at a frame rate of 30 Hz, using the MCTF coding scheme with 4 decomposition stages. Each of these spatial layers provides SNR scalability by using FGS.

The QCIF layer (layer 0) is encoded at a maximum bitrate of 80 kbit/s. By exploiting the SNR scalability provided by progressive refinement a QCIF 15 Hz bitstream can be extracted and transmitted at a bitrate ranging from 41 kbit/s (minimum quality provided without any progressive refinements) and 80 kbit/s. A reduced frame rate bitstream can also be extracted by dropping the frames labeled as B3. In combination with SNR scalability a QCIF 7.5 Hz bitstream can be extracted and transmitted with a bitrate ranging from 32 to 66 kbit/s by exploiting progressive refinements on the remaining frames.

The CIF layer (layer 1), which includes layer 0, is encoded at a maximum bitrate of 256 kbit/s. By exploiting the SNR scalability a CIF 30 Hz bitstream can be extracted with bitrates between 115 and 256 kbit/s. By exploiting the temporal scalability of the MCTF coding scheme bitstreams with reduced frame rates of 15, 7.5, 3.75, and 1.875 Hz can be extracted by dropping the sets of temporal subbands {H}0, {{H}0,{H}1}, {{H}0,{H}1,{H}2}, and {{H}0,{H}1,{H}2,{H}3}, respectively. In combination with SNR scalability the range of bitrates shown in Figure 7 can be extracted for each frame rate.

Figure 7: Example for combined scalability.

### Extended Spatial Scalability

Extended Spatial Scalability enables a generalized relation between successive spatial layers. A picture of a lower spatial layer may represent a cropped area of the higher resolution picture and the relation between successive spatial layers does not need to be dyadic. Geometrical parameters defining the cropping window and the down-sampling ratio can either be defined at the sequence level, or evolve at the picture level.

Extended Spatial Scalability includes three tools:

- cropping,

- up-sampling with horizontal and vertical inter-layer size ratios of 1.5,

- generic up-sampling (any horizontal and vertical inter-layer size ratios).

The following combinations can be considered (corresponding to different values of the syntax element extended_spatial_scalability):

| extended_spatial_scalability | allowed up-sampling ratios | cropping window |
| --- | --- | --- |
| 0 | ratio 1 ratio 2 | no crop |
| 1 | ratio 1 ratio 2 ratio 1.5 generic | crop with macroblock alignment sequence level |
| 2 | generic | crop with non macroblock alignment picture level |

Let’s consider two successive spatial layers, a low layer (base layer) and a high layer (enhancement layer), linked by the following geometrical relations (see Figure 8). Width and height of enhancement layer pictures are defined respectively as wenh and henh. Base layer pictures dimensions are defined as wbase and hbase. Base layer pictures are sub-sampled versions of pictures of dimensions wextract and hextract , partially or totally inside the enhancement layer pictures, positioned at coordinates (xorig , yorig) in the enhancement layer pictures coordinates system. In Figure 8, the enhancement and base layer pictures are divided in macroblocks. wextract / wbase and hextract / hbase correspond actually to the upsampling factors between base layer pictures and extraction pictures in enhancement layer. The parameters (xorig , yorig , wextract, hextract) completely define the geometrical relations between high and low layer pictures. In a standard dyadic scalable configuration (extended_spatial_scalability = = 0), these parameters are equal to (0, 0, 2*wbase, 2*hbase).

Figure 8: relations between enhancement layer and base layer

When using a cropping window aligned on macroblock grid and upsampling ratios of 1 or 2, the motion and texture upsampling processes as described in previous sections apply. Otherwise, either the upsampling tool with ratio 1.5 or the generic upsampling tool have to be used. These tools are described in the following.

A macroblock of enhancement layer may have either no corresponding block in the base layer (on borders of the enhancement layer picture), or it may have one or several corresponding macroblocks in the base layer, as illustrated in Figure 9.

Figure 9 : macroblock overlapping between upsampled base layer picture (dashed lines) and enhancement layer picture (solid lines)

According to the specification in the WD, inter layer prediction is only possible for macroblocks fully embedded in the scaled base layer window.

#### Inter-Layer Motion Prediction with Extended Spatial Scalability

An enhancement layer macroblock can exploit inter-layer prediction using scaled base layer motion data, using either “BASE_LAYER_MODE” or “QPEL_REFINEMENT_MODE”, as in case of dyadic spatial scalability. In case of using one of these two modes, the enhancement layer macroblock is reconstructed with default motion data deduced from the base layer.

Both for the ratio 1.5 and generic cases, the motion up-sampling is done using the following steps:

- extraction of base layer macroblocks

- sub-partitions motion inheritance

- partitions motion inheritance

- sub_mb_types derivation

- mb_type derivation

The full process is described in details in WD (section 8.4.1.6).

#### Inter-Layer texture Prediction with Extended Spatial Scalability

The inter-layer texture prediction process consists in up-sampling the base layer texture based on the quarter-pel interpolation filter specified in AVC (that is, first applying the half-pel interpolation, then a bilinear interpolation to derive the quarter-pel samples). For intra texture, this interpolation crosses block boundaries. For residual texture, interpolation does not cross transform block boundaries.

In order to consider a potential chroma phase shift between base and enhancement layers, a slightly different process can be applied for luma and chroma.

For the luma samples, the actual prediction sample pred[X, Y] at a position ( X, Y ) in the enhancement layer is deduced as follows:

pred[ X, Y ] = interp[ xI , yI ]

with

xI=(X−xorig)∗wbase∗4+2∗(wbase−wextract)/wextractyI=(Y−yorig)∗hbase∗4+2∗(hbase−hextract)/hextract

and interp is the samples array resulting from the quarter-pel interpolation of the base layer luma texture.

For the chroma samples, the actual prediction sample predC[X, Y] at a position ( X, Y ) in the enhancement layer is deduced as follows:

predC[ X, Y ] = interpC[ xI , yI ]

with

xI=(X−xorigC)∗wbaseC∗4+(2+4∗pbasex)∗wbaseC−(2+4∗pextractx)∗wextractC)/wextractCyI=(Y−yorigC)∗hbaseC∗4+(2+4∗pbasey)∗hbaseC−(2+4∗pextracty)∗hextractC)/hextractC

and interpC is the samples array resulting from the quarter-pel interpolation of the base layer chroma texture (C being either Cr or Cb). xorigC , yorigC , wextractC , hextractC , wextractC , hextractC are the chroma geometric parameters, and pbasex , pbasey , pextractx , pextracty are the horizontal and vertical chroma phase shifts in in units of quarter sampling space, with respect to the luma pictures, of the base and enhancement layers.

For generating the intra prediction signal for high-pass macroblocks coded in Intra_Base mode, the corresponding 8x8 blocks of the base layer high-pass signal are directly de-blocked and interpolated, as in case of dyadic spatial scalability. The same padding process is applied for deblocking.

##### Inter-Layer Texture Prediction (Direct Interpolation Method to Be Further Tested)

(Note: If approved in the next meeting, the subclause 1.2.5.2 will be replaced by the subclause 1.2.5.2.1.)

The inter-layer texture prediction process consists of up-sampling the base layer textures based on the direct interpolation method. For intra texture, a set of 6-tap filters is applied and the interpolation crosses transform block boundaries. The integer-based 6-tap filters are derived from the Lanczos3 filter.  For residual texture, bilinear filters are applied and the interpolation does not cross transform block boundaries.

In order to consider a potential chroma phase shift between base and enhancement layers, a slightly different process can be applied for luma and chroma.

For the luma samples, the actual prediction sample pred[X, Y] at a position ( X, Y ) in the enhancement layer is deduced as follows:

pred[ X, Y ] = interp[ xI , yI ]

with

and interp is the samples array resulting from the 1/16-sample interpolation of the base layer luma texture.

For the chroma samples, the actual prediction sample predC[X, Y] at a position ( X, Y ) in the enhancement layer is deduced as follows:

predC[ X, Y ] = interpC[ xI , yI ]

with

and interpC is the samples array resulting from the 1/16-sample interpolation of the base layer chroma texture (C being either Cr or Cb). xorigC , yorigC , wextractC , hextractC , wextractC , hextractC are the chroma geometric parameters, and pbasex , pbasey , pextractx , pextracty are the horizontal and vertical chroma phase shifts in units of quarter sampling space, with respect to the luma pictures, of the base and enhancement layers.

For generating the intra prediction signal for high-pass macroblocks coded in Intra_Base mode, the corresponding blocks of the base layer high-pass signal are directly de-blocked and interpolated, as in case of dyadic spatial scalability. The same padding process is applied for deblocking.

# Encoder Description

Single layer coding is mostly based on AVC technology with the MCTF extension.

This section summarizes the main tools used in the encoding process and should be considered as informative. An exact description of the normative tools is present in the WD [8].

## Generation of the multi-resolution inputs

In order to generate the multi-resolution input the JSVM encoder, down-sampling filters are used by the JSVM software [7].

A separable 13 tap filter h(k) is used prior to the 2D down-sampling of the input picture. This filter corresponds to filter B defined in the MPEG-4 Visual Verification Model [9]. The coefficients of h(k) are defined as:

h(k)=[ 2,0,-4,-3,5,19,26,19,5,-3,-4,0,2]/64.

## Temporal decomposition

In the JSVM software [7] the temporal decomposition relies on the basic pyramidal decomposition structure as it is described in Section 1.1 for hierarchical B pictures and MCTF. The decomposition is applied on a GOP basis as depicted e.g. in Figure 10.

The temporal decomposition structure allows for prediction over GOP boundaries. In the prediction steps, the low-pass picture of the previous GOP that is obtained after performing all N decomposition stages is used as additional reference picture for motion-compensated prediction. However, the motion-compensated update is only performed inside the GOP; i.e. the low-pass picture of the previous GOP that is used for prediction is not updated. An example for this structure using MCTF is depicted in Figure 10. Note that this decomposition structure is conceptually similar to the open-GOP structure used in hybrid video coding schemes. The delay associated with this GOP structure is identical to the delay introduced by the independent GOP structure. However, the subjectively disturbing temporal blocking artefacts are significantly reduced.

Figure 10: Example for an MCTF structure for one GOP with prediction over GOP boundaries.

### Adaptive prediction/update steps for MCTF

The decomposition structure above is directly coupled to a structural delay of 2N-1 frames at the highest temporal resolution. This delay can be adjusted by limiting the reference picture lists (list 0 and list 1) that are used for the prediction and update steps.

Let fin be the frame rate of the highest temporal level in Hz, and dmax be the maximum structural delay in seconds. Then, the delay in frames for the highest temporal resolution is given by

.

In order to enable structural encoding-decoding delays df0 that are less than 2N-1 frames, with N being the number of dyadic temporal decomposition stages, a group of pictures is partitioned into sub-groups. Neither backward prediction steps nor update steps (backward or forward) are allowed across the corresponding partition boundaries, so that these sub-groups of pictures can be encoded and decoded independently without influencing previous or following sub-groups.

Let l specify the temporal level. l is equal to zero for the first decomposition stage and is increased by one for each following dyadic temporal decomposition stage. The partitioning for the l-th decomposition level is controlled by two parameters: the partition size Gl and the sub-partition size Cl. These parameters are determined by the following algorithm, where Dl is an auxiliary variable:

for( Dl = 0; df0 >> ( Dl + l ); Dl++ );

Dl = min( Dl, N – l );

Gl = ( 1 << Dl );

Cl = max( 0, Gl – (df0 >> l ) – 1 );

Figure 11 illustrates the partitioning and the corresponding decomposition structure for a group of 16 pictures (N = 4) and a structural encoding-decoding delay df0 of 4 frames. In that case, the parameters Gl and Cl with l = 0..3 are given by

Gl = { 8, 4, 2, 1 }

Cl = { 3, 1, 0, 0 }.

Figure 11: Illustration of the GOP partitioning for a group of 16 pictures and a structural encoding-decoding delay of 4 frames at the highest temporal resolution. For clarity, only the prediction and update steps using directly neighboring reference pictures are illustrated.

Note that the partitioning is always consistent across all decomposition stages. That means, the location of the partition and sub-partition boundaries is identical for all temporal decomposition stages. And furthermore, the decomposition structure for the temporal levels 1 to N-1 and a delay of df frames is always identical to the decomposition structure for the temporal levels 0 to N-2 and a delay of df / 2 frames.

Given the parameters Gl and Cl and thus the partitioning of a group of pictures, the size of the reference picture lists that are used in the prediction and update steps is restricted in a way that any backward prediction or update across a sub-partition or partition boundary is discarded.

Let nP0[i] specify the size (in frames) for the prediction list 0 (forward prediction list) for the picture i, and let nP1[i] specify the size for the prediction list 1 (backward prediction list) for picture i. Similarly, let nU0[i] specify the size (in frames) for the update list 0 (forward update list) for the picture i, and let nU1[i] specify the size for the update list 1 (backward update list) for picture i. The picture index i is related to the low-pass size before the l-th decomposition stage is performed. The picture index 0 corresponds to the low-pass frame of the previous GOP, which is used as additional reference frame in the prediction steps. The pictures 1, 3, 5, … are predicted from the pictures 0, 2, 4, … in the prediction steps and replaced by the corresponding high-pass pictures. Thereafter, the pictures 2, 4, 6, … are updated using the high-pass pictures 1, 3, 5, … Note that picture 0 is never updated since it represents a picture of the previous GOP which is already encoded.

Given the variables Gl and Cl, the maximum sizes of the reference picture lists that are used in the prediction and update steps are determined as follows. For the following considerations, it is assumed that the default derivation process for reference picture lists as described in the WD is used. In case reference picture re-ordering is employed, the algorithm needs to be adjusted accordingly.

- Prediction list 0:

nP0[i] = ( i + 1 ) >> 1

The prediction list 0 contains the pictures that are used for forward prediction, and thus its size is only limited by the “left” GOP boundary.

- Prediction list 1:

if( ( i % Gl )  >  Cl )		nP1[i] = ( Gl  –  ( i % Gl )  +  1 ) >> 1

else				nP1[i] = ( Cl  –  ( i % Gl )  +  1 ) >> 1

The prediction list 1 specifies the pictures that can be used for backward prediction. In case the current frame with index i is contained in the second sub-partition of a GOP partition ( ( i % Gl )  >  Cl ), its size is limited by the “right” partition or GOP border, which is specified by Gl. Otherwise, the current frame is contained in the first sub-partition of a GOP partition, the size of prediction list 1 is limited by the corresponding sub-partition border, which is specified by Cl.

- Update list 0:

w = ( i  = =  0  ?  0  :  ( ( i – 1 ) % Gl )  +  1 )

if( w > Cl )			nU0[i] = ( w – Cl ) >> 1

else				nU0[i] = w >> 1

The update list 0 specifies the set of “preceding” high-pass pictures that can be used for updating the current low-pass picture. w represents a frame index inside the current GOP partition. If w is greater than Cl, that is the current frame is located inside the second sub-partition of a GOP partition, the size of update list 0 is restricted by the corresponding sub-partition boundary. Otherwise, when the current frame is located inside the first sub-partition of a GOP partition, the size of update list 0 is restricted by the “left” partition or GOP boundary.

- Update list 1:

w = ( i  = =  0  ?  0  :  ( ( i – 1 ) % Gl )  +  1 )

if( w > Cl )			nU1[i] = ( Gl – w ) >> 1

else				nU1[i] = ( i = = 0  ?  0  :  ( Cl – w + 1 ) >> 1 )

The update list 1 specifies the set of “following” high-pass pictures that can be used for updating the current low-pass picture. w represents a frame index inside the current GOP partition. If w is greater than Cl, that is the current frame is located inside the second sub-partition of a GOP partition, the size of update list 1 is restricted by the “right” partition or GOP border. Otherwise, when the current frame is located inside the first sub-partition of a GOP partition, the size of update list 1 is restricted by the corresponding sub-partition boundary. Note that the low-pass frame with index i = 0 is never updated, since it represents the low-pass picture of the previous GOP, which is already coded and cannot be modified.

### Progressive MCTF

In the progressive MCTF process, the prediction steps and update steps are interleaved at the encoder and decoder. In order to match the decoding order the following encoding process should be used. For each decomposition stage, the encoder processes the pictures of a partition in reverse display order, i.e. from the last picture of the partition in display order to the first picture of the partition in display order. The progressive MCTF structure for a GOP of 8 pictures is shown in Figure 12. The order of the prediction and update steps is indicated. If the current picture pic is to be predicted, one prediction step is performed on pic to get high-pass residual picture resPic, using low-pass reference pictures refPic. Next, one update step is performed where the update components for low-pass reference pictures refPic are derived by one scan of the motion field used in the prediction step of resPic. The low-pass reference pictures refPic are partially updated. Subsequent prediction step is performed using existing version (fully updated, partially updated or not-yet updated) of the low-pass reference pictures refPic that is currently available at that point in time, instead of using original low-pass reference pictures refPic that are not updated. The low-pass reference pictures will be progressively updated in future picture cycles.

Figure 12: Illustration of the prediction and update steps when progressive MCTF is used. The encoder order of the prediction and update steps is indicated.

It should be noted that in the current implementation of MCTF in the JSVM using conventional MCTF processing at the encoder and progressive MCTF for the decoding process does not lead to performance losses.

## CAVLC encoding process for FGS (descriptive)

[Ed. Note: This process is currently integrated into the JSVM software. However it need to be approved in the next meeting to become an integrate part of the normative process]

### Introduction

The FGS encoding process proceeds in a similar way whether using CAVLC or CABAC, in that significance and refinement passes are distinguished.  CAVLC also uses the notion of “cyclical coding” where one non-zero coefficient and all preceding zero values are encoded in each “cycle”.  In this method, the “scan index” refers to the coefficient index of the first coefficient to be coded in a given cycle.

### Initialisation

The following two initialisation vectors are written at the start of each FGS slice when using CAVLC.

#### End-of-block symbol mapping

The end-of-block (EOB) symbol mapping specifies the symbol to be used to represent an EOB for each scan index.  First, the number of scan positions where the EOB is the least probable symbol is written.  This number is capped at three and is written using a complete expGolomb code as follows:

| Num. scan pos. | Code |
| --- | --- |
| 1 | 0 |
| 2 | 10 |
| 3 | 110 |
| 0 | 111 |

For the following scan index, the symbol to be used for EOB is written using a Golomb code with k=2.  The EOB symbols for the remaining scan indexes are constrained to be monotonically decreasing, and is represented by expGolomb codes specifying the “run” of a particular value before the next decrease.  The process terminates when the EOB symbol reaches zero or the maximum scan index is reached.

For example, the EOB symbol sequence {3, 2, 2, 2, 1, 1, 0, 0, …} would involve writing the initial value of 3, followed by runs of 0, 3, and 2.

#### Start-step-stop parameter mapping

Start-step-stop codes are characterised by a parameter m.  When coding significance data, the value of m selected depends upon both the scan index and the index of the last non-zero base layer coefficient, and is capped at m=5.  This mapping follows the EOB mapping, and first involves coding the vector corresponding to the case where the last non-zero coefficient in the base is at index zero.  In order to impose the constraint that the vector be monotonically decreasing (as with the EOB mapping), values of m are inverted and encoded as 5-m.  Encoding the vector follows an identical process to the EOB mapping, i.e. the first value is encoded using a Golomb code with k=2, then symbols specifying the “run” for each value of m are coded until m reaches zero or the maximum scan index is reached.

Values for m corresponding to indexes for the last non-zero base coefficient (other than zero) are then encoded for each scan index.  The corresponding value from the first vector is used as the starting value, so that only “run” information needs to be encoded.

For example, coding the matrix

| 4 | 2 | 1 | 0 |
| --- | --- | --- | --- |
| 3 | 2 | 1 | 0 |
| 1 | 1 | 0 | 0 |
| 0 | 0 | 0 | 0 |

would initially involve coding the vector {4, 3, 1, 0} as for the EOB mapping, then coding the vectors {4, 2, 1, 0}, {3, 2, 1, 0}, {1, 1, 0, 0} and {0, 0, 0, 0}, where the initial value in each of these four vectors is inferred from the first column rather than coded explicitly.

### Significance pass

The cyclical coding process for CAVLC is very similar to that for CABAC.  When encoding the significance pass, the encoder first checks to see whether there are any significant coefficients in the block remaining to be coded.  If not, an EOB marker is emitted (see below).  Otherwise, coding of the block proceeds.  This involves incrementing an internal counter for each zero-valued significance coefficient without emitting a symbol.

Once a non-zero significance value is encountered, the number of coefficients coded in the cycle is coded using a start-step-stop code with the value of m specified in the start-step-stop parameter mapping table, and the internal zero counter is reset.  The start-step-stop code employed in CAVLC uses an expGolomb code for symbols less than or equal to m.  Symbols C with values greater than m are encoded in two parts, a prefix of ones with length

p=C−m3+m

and a “remainder” suffix of either 00, 01, or 10.  For coefficient counts less than or equal to the EOB symbol determined from the EOB mapping, the symbol is decremented.

A sign flag follows the coefficient count, as with CABAC.  The magnitude of the terminating coefficient is assumed to be unitary.

#### End-of-block coding and non-unitary magnitudes

When all significant coefficients for a block have been encoded in the significance pass, an end-of-block (EOB) condition arises, indicated by the EOB symbol.  Before writing the EOB symbol, the encoder checks to see whether any of the block’s significant coefficients written during the significance pass had a magnitude greater than one.  If not, the EOB marker is encoded using the symbol specified in the EOB mapping table.

Otherwise, the EOB symbol is the number of zero-valued coefficients remaining in the significance pass plus the number of coefficients with magnitude greater than one.  Non-zero significance pass coefficients are then scanned, with a run counter incremented for each coefficient of unit magnitude.  When a coefficient with magnitude greater than one is encountered, the run and “magnitude minus one” are each encoded using expGolomb codes, and the run counter is reset.  The number of run-level pairs to be encoded is inferred from the EOB symbol.

### Refinement pass

In the refinement pass, an entire block is encoded at once.  Refinement bits are accumulated into groups of four, and a codeword emitted according to the following table

| Symbol | Codeword |
| --- | --- |
| 0000 | 00 |
| 0001 | 010 |
| 0010 | 011 |
| 0011 | 11000 |
| 0100 | 100 |
| 0101 | 11001 |
| 0110 | 11010 |
| 0111 | 111100 |
| 1000 | 101 |
| 1001 | 11011 |
| 1010 | 11100 |
| 1011 | 111101 |
| 1100 | 11101 |
| 1101 | 111110 |
| 1110 | 1111110 |
| 1111 | 1111111 |

When the number of refinement bits is not exactly divisible by four, the remainder bits are encoded as flags.

# Operational Encoder Control

In this section, the operational encoder control implemented in the JSVM software [7] is described. Section 3.1 explains the general coding and decomposition structure, while section  describes how fine granular scalability is realized. The concept for selecting the quantization parameters that are used for encoding the subband pictures is presented in Section 3.7.

## Scalability and Decomposition Structure

The base layer is encoded using a single layer AVC coding scheme as explained in 1.1.

Several types of enhancement layers can be included to achieve the desired combined spatio-temporal-SNR scalability. These scalability dimensions are explained in Section 1.2 and, in particular, their combination in Section 1.2.4.

The extraction of a given spatio-temporal-SNR layer is controlled by the decodability dependency information provided by the syntax elements dependency_id, temporal_level, and quality_level.

The temporal_level controls the temporal scalability and is constant for a given picture. The spatial and quality scalability are controlled by the dependency_id and the quality_level. To process a layer with a dependency_id equal to N, at least one layer with dependency_id equal to N-1 is necessary. This enable two classification of layers: The one necessary for the processing of the next layer, controlled by an increase of the dependency_id, and the one whose usage is facultative, controlled by an increase of  the quality_level.

The JSVM software [7] increases the value of dependency_id when CGS or spatial scalability is used, and increase the value of quality_level for FGS layers. However such relationship is non-normative.

## Closed-loop and open-loop encoder control for hierarchical B pictures

If hierarchical B pictures are used, the ordering of the pictures for temporal decomposition at the encoder side corresponds to the decoding order at the decoder side. The pictures of the coarsest temporal resolution are encoded first, and the B pictures for the next finer temporal levels are successively inserted. The established encoder control mechanisms for hybrid coding schemes, extended for consideration of lower spatial layers and quality scalability, can be employed.

If desired, an open-loop structure can be chosen, not taking into account quantization effects in the reconstructed sequence.

For highest coding efficiency, the prediction selection can be performed closed-loop, considering the reconstruction of previously encoded pictures. When FGS layers are present, the motion-compensation loop is closed at the highest rate point. In order to improve the coding efficiency especially for FGS coding, we implemented an additional option, where we use two closed motion compensation loops at the encoder side. As depicted in Figure 13, one closed motion-compensation loop is used for the base quality, and another for the highest FGS rate.

Figure 13 : Encoder control with two closed motion compensation loops.

## Open-loop encoder control for MCTF

The application of MCTF for temporal decomposition can be seen as a filtering operation on the original input sequence. Iterative filtering is applied from the finest to the coarsest temporal resolution. Hence, the ordering of the pictures for temporal decomposition at the encoder side is inverted relative to the decoding order at the decoder side. During temporal filtering, the pictures used for prediction at fine temporal resolution are modified by the update step before being encoded. This prevents closed loop encoder control.

## Adaptive GOP structure

The coding efficiency of the temporal decomposition improves if an adaptively selected GOP size is employed that considers the temporal image characteristics of the video sequence.

Figure 14 shows some instances that adaptively-adjusted GOP structure for the improvement of coding efficiency from the fixed 16 full-GOP structure. As shown in this figure, the various AGS’s that are combinations of the groups of sub-GOP’s, such as [16], [8,8], [8,4,4], [4,4,8], [4,4,4,4], [2,2,4,8], [4,4,2,2,8], [2,2,2,2,8], [2,2,4,4,4], [2,2,2,2,4,4], and [2,2,2,2,2,2,2,2] can be applicable. The coding efficiency improves by changing the full-sized fixed GOP structure into the proper AGS structure using these available groups of sub-GOP structures.

Figure 14 : Several instances of the possible adaptive GOP structures when the full-GOP size is set as 16.

For the instances to show the coding structure variations, Figure 15 illustrates the MCTF procedure based on the AGS when the one full-sized GOP (16 pcitures) is converted into the combination of the sub-GOP’s of [8, 4, 2, 2].

Figure 15: Example for an MCTF structure using the Adaptive GOP Structure that is divided into Sub-GOP’s as [8, 4, 2, 2].

As also shown in Figure 15, the encoded bitstream of the AGS using the combination of the sub-GOP’s of [8, 4, 2, 2] consists of the encoded data of “L3(0)”, “H3(1)”, “H2(2)”, “H2(3)”, “H1(4)”, “H1(5)”, “H1(6)”, “H1(7)”, “L2(8)”, “H2(9)”, “H1(10)”, “H1(11)”, “L1(12)”, “H1(13)”, “L1(14)”, “H1(15)” frame information.

The efficient adaptive GOP structure (the combination of the sub-GOP’s) can be obtained using following procedure.

Pre-encoding is performed by varying the sub-GOP size such as 2N, 2N-1, 2N-2, 2N-3 (N≥4) in a unit of one full-sized GOP interval (normally N=4 for QCIF). The MSE (Mean Square Error) of each sub-GOP is calculated in that interval.

The “Mode Decision” procedure selects the best combination of the sub-GOP structure that produces the minimum MSE within each full-sized GOP structure. Bitstream is finally generated based on selected combination of the sub-GOP structure. This procedure is repeated for each full-sized GOP interval of the video sequence.

Figure 16: The algorithm for selecting Adaptive GOP Structure

In the “Mode Decision” procedure, the MSE of each picture is calculated by following equation.

MSE(k)=1w⋅hy=oh−1x=0w−1s[X,k]−s'[X,k]2

Where w and h represent the width and the height of the picture, s[X, k] is the pixel value of the temporally-filtered picture after performing the temporal decomposition process, s´[X, k]is the pixel value of the reconstructed temporally-filtered  picture after encoding is performed and X = (x, y)T.

Figure 17 illustrates the detailed mode decision procedure. And the Figure 16 shows the conceptual locations where to calculate the MSE of each sub-GOP used in mode decision for selecting the adaptive GOP sizes among one full-size GOP structure.

The full 2N picture-sized GOP is encoded based on the various sub-GOP size of 2N, 2N-1, 2N-2 and 2N-3 respectively. Then the MSE of each sub-GOP is calculated and then followed by the mode decision.

Figure 17: The detailed mode decision procedure

Figure 18: The conceptional locations where to calculate the MSE of each Sub-GOP for selecting the adaptive GOP size.

The following is the Pseudo Code for the mode decision procedure.

gop_mode = min { MSE_N, (MSE_N-1_1 + MSE_N-1_2)/2, (MSE_N-2_1 + MSE_N-2_2 + MSE_N-2_3 + MSE_N-2_4)/4, (MSE_N-3_1 + MSE_N-3_2 + MSE_N-3_3 + MSE_N-3_4 + MSE_N-3_5 + MSE_N-3_6 + MSE_N-3_7 + MSE_N-3_8)/8 } If (gop_mode == MSE _N) { Select 2N picture size GOP structure End of mode Decision } Else { gop_mode = min { MSE_N-1_1, (MSE_N-2_1 + MSE_N-2_2)/2, (MSE_N-3_1 + MSE_N-3_2 + MSE_N-3_3 + MSE_N-3_4)/4 } If (gop_mode == MSE_N-1_1) Select 2N-1 picture size sub-GOP structure Else { gop_mode = min { MSE_N-2_1, (MSE_N-3_1 + MSE_N-3_2)/2 } If (gop_mode == MSE_N-2_1) Select 2N-2 picture size sub-GOP structure Else { Select 2N-3 picture size sub-GOP structure (Front) Select 2N-3 picture size sub-GOP structure (Rear) } gop_mode = min { MSE_N-2_2, (MSE_N-3_3 + MSE_N-3_4)/2 } If (gop_mode == MSE_N-2_2) Select 2N-2 picture size sub-GOP structure Else { Select 2N-3 picture size sub-GOP structure (Front) Select 2N-3 picture size sub-GOP structure (Rear) } } gop_mode = min { MSE_N-1_2, (MSE_N-2_3 + MSE_N-2_4)/2, (MSE_N-3_5 + MSE_N-3_6 + MSE_N-3_7 + MSE_N-3_8)/4 } If (gop_mode == MSE_N-1_1) { Select 2N-1 picture size sub-GOP structure End of mode decision } Else { gop_mode = min { MSE_N-2_3, (MSE_N-3_5 + MSE_N-3_6)/2 } If (gop_mode == MSE_N-2_3) Select 2N-2 picture size sub-GOP structure Else { Select 2N-3 picture size sub-GOP structure (Front) Select 2N-3 picture size sub-GOP structure (Rear) } gop_mode = min { MSE_N-2_4, (MSE_N-3_7 + MSE_N-3_8)/2 } If (gop_mode == MSE_N-2_4) { Select 2N-2 picture size sub-GOP structure End of mode decision } Else { Select 2N-3 picture size sub-GOP structure (Front) Select 2N-3 picture size sub-GOP structure (Rear) End of mode decision } } }

## Motion estimation and mode decision process

The motion estimation and mode decision process in the JSVM software [7] is performed using the Lagrangian optimisation techniques as described in the JM of AVC [2], with a few additions to handle the scalability aspects.

If the current spatial layer constitutes a spatial or a CGS enhancement layer and a prediction picture exists for the current picture, the Intra_Base mode is added to the set of potential macroblock modes during the mode decision process.

For the coding of subband pictures the following difference with the AVC JM exist:

- For temporal high-pass slices, the macroblock mode decision process is only applied for intra macroblocks, which are specified by the corresponding prediction data array MP; the residual macroblock mode (residual_prediction_flag is equal to 1) is never included in the set of possible macroblock modes.

- Since for CGS enhancement slices the macroblock mode is always set to Intra_Base, no mode decision process is performed for this slice type

## Fine grain SNR scalability

Fine grain SNR scalability (FGS) is obtained using the progressive refinement representations presented in 1.2.2. The corresponding NAL units can be truncated at any arbitrary point. For any spatio-temporal resolution, a minimum bit-rate, which represents the corresponding spatial base layer representation (including the base layer representations of the lower-resolution layers), must be transmitted; these bit-rates can be adjusted in a way that the corresponding reconstructions represent the minimally acceptable video quality. Above the minimum bit-rate for a spatio-temporal resolution, any bit-rate can be extracted by truncating the progressive refinement NAL units of the corresponding spatio-temporal layer and all lower resolution layers in a suitable way.

In order to extract a requested bit-rate from the overall bit-stream, the progressive refinement NAL units are truncated at appropriate points. Two approaches for progressive refinement NAL unit truncation are presented in the following.

### Simple truncation of progressive refinement NAL units

With simple truncation, the progressive refinement packets of a refinement layer under consideration are equally truncated.

Let Rt specify the target bit-rate for a spatio-temporal resolution St-Tt, and let R0 be the base layer bit-rate for this spatio-temporal resolution, i.e. the bit-rate that corresponds to the base layer representation including all spatial lower-resolution for the temporal resolution Tt.

If R0 is greater than Rt, the requested spatio-temporal-rate point cannot be extracted from the overall bit-stream. Otherwise, the following applies

- The target rate is modified to Rt = Rt – R0

- The progressive refinement packets are processed from the lowest supported spatial resolution to the target spatial resolution St, and for each spatial resolution, the progressive refinement packets are processed from the lowest refinement layer to the highest refinement layer. For each progressive refinement layer of a spatio-temporal resolution S-Tt, the following applies:

  - Let RF be the bit-rate of the i-th progressive refinement representation for the spatio-temporal resolution S-Tt.

  - If RF is less or equal to Rt, the corresponding progressive refinement packets are fully included into the extracted bit-stream, and the target rate is modified to Rt = Rt – RF

  - Otherwise, the corresponding progressive refinement packets are truncated, and the target rate is set to zero: Rt = 0. Let L be the original length of a progressive refinement packet. During truncation, the length is set to

### Quality layers

#### Quality layers estimation

With the application of quality layers, the progressive refinement packets of a refinement layer under consideration are truncated based on their rate-distortion impact on the reconstruction.

Quality layers are assigned to each NAL unit order to perform a rate allocation among NAL units from different subbands. Various quality layers assignment may be considered. For rate-distortion optimization, such quality layers can be estimated using a Lagrangian cost functional as described below.

The distortion is calculated at some rate points for each reconstructed frame. This results in a set of rate-distortion information Rr,ti,Dr,ti. for frames at time index t, spatial resolution r, and R/D point i. In order to estimate the global distortion, the following equation is used:

Error! Objects cannot be created from editing field codes..

In the aim of defining quality layers optimized in the rate-distortion sense, the Lagrangian cost functional is used, that is

minR+λD=r,tRr,t+λwr,tDr,t.

Optimization can then be performed separately for each spatio-temporal frame for a given λ by minimizing the Lagrangian cost. For each R/D points of a frame, the λ values lying on the convex hull of rate-distortion points are kept. This provides for each frame a set of rate points with associated λ values. The λ value indicates the optimum rate Rr,t for each progressive refinement packet.  The logarithms of the λ values are then quantized to integer values in the range of [0..63] (i.e. 6 bits). These values define the quality layers.

The information on quality layers and the corresponding truncation points for the progressive refinement packets can be signalled in two ways, either using the simple_priority_id syntax element or a quality layer information SEI message.

To signal information on quality layers is to use the two-bytes header NAL unit syntax, the extension_flag must be set to 1. The simple_priority_id of a NAL unit is used to store the value of the quality layer of the NAL unit.

The second way to signal information on quality layers is to use an SEI message as specified in Annex S D.1.27.

## Quantizer selection

To optimize the bitrate allocation in the ideal case (i.e. neglecting the presence of motion and rounding effects), the quantizer step size for a given subband should be chosen to be inversely proportional to the square-root of the energy of the impulse response of the synthesis filter for the particular subband. The basic two-channel analysis step (see Section 1.1.1) can be normalized by multiplying the high-pass samples of the picture H by a factor of 1/sqrt(2) (in case of uni-directional prediction) or sqrt(23/32) (in case of bi- prediction) and the low-pass samples by a factor of sqrt(2) (in case of uni-directional prediction) or sqrt(3/2) (in case of bi- prediction). Since we neglect this normalization in the realization of the analysis and synthesis filter banks to keep the range of the samples values nearly constant, we have to take it into account during the quantization of the temporal subbands.

Let QPr0 be a given real-valued quantization parameter. For each layer and each group of pictures this quantization parameter has to be determined by the encoder control.

The quantization parameters QPH(k, i) and QPL(k, i) that are used for encoding the low- and high-pass pictures Lk[i] and Hk[i] (or their refinement representations), which are obtained after the k-th decomposition stage ( k = 0,…, n – 1 ), are set to

QPL(k, i)  =  min( 51, max( 0, Round( QPrL(k, i) ) ) ),

QPH(k, i)  =  min( 51, max( 0, Round( QPrH(k, i) ) ) ).

At this, n is the number of decomposition stages that have been applied to the considered group of pictures. The operator Round() specifies rounding to the nearest integer number. QPrL(k, i) and QPrH(k, i) represent real-valued approximations of the quantization parameters QPH(k, i) and QPL(k, i), and are calculated as follows

QPrL(k, i)  =  QPpred,L(k, i) – QPL(k, i),

QPrH(k, i)  =  QPpred,H(k, i) + QPH(k, i),

where QPpred,L(k, i) and QPpred,H(k, i) represent predictions for the real-valued quantization parameters QPrL(k, i) and QPrH(k, i). These quantization parameter predictions QPpred,L(k, i) and QPpred,H(k, i) are determined as follows.

- If k is equal to 0, QPpred,L(k, i) and QPpred,H(k, i) are set equal to the given quantization parameter QP0.

- Otherwise (k is greater than 0), QPpred,L(k, i) and QPpred,H(k, i) are set to the average of the quantization parameters { QPrL(k–1, …) } of the low-pass picture Lk-1[j], which shares the coordinate system with the regarded low- or high-pass picture Lk[i] or Hk[i], and its direct neighbours (if they exist).

The real-valued quantization parameter differences QPH(k, i) and QPL(k, i) are determined as follows

QPL(k, i) = 6 log2[3  ( ( NBi(k, i) / Ntot ) ( FACTOR_53_LP – 1 )
			 log2( 3 / 2 ) + ( NUni(k, i) / Ntot ) (FACTOR_22_LP - 1) )  ])

QPH(k, i) = 6 log2[3  ( ( NBi(k, i) / Ntot ) (FACTOR_53_HP - 1)
			 log2( 23 / 32 ) + ( NUni(k, i) / Ntot ) (FACTOR_22_HP - 1 )  ])

Ntot represents the total number of luma samples inside a picture. NBi(k, i) and NUni(k, i) specify the number of samples in the regarded low- or high-pass picture Lk[i] or Hk[i] that can be classified as bi-directional and uni-directional connected, respectively. The number of bi-directional and uni-directional connected samples NBi(k, i) and NUni(k, i) is determined by analysing the prediction data arrays {MP} (for QPH) and {MU} (for QPL) that are used in the corresponding prediction and update steps. The actual derivation process is similar to the derivation process for the prediction data arrays MU used in the update steps. FACTOR_53_LP, FACTOR_53_HP are scaling factors associated to the 5/3 structure for low-pass pictures and high-pass pictures, respectively. FACTOR_22_LP, FACTOR_22_HP are the scaling factors associated to Haar structure for low-pass pictures and high-pass pictures, respectively. The values for these factors are as follows:

For use with MCTF (including the update step)

FACTOR_53_LP = sqrt( 3 / 2 )

FACTOR_53_HP = sqrt( 23 / 32 )

FACTOR_22_LP = sqrt( 2 )

FACTOR_22_HP = sqrt( 1 / 2 )

For use with hierarchical B-frames (without an update step)

FACTOR_53_LP = sqrt( 3 / 2 )

FACTOR_53_HP = sqrt( 1  )

FACTOR_22_LP = sqrt( 2 )

FACTOR_22_HP = sqrt( 1 )

Within each temporal subband picture, the quantization parameter QP is held constant in the JSVM software. Note that using the described algorithm, the bit-allocation process (and thus the rate as well as distortion) for each layer and each group of pictures is determined by a single real-valued quantization parameter QP0.

## Rate Control

The rate control procedure can be facilitated by the usage of SNR scalability. In addition to the rate controls used in non scalable AVC, where the QP must be computed on a picture or on a GOP basis, the SNR scalability can be used to provide post-encoding rate control.

For each spatial layer the base quality layer is generated with a constant QP small enough to ensure that the rate constraints will be satisfied. Then an additional set of CGS and/or FGS scalability layers are used to reach a bitrate above the target rate. The bitrates constraints are then satisfied by using processes as described in 3.6 to reduce the final bitrates.

# Bitstream Extraction

In this section, the process of bitstream extraction is described. The description is based on the bitstream extractor provided with the JSVM software [7].

The bitstream extractor extracts a bitstream of lower temporal or spatial resolution at a given target bit rate from the input bitstream.

The encoder provides a scalability information SEI message that is transmitted at the beginning of the bitstream to provide basic information about its scalability features. The extractor could in principle generate this information from the input bitstream. This would require an additional in-depth analysis task for the bitstream extractor. Hence, the application of the scalability information SEI message simplifies the extraction process that is required to retain the target bit stream.

An output bit stream of the extractor contains the data of all lower layers that are required to be decodable from the current stream. The required decodable rate points can be defined in an ‘extraction path’, as it is done in the core experiment descriptions. An example for extraction pathes is provided in Figure 19 below. Note that information of dedicated lower layers is required for inter layer prediction.

Figure 19: Example for valid extraction points for a scalable bitstream. The dependencies of an extraction path are indicated by arrows.

## Scalability Information SEI

[Ed.Note(MW): The SEI message below is implemented in the JSVM-3 software. At the Poznan meeting, the scalability information SEI message specified in subclause D.1.24 of Annex S was tentatively adopted into the WD. If this decision is confirmed at the Nice meeting, the SEI message below is going to be removed from the specification and the software.]

### Scalability Information SEI message syntax

| scalability_information( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| max_mb_number_in_row | 5 | ue(v) |
| max_mb_number_in_column | 5 | ue(v) |
| frame_rate_unit_nominator | 5 | ue(v) |
| frame_rate_unit_denominator | 5 | ue(v) |
| max_decomposition_stages | 5 | ue(v) |
| num_spatial_layers | 5 | ue(v) |
| avc_base_layer_flag | 5 | u(1) |
| if( avc_base_layer_flag ) { |  |  |
| base_layer_decomposition_stages | 5 | ue(v) |
| } |  |  |
| non_dyadic_spatial_scalability_flag | 5 | u(1) |
| for( i = 0; i < num_spatial_layers; i++ ) { |  |  |
| if( extended_spatial_scalability_flag) { |  |  |
| frame_width_in_mbs_minus1[ i ] | 5 | ue(v) |
| frame_height_in_mbs_minus1[ i ] | 5 | ue(v) |
| } |  |  |
| else |  |  |
| spatial_resolution_factor[ i ] | 5 | ue(v) |
| temporal_resolution_factor[ i ] | 5 | ue(v) |
| } |  |  |
| } |  |  |

### Scalability Information SEI message semantics

The scalability information SEI message assists a bitstream extractor in analyzing the scalability features of the bitstream.

max_mb_number_in_row specifies the maximum number of macroblocks in a row. Thereby, the maximum horizontal resolution of a picture is determined.

max_mb_number_in_column specifies the maximum number of macroblocks in a column. Thereby, the maximum vertical resolution of a picture is determined. [Ed.Note: no interlaced considered]

frame_rate_unit_nominator and frame_rate_unit_denominator specify the frame rate at the coarsest available temporal resolution for the scalable stream. This value is used to determine the frame rate at all temporal levels. The frame rate at temporal level k is calculated as frame_rate = ( 1 << k ) * frame_rate_unit_nominator ÷ frame_rate_unit_denominator.

[Ed.Note: the calculation of the frame rate for lower decomposition levels is currently only specified for dyadic decomposition on all levels]

max_decomposition_stages specifies the maximum number of temporal decomposition stages.

num_spatial_layers specifies the number of different spatial resolutions that can be extracted from the bitstream.

avc_base_layer_flag indicates the existence of a sub-bitstream at the lowest spatial layer not conforming to the scalability extension specification.

base_layer_decomposition_stages specifies the number of temporal scalability steps that is provided by the sub-bitstream not conforming to the scalability extension specification.

spatial_resolution_factor[ i ] specifies the factor by which the spatial resolution is reduced on spatial layer i.

temporal_resolution_factor[ i ] specifies the factor by which the temporal resolution is reduced on spatial layer i.

non_dyadic_spatial_scalability_flag indicates that the factor by which the spatial resolution is reduced is not a multiple of 2.

frame_width_in_mbs_minus1[ i ] specifies the width in macroblock unit (minus 1) of the frames on spatial layer i.

frame_height_in_mbs_minus1[ i ] specifies the height in macroblock unit (minus 1) of the frames on spatial layer i.

## Analysis of the input bitstream

In order to determine the amount of bit rate spent the AUs of the stream are analyzed and the amount of bytes needed for each spatial, temporal, and quality layer is stored. The set of available layers is indicated in the scalability information SEI message.

The cumulative byte cost per spatial, temporal, and quality layer is calculated and stored.

## Bistream Extraction

For a given target bit rate at a given spatial resolution and a given frame rate, the packets that have to be extracted from the input bitstream are determined. Without further reference a layer refers to all NAL units for a given spatial layer at a given temporal level in this context. The temporal level corresponds to the number of duplications of the coarsest available frame rate that is indicated in the scalability information SEI message. The process is as follows:

- From the target bit rate, the corresponding total number of bytes is calculated.

- The applicable spatial layer and the applicable temporal level are determined.

- The amount of bytes required for the quality base layer of the target layer and the embedded lower layers is determined.

- For the lower layers, quality refinement layers are included. The layers are assigned beginning from the coarsest spatial and temporal resolution. 
If the available number of bytes is sufficiently big, all quality refinement layers of the embedded lower layers are included. 
Else, the last layer to be included is determined and truncated. The packets of the last available refinement layer are truncated as described in section  to meet the target byte budget. In this case, no quality refinement is available for the target layer.

- If the available number of bytes is sufficiently big, quality refinement layers are included for the target layer. The last available quality refinement layer is truncated to meet the target byte budget. The packets of the last available refinement layer are truncated as described in section .

If no FGS layers are included or in the case that CGS layers are provided, the truncation of layers as described in the steps 4 and 5 is not applied and only full layers are included in the extracted bitstream.

# References

- ITU-T and ISO/IEC JTC1, “Advanced Video Coding for Generic Audiovisual Services,” ITU-T 	Recommendation H.264 – ISO/IEC 14496-10 AVC, 2003.

- ITU-T and ISO/IEC JTC1, “Text Description of Joint Model Reference Encoding Methods and Decoding Concealment Methods”, JVT-N046, Jan 2005

- W. Sweldens, “A custom-design construction of biorthogonal wavelets,” J. Appl. Comp. Harm. Anal., vol. 3 (no. 2), pp. 186-200, 1996.

- ITU-T and ISO/IEC JTC1, “Generic Coding of Moving Pictures and Associated Audio Information – Part 2: Video,” ITU-T Recommendation H.262 – ISO/IEC 13818-2 (MPEG-2), 1994.

- ITU-T, “Video Coding for Low Bitrate Communication, “ ITU-T Recommendation H.263, Version 1: Nov. 1995, Version 2: Jan. 1998.

- ISO/IEC JTC1, “Information technology – Coding of audio-visual objects – Part 2: Visual,” ISO/IEC 14496-2 (MPEG-4), 1998.

- ITU-T and ISO/IEC JTC1, “JSVM 3 Software”, JVT-P203, Jul 2005.

- ITU-T and ISO/IEC JTC1, “Scalable Video Coding - Working Draft 3”, JVT-P201, Jul 2005

- ISO/IEC JTC1, “Verification Model 18.0 of MPEG-4 Visual,” ISO/IEC JTC1/WG11 Doc. N3908, Feb 2001.