| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 16th Meeting: Poznan, Poland, July, 2005 | Document: JVT-P201 Filename: input.doc |
| --- | --- |

| Title: | Scalable Video Coding – Working Draft 3 |  |  |
| --- | --- | --- | --- |
| Status: | Output Document from JVT |  |  |
| Purpose: | Text of the Joint Working Draft |  |  |
| Editor(s) / Contact(s): | Julien Reichel VisioWave (GE Security), Rte. de la Pierre 22, CH-1032 Ecublens, Switzerland Heiko Schwarz Heinrich Hertz Institute (FhG), Einsteinufer 37, D-10587 Berlin, Germany Mathias Wien Institut für Nachrichtentechnik, RWTH Aachen University, D-52056 Aachen, Germany | Tel: Fax: Email: Tel: Fax: Email: Tel: Fax: Email: | +41 (21) 695-0041 +41 (21) 695-0001 julien.reichel@visiowave.com +49 (30) 31002-226 +49 (30) 39272-00 heiko.schwarz@hhi.fhg.de +49 (241) 80-27681 +49 (241) 80-22196 wien@ient.rwth-aachen.de |
| Source: | JVT |  |  |

_____________________________

Editorial remarks:

- The consequences for the Annexes A to E need to be analysed and the required changes need to be specified (long-term item).

- All newly introduced processes are currently only described for the case that frame_mbs_only_flag is equal to 1. The inter-layer prediction processes and the motion-compensated update process in connection with the usage of interlaced features still needs to be investigated and specified.

- The motion-compensated update process as it is currently specified does not take into account the prediction weights used in the motion-compensated prediction steps. That issue need to be analysed and (possibly) specified.

- The picture duplication process for the key pictures (identified by the syntax element key_picture_flag) needs to be checked.

- The identification of the key picture in case of AVC compatible base layer needs to be corected.

- An overview of the decoding process, when what process is invoked, is missing and shall be added. This is urgently required, since the invocation of the different processes is not as obvious as for standard AVC.

- In some parts of this specification, a non-suitable “writing style” is used (e.g. uppercase names for local variables, missing spaces in formulas, unsuitable wording). The editors did not have the time to modify all these text passages delivered by various proponents (and also old parts of the text), so it has to be fixed in future versions.

Foreword	6

Introduction	6

S.1	Scope	6

S.2	Normative references	6

S.3	Definitions	6

S.4	Abbreviations	6

S.5	Conventions	6

S.5.1	Arithmetic operators	6

S.6	Source, coded, decoded and output data formats, scanning processes, and neighbouring relationships	6

S.6.1	Bitstream formats	6

S.6.2	Source, decoded, and output picture formats	7

S.6.3	Spatial subdivision of pictures and slices	7

S.6.4	Inverse scanning processes and derivation processes for neighbours	7

S.6.4.1	Inverse macroblock scanning process	7

S.6.4.2	Inverse macroblock partition and sub-partition scanning process	7

S.6.4.2.1	Inverse macroblock partition scanning process	7

S.6.4.2.2	Inverse sub-macroblock partition scanning process	7

S.6.4.3	Inverse 4x4 luma block scanning process	7

S.6.4.4	Inverse 8x8 luma block scanning process	7

S.6.5	Derivation processes for base data	7

S.6.5.1	Derivation process for base pictures	7

S.6.5.2	Derivation process for base macroblocks	7

S.6.5.3	Derivation process for base sub-macroblocks	8

S.6.5.4	Derivation process for base partitions	9

S.6.5.5	Derivation process for base quality slices	10

S.7	Syntax and semantics	11

S.7.1	Method of describing syntax in tabular form	11

S.7.2	Specification of syntax functions, categories, and descriptors	11

S.7.3	Syntax in tabular form	16

S.7.3.1	NAL unit syntax	16

S.7.3.2	Raw byte sequence payloads and RBSP trailing bits syntax	16

S.7.3.2.1	Sequence parameter set RBSP syntax	16

S.7.3.2.1.1	Scaling list syntax	19

S.7.3.2.1.2	Sequence parameter set extension RBSP syntax	19

S.7.3.2.2	Picture parameter set RBSP syntax	19

S.7.3.2.3	Supplemental enhancement information RBSP syntax	19

S.7.3.2.4	Access unit delimiter RBSP syntax	19

S.7.3.2.5	End of sequence RBSP syntax	19

S.7.3.2.6	End of stream RBSP syntax	19

S.7.3.2.7	Filler data RBSP syntax	19

S.7.3.2.8	Slice layer without partitioning RBSP syntax	19

S.7.3.2.9	Slice data partition RBSP syntax	19

S.7.3.2.10	RBSP slice trailing bits syntax	19

S.7.3.2.11	RBSP trailing bits syntax	19

S.7.3.2.12	Slice layer in scalable extension RBSP syntax	19

S.7.3.3	Slice header syntax	19

S.7.3.3.1	Reference picture list reordering syntax	20

S.7.3.3.2	Prediction weight table syntax	20

S.7.3.3.3	Decoded reference picture marking syntax	21

S.7.3.4	Slice data syntax	21

S.7.3.5	Macroblock layer syntax	21

S.7.3.6	Slice header in scalable extension syntax	22

S.7.3.7	Slice data in scalable extension syntax	23

S.7.3.8	Macroblock layer in scalable extension syntax	24

S.7.3.8.1	Macroblock prediction in scalable extension syntax	25

S.7.3.8.2	Sub-macroblock prediction in scalable extension syntax	27

S.7.3.8.3	Residual in scalable extension syntax	28

S.7.3.9	Progressive refinement slice data in scalable extension syntax	29

S.7.3.9.1	Luma coefficient in progressive refinement slice data syntax	31

S.7.3.9.2	Chroma DC coefficient in progresssive slice data syntax	33

S.7.3.9.3	Chroma AC coefficient in progressive refinement slice data syntax	35

S.7.3.9.4	Luma coefficient refinement in progressive refinement slice data syntax	37

S.7.3.9.5	Chroma DC coefficient refinement in progressive refinement slice data syntax	37

S.7.3.9.6	Chroma AC coefficient refinement in progressive refinement slice data syntax	38

S.7.4	Semantics	39

S.7.4.1	NAL unit semantics	39

S.7.4.1.1	Encapsulation of an SODB within an RBSP (informative)	40

S.7.4.1.2	Order of NAL units and association to coded pictures, access units, and video sequences	40

S.7.4.1.2.1	Order of sequence and picture parameter set RBSPs and their activation	40

S.7.4.1.2.2	Order of access units and association to coded video sequences	41

S.7.4.1.2.3	Order of NAL units and coded pictures and association to access units	41

S.7.4.1.2.4	Detection of the first VCL NAL unit of a primary coded picture	41

S.7.4.1.2.5	Order of VCL NAL units and association to coded pictures	42

S.7.4.2	Raw byte sequence payloads and RBSP trailing bits semantics	42

S.7.4.2.1	Sequence parameter set RBSP semantics	42

S.7.4.2.2	Picture parameter set RBSP semantics	44

S.7.4.2.3	Supplemental enhancement information RBSP semantics	44

S.7.4.2.4	Access unit delimiter RBSP semantics	44

S.7.4.2.5	End of sequence RBSP semantics	44

S.7.4.2.6	End of stream RBSP semantics	44

S.7.4.2.7	Filler data RBSP semantics	44

S.7.4.2.8	Slice layer without partitioning RBSP semantics	44

S.7.4.2.9	Slice data partition RBSP semantics	44

S.7.4.2.10	RBSP slice trailing bits semantics	44

S.7.4.2.11	RBSP trailing bits semantics	44

S.7.4.2.12	Slice layer in scalable extension RBSP semantics	44

S.7.4.3	Slice header semantics	44

S.7.4.4	Slice data semantics	44

S.7.4.5	Macroblock layer semantics	44

S.7.4.6	Slice header in scalable extension semantics	44

S.7.4.7	Slice data in scalable extension semantics	47

S.7.4.8	Macroblock layer in scalable extension semantics	47

S.7.4.8.1	Macroblock prediction in scalable extension semantics	51

S.7.4.8.2	Sub-macroblock prediction in scalable extension semantics	51

S.7.4.8.3	Residual in scalable extension semantics	53

S.7.4.9	Progressive refinement slice data in scalable extension semantics	54

S.8	Decoding process	56

S.8.1	NAL unit decoding process	56

S.8.2	Slice decoding process	56

S.8.2.1	Decoding process for picture order count	56

S.8.2.1.1	Decoding process for picture order count type 0	57

S.8.2.1.2	Decoding process for picture order count type 1	58

S.8.2.1.3	Decoding process for picture order count type 2	58

S.8.2.2	Decoding process for macroblock to slice group map	59

S.8.2.3	Decoding process for slice data partitioning	59

S.8.2.4	Decoding process for reference picture lists construction	59

S.8.2.4.1	Decoding process for picture numbers	59

S.8.2.4.2	Initialisation process for reference picture lists	60

S.8.2.4.2.1	Initialisation process for the reference picture list for P and SP slices in frames	60

S.8.2.4.2.2	Initialisation process for the reference picture list for P and SP slices in field	60

S.8.2.4.2.3	Initialisation process for the reference picture list for B slices in frames	60

S.8.2.4.2.4	Initialisation process for the reference picture list for B slices in fields	61

S.8.2.4.2.5	Initialisation process for reference picture lists in fields	61

S.8.2.4.2.6	Initialisation process for the reference picture list for EP slices in frames	61

S.8.2.4.2.7	Initialisation process for the reference picture list for EP slices in fields	61

S.8.2.4.2.8	Initialisation process for the reference picture list for EB slices in frames	62

S.8.2.4.2.9	Initialisation process for the reference picture list for EB slices in fields	63

S.8.2.4.3	Reordering process for reference picture lists	63

S.8.2.5	Decoded reference picture marking process	63

S.8.2.5.1	Sequence of operations for decoded reference picture marking process	63

S.8.2.5.2	Decoding process for gaps in frame_num	63

S.8.2.5.3	Sliding window decoded reference picture marking process	63

S.8.2.5.4	Adaptive memory control decoded reference picture marking process	63

S.8.2.5.4.1	Marking process of a short-term picture as “unused for reference”	63

S.8.2.5.4.2	Marking process of a long-term picture as “unused for reference”	63

S.8.2.5.4.3	Assignment process of a LongTermFrameIdx to a short-term reference picture	64

S.8.2.5.4.4	Decoding process for MaxLongTermFrameIdx	64

S.8.2.5.4.5	Marking process of all reference pictures as “unused for reference” and setting MaxLongTermFrameIdx to “no long-term frame indices”	64

S.8.2.5.4.6	Process for assigning a long-term frame index to the current picture	64

S.8.2.6	Derivation process for update picture lists	64

S.8.3	Intra prediction process	65

S.8.3.1	Intra_4x4 prediction process for luma samples	66

S.8.3.1.1	Derivation process for the Intra4x4PredMode	66

S.8.3.1.2	Intra_4x4 sample prediction	66

S.8.3.2	Intra_8x8 prediction process for luma samples	66

S.8.3.2.1	Derivation process for the Intra8x8PredMode	66

S.8.3.2.2	Intra_8x8 sample prediction	66

S.8.3.3	Intra_16x16 prediction process for luma samples	66

S.8.3.4	Intra prediction process for chroma samples	67

S.8.3.5	Sample construction process for I_PCM macroblocks	67

S.8.3.6	Intra_Base prediction process	67

S.8.3.6.1	Deblocking filter process for Intra_Base prediction	70

S.8.3.6.2	Construction process for not available or not existing samples	72

S.8.3.6.2.1	Constant border extension process	73

S.8.3.6.2.2	Diagonal border extension process	73

S.8.3.6.3	Upsampling process for Intra_Base prediction	74

S.8.3.6.4	Extended spatial scalability upsampling process for Intra_Base prediction	75

S.8.4	Inter prediction process	76

S.8.4.1	Derivation process for motion vector components and reference indices	78

S.8.4.1.1	Derivation process for luma motion vectors for skipped macroblocks in P and SP slices	79

S.8.4.1.2	Derivation process for luma motion vectors for B_Skip, B_Direct_16x16, and B_Direct_8x8	79

S.8.4.1.2.1	Derivation process for the co-located 4x4 sub-macroblock partitions	80

S.8.4.1.2.2	Derivation process for spatial direct luma motion vector and reference index prediction mode	80

S.8.4.1.2.3	Derivation process for temporal direct luma motion vector and reference index prediction mode	80

S.8.4.1.2.4	Derivation process for spatial direct luma motion vector and reference index prediction mode in scalable extension	80

S.8.4.1.3	Derivation process for luma motion vector prediction	81

S.8.4.1.3.1	Derivation process for median luma motion vector prediction	81

S.8.4.1.3.2	Derivation process for motion data of neighbouring partitions	81

S.8.4.1.4	Derivation process for chroma motion vectors	81

S.8.4.1.5	Derivation process for base luma motion vectors and reference indices	81

S.8.4.1.5.1	Derivation process for base reference indices	82

S.8.4.1.5.2	Derivation process for base luma motion vector predictions	82

S.8.4.1.5.3	Derivation process for motion vector scaling	83

S.8.4.1.6	Derivation processes for Extended Spatial Scalability	83

S.8.4.1.6.1	Derivation process for base macroblock motion data	83

S.8.4.1.6.2	Derivation process for base macroblock partition motion	84

S.8.4.1.6.3	Derivation process for base sub-macroblock partitions motion	86

S.8.4.1.6.4	Derivation process for macroblock partition label	87

S.8.4.1.6.5	Derivation process for macroblock label	88

S.8.4.1.6.6	Derivation process macroblock class index	89

S.8.4.2	Decoding process for Inter prediction samples	90

S.8.4.2.1	Reference picture selection process	90

S.8.4.2.2	Fractional sample interpolation process	90

S.8.4.2.2.1	Luma sample interpolation process	92

S.8.4.2.2.2	Chroma sample interpolation process	93

S.8.4.2.3	Weighted sample prediction process	93

S.8.4.2.3.1	Default weighted sample prediction process	93

S.8.4.2.3.2	Weighted sample prediction process	93

S.8.5	Transform coefficient decoding process and picture construction process prior to deblocking filter process	93

S.8.5.1	Specification of transform decoding process for 4x4 luma residual blocks	93

S.8.5.2	Specification of transform decoding process for luma samples of Intra_16x16 macroblock prediction mode	94

S.8.5.3	Specification of transform decoding process for 8x8 luma residual blocks	95

S.8.5.4	Specification of transform decoding process for chroma samples	96

S.8.5.5	Inverse scanning process for transform coefficients	98

S.8.5.6	Inverse scanning process for 8x8 luma transform coefficients	98

S.8.5.7	Derivation process for the chroma quantisation parameters and scaling function	98

S.8.5.8	Scaling and transformation process for luma DC transform coefficients for Intra_16x16 macroblock type	98

S.8.5.9	Scaling and transformation process for chroma DC transform coefficients	99

S.8.5.10	Scaling and transformation process for residual 4x4 luma blocks	102

S.8.5.11	Scaling and transformation process for residual 8x8 luma blocks	102

S.8.5.12	Picture construction process prior to deblocking filter process	103

S.8.5.13	Residual colour transform process	105

S.8.5.14	Residual prediction process	105

S.8.5.14.1	Upsampling process for residual prediction	108

S.8.5.14.2	Extended spatial scalability upsampling process for residual prediction	108

S.8.6	Decoding process for P macroblocks in SP slices or SI macroblocks	109

S.8.7	Deblocking filter process	109

S.8.7.1	Filtering process for block edges	109

S.8.7.2	Filtering process for a set of samples across a horizontal or vertical block edge	109

S.8.7.2.1	Derivation process for the luma content dependent boundary filtering strength	109

S.8.8	Inverse motion-compensated filtering process	110

S.8.8.1	Check for inverse prediction process	111

S.8.8.2	Check for inverse update process	112

S.8.8.3	Motion-compensated update process	113

S.8.8.3.1	Derivation process of update data	115

S.8.8.3.2	Update picture selection process	118

S.8.8.3.3	Low-complexity motion-compensated update process	119

S.8.8.3.3.1	Integer-luma-sample motion compensation process	121

S.8.8.3.3.2	Low-complexity fractional sample interpolation process for luma samples	122

S.8.8.4	Motion-compensated prediction process	125

S.9	Parsing process	126

S.9.1	Parsing process for Exp-Golomb codes	126

S.9.2	CAVLC parsing process for transform coefficient levels	126

S.9.3	CABAC parsing process for slice data	126

S.9.3.1	Initialisation process	127

S.9.3.1.1	Initialisation process for context variables	127

S.9.3.1.2	Initialisation process for the arithmetic decoding engine	128

S.9.3.2	Binarization process	128

S.9.3.2.1	Unary (U) binarization process	129

S.9.3.2.2	Truncated unary (TU) binarization process	129

S.9.3.2.3	Concatenated unary/ k-th order Exp-Golomb (UEGk) binarization process	129

S.9.3.2.4	Fixed-length (FL) binarization process	129

S.9.3.2.5	Binarization process for macroblock type and sub-macroblock type	129

S.9.3.2.6	Binarization process for coded block pattern	130

S.9.3.2.7	Binarization process for mb_qp_delta	130

S.9.3.2.8	Signed truncated unary (STU) binarization proces	130

S.9.3.3	Decoding process flow	130

S.9.3.3.1	Derivation process for ctxIdx	130

S.9.3.3.1.1	Assignment process of ctxIdxInc using neighbouring syntax elements	131

S.9.3.3.1.2	Assignment process of ctxIdxInc using prior decoded bin values	132

S.9.3.3.1.3	Assignment process of ctxIdxInc for syntax elements significant_coeff_flag, last_significant_coeff_flag, and coeff_abs_level_minus1	132

S.9.3.3.2	Arithmetic decoding process	134

S.9.3.3.2.1	Arithmetic decoding process for a binary decision	134

S.9.3.3.2.2	Renormalization process in the arithmetic decoding engine	134

S.9.3.3.2.3	Bypass decoding process for binary decisions	134

S.9.3.3.2.4	Decoding process for binary decisions before termination	134

# Foreword

The specification of this clause in AVC shall apply.

# Introduction

The specification of this clause in AVC shall apply.

# Scope

The specification of this clause in AVC shall apply.

# Normative references

The specification of this clause in AVC shall apply.

# Definitions

The specification of this clause in AVC shall apply with the following modifications.

[Ed. Note(JR/HS): Here, we need to modify and add some definitions, e.g. (first rough ideas)

access unit: A set of NAL units containing all coded slice or slice data partition NAL units having the same value of picture order count. In addition to the coded slice or slice data partitioning NAL units, an access unit may also contain other NAL units not containing slices or slice data partitions.

picture: A picture is decoded from a set of NAL units with an identical value of picture order count, dependency_id, and quality_level.

residual picture: a picure composed of residual and/or decoded samples or data elements.

update: An embodiment of the update process.

update process: The use of a predictior to provide an update of the sample value or data element of a picture.

]

# Abbreviations

The specification of this clause in AVC shall apply.

# Conventions

The specification of this clause in AVC shall apply.

## Arithmetic operators

The specification of this subclause in AVC shall apply with the following addition.

x // y	Simplified form of division, defined for integers x and y with y > 0.

x // y = ( x * z( y ) ) >> n( y ) , with	n( y ) = floor( Log2( y ) ) + 15

z( y ) = ( ( 1 << n ( y ) ) + y / 2 ) / y

# Source, coded, decoded and output data formats, scanning processes, and neighbouring relationships

## Bitstream formats

The specification of this subclause in AVC shall apply.

## Source, decoded, and output picture formats

The specification of this subclause in AVC shall apply.

## Spatial subdivision of pictures and slices

The specification of this subclause in AVC shall apply.

## Inverse scanning processes and derivation processes for neighbours

The specification of this subclause in AVC shall apply.

### Inverse macroblock scanning process

The specification of this subclause in AVC shall apply.

### Inverse macroblock partition and sub-partition scanning process

The specification of this subclause in AVC shall apply.

#### Inverse macroblock partition scanning process

The specification of this subclause in AVC shall apply.

#### Inverse sub-macroblock partition scanning process

The specification of this subclause in AVC shall apply.

### Inverse 4x4 luma block scanning process

The specification of this subclause in AVC shall apply.

### Inverse 8x8 luma block scanning process

The specification of this subclause in AVC shall apply.

## Derivation processes for base data

This subclause specifies derivation processes for base data.

[Ed. Note(HS): All processes in this subclause are only specified for the case that frame_mbs_only_flag is equal to 1 for all activated sequence parameter sets.]

### Derivation process for base pictures

This process can only be invoked when the value of the syntax element base_id_plus1 is greater than 0.

Output of this process is a base picture basePic.

The picture basePic can be used for predicting motion data, sample values and/or residual data of the current picture CurrPic.  The base picture is represented by decoded samples and residual samples as well as its syntax elements, prediction utilization flags, reference picture indices, and motion vectors.

The picture basePic, for which all of the following conditions are true, shall be the output.

- The syntax element dependency_id for the picture basePic is equal to DependencyIdBase

- The syntax element quality_level for the picture basePic is equal to QualityLevelBase.

NOTE – The syntax elements dependency_id and quality_level are either transmitted in the NAL units header or inferred.

- The value of PicOrderCnt( basePic ) is equal to the value of PicOrderCnt( CurrPic ).

### Derivation process for base macroblocks

This process can only be invoked when the value of the syntax element base_id_plus1 is greater than 0.

Input to this process is a macroblock address mbAddr.

Outputs of this process are

- a base picture basePic,

- an array of macroblock addresses aMbAddrBase identifying the base macroblocks inside the base picture basePic.

The derivation process for base pictures in subclause S.7.5.1 is invoked with basePic as output.

Let mbIdxX and mbIdxY be defined as follows.

- mbIdxX = mbAddr % PicWidthInMbs

- mbIdxY = mbAddr / PicWidthInMbs

The array of macroblock addresses aMbAddrBase is derived as follows.

- If SpatialScalabilityType is equal to 0,

- Let aMbAddrBase[ 1 ] = aMbAddrBase[ 2 ] = aMbAddrBase[ 3 ] = -1. and

- aMbAddrBase[ 0 ] = BasePicWidthInMbs * ( mbIdxY – ScaledBaseTopOffset / 16 ) +
                                  mbIdxX – ScaledBaseLeftOffset / 16

- Otherwise, if SpatialScalabilityType is equal to 1,

- Let aMbAddrBase[ 1 ] = aMbAddrBase[ 2 ] = aMbAddrBase[ 3 ] = -1, and

aMbAddrBase[ 0 ] = BasePicWidthInMbs *  ( (mbIdxY - ScaledBaseTopOffset / 16) / 2 ) +
                                  (mbIdxX – ScaledBaseLeftOffset / 16) / 2

- Otherwise ( SpatialScalabilityType is greater than 1 ), the following applies.

- Let aMbAddrBase[ 1 ] = aMbAddrBase[ 2 ] = aMbAddrBase[ 3 ] = -1.

- If SpatialScalabilityType is equal to 2, the following applies.

xB = ( ( 16 * mbIdxX – ScaledBaseLeftOffset ) * 2 / 3

yB = ( ( 16*mbIdxY – ScaledBaseTopOffset ) * 2 / 3

- Otherwise, the following applies.

xB = ( ( 16 * mbIdxX – ScaledBaseLeftOffset ) * BasePicWidth + BasePicWidth / 2 ) // ScaledBaseWidth

yB = ( ( 16* mbIdxY – ScaledBaseTopOffset ) * BasePicHeight + BasePicHeight / 2 ) // ScaledBaseHeight

- aMbAddrBase[ 0 ] = BasePicWidthInMbs *  ( yB / 16 ) + xB / 16

- The derivation process for class index in subclause S.9.4.1.6.6 is invoked with mbAddr as input, mbClassIdx as output.

- If mbClassIdx is equal to Vert_X (with X =0..1),

aMbAddrBase[ 1 ] = aMbAddrBase [ 0 ] + 1

- Otherwise, if mbClassIdx is equal to Hori_X (with X =0..1),

aMbAddrBase[ 1 ] = aMbAddrBase[ 0 ] + BasePicWidthInMbs

- Otherwise, if mbClassIdx is equal to Center,

aMbAddrBase[ 1 ] = aMbAddrBase[ 0 ] + 1

aMbAddrBase[ 2 ] = aMbAddrBase[ 0 ] + BasePicWidthInMbs

aMbAddrBase[ 3 ] = aMbAddrBase[ 0 ] + BasePicWidthInMbs + 1

### Derivation process for base sub-macroblocks

This process can only be invoked when the value of the syntax element base_id_plus1 is greater than 0 and the value of SpatialScalabilityType is equal to 1.

Input to this process is a macroblock address mbAddr.

Outputs of this process are

- a base picture basePic,

- a macroblock address mbAddrBase identifying the base macroblock inside the base picture basePic.

- a macroblock partition index mbPartIdxBase identifying the base macroblock partition inside the base macroblock mbAddrBase in the base picture basePic.

The derivation process for base macroblocks in subclause S.7.5.2 is invoked with mbAddr as input and the output is assigned to the picture basePic and the array of macroblock addresses aMbAddrBase.

Let mbAddrBase set to be equal to aMbAddrBase[ 0 ].

The macroblock partition index mbPartIdxBase is derived as follows.

- The base luma location ( xB, yB ) is derived as follows

xB = 8 * ( ( ( mbAddr % PicWidthInMbs ) - ScaledBaseLeftOffset / 16 ) % 2)

yB = 8 * ( ( ( mbAddr / PicWidthInMbs ) - ScaledBaseTopOffset / 16 ) % 2)

- The macroblock partition index of the macroblock partition in the macroblock mbAddrBase in the picture basePic covering the luma location ( xB, yB ) shall be assigned to mbPartIdxBase.

### Derivation process for base partitions

This process can only be invoked when the value of the syntax element base_id_plus1 is greater than 0.

Inputs to this process are

- a macroblock address mbAddr,

- a macroblock partition index mbPartIdx,

- a sub-macroblock partition index subMbPartIdx (when applicable).

Outputs of this process are

- a base picture basePic,

- a macroblock address mbAddrBase identifying the base macroblock inside the base picture basePic,

- a macroblock partition index mbPartIdxBase identifying the base macroblock partition inside the base macroblock mbAddrBase in the base picture basePic.

- a sub-macroblock partition index subMbPartIdxBase identifying the base sub-macroblock partition inside the macroblock partition mbPartIdxBase of the base macroblock mbAddrBase in the base picture basePic.

The derivation process for base macroblocks in subclause S.7.5.2 is invoked with mbAddr as input and the output is assigned to the picture basePic and the array of macroblock addresses aMbAddrBase.

The macroblock partition index mbPartIdxBase and the sub-macroblock partition index subMbPartIdxBase are derived as follows.

- If SpatialScalabilityType is lower than 2, the following applies.

- mbAddrBase is set to aMbAddrBase[ 0].

- The inverse macroblock macroblock partition scanning process in subclause S.7.4.2.1 is invoked with mbPartIdx as the input and ( x, y ) as the output.

- The location of the upper-left luma sample inside a macroblock partition ( xS, yS ) is derived as follows.

- If mb_type is equal to P_8x8, P_8x8ref0, or B_8x8, the inverse sub-macroblock partition scanning process in subclause S.7.4.2.2 is invoked with subMbPartIdx as the input and ( xS, yS ) as the output.

- Otherwise, ( xS, yS ) are set equal to ( 0, 0 ).

- The base luma location ( xB, yB ) is derived as follows.

- If SpatialScalabilityType is equal to 0,

xB = x + xS

yB = y + yS

- Otherwise,

xB = 8 * ( ( ( mbAddr % PicWidthInMbs ) - ScaledBaseLeftOffset /  16 ) % 2) + ( x + xS ) / 2

yB = 8 * ( ( ( mbAddr / PicWidthInMbs ) - ScaledBaseTopOffset /  16 ) % 2) + ( y + yS ) / 2

- The macroblock partition index of the macroblock partition in the macroblock mbAddrBase in the picture basePic covering the luma location ( xB, yB ) shall be assigned to mbPartIdxBase.

- The sub-macroblock partition index of the sub-macroblock partition inside the macroblock partition mbPartIdxBase of the macroblock mbAddrBase in the picture basePic covering the luma location ( xB, yB ) shall be assigned to subMbPartIdxBase.

- Otherwise, if SpatialScalabilityType is equal to 2, the following applies.

- The derivation for macroblock class index in subclause S.9.4.1.6.6 is invoked with mbAddr as input and the output is assigned to mbClassIdx.

- Let b4x4Idx be the 4x4 block index inside the macroblock related to the sub-macroblock partition index subMbPartIdx inside the macroblock partition mbPartIdx of the macroblock mbAddr in the picture basePic.  The b4x4Idx  values are in range 0..15 (cf figure 6-6).

- Let mbAddrBase be derived as follows.

- If mbClassIdx is equal to Corner_X (with X = 0..3 ), mbAddrBase = aMbAddrBase[ 0 ]

- Otherwise, if mbClassIdx is equal to Vert_X (with X = 0,1 ), mbAddrBase = aMbAddrBase[ mbPartIdx%2 ]

- Otherwise, if mbClassIdx is equal to Hori_X (with X = 0,1 ), mbAddrBase = aMbAddrBase[mbPartIdx/2]

- Otherwise, mbAddrBase = aMbAddrBase[ mbPartIdx ]

- Let b4x4IdxBase be the sub-partition index inside the macroblock mbAddrBase corresponding to the sub-macroblock partition index b4x4Idx. b4x4IdxBase is directly given by Table S-6-1 as a function of mbClassIdx and b4x4Idx.

- mbPartIdxBase and subMbPartIdxBase are directly deduced from b4x4IdxBase.  mbPartIdxBase is the index of the macroblock partition of mbAddrBase covering the 4x4 block of index b4x4IdxBase. subMbPartIdxBase is the index of the sub-partition, if it exists, of partition mbPartIdxBase, covering the 4x4 block of index b4x4IdxBase.

Table S-6-1 – b4x4IdxBase as a function of b4x4Idx and mbClassIdx

|  |  | mbSubPartIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| mbClassIdx | Corner_0 | 0 | 0 | 1 | 2 | 0 | 0 | 1 | 2 | 4 | 4 | 5 | 6 | 8 | 8 | 9 | 10 |
|  | Corner_1 | 1 | 2 | 3 | 3 | 1 | 2 | 3 | 3 | 5 | 6 | 7 | 7 | 9 | 10 | 11 | 11 |
|  | Corner_2 | 4 | 4 | 5 | 6 | 8 | 8 | 9 | 10 | 12 | 12 | 13 | 14 | 12 | 12 | 13 | 14 |
|  | Corner_3 | 5 | 6 | 7 | 7 | 9 | 10 | 11 | 11 | 13 | 14 | 15 | 15 | 13 | 14 | 15 | 15 |
|  | Vert_0 | 3 | 3 | 0 | 0 | 3 | 3 | 0 | 0 | 7 | 7 | 4 | 4 | 11 | 11 | 8 | 8 |
|  | Vert_1 | 7 | 7 | 4 | 4 | 11 | 11 | 8 | 8 | 15 | 15 | 12 | 12 | 15 | 15 | 12 | 12 |
|  | Hori_0 | 12 | 12 | 13 | 14 | 12 | 12 | 13 | 14 | 0 | 0 | 1 | 2 | 0 | 0 | 1 | 2 |
|  | Hori_1 | 13 | 14 | 15 | 15 | 13 | 14 | 15 | 15 | 1 | 2 | 3 | 3 | 1 | 2 | 3 | 3 |
|  | Center | 15 | 15 | 12 | 12 | 15 | 15 | 12 | 12 | 3 | 3 | 0 | 0 | 3 | 3 | 0 | 0 |

### Derivation process for base quality slices

This process can only be invoked when the value of the syntax element slice_type is equal to PR.

Input to this process is

- a macroblock address mbAddr

- a variable baseQualityLevel

Output of this process is a base quality slice baseQualitySlice.

The slice baseQualitySlice is represented by its transform coefficients lists LumaLevel[ ], LumaLevel8x8[ ], Intra16x16DCLevel, Intra16x16ACLevel[ ], ChromaDCLevel[ ], and ChromaACLevel[ ][ ] for each macroblock as well as by its syntax elements and its derived motion data.

The slice baseQualitySlice, for which all of the following conditions are true, shall be the output.

- The syntax element dependency_id is equal to baseQualityDependencyId with baseQualityDependencyId being the syntax element dependency_id of the base quality slice baseQualitySlice.

- The syntax element quality_level of the base quality slice baseQualitySlice is equal to baseQualityLevel.

NOTE – The syntax elements dependency_id and quality_level are either transmitted in the NAL unit header or inferred.

- The value of PicOrderCnt( baseQualitySlice ) is equal to the value of PicOrderCnt( CurrPic ).

- The slice baseQualitySlice covers the macroblock with macroblock address mbAddr.

# Syntax and semantics

## Method of describing syntax in tabular form

The specification of this subclause in AVC shall apply.

## Specification of syntax functions, categories, and descriptors

The specification of this subclause in AVC shall be extended by the following paragraphs.

spatial_scalability_type( ) is specified as follows.

- If one of the following conditions is true, the return value of spatial_scalability_type ( ) is equal to 0.

- the syntax element base_id_plus1 is equal to 0

- all the following conditions are true:

- extended_spatial_scalability is less than 2

- ScaledBaseWidth is equal to BasePicWidth

- ScaledBaseHeight is equal to BasePicHeight

- ScaledBaseLeftOffset % 16 is equal to 0

- ScaledBaseTopOffset % 16 is equal to 0

- chroma_format_idc is equal to BaseChromaFormatIdc

- Otherwise, if all the following condition are true, the return value of spatial_scalability_type ( ) is equal to 1.

- extended_spatial_scalability is less than 2

- ScaledBaseWidth  is equal to 2 * BasePicWidth

- ScaledBaseHeight is equal to 2 * BasePicHeight

- ScaledBaseLeftOffset % 16 is equal to 0

- ScaledBaseTopOffset % 16 is equal to 0

- chroma_format_idc is equal to BaseChromaFormatIdc

- Otherwise, if all the following condition are true, the return value of spatial_scalability_type ( ) is equal to 2.

- extended_spatial_scalability is less than 2

- 2 * ScaledBaseWidth is equal to 3 * BasePicWidth

- 2*ScaledBaseHeight is equal to 3 * BasePicHeight

- ScaledBaseLeftOffset % 16 is equal to 0

- ScaledBaseTopOffset % 16 is equal to 0

- chroma_format_idc is equal to BaseChromaFormatIdc

- Otherwise, the return value of spatial_scalability_type ( ) is equal to 3.

intra_base_mb( mbAddr )  is specified as follows.

- The derivation process for base macroblocks in subclause S.7.5.2 is invoked with mbAddr as input and the output is assigned to the picture basePic and the the array of macroblock addresses aMbAddrBase.

- The return value of the function intra_base_mb( mbAddr ) is derived as follows.

- If all macroblock with address aMbAddrBase[ i ] with i = in 0..3 not equal to -1, which in the picture basePic are I macroblocks, the return value of intra_base_mb( mbAddr ) is equal to TRUE.

- Otherwise, the return value of intra_base_mb( mbAddr ) is FALSE.

base_mb_part_pred_mode( mbAddr, mbPartIdx )  is specified as follows.

- If SpatialScalabilityType is lower than 2, the following applies.

- The derivation process for base partitions in subclause S.7.5.4 is invoked with mbAddr and mbPartIdx as inputs and the output is assigned to the picture basePic, the macroblock address mbAddrBase, the macroblock partition index mbPartIdxBase, and the sub-macroblock partition index subMbPartIdxBase.

- Let predFlagL0Base and predFlagL1Base be the values of the prediction utilization flags predFlagL0 and predFlagL1, respectively, of the macroblock or sub-macroblock partition specified by mbAddrBase\mbPartIdxBase\subMbPartIdxBase.

- The return value of the function base_mb_part_pred_mode( mbAddr, mbPartIdx ) is derived as follows.

- If predFlagL0Base is equal to 1 and predFlagL1Base is equal to 1, the return value of base_mb_part_pred_mode( mbAddr, mbPartIdx ) is BiPred.

- Otherwise, if predFlagL0Base is equal to 1, the return value of base_mb_part_pred_mode( mbAddr, mbPartIdx ) is Pred_L0.

- Otherwise, the return value of base_mb_part_pred_mode( mbAddr, mbPartIdx ) is Pred_L1.

- Otherwise ( SpatialScalabilityType is greater than or equal to 2), the following applies.

- The derivation for base reference indices in S.9.4.1.5.1 is invoked with CurrMbAddr, mbPartIdx and a list suffix LX, with X being replaced by either 0 and 1, as inputs and the output is assigned to the reference index refIdxLX.

- If refIdxL0 is equal to 1 and refIdxL1 is equal to 1,

- the return value of  base_mb_part_pred_mode( mbAddr, mbPartIdx ) is BiPred.

- Otherwise, if refIdxL0 is not equal to 1,

- the return value of  base_mb_part_pred_mode( mbAddr, mbPartIdx ) is Pred_L0.

- Otherwise, the return value of  base_mb_part_pred_mode( mbAddr, mbPartIdx ) is Pred_L1.

more_cabac_data_in_nal_unit( )  is specified as follows.

- If more_rbsp_data( ) returns true, the return value of this function is TRUE.

- Otherwise, the return value of this function is FALSE.

base_quality_mb_transform_size_specified( mbAddr )  is specified as follows.

- For any value i with i = 0..quality_level – 1 the following applies.

- The derivation process for base quality slices in subclause S.7.5.5 is invoked with mbAddr and baseQualityLevel equal to i as input and the output is assigned to baseQualitySlice[ i ].

- Let mbTypeBaseQuality be the syntax element mb_type of the macroblock mbAddr in the slice baseQualitySlice[ 0 ].

- Let subMbTypeBaseQuality[ k ] with k = 0..3 be the syntax element sub_mb_type[ k ] of the macroblock mbAddr in the slice baseQualitySlice[ 0 ].

- The return value of base_quality_mb_transform_size_specified( mbAddr ) is derived as follows.

- If any of the following conditions are true, the return value of base_quality_mb_transform_size_specified( mbAddr ) is equal to TRUE.

- MbPartPredMode( mbTypeBaseQuality, 0 ) is equal to Intra_4x4, Intra_8x8, or Intra_16x16.

- mbTypeBaseQuality is equal to P_8x8 or P_8x8ref0, and any of the variables subMbTypeBaseQuality[ k ] with k = 0..3 is not equal to P_L0_8x8.

- mbTypeBaseQuality is equal to B_8x8 and any of the following conditions is true.

- NumSubMbPart( subMbTypeBaseQuality[ k ] ) is greater than 1 for any k = 0...3.

- SubMbPredMode( subMbTypeBaseQuality[ k ] ) is equal to Direct for any k = 0..3, and direct_8x8_inference_flag is equal to 0.

- The syntax element transform_size_8x8_flag was transmitted for the macroblock with macroblock address mbAddr in any of the slices baseQualitySlice[ i ] with i = 0.. quality_level – 1.

- Otherwise, the return value of base_quality_mb_transform_size_specified( mbAddr ) is equal to FALSE.

base_quality_transform_size_8x8_flag( mbAddr )  is specified as follows.

- The derivation process for base quality slices in subclause S.7.5.5 is invoked with mbAddr and baseQualityLevel equal to quality_level – 1 as input and the output is assigned to baseQualitySlice.

- The return value of base_quality_transform_size_8x8_flag( mbAddr ) is equal to the syntax element transform_size_8x8_flag of the macroblock with macroblock address mbAddr in the base quality slice baseQualitySlice.

base_mb_sig( mbAddr )  is specified as follows.

- For any value i with i = 0..quality_level – 1 the following applies.

- The derivation process for base quality slices in subclause S.7.5.5 is invoked with mbAddr and baseQualityLevel equal to i as input and the output is assigned to baseQualitySlice[ i ].

- The return value of base_mb_sig( mbAddr ) is derived as follows.

- If any of the following conditions is true for any of the slices baseQualitySlice[ i ] with i = 0..quality_level – 1, the return value of base_mb_sig( mbAddr ) is equal to TRUE.

- The variable CodedBlockPatternLuma is greater than 0.

- The variable CodedBlockPatternChroma is greater than 0.

- Otherwise, the return value of base_mb_sig( mbAddr ) is equal to FALSE.

base_luma_level( mbAddr, i8x8, i4x4, scanIdx )  is specified as follows.

- A variable i is set equal to 0.

- A variable result is set equal to 0.

- The return value of base_luma_level( mbAddr, i8x8, i4x4, scanIdx ) is derived as follows.

- The derivation process for base quality slices in subclause S.7.5.5 is invoked with mbAddr and baseQualityLevel equal to i as input and the output is assigned to baseQualitySlice[ i ].

- When the transform coefficient level LumaLevel[ 4 * i8x8 + i4x4 ][ scanIdx ] of the macroblock mbAddr is not equal to 0, the following applies

- The variable result is set equal to LumaLevel[ 4 * i8x8 + i4x4 ][ scanIdx ].

- The derivation process is continued with step 4.

- When the variable i is less than ( quality_level – 1 ), the following applies.

- The variable i is incremented by 1: i = i + 1.

- The derivation process is continued with step 1.

- The return value of base_luma_level( mbAddr, i8x8, i4x4, scanIdx ) is set equal to the value of the variable result.

base_luma_level8x8( mbAddr, i8x8, scanIdx8x8 )  is specified as follows.

- A variable i is set equal to 0.

- A variable result is set equal to 0.

- The return value of base_luma8x8_level( mbAddr, i8x8, scanIdx8x8 ) is derived as follows.

- The derivation process for base quality slices in subclause S.7.5.5 is invoked with mbAddr and baseQualityLevel equal to i as input and the output is assigned to baseQualitySlice[ i ].

- When the transform coefficient level LumaLevel8x8[ i8x8 ][ scanIdx8x8 ] of the macroblock mbAddr is not equal to 0, the following applies

- The variable result is set equal to LumaLevel8x8[ i8x8 ][ scanIdx8x8 ].

- The derivation process is continued with step 4.

- When the variable i is less than ( quality_level – 1 ), the following applies.

- The variable i is incremented by 1: i = i + 1.

- The derivation process is continued with step 1.

- The return value of base_luma8x8_level( mbAddr, i8x8, scanIdx8x8 ) is set equal to the value of the variable result.

base_chromaDC_level( mbAddr, iCbCr, dcIdx )  is specified as follows.

- A variable i is set equal to 0.

- A variable result is set equal to 0.

- The return value of base_chromaDC_level_sig( mbAddr, iCbCr, dcIdx ) is derived as follows.

- The derivation process for base quality slices in subclause S.7.5.5 is invoked with mbAddr and baseQualityLevel equal to i as input and the output is assigned to baseQualitySlice[ i ].

- When the transform coefficient level ChromaDCLevel[ iCbCr ][ dcIdx ] of the macroblock mbAddr is not equal to 0, the following applies

- The variable result is set equal to ChromaDCLevel[ iCbCr ][ dcIdx ].

- The derivation process is continued with step 4.

- When the variable i is less than ( quality_level – 1 ), the following applies.

- The variable i is incremented by 1: i = i + 1.

- The derivation process is continued with step 1.

- The return value of base_chromaDC_level_sig( mbAddr, iCbCr, dcIdx ) is set equal to the value of the variable result.

base_chromaAC_level( mbAddr, iCbCr, i8x8, i4x4, scanIdx )  is specified as follows.

- A variable i is set equal to 0.

- A variable result is set equal to 0.

- The return value of base_chromaAC_level_sig( mbAddr, iCbCr, i8x8, i4x4, scanIdx ) is derived as follows.

- The derivation process for base quality slices in subclause S.7.5.5 is invoked with mbAddr and baseQualityLevel equal to i as input and the output is assigned to baseQualitySlice[ i ].

- When the transform coefficient level ChromaACLevel[ iCbCr ][ 4 * i8x8 + i4x4 ][ scanIdx ] of the macroblock mbAddr is not equal to 0, the following applies

- The variable result is set equal to ChromaACLevel[ iCbCr ][ 4 * i8x8 + i4x4 ][ scanIdx ].

- The derivation process is continued with step 4.

- When the variable i is less than ( quality_level – 1 ), the following applies.

- The variable i is incremented by 1: i = i + 1.

- The derivation process is continued with step 1.

- The return value of base_chromaAC_level_sig( mbAddr, iCbCr, i8x8, i4x4, scanIdx ) is set equal to the value of the variable result.

in_crop_window( mbAddr )  is specified as follows.

- Let mbIdxX and mbIdxY be defined as follows.

- mbIdxX is equal to mbAddr % PicWidthInMbs

- mbIdxY is equal to mbAddr / PicWidthInMbs

- If all of the following conditions are true, the return value of in_crop_window ( ) is equal to TRUE.

- mbIdxX is greater than or equal to ( ScaledBaseLeftOffset + 15 ) / 16

- mbIdxX is less than or equal to ( ScaledBaseLeftOffset + ScaledBaseWidth - 1 ) / 16

- mbIdxY is greater than or equal to ( ScaleBaseTopOffset + 15 ) / 16

- mbIdxY is less than or equal to ( ScaleBaseTopOffset + ScaledBaseHeight - 1 ) / 16

- Otherwise, the return value of in_crop_window ( ) is equal to FALSE..

## Syntax in tabular form

The specification of this subclause in AVC shall be extended as specified in the following subclauses.

### NAL unit syntax

The specification of this subclause in AVC shall be replaced by the following specification.

| nal_unit( NumBytesInNALunit ) { | C | Descriptor |
| --- | --- | --- |
| forbidden_zero_bit | All | f(1) |
| nal_ref_idc | All | u(2) |
| nal_unit_type | All | u(5) |
| nalUnitHeaderBytes = 1 |  |  |
| if( nal_unit_type = = 20 ¦ ¦ nal_unit_type = = 21 ) { |  |  |
| simple_priority_id | All | u(6) |
| discardable_flag | All | u(1) |
| extension_flag | All | u(1) |
| if( extension_flag = = 1 ) { |  |  |
| temporal_level | All | u(3) |
| dependency_id | All | u(3) |
| quality_level | All | u(2) |
| nalUnitHeaderBytes++ |  |  |
| } |  |  |
| nalUnitHeaderBytes++ |  |  |
| } |  |  |
| NumBytesInRBSP = 0 |  |  |
| for( i = nalUnitHeaderBytes; i < NumBytesInNALunit; i++ ) { |  |  |
| if( i + 2 < NumBytesInNALunit && next_bits( 24 ) = = 0x000003 ) { |  |  |
| rbsp_byte[ NumBytesInRBSP++ ] | All | b(8) |
| rbsp_byte[ NumBytesInRBSP++ ] | All | b(8) |
| i += 2 |  |  |
| emulation_prevention_three_byte /* equal to 0x03 */ | All | f(8) |
| } else |  |  |
| rbsp_byte[ NumBytesInRBSP++ ] | All | b(8) |
| } |  |  |
| } |  |  |

### Raw byte sequence payloads and RBSP trailing bits syntax

#### Sequence parameter set RBSP syntax

The specification of this subclause in AVC shall be replaced by the following specification.

| seq_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| profile_idc | 0 | u(8) |
| constraint_set0_flag | 0 | u(1) |
| constraint_set1_flag | 0 | u(1) |
| constraint_set2_flag | 0 | u(1) |
| constraint_set3_flag | 0 | u(1) |
| reserved_zero_4bits /* equal to 0 */ | 0 | u(4) |
| level_idc | 0 | u(8) |
| seq_parameter_set_id | 0 | ue(v) |
| if( profile_idc = = 83 ) { |  |  |
| nal_unit_extension_flag | 0 | u(1) |
| if( nal_unit_extension_flag = = 0 ) { |  |  |
| number_of_simple_priority_id_values_minus1 | 0 | ue(v) |
| for( i = 0; i <= number_of_simple_priority_id_values_minus1; i++ ) { |  |  |
| priority_id | 0 | u(6) |
| temporal_level_list[ priority_id ] | 0 | u(3) |
| dependency_id_list[ priority_id ] | 0 | u(3) |
| quality_level_list[ priority_id ] | 0 | u(2) |
| } |  |  |
| } |  |  |
| low_complexity_update_flag | 0 | u(1) |
| } |  |  |
| if( profile_idc = = 100 ¦ ¦ profile_idc = = 110 ¦ ¦ profile_idc = = 122 ¦ ¦ profile_idc = = 144 ¦ ¦ profile_idc = = 83 ) ) { |  |  |
| chroma_format_idc | 0 | ue(v) |
| if( chroma_format_idc = = 3 ) |  |  |
| residual_colour_transform_flag | 0 | u(1) |
| bit_depth_luma_minus8 | 0 | ue(v) |
| bit_depth_chroma_minus8 | 0 | ue(v) |
| qpprime_y_zero_transform_bypass_flag | 0 | u(1) |
| seq_scaling_matrix_present_flag | 0 | u(1) |
| if( seq_scaling_matrix_present_flag ) |  |  |
| for( i = 0; i < 8; i++ ) { |  |  |
| seq_scaling_list_present_flag[ i ] | 0 | u(1) |
| if( seq_scaling_list_present_flag[ i ] ) |  |  |
| if( i < 6 ) |  |  |
| scaling_list( ScalingList4x4[ i ], 16, UseDefaultScalingMatrix4x4Flag[ i ]) | 0 |  |
| else |  |  |
| scaling_list( ScalingList8x8[ i – 6 ], 64, UseDefaultScalingMatrix8x8Flag[ i – 6 ] ) | 0 |  |
| } |  |  |
| } |  |  |
| log2_max_frame_num_minus4 | 0 | ue(v) |
| pic_order_cnt_type | 0 | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |  |
| log2_max_pic_order_cnt_lsb_minus4 | 0 | ue(v) |
| else if( pic_order_cnt_type = = 1 ) { |  |  |
| delta_pic_order_always_zero_flag | 0 | u(1) |
| offset_for_non_ref_pic | 0 | se(v) |
| offset_for_top_to_bottom_field | 0 | se(v) |
| num_ref_frames_in_pic_order_cnt_cycle | 0 | ue(v) |
| for( i = 0; i < num_ref_frames_in_pic_order_cnt_cycle; i++ ) |  |  |
| offset_for_ref_frame[ i ] | 0 | se(v) |
| } |  |  |
| num_ref_frames | 0 | ue(v) |
| gaps_in_frame_num_value_allowed_flag | 0 | u(1) |
| pic_width_in_mbs_minus1 | 0 | ue(v) |
| pic_height_in_map_units_minus1 | 0 | ue(v) |
| frame_mbs_only_flag | 0 | u(1) |
| if( !frame_mbs_only_flag ) |  |  |
| mb_adaptive_frame_field_flag | 0 | u(1) |
| direct_8x8_inference_flag | 0 | u(1) |
| frame_cropping_flag | 0 | u(1) |
| if( frame_cropping_flag ) { |  |  |
| frame_crop_left_offset | 0 | ue(v) |
| frame_crop_right_offset | 0 | ue(v) |
| frame_crop_top_offset | 0 | ue(v) |
| frame_crop_bottom_offset | 0 | ue(v) |
| } |  |  |
| if ( profile_idc = = 83 ){ |  |  |
| extended_spatial_scalability | 0 | u(2) |
| if( extended_spatial_scalability > 0 ) { |  |  |
| if ( chroma_format_idc > 0 ) { |  |  |
| chroma_phase_x_plus1 | 0 | u(2) |
| chroma_phase_y_plus1 | 0 | u(2) |
| } |  |  |
| if( extended_spatial_scalability = = 1 ) { |  |  |
| scaled_base_left_offset | 0 | se(v) |
| scaled_base_top_offset | 0 | se(v) |
| scaled_base_right_offset | 0 | se(v) |
| scaled_base_bottom_offset | 0 | se(v) |
| } |  |  |
| } |  |  |
| } |  |  |
| vui_parameters_present_flag | 0 | u(1) |
| if( vui_parameters_present_flag ) |  |  |
| vui_parameters( ) | 0 |  |
| rbsp_trailing_bits( ) | 0 |  |
| } |  |  |

##### Scaling list syntax

The specification of this subclause in AVC shall apply.

##### Sequence parameter set extension RBSP syntax

The specification of this subclause in AVC shall apply.

#### Picture parameter set RBSP syntax

The specification of this subclause in AVC shall apply.

#### Supplemental enhancement information RBSP syntax

The specification of this subclause in AVC shall apply.

#### Access unit delimiter RBSP syntax

The specification of this subclause in AVC shall apply.

#### End of sequence RBSP syntax

The specification of this subclause in AVC shall apply.

#### End of stream RBSP syntax

The specification of this subclause in AVC shall apply.

#### Filler data RBSP syntax

The specification of this subclause in AVC shall apply.

#### Slice layer without partitioning RBSP syntax

The specification of this subclause in AVC shall apply.

#### Slice data partition RBSP syntax

The specification of this subclause in AVC shall apply.

#### RBSP slice trailing bits syntax

The specification of this subclause in AVC shall apply.

#### RBSP trailing bits syntax

The specification of this subclause in AVC shall apply.

#### Slice layer in scalable extension RBSP syntax

| slice_layer_in_scalable_extension_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| slice_header_in_scalable_extension( ) | 2 |  |
| if( slice_type != PR ) |  |  |
| slice_data_in_scalable_extension( ) | 2 ¦ 3 ¦ 4 |  |
| else |  |  |
| progressive_refinement_slice_data_in_scalable_extension( ) | 2 ¦ 3 ¦ 4 |  |
| rbsp_slice_trailing_bits( ) | 2 |  |
| } |  |  |

### Slice header syntax

The specification of this subclause in AVC shall apply.  The subclauses are modified as specified in the following.

#### Reference picture list reordering syntax

The specification of this subclause in AVC shall be replaced by the following specification.

| ref_pic_list_reordering( ) { | C | Descriptor |
| --- | --- | --- |
| if( slice_type != I && slice_type != SI && slice_type != EI ) { |  |  |
| ref_pic_list_reordering_flag_l0 | 2 | u(1) |
| if( ref_pic_list_reordering_flag_l0 ) |  |  |
| do { |  |  |
| reordering_of_pic_nums_idc | 2 | ue(v) |
| if( reordering_of_pic_nums_idc = = 0 ¦ ¦ reordering_of_pic_nums_idc = = 1 ) |  |  |
| abs_diff_pic_num_minus1 | 2 | ue(v) |
| else if( reordering_of_pic_nums_idc = = 2 ) |  |  |
| long_term_pic_num | 2 | ue(v) |
| } while( reordering_of_pic_nums_idc != 3 ) |  |  |
| } |  |  |
| if( slice_type = = B ¦ ¦ slice_type = = EB ) { |  |  |
| ref_pic_list_reordering_flag_l1 | 2 | u(1) |
| if( ref_pic_list_reordering_flag_l1 ) |  |  |
| do { |  |  |
| reordering_of_pic_nums_idc | 2 | ue(v) |
| if( reordering_of_pic_nums_idc = = 0 ¦ ¦ reordering_of_pic_nums_idc = = 1 ) |  |  |
| abs_diff_pic_num_minus1 | 2 | ue(v) |
| else if( reordering_of_pic_nums_idc = = 2 ) |  |  |
| long_term_pic_num | 2 | ue(v) |
| } while( reordering_of_pic_nums_idc != 3 ) |  |  |
| } |  |  |
| } |  |  |

#### Prediction weight table syntax

The specification of this subclause in AVC shall be replaced by the following specification.

| pred_weight_table( ) { | C | Descriptor |
| --- | --- | --- |
| luma_log2_weight_denom | 2 | ue(v) |
| chroma_log2_weight_denom | 2 | ue(v) |
| for( i = 0; i <= num_ref_idx_l0_active_minus1; i++ ) { |  |  |
| luma_weight_l0_flag | 2 | u(1) |
| if( luma_weight_l0_flag ) { |  |  |
| luma_weight_l0[ i ] | 2 | se(v) |
| luma_offset_l0[ i ] | 2 | se(v) |
| } |  |  |
| chroma_weight_l0_flag | 2 | u(1) |
| if( chroma_weight_l0_flag ) |  |  |
| for( j =0; j < 2; j++ ) { |  |  |
| chroma_weight_l0[ i ][ j ] | 2 | se(v) |
| chroma_offset_l0[ i ][ j ] | 2 | se(v) |
| } |  |  |
| } |  |  |
| if( slice_type = = B ¦ ¦ slice_type = = EB ) |  |  |
| for( i = 0; i <= num_ref_idx_l1_active_minus1; i++ ) { |  |  |
| luma_weight_l1_flag | 2 | u(1) |
| if( luma_weight_l1_flag ) { |  |  |
| luma_weight_l1[ i ] | 2 | se(v) |
| luma_offset_l1[ i ] | 2 | se(v) |
| } |  |  |
| chroma_weight_l1_flag | 2 | u(1) |
| if( chroma_weight_l1_flag ) |  |  |
| for( j = 0; j < 2; j++ ) { |  |  |
| chroma_weight_l1[ i ][ j ] | 2 | se(v) |
| chroma_offset_l1[ i ][ j ] | 2 | se(v) |
| } |  |  |
| } |  |  |
| } |  |  |

#### Decoded reference picture marking syntax

The specification of this subclause in AVC shall be replaced by the following specification.

| dec_ref_pic_marking( ) { | C | Descriptor |
| --- | --- | --- |
| if( nal_unit_type = = 5 ¦¦ nal_unit_type = = 21 ) { |  |  |
| no_output_of_prior_pics_flag | 2 ¦ 5 | u(1) |
| long_term_reference_flag | 2 ¦ 5 | u(1) |
| } else { |  |  |
| adaptive_ref_pic_marking_mode_flag | 2 ¦ 5 | u(1) |
| if( adaptive_ref_pic_marking_mode_flag ) |  |  |
| do { |  |  |
| memory_management_control_operation | 2 ¦ 5 | ue(v) |
| if( memory_management_control_operation = = 1 ¦ ¦ memory_management_control_operation = = 3 ) |  |  |
| difference_of_pic_nums_minus1 | 2 ¦ 5 | ue(v) |
| if(memory_management_control_operation = = 2 ) |  |  |
| long_term_pic_num | 2 ¦ 5 | ue(v) |
| if( memory_management_control_operation = = 3 ¦ ¦ memory_management_control_operation = = 6 ) |  |  |
| long_term_frame_idx | 2 ¦ 5 | ue(v) |
| if( memory_management_control_operation = = 4 ) |  |  |
| max_long_term_frame_idx_plus1 | 2 ¦ 5 | ue(v) |
| } while( memory_management_control_operation != 0 ) |  |  |
| } |  |  |
| } |  |  |

### Slice data syntax

The specification of this subclause in AVC shall apply.

### Macroblock layer syntax

The specification of this subclause in AVC shall apply.

### Slice header in scalable extension syntax

| slice_header_in_scalable_extension( ) { | C | Descriptor |
| --- | --- | --- |
| first_mb_in_slice | 2 | ue(v) |
| slice_type | 2 | ue(v) |
| pic_parameter_set_id | 2 | ue(v) |
| if( slice_type = = PR ) { |  |  |
| num_mbs_in_slice_minus1 | 2 | ue(v) |
| luma_chroma_sep_flag | 2 | u(1) |
| } |  |  |
| frame_num | 2 | u(v) |
| if( !frame_mbs_only_flag ) { |  |  |
| field_pic_flag | 2 | u(1) |
| if( field_pic_flag ) |  |  |
| bottom_field_flag | 2 | u(1) |
| } |  |  |
| if( nal_unit_type = = 21 ) |  |  |
| idr_pic_id | 2 | ue(v) |
| if( pic_order_cnt_type = = 0 ) { |  |  |
| pic_order_cnt_lsb | 2 | u(v) |
| if( pic_order_present_flag && !field_pic_flag ) |  |  |
| delta_pic_order_cnt_bottom | 2 | se(v) |
| } |  |  |
| if( pic_order_cnt_type = = 1 && !delta_pic_order_always_zero_flag ) { |  |  |
| delta_pic_order_cnt[ 0 ] | 2 | se(v) |
| if( pic_order_present_flag && !field_pic_flag ) |  |  |
| delta_pic_order_cnt[ 1 ] | 2 | se(v) |
| } |  |  |
| if( slice_type != PR ) { |  |  |
| if( redundant_pic_cnt_present_flag ) |  |  |
| redundant_pic_cnt | 2 | ue(v) |
| if( slice_type = = EB ) |  |  |
| direct_spatial_mv_pred_flag | 2 | u(1) |
| key_picture_flag | 2 | u(1) |
| decomposition_stages | 2 | ue(v) |
| base_id_plus1 | 2 | ue(v) |
| if( base_id_plus1 != 0 ) { |  |  |
| adaptive_prediction_flag | 2 | u(1) |
| } |  |  |
| if( slice_type = = EP ¦ ¦ slice_type = = EB ) { |  |  |
| num_ref_idx_active_override_flag | 2 | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |  |
| num_ref_idx_l0_active_minus1 | 2 | ue(v) |
| if( slice_type = = EB ) |  |  |
| num_ref_idx_l1_active_minus1 | 2 | ue(v) |
| } |  |  |
| } |  |  |
| ref_pic_list_reordering( ) | 2 |  |
| for( decLvl = temporal_level; decLvl < decomposition_stages; decLvl++ ) { |  |  |
| num_ref_idx_update_l0_active[ decLvl + 1 ] | 2 | ue(v) |
| num_ref_idx_update_l1_active[ decLvl + 1 ] | 2 | ue(v) |
| } |  |  |
| if( ( weighted_pred_flag && slice_type = = EP ) ¦ ¦ ( weighted_bipred_idc = = 1 && slice_type = = EB ) ) |  |  |
| pred_weight_table( ) | 2 |  |
| if( nal_ref_idc != 0 ) |  |  |
| dec_ref_pic_marking( ) | 2 |  |
| if( entropy_coding_mode_flag && slice_type != EI ) |  |  |
| cabac_init_idc | 2 | ue(v) |
| } |  |  |
| slice_qp_delta | 2 | se(v) |
| if( deblocking_filter_control_present_flag ) { |  |  |
| disable_deblocking_filter_idc | 2 | ue(v) |
| if( disable_deblocking_filter_idc != 1 ) { |  |  |
| slice_alpha_c0_offset_div2 | 2 | se(v) |
| slice_beta_offset_div2 | 2 | se(v) |
| } |  |  |
| } |  |  |
| if( slice_type != PR ) |  |  |
| if( num_slice_groups_minus1 > 0 && slice_group_map_type >= 3 && slice_group_map_type <= 5) |  |  |
| slice_group_change_cycle | 2 | u(v) |
| if( slice_type != PR && extended_spatial_scalability > 0 ) { |  |  |
| if ( chroma_format_idc > 0 ) { |  |  |
| base_chroma_phase_x_plus1 | 2 | u(2) |
| base_chroma_phase_y_plus1 | 2 | u(2) |
| } |  |  |
| if( extended_spatial_scalability = = 2 ) { |  |  |
| scaled_base_left_offset | 2 | se(v) |
| scaled_base_top_offset | 2 | se(v) |
| scaled_base_right_offset | 2 | se(v) |
| scaled_base_bottom_offset | 2 | se(v) |
| } |  |  |
| } |  |  |
| SpatialScalabilityType = spatial_scalability_type( ) |  |  |
| } |  |  |

### Slice data in scalable extension syntax

| slice_data_in_scalable_extension( ) { | C | Descriptor |
| --- | --- | --- |
| while( !byte_aligned( ) ) |  |  |
| cabac_alignment_one_bit | 2 | f(1) |
| CurrMbAddr = first_mb_in_slice * ( 1 + MbaffFrameFlag ) |  |  |
| do { |  |  |
| if( MbaffFrameFlag && ( CurrMbAddr % 2 = = 0 ) ) |  |  |
| mb_field_decoding_flag | 2 | ae(v) |
| macroblock_layer_in_scalable_extension( ) | 2 ¦ 3 ¦ 4 |  |
| if( MbaffFrameFlag && ( CurrMbAddr % 2 = = 0 ) ) |  |  |
| moreDataFlag = 1 |  |  |
| else { |  |  |
| end_of_slice_flag | 2 | ae(v) |
| moreDataFlag = ! end_of_slice_flag |  |  |
| } |  |  |
| CurrMbAddr = NextMbAddress( CurrMbAddr ) |  |  |
| } while( moreDataFlag ) |  |  |
| } |  |  |

### Macroblock layer in scalable extension syntax

| macroblock_layer_in_scalable_extension( ) { | C | Descriptor |
| --- | --- | --- |
| if( in_crop_window( CurrMbAddr ) ) |  |  |
| if( base_id_plus1 != 0 && adaptive_prediction_flag && in_crop_window( CurrMbAddr ) ) { |  |  |
| base_mode_flag | 2 | ae(v) |
| if( ! base_mode_flag && SpatialScalabilityType > 0 && ! intra_base_mb( CurrMbAddr ) ) |  |  |
| base_mode_refinement_flag | 2 | ae(v) |
| } |  |  |
| } |  |  |
| if( ! base_mode_flag && ! base_mode_refinement_flag ) { |  |  |
| mb_type | 2 | ae(v) |
| if( mb_type = = I_NxN && base_id_plus1 != 0 ) |  |  |
| intra_base_flag | 2 | ae(v) |
| } |  |  |
| if( mb_type = = I_PCM ) { |  |  |
| while( !byte_aligned( ) ) |  |  |
| pcm_alignment_zero_bit | 2 | f(1) |
| for( i = 0; i < 256; i++ ) |  |  |
| pcm_sample_luma[ i ] | 2 | u(v) |
| for( i = 0; i < 2 * MbWidthC * MbHeightC; i++ ) |  |  |
| pcm_sample_chroma[ i ] | 2 | u(v) |
| } else { |  |  |
| NoSubMbPartSizeLessThan8x8Flag = 1 |  |  |
| if( mb_type != I_NxN && MbPartPredMode( mb_type, 0 ) != Intra_16x16 && NumMbPart( mb_type ) = = 4 ) { |  |  |
| sub_mb_pred_in_scalable_extension( mb_type ) | 2 |  |
| for( mbPartIdx = 0; mbPartIdx < 4; mbPartIdx++ ) |  |  |
| if( sub_mb_type[ mbPartIdx ] != B_Direct_8x8 ) { |  |  |
| if( NumSubMbPart( sub_mb_type [ mbPartIdx ] ) > 1 ) |  |  |
| NoSubMbPartSizeLessThan8x8Flag = 0 |  |  |
| } else if( !direct_8x8_inference_flag ) |  |  |
| NoSubMbPartSizeLessThan8x8Flag = 0 |  |  |
| } else { |  |  |
| if( transform_8x8_mode_flag && mb_type = = I_NxN ) |  |  |
| transform_size_8x8_flag | 2 | ae(v) |
| mb_pred_in_scalable_extension( mb_type ) | 2 |  |
| } |  |  |
| if( MbPartPredMode( mb_type, 0 ) != Intra_16x16 ) { |  |  |
| coded_block_pattern | 2 | ae(v) |
| if( CodedBlockPatternLuma > 0 && transform_8x8_mode_flag && mb_type != I_NxN && NoSubMbPartSizeLessThan8x8Flag && !( MbPartPredMode( mb_type, 0 ) = = B_Direct_16x16 && !direct_8x8_inference_flag ) ) |  |  |
| transform_size_8x8_flag | 2 | ae(v) |
| } |  |  |
| if( CodedBlockPatternLuma > 0 ¦ ¦ CodedBlockPatternChroma > 0 ¦ ¦ MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) { |  |  |
| mb_qp_delta | 2 | ae(v) |
| residual_in_scalable_extension( ) | 3 ¦ 4 |  |
| } |  |  |
| } |  |  |
| } |  |  |

#### Macroblock prediction in scalable extension syntax

| mb_pred_in_scalable_extension( mb_type ) { | C | Descriptor |
| --- | --- | --- |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_4x4 ¦ ¦ MbPartPredMode( mb_type, 0 ) = = Intra_8x8 ¦ ¦ MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) { |  |  |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_4x4 ) |  |  |
| for( luma4x4BlkIdx = 0; luma4x4BlkIdx < 16; luma4x4BlkIdx++ ) { |  |  |
| prev_intra4x4_pred_mode_flag[ luma4x4BlkIdx ] | 2 | ae(v) |
| if( !prev_intra4x4_pred_mode_flag[ luma4x4BlkIdx ] ) |  |  |
| rem_intra4x4_pred_mode[ luma4x4BlkIdx ] | 2 | ae(v) |
| } |  |  |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_8x8 ) |  |  |
| for( luma8x8BlkIdx = 0; luma8x8BlkIdx < 4; luma8x8BlkIdx++ ) { |  |  |
| prev_intra8x8_pred_mode_flag[ luma8x8BlkIdx ] | 2 | ae(v) |
| if( !prev_intra8x8_pred_mode_flag[ luma8x8BlkIdx ] ) |  |  |
| rem_intra8x8_pred_mode[ luma8x8BlkIdx ] | 2 | ae(v) |
| } |  |  |
| if( chroma_format_idc != 0 ) |  |  |
| intra_chroma_pred_mode | 2 | ae(v) |
| } else if( MbPartPredMode( mb_type, 0 ) != Direct && MbPartPredMode( mb_type, 0 ) != Intra_Base && ! base_mode_flag ) { |  |  |
| if( !base_mode_refinement_flag ) { |  |  |
| if( base_id_plus1 != 0 ) { |  |  |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( MbPartPredMode( mb_type, mbPartIdx ) != Pred_L1 && base_mb_part_pred_mode( mb_type, mbPartIdx ) != Pred_L1 ) |  |  |
| motion_prediction_flag_l0[ mbPartIdx ] | 2 | ae(v) |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( MbPartPredMode( mb_type, mbPartIdx ) != Pred_L0 && base_mb_part_pred_mode( mb_type, mbPartIdx ) != Pred_L0 ) |  |  |
| motion_prediction_flag_l1[ mbPartIdx ] | 2 | ae(v) |
| } |  |  |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( ( num_ref_idx_l0_active_minus1 > 0 ¦ ¦ mb_field_decoding_flag ) && MbPartPredMode( mb_type, mbPartIdx ) != Pred_L1 && ! motion_prediction_flag_l0[ mbPartIdx ] ) |  |  |
| ref_idx_l0[ mbPartIdx ] | 2 | ae(v) |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( ( num_ref_idx_l1_active_minus1 > 0 ¦ ¦ mb_field_decoding_flag ) && MbPartPredMode( mb_type, mbPartIdx ) != Pred_L0 && ! motion_prediction_flag_l1[ mbPartIdx ] ) |  |  |
| ref_idx_l1[ mbPartIdx ] | 2 | ae(v) |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( MbPartPredMode ( mb_type, mbPartIdx ) != Pred_L1 ) |  |  |
| for( compIdx = 0; compIdx < 2; compIdx++ ) |  |  |
| mvd_l0[ mbPartIdx ][ 0 ][ compIdx ] | 2 | ae(v) |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( MbPartPredMode( mb_type, mbPartIdx ) != Pred_L0 ) |  |  |
| for( compIdx = 0; compIdx < 2; compIdx++ ) |  |  |
| mvd_l1[ mbPartIdx ][ 0 ][ compIdx ] | 2 | ae(v) |
| } else { /* base_mode_refinement_flag */ |  |  |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( MbPartPredMode ( mb_type, mbPartIdx ) != Pred_L1 ) |  |  |
| for( compIdx = 0; compIdx < 2; compIdx++ ) |  |  |
| mvd_ref_l0[ mbPartIdx ][ 0 ][ compIdx ] | 2 | ae(v) |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( MbPartPredMode( mb_type, mbPartIdx ) != Pred_L0 ) |  |  |
| for( compIdx = 0; compIdx < 2; compIdx++ ) |  |  |
| mvd_ref_l1[ mbPartIdx ][ 0 ][ compIdx ] | 2 | ae(v) |
| } |  |  |
| } |  |  |
| } |  |  |

#### Sub-macroblock prediction in scalable extension syntax

| sub_mb_pred_in_scalable_extension( MbType ) { | C | Descriptor |
| --- | --- | --- |
| if( !base_mode_flag && !base_mode_refinement_flag ) { |  |  |
| for( mbPartIdx = 0; mbPartIdx < 4; mbPartIdx++ ) |  |  |
| sub_mb_type[ mbPartIdx ] | 2 | ae(v) |
| if( base_id_plus1 != 0 ) { |  |  |
| for( mbPartIdx = 0; mbPartIdx < 4; mbPartIdx++ ) |  |  |
| if( SubMbPredMode( sub_mb_type[ mbPartIdx ] ) != Pred_L1 && base_mb_part_pred_mode( CurrMbAddr, mbPartIdx ) != Pred_L1 ) |  |  |
| motion_prediction_flag_l0[ mbPartIdx ] | 2 | ae(v) |
| for( mbPartIdx = 0; mbPartIdx < 4; mbPartIdx++ ) |  |  |
| if( SubMbPredMode( sub_mb_type[ mbPartIdx ] ) != Pred_L0 && base_mb_part_pred_mode( CurrMbAddr, mbPartIdx ) != Pred_L0 ) |  |  |
| motion_prediction_flag_l1[ mbPartIdx ] | 2 | ae(v) |
| } |  |  |
| for( mbPartIdx = 0; mbPartIdx < 4; mbPartIdx++ ) |  |  |
| if( ( num_ref_idx_l0_active_minus1 > 0 ¦ ¦ mb_field_decoding_flag ) && sub_mb_type[ mbPartIdx ] != B_Direct_8x8 && SubMbPredMode( sub_mb_type[ mbPartIdx ] ) != Pred_L1 && !motion_prediction_flag_l0[ mbPartIdx ] ) |  |  |
| ref_idx_l0[ mbPartIdx ] | 2 | ae(v) |
| for( mbPartIdx = 0; mbPartIdx < 4; mbPartIdx++ ) |  |  |
| if( (num_ref_idx_l1_active_minus1 > 0 ¦ ¦ mb_field_decoding_flag ) && sub_mb_type[ mbPartIdx ] != B_Direct_8x8 && SubMbPredMode( sub_mb_type[ mbPartIdx ] ) != Pred_L0 && !motion_prediction_flag_l1[ mbPartIdx ] ) |  |  |
| ref_idx_l1[ mbPartIdx ] | 2 | ae(v) |
| for( mbPartIdx = 0; mbPartIdx < 4; mbPartIdx++ ) |  |  |
| if( sub_mb_type[ mbPartIdx ] != B_Direct_8x8 && SubMbPredMode( sub_mb_type[ mbPartIdx ] ) != Pred_L1 ) |  |  |
| for( subMbPartIdx = 0; subMbPartIdx < NumSubMbPart( sub_mb_type[ mbPartIdx ] ); subMbPartIdx++) |  |  |
| for( compIdx = 0; compIdx < 2; compIdx++ ) |  |  |
| mvd_l0[ mbPartIdx ][ subMbPartIdx ][ compIdx ] | 2 | ae(v) |
| for( mbPartIdx = 0; mbPartIdx < 4; mbPartIdx++ ) |  |  |
| if( sub_mb_type[ mbPartIdx ] != B_Direct_8x8 && SubMbPredMode( sub_mb_type[ mbPartIdx ] ) != Pred_L0 ) |  |  |
| for( subMbPartIdx = 0; subMbPartIdx < NumSubMbPart( sub_mb_type[ mbPartIdx ] ); subMbPartIdx++) |  |  |
| for( compIdx = 0; compIdx < 2; compIdx++ ) |  |  |
| mvd_l1[ mbPartIdx ][ subMbPartIdx ][ compIdx ] | 2 | ae(v) |
| } else if( base_mode_refinement_flag ) { |  |  |
| for( mbPartIdx = 0; mbPartIdx < 4; mbPartIdx++ ) |  |  |
| if( SubMbPredMode( sub_mb_type[ mbPartIdx ] ) != Pred_L1 ) |  |  |
| for( subMbPartIdx = 0; subMbPartIdx < NumSubMbPart( sub_mb_type[ mbPartIdx ] ); subMbPartIdx++) |  |  |
| for( compIdx = 0; compIdx < 2; compIdx++ ) |  |  |
| mvd_ref_l0[ mbPartIdx ][ subMbPartIdx ][ compIdx ] | 2 | ae(v) |
| for( mbPartIdx = 0; mbPartIdx < 4; mbPartIdx++ ) |  |  |
| if( SubMbPredMode( sub_mb_type[ mbPartIdx ] ) != Pred_L0 ) |  |  |
| for( subMbPartIdx = 0; subMbPartIdx < NumSubMbPart( sub_mb_type[ mbPartIdx ] ); subMbPartIdx++) |  |  |
| for( compIdx = 0; compIdx < 2; compIdx++ ) |  |  |
| mvd_ref_l1[ mbPartIdx ][ subMbPartIdx ][ compIdx ] | 2 | ae(v) |
| } |  |  |
| } |  |  |

#### Residual in scalable extension syntax

| residual_in_scalable_extension( ) { | C | Descriptor |
| --- | --- | --- |
| if( base_id_plus1 != 0 && adaptive_prediction_flag && MbPartPredType( mb_type, 0 ) != Intra_16x16 && MbPartPredType( mb_type, 0 ) != Intra_8x8 && MbPartPredType( mb_type, 0 ) != Intra_4x4 && MbPartPredType( mb_type, 0 ) != Intra_Base ) |  |  |
| residual_prediction_flag | 3 ¦ 4 | ae(v) |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) |  |  |
| residual_block_cabac( Intra16x16DCLevel, 16 ) | 3 |  |
| for( i8x8 = 0; i8x8 < 4; i8x8++ ) /* each luma 8x8 block */ |  |  |
| if( !transform_size_8x8_flag ) |  |  |
| for( i4x4 = 0; i4x4 < 4; i4x4++ ) { /* each 4x4 sub-block of block */ |  |  |
| if( CodedBlockPatternLuma & ( 1 << i8x8 ) ) |  |  |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) |  |  |
| residual_block_cabac( Intra16x16ACLevel[i8x8 * 4 + i4x4], 15 ) | 3 |  |
| else |  |  |
| residual_block_cabac( LumaLevel[ i8x8 * 4 + i4x4 ], 16 ) | 3 ¦ 4 |  |
| else if( MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) |  |  |
| for( i = 0; i < 15; i++ ) |  |  |
| Intra16x16ACLevel[ i8x8 * 4 + i4x4 ][ i ] = 0 |  |  |
| else |  |  |
| for( i = 0; i < 16; i++ ) |  |  |
| LumaLevel[ i8x8 * 4 + i4x4 ][ i ] = 0 |  |  |
| } |  |  |
| else if( CodedBlockPatternLuma & ( 1 << i8x8 ) ) |  |  |
| residual_block_cabac( LumaLevel8x8[ i8x8 ], 64 ) | 3 ¦ 4 |  |
| else |  |  |
| for( i = 0; i < 64; i++ ) |  |  |
| LumaLevel8x8[ i8x8 ][ i ] = 0 |  |  |
| if( chroma_format_idc != 0 ) { |  |  |
| NumC8x8 = 4 / ( SubWidthC * SubHeightC ) |  |  |
| for( iCbCr = 0; iCbCr < 2; iCbCr++ ) |  |  |
| if( CodedBlockPatternChroma & 3 ) /* chroma DC residual present */ |  |  |
| residual_block_cabac( ChromaDCLevel[ iCbCr ], 4 * NumC8x8 ) | 3 ¦ 4 |  |
| else |  |  |
| for( i = 0; i < 4 * NumC8x8; i++ ) |  |  |
| ChromaDCLevel[ iCbCr ][ i ] = 0 |  |  |
| for( iCbCr = 0; iCbCr < 2; iCbCr++ ) |  |  |
| for( i8x8 = 0; i8x8 < NumC8x8; i8x8++ ) |  |  |
| for( i4x4 = 0; i4x4 < 4; i4x4++ ) |  |  |
| if( CodedBlockPatternChroma & 2 ) /* chroma AC residual present */ |  |  |
| residual_block_cabac( ChromaACLevel[ iCbCr ][ i8x8*4+i4x4 ], 15) | 3 ¦ 4 |  |
| else |  |  |
| for( i = 0; i < 15; i++ ) |  |  |
| ChromaACLevel[ iCbCr ][ i8x8*4+i4x4 ][ i ] = 0 |  |  |
| } |  |  |

### Progressive refinement slice data in scalable extension syntax

| progressive_refinement_slice_data_in_scalable_extension( ) { | C | Descriptor |
| --- | --- | --- |
| startMbAddr = first_mb_in_slice * ( 1 + MbaffFrameFlag ) |  |  |
| lastMbAddr = startMbAddr |  |  |
| for( i = 0; i < num_mbs_in_slice_minus1; i++ ) |  |  |
| lastMbAddr = NextMbAddr( lastMbAddr ) |  |  |
| CurrMbAddr = startMbAddr |  |  |
| while( CurrMbAddr <= lastMbAddr ) { /* initialisation */ |  |  |
| for( i8x8 = 0; i8x8 < 4; i8x8++ ) { |  |  |
| for( scanIdx8x8 = 0; scanIdx8x8 < 64; scanIdx8x8++ ) |  |  |
| LumaLevel8x8[ i8x8 ][ scanIdx8x8 ] = NotCoded |  |  |
| for( i4x4 = 0; i4x4 < i; i4x4++ ) { |  |  |
| coded_block_flag_luma[ 4 * i8x8 + i4x4 ] = NotCoded |  |  |
| for( scanIdx = 0; scanIdx < 16; scanIdx++ ) |  |  |
| LumaLevel[ 4*i8x8 + i4x4 ][ scanIdx ] = NotCoded |  |  |
| } |  |  |
| } |  |  |
| for( i = 0; i < 6; i++ ) |  |  |
| coded_block_pattern_bit[ i ] = NotCoded |  |  |
| mb_qp_delta = NotCoded |  |  |
| if( transform_8x8_mode_flag ) { |  |  |
| if( base_quality_mb_transform_size_specified( CurrMbAddr ) |  |  |
| transform_size_8x8_flag = base_quality_transform_size_8x8_flag( CurrMbAddr ) |  |  |
| else |  |  |
| transform_size_8x8_flag = NotCoded |  |  |
| } else |  |  |
| transform_size_8x8_flag = 0 |  |  |
| TransformSize8x8Flag = ( transform_size_8x8_flag = = NotCoded ? 0 : transform_size_8x8_flag ) |  |  |
| if( chroma_format_idc != 0 ) { |  |  |
| NumC8x8 = 4 / ( SubWidthC * SubHeightC ) |  |  |
| for( iCbCr = 0; iCbCr < 2; iCbCr++ ) { |  |  |
| coded_block_flag_chromaDC[ iCbCr ] = NotCoded |  |  |
| for( dcIdx = 0; dcIdx < 4 * NumC8x8; dcIdx++ ) |  |  |
| ChromaDCLevel[ iCbCr ][ dcIdx ] = NotCoded |  |  |
| for( i8x8 = 0; i8x8 < NumC8x8; i8x8++ ) |  |  |
| for( i4x4 = 0; i4x4 < 4; i4x4++ ) { |  |  |
| coded_block_flag_chromaAC[ iCbCr ][4 * i8x8 + i4x4] = NotCoded |  |  |
| for( scanIdx = 0; scanIdx < 15; scanIdx++ ) |  |  |
| ChromaACLevel[iCbCr ][ 4 * i8x8 + i4x4 ][ scanIdx ] = NotCoded |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| CurrMbAddr = NextMbAddress( CurrMbAddr ) |  |  |
| } |  |  |
| scanIdxLuma = 0 |  |  |
| scanIdxChromaDC = 0 |  |  |
| scanIdxChromaAC = 1 |  |  |
| refPassLumaFlag = 0 |  |  |
| refPassChromaDCFlag = 0 |  |  |
| refPassChromaACFlag = 0 |  |  |
| cycleNum = 0 |  |  |
| chromaStartCycle = 1 |  |  |
| while ( scanIdxLuma < 16 ¦ ¦ scanIdxChromaDC < 4 ¦ ¦ scanIdxChromaAC < 16 ) { |  |  |
| allowChromaDCFlag = ( cycleNum = = 0 ) ¦ ¦ ( ( cycleNum >= chromaStartCycle ) && ( ( chromaStartCycle – cycleNum ) % 2 = = 0 ) ) |  |  |
| allowChromaACFlag = ( cycleNum > 0 ) && ( ( cycleNum = = chromaStartCycle ) ¦ ¦ ( ( cycleNum >= chromaStartCycle ) && ( ( cycleNum – chromaStartCycle ) % 3 = = 1 ) ) ) |  |  |
| sigCompleteLumaFlag = 1 |  |  |
| sigCompleteChromaDCFlag = allowChromaDCFlag |  |  |
| sigCompleteChromaACFlag = allowChromaACFlag |  |  |
| CurrMbAddr = startMbAddr |  |  |
| while( CurrMbAddr <= lastMbAddr ) { |  |  |
| for( blkIdx4x4 = 0; blkIdx4x4 < 16; blkIdx4x4++ ) { /* luma */ |  |  |
| if( !refPassLumaFlag ) |  |  |
| luma_coefficient( blkIdx4x4, sigCompleteLumaFlag ) | 3 ¦ 4 |  |
| else if ( scanIdxLuma < 16 ) |  |  |
| luma_coefficient_ref( blkIdx4x4, scanIdx ) | 3 ¦ 4 |  |
| } |  |  |
| if( chroma_format_idc != 0 && ( luma_chroma_sep_flag = = 0 ¦ ¦ scanIdxLuma = = 16 ) ) { |  |  |
| NumC8x8 = 4 / ( SubWidthC * SubHeightC ) |  |  |
| if( allowChromaDCFlag ) /* chromaDC */ |  |  |
| for( iCbCr = 0; iCbCr < 2; iCbCr++ ) { |  |  |
| if ( !refPassChromaDCFlag ) |  |  |
| chromaDC_coefficient( iCbCr, sigCompleteChromaDCFlag ) | 3 ¦ 4 |  |
| else if( scanIdxChromaDC < 4 ) |  |  |
| chromaDC_coefficient_ref( iCbCr, dcIdx ) | 3 ¦ 4 |  |
| } |  |  |
| if( allowChromaACFlag ) /* chromaAC */ |  |  |
| for( iCbCr = 0; iCbCr < 2; iCbCr++ ) |  |  |
| for( blkIdx4x4 = 0; blkIdx4x4 < 4*NumC8x8; blkIdx4x4++ ) { |  |  |
| if ( !refPassChromaACFlag ) |  |  |
| chromaAC_coefficient( iCbCr, blkIdx4x4, sigCompleteChromaACFlag ) | 3 ¦ 4 |  |
| else if ( scanIdxChromaAC < 16 ) |  |  |
| chromaAC_coefficient_ref( iCbCr, blkIdx4x4, scanIdx ) | 3 ¦ 4 |  |
| } |  |  |
| } |  |  |
| CurrMbAddr = NextMbAddr( CurrMbAddr ) |  |  |
| } |  |  |
| if( refPassLumaFlag ) |  |  |
| scanIdxLuma++ |  |  |
| if( refPassChromaDCFlag && allowChromaDCFlag ) |  |  |
| scanIdxChromaDC++ |  |  |
| if( refPassChromaACFlag && allowChromaACFlag ) |  |  |
| scanIdxChromaAC++ |  |  |
| if( cycleNum = = 0 && luma_chroma_sep_flag = = 0 ) { |  |  |
| cycle_start_symbol | 3 ¦ 4 | ae(v) |
| if( cycle_start_symbol = = 1 ) { |  |  |
| cycle_start_symbol_ext | 3 ¦ 4 | ae(v) |
| chromaStartCycle = 2 + cycle_start_symbol_ext |  |  |
| } |  |  |
| } |  |  |
| refPassLumaFlag ¦= sigCompleteLumaFlag |  |  |
| refPassChromaDCFlag ¦= sigCompleteChromaDCFlag |  |  |
| refPassChromaACFlag ¦= sigCompleteChromaACFlag |  |  |
| cycleNum++ |  |  |
| } |  |  |
| if( moreCabacDataInNALUnit( ) ) |  |  |
| end_of_progressive_refinement_slice_flag | 2 | ae(v) |
| } |  |  |

#### Luma coefficient in progressive refinement slice data syntax

| luma_coefficient( blkIdx4x4, endCyclesFlag ) { | C | Descriptor |
| --- | --- | --- |
| i8x8 = blkIdx4x4 / 4 |  |  |
| i4x4 = blkIdx4x4 % 4 |  |  |
| if( moreCabacDataInNALUnit( ) && coded_block_pattern_bit[ i8x8 ] = = NotCoded ) { |  |  |
| coded_block_pattern_bit[ i8x8 ] | 3 ¦ 4 | ae(v) |
| if( coded_block_pattern_bit[ i8x8 ] = = 0 ) { |  |  |
| for( j = 0; j < 4; j++ ) |  |  |
| for( i = 0; i < 16; i++ ) |  |  |
| LumaLevel[ 4 * i8x8 + j ][ i ] = 0 |  |  |
| for( i = 0; i < 64; i++ ) |  |  |
| LumaLevel8x8[ i8x8 ][ i ] = 0 |  |  |
| } else { |  |  |
| if( moreCabacDataInNALUnit( ) && ! base_mb_sig( CurrMbAddr ) && mb_qp_delta = = NotCoded ) |  |  |
| mb_qp_delta | 2 | ae(v) |
| if( moreCabacDataInNALUnit( ) && transform_size_8x8_flag = = NotCoded ) { |  |  |
| transform_size_8x8_flag | 2 | ae(v) |
| TransformSize8x8Flag = transform_size_8x8_flag |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| if( coded_block_pattern_bit[ i8x8 ] = = 1 ) { |  |  |
| if( ! TranformSize8x8Flag ) { |  |  |
| if( moreCabacDataInNALUnit( ) && coded_block_flag_luma[ blkIdx4x4 ] = = NotCoded ) { |  |  |
| coded_block_flag_luma[ blkIdx4x4 ] | 3 ¦ 4 | ae(v) |
| if( coded_block_flag_luma[ blkIdx4x4 ] = = 0 ) |  |  |
| for( i = 0; i < 16; i++ ) |  |  |
| LumaLevel[ blkIdx4x4 ][ i ] = 0 |  |  |
| } |  |  |
| if( coded_block_flag_luma[ blkIdx4x4 ] = = 1 ) { |  |  |
| scanIdx = 16 |  |  |
| scanIdxStart = 16 |  |  |
| for( idx = 0; idx < 16; idx++ ) { |  |  |
| if( scanIdx = = 16 && LumaLevel[ blkIdx4x4 ][ idx ] = = NotCoded && base_luma_level( CurrMbAddr, i8x8, i4x4, idx ) = = 0 ) |  |  |
| scanIdx = idx |  |  |
| if( scanIdxStart = = 16 && base_luma_level( CurrMbAddr, i8x8, i4x4, idx ) = = 0 ) |  |  |
| scanIdxStart = idx |  |  |
| } |  |  |
| if( scanIdx != 16 ) |  |  |
| endCyclesFlag = 0 |  |  |
| if( scanIdx > scanIdxStart && moreCabacDataInNALUnit( ) ) { |  |  |
| last_significant_coeff_flag | 3 ¦ 4 | ae(v) |
| if( last_significant_coeff_flag ) { |  |  |
| for( i = scanIdx; i < 16; i++ ) |  |  |
| LumaLevel[ blkIdx4x4 ][ i ] = 0 |  |  |
| scanIdx = 16 |  |  |
| } |  |  |
| } |  |  |
| while( scanIdx < 16 ) { |  |  |
| if( moreCabacDataInNALUnit( ) ) |  |  |
| significant_coeff_flag | 3 ¦ 4 | ae(v) |
| if( significant_coeff_flag ) { |  |  |
| if( moreCabacDataInNALUnit( ) ) |  |  |
| coeff_sign_flag | 3 ¦ 4 | ae(v) |
| if( moreCabacDataInNALUnit( ) ) { |  |  |
| coeff_abs_level_minus1 | 3 ¦ 4 | ae(v) |
| LumaLevel[ blkIdx4x4 ][ scanIdx ] = |  |  |
| (coeff_abs_level_minus1 + 1) * (1 – 2 * coeff_sign_flag) |  |  |
| } |  |  |
| scanIdx = 16 |  |  |
| } else { |  |  |
| LumaLevel[ blkIdxi4x4 ][ scanIdx ] = 0 |  |  |
| scanIdx++ |  |  |
| while( scanIdx < 16 && base_luma_level( CurrMbAddr, i8x8, i4x4, scanIdx ) = = 1 ) |  |  |
| scanIdx++ |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } else { |  |  |
| scanIdx8x8 = 64 |  |  |
| scanIdxStart8x8 = 64 |  |  |
| for( idx8x8 = 0; idx8x8 < 64; idx8x8++ ) { |  |  |
| if( scanIdx8x8 = = 64 && LumaLevel8x8[ i8x8 ][ idx8x8 ] = = NotCoded && base_luma_level8x8( CurrMbAddr, i8x8, idx8x8 ) = = 0 ) |  |  |
| scanIdx8x8 = idx8x8 |  |  |
| if( scanIdxStart8x8 = = 64 && base_luma_level8x8( CurrMbAddr, i8x8, idx8x8 ) = = 0 ) |  |  |
| scanIdxStart8x8 = idx8x8 |  |  |
| } |  |  |
| if( scanIdx8x8 > scanIdxStart8x8 && moreCabacDataInNALUnit( ) ) |  |  |
| last_significant_coeff_flag | 3 ¦ 4 | ae(v) |
| if( last_significant_coeff_flag ) { |  |  |
| for( i = scanIdx8x8; i < 64; i++ ) |  |  |
| LumaLevel8x8[ i8x8 ][ i ] = 0 |  |  |
| scanIdx8x8 = 64 |  |  |
| } |  |  |
| } |  |  |
| while( scanIdx8x8 < 64 ) { |  |  |
| if( moreCabacDataInNALUnit( ) ) |  |  |
| significant_coeff_flag | 3 ¦ 4 | ae(v) |
| if( significant_coeff_flag ) { |  |  |
| if( moreCabacDataInNALUnit( ) ) |  |  |
| coeff_sign_flag | 3 ¦ 4 | ae(v) |
| if( moreCabacDataInNALUnit( ) ) { |  |  |
| coeff_abs_level_minus1 | 3 ¦ 4 | ae(v) |
| LumaLevel[ i8x8 ][ scanIdx8x8 ] = |  |  |
| (coeff_abs_level_minus1 + 1) * (1 – 2 * coeff_sign_flag) |  |  |
| } |  |  |
| scanIdx = 64 |  |  |
| } else { |  |  |
| LumaLevel8x8[ i8x8 ][ scanIdx8x8 ] = 0 |  |  |
| scanIdx8x8++ |  |  |
| while( scanIdx8x8 < 64 && base_luma_level8x8( CurrMbAddr, i8x8, scanIdx8x8 ) ) { |  |  |
| scanIdx8x8++ |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

#### Chroma DC coefficient in progresssive slice data syntax

| chromaDC_coefficient( iCbCr, endCyclesFlag ) { | C | Descriptor |
| --- | --- | --- |
| NumC8x8 = 4 / ( SubWidthC * SubHeightC ) |  |  |
| dcIdx = 4 |  |  |
| dcIdxStart = 4 |  |  |
| for( idx = 0; idx < 4; idx++ ) { |  |  |
| if( dcIdx = = 4 && ChromaDCLevel[ iCbCr ][ idx ] = = NotCoded && base_chromaDC_level( CurrMbAddr, iCbCr, idx ) = = 0 ) |  |  |
| dcIdx = idx |  |  |
| if( dcIdxStart = = 4 && base_chromaDC_level( CurrMbAddr, iCbCr, idx ) = = 0 ) |  |  |
| dcIdxStart = idx |  |  |
| } |  |  |
| if( dcIdx != 4 ) |  |  |
| endCyclesFlag = 0 |  |  |
| if( moreCabacDataInNALUnit( ) && coded_block_pattern_bit[ 4 ] = = NotCoded ) { |  |  |
| coded_block_pattern_bit[ 4 ] | 3 ¦ 4 | ae(v) |
| if( coded_block_pattern_bit[ 4 ] = = 0 ) { |  |  |
| for( j = 0; j < 2; j++ ) { |  |  |
| for( i = 0; i < 4 * NumC8x8; i++ ) |  |  |
| ChromaDCLevel[ j ][ i ] = 0 |  |  |
| for( i8x8 = 0; i8x8 < NumC8x8; i8x8++ ) |  |  |
| for( i4x4 = 0; i4x4 < 4; i4x4++ ) |  |  |
| for( scanIdx = 0; scanIdx < 15; scanIdx++ ) |  |  |
| ChromaACLevel[ j ][ 4 * i8x8 + i4x4 ][ scanIdx ] = 0 |  |  |
| } |  |  |
| } else { |  |  |
| if( moreCabacDataInNALUnit( ) && ! base_mb_sig( CurrMbAddr ) && mb_qp_delta = = NotCoded ) |  |  |
| mb_qp_delta | 2 | ae(v) |
| } |  |  |
| } |  |  |
| if( coded_block_pattern_bit[ 4 ] = = 1 ) { |  |  |
| if( moreCabacDataInNALUnit( ) && coded_block_flag_chromaDC[ iCbCr ] = = NotCoded ) { |  |  |
| coded_block_flag_chromaDC[ iCbCr ] | 3 ¦ 4 | ae(v) |
| if( coded_block_flag_chromaDC[ iCbCr ] == 0 ) |  |  |
| for( i = 0; i < 4 * NumC8x8; i++ ) |  |  |
| ChromaDCLevel[ iCbCr ][ i ] = 0 |  |  |
| } |  |  |
| if( coded_block_flag_chromaDC[ iCbCr ] = = 1 ) { |  |  |
| if( dcIdx > dcIdxStart && moreCabacDataInNALUnit( ) ) { |  |  |
| last_significant_coeff_flag | 3 ¦ 4 | ae(v) |
| if( last_significant_coeff_flag ) { |  |  |
| for( i = dcIdx; i < 4 * NumC8x8; i++ ) |  |  |
| ChromaDCLevel[ iCbCr ][ i ] = 0 |  |  |
| dcIdx = 4 |  |  |
| } |  |  |
| } |  |  |
| while( dcIdx < 4 ) { |  |  |
| if( moreCabacDataInNALUnit( ) ) |  |  |
| significant_coeff_flag | 3 ¦ 4 | ae(v) |
| if( significant_coeff_flag ) { |  |  |
| if( moreCabacDataInNALUnit( ) ) |  |  |
| coeff_sign_flag | 3 ¦ 4 | ae(v) |
| if( moreCabacDataInNALUnit( ) ) { |  |  |
| coeff_abs_level_minus1 | 3 ¦ 4 | ae(v) |
| ChromaDCLevel[ iCbCr ] [ dcIdx ] = |  |  |
| ( coeff_abs_level_minus1 + 1 ) * ( 1 – 2 * coeff_sign_flag ) |  |  |
| } |  |  |
| dcIdx++ |  |  |
| } else { |  |  |
| ChromaDCLevel[ iCbCr ][ dcIdx ] = 0 |  |  |
| dcIdx++ |  |  |
| while( dcIdx < 4 && base_chromaDC_level( CurrMbAddr, iCbCr, dcIdx ) = = 1 ) |  |  |
| dcIdx++ |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

#### Chroma AC coefficient in progressive refinement slice data syntax

| chromaAC_coefficient( iCbCr, blkIdx4x4, endCyclesFlag ) { | C | Descriptor |
| --- | --- | --- |
| NumC8x8 = 4 / ( SubWidthC * SubHeightC ) |  |  |
| i8x8 = blkIdx4x4 / 4 |  |  |
| i4x4 = blkIdx4x4 % 4 |  |  |
| scanIdx = 16 |  |  |
| scanIdxStart = 16 |  |  |
| for( idx = 0; idx < 16; idx++ ) { |  |  |
| if( scanIdx == 16 && ChromaACLevel[ iCbCr ][ blkIdx4x4 ][ idx ] == NotCoded && base_chromaAC_level( CurrMbAddr, iCbCr, i8x8, i4x4, idx ) = = 0 ) |  |  |
| scanIdx = idx |  |  |
| if( scanIdxStart == 16 && base_chromaAC_level( CurrMbAddr, iCbCr, i8x8, i4x4, idx ) = = 0) |  |  |
| scanIdxStart = idx |  |  |
| } |  |  |
| if( scanIdx != 16 ) |  |  |
| endCyclesFlag = 0 |  |  |
| if( moreCabacDataInNALUnit( ) && coded_block_pattern_bit[ 4 ] = = NotCoded ) { |  |  |
| coded_block_pattern_bit[ 4 ] | 3 ¦ 4 | ae(v) |
| if( coded_block_pattern_bit[ 4 ] = = 0 ) { |  |  |
| for( j = 0; j < 2; j++ ) { |  |  |
| for( i = 0; i < 4 * NumC8x8; i++ ) |  |  |
| ChromaDCLevel[ j ][ i ] = 0 |  |  |
| for( i = 0; i < NumC8x8; i++ ) |  |  |
| for( k = 0; k < 4; k++ ) |  |  |
| for( l = 0; l < 15; l++ ) |  |  |
| ChromaACLevel[ j ][ 4 * i + k ][ l ] = 0 |  |  |
| } |  |  |
| } else { |  |  |
| if( moreCabacDataInNALUnit( ) && ! base_mb_sig( CurrMbAddr ) && mb_qp_delta = = NotCoded ) |  |  |
| mb_qp_delta |  |  |
| } |  |  |
| } |  |  |
| if( coded_block_pattern_bit[ 4 ] = = 1 && moreCabacDataInNALUnit( ) && coded_block_pattern_bit[ 5 ] = = NotCoded ) { |  |  |
| coded_block_pattern_bit[ 5 ] | 3 ¦ 4 | ae(v) |
| if( coded_block_pattern_bit[ 5 ] = = 0 ) { |  |  |
| for( j = 0; j < 2; j++ ) |  |  |
| for( i = 0; i < NumC8x8; i++ ) |  |  |
| for( k = 0; k < 4; k++ ) |  |  |
| for( l = 0; l < 15; l++ ) |  |  |
| ChromaACLevel[ j ][ 4 * i + k ][ l ] = 0 |  |  |
| } |  |  |
| } |  |  |
| if( coded_block_pattern_bit[ 5 ] = = 1 ) { |  |  |
| if( moreCabacDataInNALUnit( ) && coded_block_flag_chromaAC[ iCbCr ][ blkIdx4x4] = = NotCoded ) { |  |  |
| coded_block_flag_chromaAC[ iCbCr ][ blkIdx4x4 ] | 3 ¦ 4 | ae(v) |
| if( coded_block_flag_chromaAC[ iCbCr ][ blkIdx4x4 ] = = 0 ) |  |  |
| for( k = 0; k < 15; k++ ) |  |  |
| ChromaACLevel[ iCbCr ][ blkIdx4x4 ][ k ] = 0 |  |  |
| } |  |  |
| if( coded_block_flag_chromaAC[ iCbCr ][ blkIdx4x4 ] = = 1 ) { |  |  |
| if( scanIdx > scanIdxStart && moreCabacDataInNALUnit( ) ) { |  |  |
| last_significant_coeff_flag | 3 ¦ 4 | ae(v) |
| if( last_significant_coeff_flag ) { |  |  |
| for( i = scanIdx; i < 15; i++ ) |  |  |
| ChromaACLevel[ iCbCr ][ blkIdx4x4 ][ i ] = 0 |  |  |
| scanIdx = 16 |  |  |
| } |  |  |
| } |  |  |
| while( scanIdx < 16 ) { |  |  |
| if( moreCabacDataInNALUnit( ) ) |  |  |
| significant_coeff_flag | 3 ¦ 4 | ae(v) |
| if( significant_coeff_flag ) { |  |  |
| if( moreCabacDataInNALUnit( ) ) |  |  |
| coeff_sign_flag | 3 ¦ 4 | ae(v) |
| if( moreCabacDataInNALUnit( ) ) { |  |  |
| coeff_abs_level_minus1 | 3 ¦ 4 | ae(v) |
| ChromaACLevel[ iCbCr ][ blkIdx4x4 ][ scanIdx ] = |  |  |
| (coeff_abs_level_minus1 + 1) * (1 – 2 * coeff_sign_flag) |  |  |
| } |  |  |
| scanIdx = 16 |  |  |
| } else { |  |  |
| ChromaACLevel[ iCbCr ][ blkIdx4x4 ][ scanIdx ] = 0 |  |  |
| scanIdx++ |  |  |
| while( scanIdx < 16 && base_chromaAC_level( CurrMbAddr, iCbCr, i8x8, 4x4, scanIdx ) = = 1 ) |  |  |
| scanIdx++ |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

#### Luma coefficient refinement in progressive refinement slice data syntax

| luma_coefficient_ref( blkIdx4x4, scanIdx ) { | C | Descriptor |
| --- | --- | --- |
| i8x8 = blkIdx4x4 / 4 |  |  |
| i4x4 = blkIdx4x4 % 4 |  |  |
| scanIdx8x8 = 4 * scanIdx + i4x4 |  |  |
| if( ( ! TransformSize8x8Flag && base_luma_level( CurrMbAddr, i8x8, i4x4, scanIdx ) != 0 ) ¦ ¦ ( TransformSize8x8Flag && base_luma_level8x8( CurrMbAddr, i8x8, scanIdx8x8 ) != 0 ) ) { |  |  |
| coeffLevel = 0 |  |  |
| if( ! TransformSize8x8Flag ) |  |  |
| bLSign = ( base_luma_level( CurrMbAddr, i8x8, i4x4, scanIdx ) < 0 ? 1 : 0 ) |  |  |
| else |  |  |
| bLSign = ( base_luma_level8x8( CurrMbAddr, i8x8, scanIdx8x8 ) < 0 ? 1 : 0 ) |  |  |
| if( moreCabacDataInNALUnit( ) ) |  |  |
| coeff_refinement_flag | 3 ¦ 4 | ae(v) |
| if( coeff_refinement_flag ) { |  |  |
| if( moreCabacDataInNALUnit( ) ) { |  |  |
| coeff_refinement_direction_flag | 3 ¦ 4 | ae(v) |
| coeffLevel = ( ( coeff_refinement_direction_flag ^ bLSign ) ? –1 : 1 ) |  |  |
| } |  |  |
| } |  |  |
| if( ! TransformSize8x8Flag ) |  |  |
| LumaLevel[ blkIdx4x4 ][ scanIdx ] = coeffLevel |  |  |
| else |  |  |
| LumaLevel8x8[ i8x8 ][ scanIdx8x8 ] = coeffLevel |  |  |
| } |  |  |
| } |  |  |

#### Chroma DC coefficient refinement in progressive refinement slice data syntax

| chromaDC_coefficient_ref( iCbCr, dcIdx ) { | C | Descriptor |
| --- | --- | --- |
| if( base_chromaDC_level( CurrMbAddr, iCbCr, dcIdx ) ) { |  |  |
| if( moreCabacDataInNALUnit( ) ) |  |  |
| coeff_refinement_flag | 3 ¦ 4 | ae(v) |
| if( coeff_refinement_flag ) { |  |  |
| if( moreCabacDataInNALUnit( ) ) { |  |  |
| coeff_refinement_direction_flag | 3 ¦ 4 | ae(v) |
| bLSign = ( base_chromaDC_level( CurrMbAddr, iCbCr, dcIdx ) < 0 ? 1 : 0 ) |  |  |
| ChromaDCLevel[ iCbCr ][ dcIdx ] = ( ( coeff_refinement_direction_flag ^ bLSign ) ? –1 : 1 ) |  |  |
| } |  |  |
| } else |  |  |
| ChromaDCLevel[ iCbCr ][ dcIdx ] = 0 |  |  |
| } |  |  |
| } |  |  |

#### Chroma AC coefficient refinement in progressive refinement slice data syntax

| chromaAC_coefficient_ref( iCbCr, blkIdx4x4, scanIdx ) { | C | Descriptor |
| --- | --- | --- |
| NumC8x8 = 4 / ( SubWidthC * SubHeightC ) |  |  |
| i8x8 = blkIdx4x4 / 4 |  |  |
| i4x4 = blkIdx4x4 % 4 |  |  |
| if( base_chromaAC_level( CurrMbAddr, iCbCr, i8x8, i4x4, scanIdx ) != 0 ) { |  |  |
| if( moreCabacDataInNALUnit( ) ) |  |  |
| coeff_refinement_flag | 3 ¦ 4 | ae(v) |
| if( coeff_refinement_flag ) { |  |  |
| if( moreCabacDataInNALUnit( ) ) |  |  |
| coeff_refinement_direction_flag | 3 ¦ 4 | ae(v) |
| bLSign = (base_chromaAC_level( CurrMbAddr, iCbCr, i8x8, i4x4, scanIdx ) < 0 ? 1 : 0 ) |  |  |
| ChromaACLevel[ iCbCr ][ blkIdx4x4 ][ scanIdx ] = ( ( coeff_refinement_direction_flag ^ bLSign ) ? –1 : 1 ) |  |  |
| } |  |  |
| } else |  |  |
| ChromaACLevel[ iCbCr ][ blkIdx4x4 ][ dcIdx ] = 0 |  |  |
| } |  |  |
| } |  |  |

## Semantics

### NAL unit semantics

The specification of this subclause in AVC shall apply with the following modifications.

1) Replace the following paragraph of this subclause in AVC

nal_unit_type specifies the type of RBSP data structure contained in the NAL unit as specified in Table 7-1.  VCL NAL units are specified as those NAL units having nal_unit_type equal to 1 to 5, inclusive. All remaining NAL units are called non-VCL NAL units.

with the following

nal_unit_type specifies the type of RBSP data structure contained in the NAL unit as specified in Table 7-1.  VCL NAL units are specified as those NAL units having nal_unit_type equal to 1 to 5, inclusive, or equal to 20 to 21, inclusive.  All remaining NAL units are called non-VCL NAL units.

2) Replace Table 7-1 of this subclause in AVC with the following table.

| nal_unit_type | Content of NAL unit and RBSP syntax structure | C |
| --- | --- | --- |
| 0 | Unspecified |  |
| 1 | Coded slice of a non-IDR picture slice_layer_without_partitioning_rbsp( ) | 2, 3, 4 |
| 2 | Coded slice data partition A slice_data_partition_a_layer_rbsp( ) | 2 |
| 3 | Coded slice data partition B slice_data_partition_b_layer_rbsp( ) | 3 |
| 4 | Coded slice data partition C slice_data_partition_c_layer_rbsp( ) | 4 |
| 5 | Coded slice of an IDR picture slice_layer_without_partitioning_rbsp( ) | 2, 3 |
| 6 | Supplemental enhancement information (SEI) sei_rbsp( ) | 5 |
| 7 | Sequence parameter set seq_parameter_set_rbsp( ) | 0 |
| 8 | Picture parameter set pic_parameter_set_rbsp( ) | 1 |
| 9 | Access unit delimiter access_unit_delimiter_rbsp( ) | 6 |
| 10 | End of sequence end_of_seq_rbsp( ) | 7 |
| 11 | End of stream end_of_stream_rbsp( ) | 8 |
| 12 | Filler data filler_data_rbsp( ) | 9 |
| 13 | Sequence parameter set extension seq_parameter_set_extension_rbsp( ) | 10 |
| 14..18 | Reserved |  |
| 19 | Coded slice of an auxiliary coded picture without partitioning slice_layer_without_partitioning_rbsp( ) | 2, 3, 4 |
| 20 | Coded slice of a non-IDR picture in scalable extension slice_layer_in_scalable_extension_rbsp( ) | 2, 3, 4 |
| 21 | Coded slice of an IDR picture in scalable extension slice_layer_in_scalable_extension_rbsp( ) | 2, 3 |
| 22..23 | Reserved |  |
| 24..31 | Unspecified |  |

3) Insert the following paragraphs before “rbsp_byte[ i ] …” of this subclause in AVC:

When the value of nal_unit_type is equal to 21 for a NAL unit containing a slice of a coded picture, the value of nal_unit_type shall be 21 in all other VCL NAL units of the same coded picture. Such a picture is referred to as an IDR picture in scalable extension.

Either all pictures with an identical value of picture order count but different values of dependency_id or quality_level, are coded as IDR pictures, or no picture for a specific value of picture order count is coded as IDR picture.

simple_priority_id  specifies a priority identifier for the NAL unit. When extension_flag is equal to 0, simple_priority_id is used for inferring the values of dependency_id, temporal_level, and quality_level. When simple_priority_id is not present, it shall be inferred to be equal to 0.

NOTE – When extension_flag is equal to 1, simple_priority_id is not used by the decoding process specified in this Recommendation | International Standard; when extension_flag is equal to 0, it is only used for inferring the values of dependency_id, temporal_level, and quality_level. The syntax element simple_priority_id may be used as determined by the application.

discardable_flag  equal to 1 specifies that the content of the NAL unit (currDependencyId = dependency_id) is not used in the decoding process of NAL units with dependency_id > currDependencyId.  discardable_flag equal to 0 indicates that the content of the NAL unit (currDependencyId = dependency_id) is used in the decoding process of NAL units with dependency_id > currDependencyId. [Ed. Note(SP/HS): Currently this flag is not required by the decoding process, it represents mainly high-level information for applications. The discardable feature shall also be reflected by the syntax element base_id_plus1. The discardable_flag may be used in future versions of the JSVM to indicate discardability of fragemented NAL units for progressive refinement slices in order to support arbitrary rate points for inter-layer prediction.]

extension_flag  equal to 1 indicates that the syntax elements dependency_id, temporal_level, and quality_level are present in the NAL unit.

dependency_id  specifies a dependency identifier for the current picture. When dependency_id is not present, it shall be inferred to be equal to dependency_id_list[ simple_priority_id ]. The dependency_id is used in the decoding process for picture order count (subclause S.9.2.1), the decoding process for reference lists (subclause S.9.2.4), the decoded reference picture marking process (subclause S.9.2.5), the derivation process for update picture lists (subclause S.9.2.6), and for identifying base pictures that are used for inter-layer prediction of motion and/or texture data.

temporal_level  specifies a temporal level for the current picture. When temporal_level is not present, it shall be inferred to be equal to temporal_level_list[ simple_priority_id ]. The temporal_level is used in the decoding process for reference lists (subclause S.9.2.4) and the derivation process for update picture lists (subclause S.9.2.6).

quality_level  specifies a quality level for the current NAL unit. When quality_level is not present, it shall be inferred to be equal to quality_level_list[ simple_priority_id ]. The quality_level is used in connection with the end_of_progressive_refinement_slice_flag of previous NAL units in decoding order for determining whether a NAL unit containing a PR slice can be decoded.

#### Encapsulation of an SODB within an RBSP (informative)

The specification of this subclause in AVC shall apply.

#### Order of NAL units and association to coded pictures, access units, and video sequences

The specification of this subclause in AVC shall apply.

        - Order of sequence and picture parameter set RBSPs and their activation

The specification of this subclause in AVC shall apply with the following modifications..

1) Insert the following at the beginning of this subclause.

The processes and constraints described in this subclause apply only for the NAL units with an identical value of dependency_id. [Ed. Note(YKW/HS): The text about the activation of picture parameter sets need to be checked and possibly modified.]

- NOTE – More than one sequence parameter set RBSP or picture parameter set RBSP may be considered active at any given moment during the operation of the decoding process.

chroma_format_idc shall be identical for all activated sequence parameter sets (for all values of dependency_id).

        - Order of access units and association to coded video sequences

The specification of this subclause in AVC shall apply with the following modifications.

1) Replace the following paragraph of this subclause in AVC

The values of picture order count for the coded pictures in consecutive access units in decoding order containing non-reference pictures shall be non-decreasing.

with the following.

The values of picture order count for the coded pictures with identical values of dependency_id and quality_level in consecutive access units in decoding order and containing non-reference pictures shall be non-decreasing.

        - Order of NAL units and coded pictures and association to access units

The specification of this subclause in AVC shall apply with the following modifications.

[Ed. Note(HS/YKW): This subclause needs to be carefully checked and the necessary restrictions for scalable bitstreams need to be analyzed.  It is likely that defintion of a sub-access-unit conceptually same as access unit in AVC is needed to clearly specify the order of NAL units and coded pictures and association to sub-access-unit and the order of sub-access-units and association to access units. Here are some first rough ideas:

1) Replace the first sentence (copied below) this subclause in AVC

An access unit consists of one primary coded picture, zero or more corresponding redundant coded pictures, and zero or more non-VCL NAL units.

with the following .

An access unit consists of one or more primary coded pictures, zero or more corresponding redundant coded pictures with the same value of picture order count, and zero or more non-VCL NAL units.

- Progressive refinement NAL units with quality_level equal to qualityLevel shall immediately follow the NAL units with identical values of picture order count and dependency_id, and with quality_level equal to (qualityLevel – 1).

- The NAL units with dependency_id equal to dependencyId shall immediately follow the NAL units with an identical value of picture order count and with dependency_id equal to or less than (dependencyId – 1).

]

        - Detection of the first VCL NAL unit of a primary coded picture

The specification of this subclause in AVC shall apply with the following modifications.

[Ed. Note(YKW): This subclause needs to be carefully checked and the necessary restrictions for scalable bitstreams need to be analyzed. Here are some initial ideas.

1) Replace the following paragraph of this subclause in AVC

- Any coded slice NAL unit or coded slice data partition A NAL unit of the primary coded picture of the current access unit shall be different from any coded slice NAL unit or coded slice data partition A NAL unit of the primary coded picture of the previous access unit in one or more of the following ways.

with the following

- Any coded slice NAL unit or coded slice data partition A NAL unit of a primary coded picture of the current access unit shall be different from any coded slice NAL unit or coded slice data partition A NAL unit of another primary coded picture in the same or previous access unit in one or more of the following ways.

The following bullet items could be added after the last bullet item in this subclasue in AVC:

- dependency_id differs in value

- temporal_level differs in value

- quality_level differs in value

- nal_unit_type differs in value with one of the nal_unit_type values being equal to 21

- nal_unit_type is equal to 21 for both and idr_pic_id differs in value

]

##### Order of VCL NAL units and association to coded pictures

The specification of this subclause in AVC shall apply with the following modifications.

1) Replace the following paragraph of this subclause in AVC

NAL units having nal_unit_type in the range of 20 to 23, inclusive, which are reserved, shall not precede the first VCL NAL unit of the primary coded picture within the access unit (when specified in the future by ITU-T | ISO/IEC).

with the following

NAL units having nal_unit_type in the range of 22 to 23, inclusive, which are reserved, shall not precede the first VCL NAL unit of the primary coded picture within the access unit (when specified in the future by ITU-T | ISO/IEC).

### Raw byte sequence payloads and RBSP trailing bits semantics

#### Sequence parameter set RBSP semantics

The specification of this subclause in AVC shall apply with the following modifications.

[Ed. Note(YKW): The “bitstream” mentioned in the semantics of sequence parameter set parameters refers to the bitstream within a coded video sequence consisting of all the NAL units of the scalable layer that refers to the sequence parameter set and all the NAL units of the (required?) lower scalable layers. This needs to be refined.]

1) Insert the following before the paragraph starting with “chroma_format_idc  specifies ..”.

nal_unit_extension_flag  equal to 0 specifies that the parameters that specify the mapping of simple_priority_id to (dependency_id, temporal_level, quality_id) follow next in the sequence parameter set. nal_unit_extension_flag equal to 1 specifies that the parameters that specify the mapping of simple_priority_id to (dependency_id, temporal_level, quality_level) are not present.  When nal_unit_extension_flag is not present, it shall be inferred to be equal to 1.

The NAL unit syntax element extension_flag of all NAL units with nal_unit_type equal to 20 and 21 that reference the current sequence parameter set shall be equal to nal_unit_extension_flag.

NOTE – When profile_idc is not equal to 83, the syntax element extension_flag of all NAL units with nal_unit_type equal to 20 and 21 that reference the current sequence parameter set shall be equal to 1.

number_of_simple_priority_id_values_minus1  plus 1 specifies the number of values for simple_priority_id, for which a mapping to (dependency_id, temporal_level, quality_level) is specified by the parameters that follow next in the sequence parameter set. The value of number_of_simple_priority_id_values_minus1 shall be in the range of 0 to 63, inclusive.

priority_id, dependency_id_list[ priority_id ], temporal_level_list[ priority_id ], quality_level_list[ priority_id ]  specify the inferring process for the syntax elements dependency_id, temporal_level, and quality_level as specified in subclause S.8.4.1.

For all values of priority_id, for which dependency_list[ priority_id ], temporal_level_list[ priority_id ], and quality_level_list[ priority_id ] are not present, dependency_list[ priority_id ], temporal_level_list[ priority_id ], and quality_level_list[ priority_id ] shall be inferred to be equal to 0.

low_complexity_update_flag specifies the method used for the MCTF update step. If low_complexity_update_flag is equal to 1, a low-complexity motion-compensated update process is employed; otherwise (low_complexity_update_flag is equal to 0), a higher-complexity motion-compensated update is performed. [Ed. Note(HS): It is very likely that one of the update methods will be removed in the future.]

1) Insert the following before the paragraph starting with “vui_parameters_present_flag equal to 1 specifies ..”..

extended_spatial_scalability specifies the presence of syntax elements related to geometrical parameters for the base layer upsampling.  When extended_spatial_scalability is equal to 0, no geometrical parameter is present in the bitstream.  When extended_spatial_scalability is equal to 1, geometrical parameters are present in the sequence parameter set.  When extended_spatial_scalability is equal to 2, geometrical parameters are present in slice_data_in_scalable_extension.  The value of 3 is reserved for extended_spatial_scalability.  When extended_spatial_scalability is not present, it shall be inferred to be equal to 0.

scaled_base_left_offset specifies the horizontal offset between the upper-left pixel of an upsampled base layer picture and the upper-left pixel of a picture of the current layer in units of two luma samples. When scaled_base_left_offset is not present, it shall be inferred to be equal to 0.

The variable ScaledBaseLeftOffset is defined as follow:

ScaledBaseLeftOffset  = 2 * scaled_base_left_offset

The variable ScaledBaseLeftOffsetC is defined as follow:

ScaledBaseLeftOffsetC  = ScaledBaseLeftOffset / SubWidthC

scaled_base_top_offset specifies vertical offset of the upper-left pixel of an upsampled base layer picture and the upper-left pixel of a picture of the current layer in units of two luma samples. When scaled_base_top_offset is not present, it shall be inferred to be equal to 0.

The variable ScaledBaseTopOffset is defined as follow:

ScaledBaseTopOffset  = 2 * scaled_base_top_offset

The variable ScaledBaseTopOffsetC is defined as follow:

ScaledBaseTopOffsetC = ScaledBaseTopOffset / SubHeightC

scaled_base_right_offset specifies the horizontal offset between the bottom-right pixel of an upsampled based layer picture and the bottom-right pixel of a picture of the current layer in units of two luma samples. When scaled_base_right_offset is not present, it shall be inferred to be equal to 0.

The variable ScaledBaseRightOffset is defined as follow:

ScaledBaseRightOffset = 2 * scaled_base_right_offset

The variable ScaledBaseWidth is defined as follow:

ScaledBaseWidth 	= PicWidthInMbs * 16 – ScaledBaseLeftOffset – ScaledBaseRightOffset

The variable ScaledBaseWidthC is defined as follow:

ScaledBaseWidthC = ScaledBaseWidth  / SubWidthC

scaled_base_bottom_offset specifies the vertical offset between the bottom-right pixel of an upsampled based layer picture and the bottom-right pixel of a picture of the current layer in units of two luma samples.  When scaled_base_bottom_offset is not present, it shall be inferred to be equal to 0.

The variable ScaledBaseBottomOffset is defined as follow:

ScaledBaseBottomOffset = 2 * scaled_base_bottom_offset

The variable ScaledBaseHeight is defined as follow:

ScaledBaseHeight	 = PicHeightInMbs * 16 – ScaledBaseTopOffset – ScaledBaseBottomOffset

The variable ScaledBaseHeightC is defined as follow:

ScaledBaseHeightC = ScaledBaseHeight / SubHeightC

chroma_phase_x_plus1 specifies the horizontal phase shift of the chroma components in units of quarter sampling space in the horizontal direction of a picture of the current layer. When chroma_phase_x_plus1 is not present, it shall be inferred to be equal to 1.

chroma_phase_y_plus1 specifies the vertical phase shift of the chroma components in units of quarter sampling space in the vertical direction of a picture of the current layer. When chroma_phase_y_plus1 is not present, it shall be inferred to be equal to 1.

Note: The chroma phase parameters, chroma_phase_x_plus1 and chroma_phase_y_plus1 are in range 0..2. The value of 3 is reserved for all these chroma phase parameters.

#### Picture parameter set RBSP semantics

The specification of this subclause in AVC shall apply.

#### Supplemental enhancement information RBSP semantics

The specification of this subclause in AVC shall apply.

#### Access unit delimiter RBSP semantics

The specification of this subclause in AVC shall apply.

#### End of sequence RBSP semantics

The specification of this subclause in AVC shall apply.

#### End of stream RBSP semantics

The specification of this subclause in AVC shall apply.

#### Filler data RBSP semantics

The specification of this subclause in AVC shall apply.

#### Slice layer without partitioning RBSP semantics

The specification of this subclause in AVC shall apply.

#### Slice data partition RBSP semantics

The specification of this subclause in AVC shall apply.

#### RBSP slice trailing bits semantics

The specification of this subclause in AVC shall apply.

#### RBSP trailing bits semantics

The specification of this subclause in AVC shall apply.

#### Slice layer in scalable extension RBSP semantics

The slice layer in scalable extension consists of a slice header in scalable extension and slice data.  If the syntax element slice_type is equal to PR, the slice data are progressive refinement slice data in scalable extension; otherwise, the slice data are slice data in scalable extension.

### Slice header semantics

The specification of this subclause in AVC shall apply with the following additions.

For the semantics and decoding process of frame_num and idr_pic_id, only pictures with dependency_id equal to the value of dependency_id of the current slice are considered.

### Slice data semantics

The specification of this subclause in AVC shall apply.

### Macroblock layer semantics

The specification of this subclause in AVC shall apply.

### Slice header in scalable extension semantics

When present, the value of the slice header in scalable extension syntax elements pic_parameter_set_id, frame_num, field_pic_flag, bottom_field_flag, idr_pic_id, pic_order_cnt_lsb, delta_pic_order_cnt_bottom, delta_pic_order_cnt[ 0 ], delta_pic_order_cnt[ 1 ], and slice_group_change_cycle shall be the same in all slice headers of a coded picture.

first_mb_in_slice has the same semantics as first_mb_in_slice in subclause S.8.4.3.

slice_type specifies the coding type of the slice according to Table S-7-2.

Table S-7-2 – Name association to slice_type for NAL units with nal_unit_type equal to 20 or 21.

| - slice_type | - Name of slice_type |
| --- | --- |
| - 0 | - EB (B slice in scalable extension) |
| - 1 | - EP (P slice in scalable extension) |
| - 2 | - EI (I slice in scalable extension) |
| - 3 | - PR (progressive refinement slice) |

[Ed. Note(JR): Shouldn’t we modify the syntax in order that slice_type = 0 correspond to EP and slice_type=1 corespond to EB, in order to be compatible with the AVC meaning of slice_type?]

pic_parameter_set_id has the same semantics as pic_parameter_set_id in subclause S.8.4.3.

num_mbs_in_slice_minus1  plus 1 specifies the number of macroblocks in the progressive refinement slice.

luma_chroma_sep_flag  equal to 0 specifies that any chroma transform coefficient levels to be decoded for the current macroblock will immediately follow the luma transform coefficient levels. luma_chroma_sep_flag equal to 1 specifies that all luma transform coefficient levels for the slice are to be decoded first, followed by all chroma transform coefficient values.

frame_num has the same semantics as frame_num in subclause S.8.4.3.

field_pic_flag has the same semantics as field_pic_flag in subclause S.8.4.3.

bottom_field_flag has the same semantics as bottom_field_flag in subclause S.8.4.3.

idr_pic_id has the same semantics as idr_pic_id in subclause S.8.4.3.

pic_order_cnt_lsb has the same semantics as pic_order_cnt_lsb in subclause S.8.4.3.

delta_pic_order_cnt_bottom has the same semantics as delta_pic_order_cnt_bottom in subclause S.8.4.3.

delta_pic_order_cnt[ 0 ] has the same semantics as delta_pic_order_cnt[ 0 ] in subclause S.8.4.3.

delta_pic_order_cnt[ 1 ] has the same semantics as delta_pic_order_cnt[ 1 ] in subclause S.8.4.3.

redundant_pic_cnt has the same semantics as redundant_pic_cnt in subclause S.8.4.3.

direct_spatial_mv_pred_flag has the same semantics as direct_spatial_mv_pred_flag in subclause S.8.4.3.

key_picture_flag is used to identify slices that belong to a key picture. When key_picture_flag is not present, it shall be inferred to be equal to 0.   [Ed. Note(HS): Probably, we need to add that the syntax elements key_picture_flag of all slices for a dependency_id, quality_level, and picture order count (so, basically for a picture) need to be identical.]  [Ed. Note(JR): In the current JSVM software, when key_picture_flag is not present, in the decoder it is inferred to be equal to 1 if adaptive_ref_pic_marking_mode_flag is equal to one or if nal_unit_type == 5 otherwise it is infered to be equal to 0. However this solution impose some constraints on the AVC layer and another  solution should be found.]]

decomposition_stages specifies a parameter that is used for controlling the motion-compensated reconstruction process.

A variable NumberOfUpdateLevel is derived as follow

NumberOfUpdateLevel = decomposition_stages - temporal_level

[Ed. Note(HS): This syntax element is probably only required for specifying the number of active entries for the various update lists (num_ref_idx_update_lX_active[ ]). In that case, it should be renamed and moved inside the slice header in scalable extension syntax.]

base_id_plus1  minus 1 specifies the value of dependency_id and quality_level for base pictures that are used for inter-layer prediction of coding mode, motion, samples values, and/or residual values of the current slice. base_id_plus1 equal to 0 specifies that no inter-layer prediction (of coding mode, motion, sample value, and/or residual prediction) is used for the current slice. base_id_plus1 greater than 0 specifies that an inter-layer prediction (of coding mode, motion, sample value, and/or residual prediction) may be used for the current slice when signalled in the macroblock layer.

If base_id_plus1 is greater than 0, the variables DependencyIdBase and QualityLevelBase are derived as follows.:

DependencyIdBase = ( base_id_plus1 – 1 ) >> 2
	QualityLevelBase   = ( base_id_plus1 – 1 ) & 3.

Otherwise (base_id_plus1 is equal to 0), DependencyIdBase and QualityLevelBase are set equal to –1.

If base_id_plus1 is greater than 0, the variables BasePicWidthInMbs, BaseChromaFormatIdc, BasePicWidth, BasePicHeight, BasePicWidthC, BasePicHeightC,  BaseMbWidthC and BaseMbHeightC are defined as follow:

- BasePicWidthInMbs is equal to basePicWidthInMbsMinus1 + 1, with basePicWidthInMbsMinus1 being the syntax element pic_width_in_mbs_minus1 of the active sequence parameter set for the pictures with dependency_id equal to DependencyIdBase

- BaseChromaFormatIdc is equal to the syntax element chroma_format_idc of the active sequence parameter set for the pictures with dependency_id equal to DependencyIdBase.

- BasePicWidth is equal to variable PicWidthInSamplesL of the active sequence parameter set for the pictures with dependency_id equal to DependencyIdBase.

- BasePicHeight is equal to variable PicHeightInSamplesL of the active sequence parameter set for the pictures with dependency_id equal to DependencyIdBase.

- BasePicWidthC is equal to variable PicWidthInSamplesC of the active sequence parameter set for the pictures with dependency_id equal to DependencyIdBase.

- BasePicHeightC is equal to variable PicHeightInSamplesC of the active sequence parameter set for the pictures with dependency_id equal to DependencyIdBase.

- BaseMbWidthC is equal to variable MbWidthC of the active sequence parameter set for the pictures with dependency_id equal to DependencyIdBase.

- BaseMbHeightC is equal to variable MbHeightC of the active sequence parameter set for the pictures with dependency_id equal to DependencyIdBase.

adaptive_prediction_flag specifies the presence of syntax elements in the macroblock layer in scalable extension. When this syntax element is not present, it shall be inferred to be equal to 0.

num_ref_idx_active_override_flag has the same semantics as num_ref_idx_active_override_flag in subclause S.8.4.3.

num_ref_idx_l0_active_minus1 has the same semantics as num_ref_idx_l0_active_minus1 in subclause S.8.4.3.

num_ref_idx_l1_active_minus1 has the same semantics as num_ref_idx_l1_active_minus1 in subclause S.8.4.3.

num_ref_idx_update_l0_active[ i ] specifies the maximum reference index for update picture list 0 that shall be used for the update process that uses pictures for which temporal_level is equal to i.

num_ref_idx_update_l1_active[ i ] specifies the maximum reference index for update picture list 1 that shall be used for the update process that uses pictures for which temporal_level is equal to i.

cabac_init_idc has the same semantics as cabac_init_idc in subclause S.8.4.3.

slice_qp_delta has the same semantics as slice_qp_delta in subclause S.8.4.3.

disable_deblocking_filter_idc has the same semantics as disable_deblocking_filter_idc in subclause S.8.4.3.

slice_alpha_c0_offset_div2 has the same semantics as slice_alpha_c0_offset_div2 in subclause S.8.4.3.

slice_beta_c0_offset_div2 has the same semantics as slice_beta_c0_offset_div2 in subclause S.8.4.3.

slice_group_change_cycle has the same semantics as slice_group_change_cycle in subclause S.8.4.3.

base_chroma_phase_x_plus1 specifies the horizontal phase shift of the chroma components in units of quarter sampling space in the horizontal direction of the pictures with dependency_id equal to DependencyIdBase. When base_chroma_phase_x_plus1 is not present, it shall be inferred to be equal to 1.

base_chroma_phase_y_plus1 specifies the vertical phase shift of the chroma components in units of quarter sampling space in the vertical direction of the pictures with dependency_id equal to DependencyIdBase. When base_chroma_phase_y_plus1 is not present, it shall be inferred to be equal to 1.

scaled_base_left_offset specifies the horizontal offset between the upper-left pixel of the upsampled base layer picture and the upper-left pixel of the current picture in units of two luma samples of current picture.  When scaled_base_left_offset is not present, it shall be inferred to be equal to 0.

scaled_base_top_offset specifies vertical offset of the upper-left pixel of the upsampled base layer picture and the upper-left pixel of the current picture in units of two luma samples of current picture.  When scaled_base_top_offset is not present, it shall be inferred to be equal to 0.

scaled_base_right_offset specifies the horizontal offset between the bottom-right pixel of the upsampled based layer picture and the bottom-right pixel of the current picture in units of two luma samples of current picture.  When scaled_base_right_offset is not present, it shall be inferred to be equal to 0.

scaled_base_bottom_offset specifies the vertical offset between the bottom-right pixel of the upsampled based layer picture and the bottom-right pixel of the current picture in units of two luma samples of current picture.  When scaled_base_bottom_offset is not present, it shall be inferred to be equal to 0.

Note: These geometrical parameters, if present in the slice data in scalable extension semantics, shall apply to the current slice, and the parameters shall be consistent for slices of the same layer within an access unit. These parameters shall be associated to the current picture if the picture is to be used as reference picture

### Slice data in scalable extension semantics

cabac_alignment_one_bit has the same semantics as cabac_alignment_one_bit in subclause S.8.4.4.

mb_field_decoding_flag has the same semantics as mb_field_decoding_flag in subclause S.8.4.4.

end_of_slice_flag has the same semantics as end_of_slice_flag in subclause S.8.4.4.

The function NextMbAddress( ) used in the slice data in scalable extension syntax table is specified in subclause S.9.2.2.

### Macroblock layer in scalable extension semantics

base_mode_flag equal to 1 specifies that mb_type for the current macroblock and where applicable the reference indices and motion vectors shall be inferred in dependence of the corresponding base macroblocks.  base_mode_flag equal to 0 specifies that mb_type is not inferred unless the base_mode_refinement_flag is equal to 1.

When base_mode_flag is not present, base_mode_flag shall be inferred to be equal to 0.

When base_mode_flag is not present, base_mode_flag shall be inferred as follows.

- If base_id_plus1 is equal to 0, the value of base_mode_flag shall be inferred to be equal to 0.

- Otherwise (base_id_plus1 is greater than 0), the value of base_mode_flag shall be inferred to be equal to 1.

base_mode_refinement_flag equal to 1 specifies that mb_type for the current macroblock and the reference indices shall be inferred in dependence of the corresponding base macroblocks. Furthermore, base_mode_refinement_flag equal to 1 specifies that the motion vectors are specified by quarter-sample refinements that are added to motion vectors predictors which are derived using the motion vectors of the base macroblock. base_mode_refinement_flag equal to 0 specifies that mb_type is not inferred.

When base_mode_refinement_flag is not present, base_mode_refinement_flag shall be inferred to be equal to 0.

mb_type specifies the macroblock type. The semantics of mb_type depend on the slice type.

Tables and semantics are specified for the various macroblock types for EI, EP, and EB slices. Each table presents the value of mb_type, the name of mb_type, the number of macroblock partitions used (given by NumMbPart( mb_type ) function), the prediction mode of the macroblock (when it is not partitioned) or the first partition (given by the MbPartPredMode( mb_type, 0 ) function) and the prediction mode of the second partition (given by the MbPartPredMode( mb_type, 1 ) function). When a value is not applicable it is designated by “na”. In the text, the value of mb_type may be referred to as the macroblock type and a value X of MbPartPredMode( ) may be referred to in the text by “X macroblock (partition) prediction mode” or as “X prediction macroblocks”.

Table S-7-3 shows the allowed collective macroblock types for each slice_type.

Table S-7-3 – Allowed collective macroblock types for slice_type.

| - slice_type | - allowed collective macroblock types |
| --- | --- |
| EI (slice) | I (see Table S-7-4) (macroblock types) |
| EP (slice) | P (see Table 7-13 of the AVC text) and I (see Table S-7-4) (macroblock types) |
| EB (slice) | B (see Table 7-14 of the AVC text) and I (see Table S-7-4) (macroblock types) |

When mb_type is not present, mb_type shall be inferred as follows.

- If SpatialScalabilityType is equal to 0, the following applies.

- The derivation process for base macroblocks in subclause S.7.5.2 is invoked with CurrMbAddr as input and the output is assigned to the picture basePic and the array of macroblock addresses aMbAddrBase.

- Let mbAddrBase set to be equal to aMbAddrBase[0].

- Let mbTypeBase be the value of the syntax element mb_type of the macroblock with macroblock address mbAddrBase in the picture basePic.

- Let mbTypeBaseSuffix be the name of mbTypeBase without the two first characters.

NOTE – Example: When mbTypeBase is equal to B_L0_L0_16x8, mbTypeBaseSuffix is equal to L0_L0_16x8.

- Let S be a variable that is derived as follows.

- If slice_type is equal to EI, S shall be equal to I.

- Otherwise, if slice_type is equal to EP, S shall be equal to P.

- Otherwise (slice_type is equal to EB), S shall be equal to B.

- The syntax element mb_type is inferred as follows.

- If mbTypeBase specifies an I macroblock type, mb_type shall be inferred to be equal to I_NxN.

- Otherwise, if mbTypeBase is equal to P_Skip, mb_type shall be inferred to be equal to S_L0_16x16.

- Otherwise, if mbTypeBase is equal to B_Skip or B_Direct_16x16, mb_type shall be inferred to be equal to B_8x8.

- Otherwise, mb_type shall be inferred to be equal to S_mbTypeBaseSuffix.

- Otherwise, if SpatialScalabilityType is equal to 1, the following applies.

- The derivation process for base sub-macroblocks in subclause S.7.5.3 is invoked with CurrMbAddr as input and the output is assigned to the picture basePic, the macroblock address mbAddrBase, and the macroblock partition index mbPartIdxBase.

- Let mbTypeBase be the value of the syntax element mb_type of the macroblock with macroblock address mbAddrBase in the picture basePic.

- Let subMbTypeBase be the value of the syntax element sub_mb_type[ mbPartIdxBase ] of the macroblock mbAddrBase inside the picture basePic.

- Let predFlagL0Base and predFlagL1Base be the variables predFlagL0 and predFlagL1, respectively, of the macroblock partition mbPartIdxBase of the macroblock mbAddrBase inside the picture basePic.

- Let S be a variable that is derived as follows.

- If slice_type is equal to EI, S shall be equal to I.

- Otherwise, if slice_type is equal to EP, S shall be equal to P.

- Otherwise (slice_type is equal to EB), S shall be equal to B.

- The syntax element mb_type is inferred as follows.

- If mbTypeBase specifies an I macroblock type, mb_type shall be inferred to be equal to I_NxN.

- Otherwise, if mbTypeBase is equal to B_Skip or equal to B_Direct_16x16, the following applies.

- If the syntax element direct_8x8_inference_flag for the picture basePic is equal to 1, the following applies.

- If predFlagL0Base is equal to 1 and predFlagL1Base is equal to 1, mb_type shall be inferred to be equal to S_Bi_16x16.

- Otherwise, if predFlagL0Base is equal to 1, mb_type shall be inferred to be equal to S_L0_16x16.

- Otherwise ( predFlagL0Base is not equal to 1 ), mb_type shall be inferred to be equal to S_L1_16x16.

- Otherwise ( direct_8x8_inference_flag for the picture basePic is not equal to 1 ), mb_type shall be inferred to be equal to S_8x8.

- Otherwise, if mbTypeBase is equal to P_8x8 or equal to P_8x8ref0, the following applies.

- If subMbTypeBase is equal to P_L0_8x8, mb_type shall be inferred to be equal to S_L0_16x16.

- Otherwise, if subMbTypeBase is equal to P_L0_8x4, mb_type shall be inferred to be equal to S_L0_16x8.

- Otherwise, if subMbTypeBase is equal to P_L0_4x8, mb_type shall be inferred to be equal to S_L0_8x16.

- Otherwise (subMbTypeBase is equal to P_L0_4x4), mb_type shall be inferred to be equal to S_8x8.

- Otherwise, if mbTypeBase is equal to B_8x8, the following applies

- If subMbTypeBase is equal to B_X_8x8 (with X being replace be either L0, L1, or Bi), mb_type shall be inferred to be equal to S_X_16x16.

- Otherwise, if subMbTypeBase is equal to B_X_8x4 (with X being replace be either L0, L1, or Bi), mb_type shall be inferred to be equal to S_X_16x8.

- Otherwise, if subMbTypeBase is equal to B_X_4x8 (with X being replace be either L0, L1, or Bi), mb_type shall be inferred to be equal to S_X_8x16.

- Otherwise, if subMbTypeBase is equal to B_X_4x4 (with X being replace be either L0, L1, or Bi), mb_type shall be inferred to be equal to S_ 8x8.

- Otherwise (subMbTypeBase is equal to B_Direct_8x8),

- If the syntax element direct_8x8_inference_flag for the picture basePic is equal to 1, the following applies.

- If predFlagL0Base is equal to 1 and predFlagL1Base is equal to 1, mb_type shall be inferred to be equal to S_Bi_16x16.

- Otherwise, if predFlagL0Base is equal to 1, mb_type shall be inferred to be equal to S_L0_16x16.

- Otherwise, mb_type shall be inferred to be equal to S_L1_16x16.

- Otherwise, mb_type shall be inferred to be equal to S_8x8.

- Otherwise, the following applies.

- If predFlagL0Base is equal to 1 and predFlagL1Base is equal to 1, mb_type shall be inferred to be equal to S_Bi_16x16.

- Otherwise, if predFlagL0Base is equal to 1, mb_type shall be inferred to be equal to S_L0_16x16.

- Otherwise, mb_type shall be inferred to be equal to S_L1_16x16.

- Otherwise, if SpatialScalabilityType is greater than 1, the following applies.

- Subclause S.9.4.1.6.1 is invoked for inheriting motion data with CurrMbAddr as input and mb_type as output.

The bitstream shall not contain syntax elements that results in inferred macroblock types that are unvalid.

NOTE – The inferrence of unvalid syntax elements mb_type can occur when the slice type of the current slice is EI and the slice type of the slice which contains the base macroblock is equal to P, EP, B, or EB, or when the slice type of the current slice is EP and the slice type of the slice which contains the base macroblock is equal to B or EB. As an example, a macroblock type P_L1_16x16 is unvalid.

intra_base_flag equal to 1specifies the macroblock type I_BL.

When intra_base_flag is not present, intra_base_flag shall be inferred as follows.

- If base_mode_flag is equal to 1 and mb_type is equal to I_NxN, intra_base_flag shall be inferred to be equal to 1.

- Otherwise, intra_base_flag shall be inferred to be equal to 0.

transform_size_8x8_flag has the same semantics as transform_size_8x8_flag in subclause S.8.4.5.

Macroblock types that may be collectively referred to as I macroblock types are specified in Table S-7-4.

The macroblock types for EI slices are all I macroblock types.

Table S-7-4 – Macroblock types for EI slices.

| mb_type | Name of mb_type | transform_size_8x8_flag | intra_base_flag | MbPartPredMode ( mb_type, 0 ) | Intra16x16PredMode | CodedBlockPatternChroma | CodedBlockPatternLuma |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | I_NxN | 0 | 0 | Intra_4x4 | na | see AVC text | see AVC text |
| 0 | I_NxN | 1 | 0 | Intra_8x8 | na | see AVC text | see AVC text |
| 0 | I_BL | na | 1 | Intra_Base | na | see AVC text | see AVC text |
| 1 | I_16x16_0_0_0 | na | na | Intra_16x16 | 0 | 0 | 0 |
| 2 | I_16x16_1_0_0 | na | na | Intra_16x16 | 1 | 0 | 0 |
| 3 | I_16x16_2_0_0 | na | na | Intra_16x16 | 2 | 0 | 0 |
| 4 | I_16x16_3_0_0 | na | na | Intra_16x16 | 3 | 0 | 0 |
| 5 | I_16x16_0_1_0 | na | na | Intra_16x16 | 0 | 1 | 0 |
| 6 | I_16x16_1_1_0 | na | na | Intra_16x16 | 1 | 1 | 0 |
| 7 | I_16x16_2_1_0 | na | na | Intra_16x16 | 2 | 1 | 0 |
| 8 | I_16x16_3_1_0 | na | na | Intra_16x16 | 3 | 1 | 0 |
| 9 | I_16x16_0_2_0 | na | na | Intra_16x16 | 0 | 2 | 0 |
| 10 | I_16x16_1_2_0 | na | na | Intra_16x16 | 1 | 2 | 0 |
| 11 | I_16x16_2_2_0 | na | na | Intra_16x16 | 2 | 2 | 0 |
| 12 | I_16x16_3_2_0 | na | na | Intra_16x16 | 3 | 2 | 0 |
| 13 | I_16x16_0_0_1 | na | na | Intra_16x16 | 0 | 0 | 15 |
| 14 | I_16x16_1_0_1 | na | na | Intra_16x16 | 1 | 0 | 15 |
| 15 | I_16x16_2_0_1 | na | na | Intra_16x16 | 2 | 0 | 15 |
| 16 | I_16x16_3_0_1 | na | na | Intra_16x16 | 3 | 0 | 15 |
| 17 | I_16x16_0_1_1 | na | na | Intra_16x16 | 0 | 1 | 15 |
| 18 | I_16x16_1_1_1 | na | na | Intra_16x16 | 1 | 1 | 15 |
| 19 | I_16x16_2_1_1 | na | na | Intra_16x16 | 2 | 1 | 15 |
| 20 | I_16x16_3_1_1 | na | na | Intra_16x16 | 3 | 1 | 15 |
| 21 | I_16x16_0_2_1 | na | na | Intra_16x16 | 0 | 2 | 15 |
| 22 | I_16x16_1_2_1 | na | na | Intra_16x16 | 1 | 2 | 15 |
| 23 | I_16x16_2_2_1 | na | na | Intra_16x16 | 2 | 2 | 15 |
| 24 | I_16x16_3_2_1 | na | na | Intra_16x16 | 3 | 2 | 15 |
| 25 | I_PCM | na | na | na | na | na | na |

Macroblock types that may be collectively referred to as P macroblock types are specified in Table 7-13 of the AVC text.

The macroblock types for EP slices are specified in Table 7-13 of the AVC text and Table S-7-4. mb_type values 0 to 4 are specified in Table S-7-4 and mb_type values 5 to 30 are specified in Table 7-13 of the AVC text, indexed by subtracting 5 from the value of mb_type.

Macroblock types that may be collectively referred to as B macroblock types are specified in Table 7-14 of the AVC text.

The macroblock types for EB slices are specified in Table 7-14 of the AVC text and Table S-7-4. mb_type values 0 to 22 are specified in Table S-7-4 and mb_type values 23 to 48 are specified in Table 7-14 of the AVC text, indexed by subtracting 5 from the value of mb_type.

pcm_alignment_zero_bit has the same semantics as pcm_alignment_bit_zero in subclause S.8.4.5.

pcm_sample_luma[ i ] has the same semantics as pcm_sample_luma[ i ] in subclause S.8.4.5.

pcm_sample_chroma[ i ] has the same semantics as pcm_sample_chroma[ i ] in subclause S.8.4.5.

coded_block_pattern has the same semantics as coded_block_pattern in subclause S.8.4.5.

mb_qp_delta has the same semantics as mb_qp_delta in subclause S.8.4.5.

#### Macroblock prediction in scalable extension semantics

prev_intra4x4_pred_mode_flag[ luma4x4BlkIdx ] and rem_intra4x4_pred_mode[ luma4x4BlkIdx ] have the same semantics as prev_intra4x4_pred_mode_flag[ luma4x4BlkIdx ] and rem_intra4x4_pred_mode[ luma4x4BlkIdx ], respectively, in subclause S.8.4.5.

prev_intra8x8_pred_mode_flag[ luma8x8BlkIdx ] and rem_intra8x8_pred_mode[ luma8x8BlkIdx ] have the same semantics as prev_intra8x8_pred_mode_flag[ luma8x8BlkIdx ] and rem_intra8x8_pred_mode[ luma8x8BlkIdx ], respectively, in subclause S.8.4.5.

intra_chroma_pred_mode has the same semantics as intra_chroma_pred_mode in subclause S.8.4.5.

motion_prediction_flag_l0[ mbPartIdx ] equal to 1 specifies that the (possibly scaled) base motion vector(s) is/are used as motion vector predictor(s) for the list 0 motion vector(s) of the macroblock partition mbPartIdx. When motion_prediction_flag_l0[ mbPartIdx ] is equal to 1, the reference index ref_idx_l0[ mbPartIdx ] is inferred using the base reference index and the components of the motion vector(s) mvL0 are obtained by adding the motion vector differences mvd_l0[ mbPartIdx ][ ] to the (possibly scaled) base motion vectors.

When motion_prediction_flag_l0[ mbPartIdx ] is not present, motion_prediction_flag_l0[ mbPartIdx ] shall be inferred to be equal to 0.

motion_prediction_flag_l1[ mbPartIdx ] has the same semantics as motion_prediction_flag_l0[ mbPartIdx ], with l0 and list 0 replaced by l1 and list 1, respectively.

ref_idx_l0[ mbPartIdx ] has the same semantics as ref_idx_l0[ mbPartIdx ][ 0 ][ compIdx ] in subclause S.8.4.5.

ref_idx_l1[ mbPartIdx ] has the same semantics as ref_idx_l1[ mbPartIdx ][ 0 ][ compIdx ] in subclause S.8.4.5.

mvd_l0[ mbPartIdx ][ 0 ][ compIdx ] has the same semantics as mvd_l0[ mbPartIdx ][ 0 ][ compIdx ] in subclause S.8.4.5.

mvd_l1[ mbPartIdx ][ 0 ][ compIdx ] has the same semantics as mvd_l1[ mbPartIdx ][ 0 ][ compIdx ] in subclause S.8.4.5.

mvd_ref_l0[ mbPartIdx ][ 0 ][ compIdx ] has the same semantics as mvd_l0[ mbPartIdx ][ 0 ][ compIdx ] in this subclause, except that the value of mvd_ref_l0[ mbPartIdx ][ 0 ][ compIdx ] shall be in the range of -1 to 1, inclusive.

mvd_ref_l1[ mbPartIdx ][ 0 ][ compIdx ]  has the same semantics as mvd_ref_l0[ mbPartIdx ][ 0 ][ compIdx ] with l0 replaced by l1.

#### Sub-macroblock prediction in scalable extension semantics

sub_mb_type[ mbPartIdx ] specifies the sub-macroblock types.

Tables and semantics are specified for the various sub-macroblock types for EP and EB slices. Each table presents the value of sub_mb_type, the name of sub_mb_type, the number of sub-macroblock partitions used (given by the NumSubMbPart( sub_mb_type ) function), and the prediction mode of the sub-macroblock (given by the SubMbPredMode( sub_mb_type ) function). In the text, the value of sub_mb_type may be referred to by “sub-macroblock type”. In the text, the value of SubMbPredMode( ) may be referred to by “sub-macroblock prediction mode”.

When sub_mb_type[ mbPartIdx ] is not present and mb_type is equal to P_8x8, P_8x8ref0, or B_8x8, sub_mb_type[ mbPartIdx ] shall be inferred as follows.

If SpatialScalability is lower than 2, the following applies.

- The derivation process for base partitions in subclause S.7.5.4 is invoked with CurrMbAddr and mbPartIdx as inputs and the output is assigned to the picture basePic, the macroblock address mbAddrBase, the macroblock partition index mbPartIdxBase, and the sub-macroblock partition index subMbPartIdxBase.

- Let mbTypeBase be the value of the syntax element mb_type of the macroblock with macroblock address mbAddrBase in the picture basePic.

- Let subMbTypeBase be the value of the syntax element sub_mb_type[ mbPartIdxBase ] of the macroblock mbAddrBase inside the picture basePic.

- Let predFlagL0Base and predFlagL1Base be the variables predFlagL0 and predFlagL1, respectively, of the macroblock partition mbPartIdxBase of the macroblock mbAddrBase inside the picture basePic.

- Let S be a variable that is derived as follows.

- If slice_type is equal to EI, S shall be equal to I.

- Otherwise, if slice_type is equal to EP, S shall be equal to P.

- Otherwise (slice_type is equal to EB), S shall be equal to B.

- The syntax element sub_mb_type[ mbPartIdx ] is inferred as follows.

- If SpatialScalability is equal to 0, the following applies.

- Let subMbPartSize be a variable that is derived as follows.

- If mbTypeBase is equal to B_Skip or B_Direct_16x16, the following applies.

- If the syntax element direct_8x8_inference_flag for the picture basePic is equal to 1, subMbPartSize is set equal to 8x8.

- Otherwise, subMbPartSize is set equal to 4x4.

- Otherwise, subMbPartSize is set equal to AxB, with A being SubMbPartWidth( subMbTypeBase ) and B being SubMbPartHeight( subMbTypeBase ).

- The syntax element sub_mb_type[ mbPartIdx ] is inferred as follows.

- If predFlagL0Base is equal to 1 and predFlagL1Base is equal to 1, sub_mb_type[ mbPartIdx ] shall be inferred to be equal to S_Bi_subMbPartSize.

- Otherwise, if predFlagL0Base is equal to 1, sub_mb_type[ mbPartIdx ] shall be inferred to be equal to S_L0_subMbPartSize.

- Otherwise, sub_mb_type[ mbPartIdx ] shall be inferred to be equal to S_L1_subMbPartSize.

- Otherwise (SpatialScalability is equal to 1), the following applies.

- The syntax element sub_mb_type[ mbPartIdx ] is inferred as follows.

- If predFlagL0Base is equal to 1 and predFlagL1Base is equal to 1, sub_mb_type[ mbPartIdx ] shall be inferred to be equal to S_Bi_8x8.

- Otherwise, if predFlagL0Base is equal to 1, sub_mb_type[ mbPartIdx ] shall be inferred to be equal to S_L0_8x8.

- Otherwise, sub_mb_type[ mbPartIdx ] shall be inferred to be equal to S_L1_8x8.

Otherwise, ( SpatialScalability is greater than or equal to 2 )

- The derivation for macroblock partitions label in subclause S.9.4.1.6.4 is invoked with CurrMbAddr and mbPartIdx as inputs and the output is assigned to mbPartLabel.

- The derivation for base reference indices in S.9.4.1.5.1 is invoked with CurrMbAddr, mbPartIdx and a list suffix LX (with X being replaced by 0 and 1) as inputs and the output is assigned to refIdxLX.

- Let mbPartSize be a variable derived as follows.

- If mbPartLabel is equal to INTRA, mbPartSize is set to 8x8

- Otherwise, mbPartSize is set to the last part of the name of mbPartLabel.

- Let S be a variable that is derived as follows.

- If slice_type is equal to IE, S shall be equal to I.

- Otherwise, if slice_type is equal to PE, S shall be equal to P.

- Otherwise ( slice_type is equal to BE ), S shall be equal to B.

- Let Z be a variable that is derived as follows.

- If  refIdxL0 is equal to 1 and refIdxL1 is equal to 1,  Z is set equal to Bi

- Otherwise, if refIdxL0 is not equal to 1, Z is set equal to L0

- Otherwise, Z is set equal to L1.

- Then, sub_mb_type[ mbPartIdx ] shall be inferred to be equal to S_Z_mbPartSize.

The bitstream shall not contain syntax elements that results in inferred sub-macroblock types that are unvalid.

NOTE – The inferrence of unvalid syntax elements sub_mb_type[ mbPartIdx ] can occur when the slice type of the current slice is EI and the slice type of the slice which contains the base macroblock is equal to P, EP, B, or EB, or when the slice type of the current slice is EP and the slice type of the slice which contains the base macroblock is equal to B or EB. As an example, a sub-macroblock type P_L1_8x8 is unvalid.

The sub-macroblock types for P macroblock types are specified in Table 7–17 of the AVC text.

The sub-macroblock types for B macroblock types are specified in Table 7–18 of the AVC text.

motion_prediction_flag_l0[ mbPartIdx ]  has the same semantics as motion_prediction_flag_l0[ mbPartIdx ] in subclause S.8.4.8.1.

motion_prediction_flag_l1[ mbPartIdx ]  has the same semantics as motion_prediction_flag_l1[ mbPartIdx ] in subclause S.8.4.8.1.

ref_idx_l0[ mbPartIdx ]  has the same semantics as ref_idx_l0[ mbPartIdx ] in subclause S.8.4.8.1.

ref_idx_l1[ mbPartIdx ]  has the same semantics as ref_idx_l1[ mbPartIdx ] in subclause S.8.4.8.1.

mvd_l0[ mbPartIdx ][ subMbPartIdx ][ compIdx ]  has the same semantics as mvd_l0 in subclause S.8.4.8.1, except that it is applied to the sub-macroblock partition index with SubMbPartIdx. The indices mbPartIdx and subMbPartIdx specify to which macroblock partition and sub-macroblock partition mvd_l0 is assigned.

mvd_l1[ mbPartIdx ][ subMbPartIdx ][ compIdx ]  has the same semantics as mvd_l1 in subclause S.8.4.8.1, except that it is applied to the sub-macroblock partition index with SubMbPartIdx. The indices mbPartIdx and subMbPartIdx specify to which macroblock partition and sub-macroblock partition mvd_l1 is assigned.

mvd_ref_l0[ mbPartIdx ][ subMbPartIdx ][ compIdx ]  has the same semantics as mvd_ref_l0 in subclause S.8.4.8.1, except that it is applied to the sub-macroblock partition index with SubMbPartIdx. The indices mbPartIdx and subMbPartIdx specify to which macroblock partition and sub-macroblock partition mvd_ref_l1 is assigned.

mvd_ref_l0[ mbPartIdx ][ subMbPartIdx ][ compIdx ]  has the same semantics as mvd_ref_l1 in subclause S.8.4.8.1, except that it is applied to the sub-macroblock partition index with SubMbPartIdx. The indices mbPartIdx and subMbPartIdx specify to which macroblock partition and sub-macroblock partition mvd_ref_l1 is assigned.

#### Residual in scalable extension semantics

residual_prediction_flag equal to 1 specifies that the residual signal is predicted by the (possibly upsampled) reconstructed residual signal of the base macroblock or sub-macroblock.  residual_prediction_flag equal to 0 specifies that the residual signal is not predicted.

When the syntax element residual_prediction is not present, residual_prediction_flag is inferred as follows.

- If mb_type does not specify an I macroblock type and base_id_plus1 is greater than 0, residual_prediction_flag is inferred to be equal to 1.

- Otherwise, residual_prediction_flag is inferred to be equal to 0.

Depending on mb_type, luma or chroma, and chroma format, the syntax structure residual_block_cabac( coeffLevel, maxNumCoeff ) is used with the arguments coeffLevel, which is a list containing the maxNumCoeff transform coefficient levels that are parsed in residual_block_cabac( ) and maxNumCoeff as follows.

- Depending on MbPartPredMode( mb_type, 0 ), the following applies.

- If MbPartPredMode( mb_type, 0 ) is equal to Intra_16x16, the transform coefficient levels are parsed into the list Intra16x16DCLevel and into the 16 lists Intra16x16ACLevel[ i ]. Intra16x16DCLevel contains the 16 transform coefficient levels of the DC transform coefficient levels for each 4x4 luma block. For each of the 16 4x4 luma blocks indexed by i = 0..15, the 15 AC transform coefficients levels of the i-th block are parsed into the i-th list Intra16x16ACLevel[ i ].

- Otherwise (MbPartPredMode( mb_type, 0 ) is not equal to Intra_16x16), the following applies.

- If transform_size_8x8_flag is equal to 0, for each of the 16 4x4 luma blocks indexed by i = 0..15, the 16 transform coefficient levels of the i-th block are parsed into the i-th list LumaLevel[ i ].

- Otherwise (transform_size_8x8_flag is equal to 1), the 64 transform coefficient levels of the i8x8-th block are parsed into the i8x8-th list LumaLevel8x8[ i8x8 ].

- For each chroma component, indexed by iCbCr = 0..1, the DC transform coefficient levels of the 4 * NumC8x8 4x4 chroma blocks are parsed into the iCbCr-th list ChromaDCLevel[ iCbCr ].

- For each of the 4x4 chroma blocks, indexed by i4x4 = 0..3 and i8x8 = 0..NumC8x8  1, of each chroma component, indexed by iCbCr = 0..1, the 15 AC transform coefficient levels are parsed into the (i8x8*4 + i4x4)-th list of the iCbCr-th chroma component ChromaACLevel[ iCbCr ][ i8x8*4 + i4x4 ].

### Progressive refinement slice data in scalable extension semantics

Within a progressive refinement slice, the syntax elements for representening the transform coefficient levels for a slice are not parsed in the default scanning order. Syntax elements of various macroblocks are interleaved. The current macroblock is specified by the variable CurrMbAddr as specified in the syntax diagram in in subclause S.8.3.9.

Depending on luma or chroma, the parsing of transform coefficient levels proceeds as follows

- Depending on the value of value of transform_size_8x8_flag for the current macroblock CurrMbAddr, which is either decoded or inferred using the base representation data as specified in subclause S.8.3.9, the following applies.

- If transform_size_8x8_flag is equal to 0, for each of the 16 4x4 luma blocks indexed by i = 0..15, the 16 transform coefficient levels of the i-th block are parsed into the i-th list LumaLevel[ i ].

- Otherwise, the 64 transform coefficient levels of the i8x8-th block are parsed into the i8x8-th list LumaLevel8x8[ i8x8 ]

- When chroma_format_idc is not equal to 0, for each chroma component, indexed by iCbCr = 0..1, the DC transform coefficient levels of the 4 * NumC8x8 4x4 chroma blocks are parsed into the iCbCr-th list ChromaDCLevel[ iCbCr ].

- When chroma_format_idc is not equal to 0, for each of the 4x4 chroma blocks, indexed by i4x4 = 0..3 and i8x8 = 0..NumC8x8  1, of each chroma component, indexed by iCbCr = 0..1, the 15 AC transform coefficient levels are parsed into the (i8x8*4 + i4x4)-th list of the iCbCr-th chroma component ChromaACLevel[ iCbCr ][ i8x8*4 + i4x4 ].

After a progressive refinement slice is completely parsed, all syntax elements and derived variables, to which a value NotCoded was assigned during the parsing process, are set equal to 0.

cycle_start_symbol  indicates whether decoding of chroma AC refinement values should be deferred beyond one cycle.

cycle_start_symbol_ext  indicates the number of cycles by which chroma AC refinement values should be deferred.

end_of_progressive_refinement_slice_flag equal to 1 specifies that the progressive refinement slice has been completely decoded.

When end_of_progressive_refinement_slice_flag is not present, end_of_progressive_refinement_slice_flag shall be inferred to be equal to 0.

When end_of_progressive_refinement_slice_flag is equal to 0, the bitstream shall not contain any progressive refinement slices prSlice for which all of the following conditions are true

- The variable dependency_id is equal to the value of the variable dependency_id for the slice with end_of_progressive_refinement_slice_flag equal to 0.

- The variable quality_level is greater than the value of the variable quality_level for the slice with end_of_progressive_refinement_slice_flag equal to 0.

- PicOrderCnt( prSlice ) is equal to PicOrderCnt( p0Slice ) with p0Slice being the slice with end_of_progressive_refinement_slice_flag equal to 0.

- bottom_field_flag is equal to the value of bottom_field_flag for the slice with end_of_progressive_refinement_slice_flag equal to 0.

coded_block_pattern_bit[ i ] represents the i-th bit (starting from the LSB) of the syntax element coded_block_pattern as it is specified in subclause S.8.4.5. If the syntax element is not present, it shall be inferred to be equal to 0.

The combined syntax element coded_block_pattern is derived as follows:

coded_block_pattern = 0
for( i = 0; i < 6; i++ )    coded_block_pattern += ( coded_block_pattern_bit[ i ] << i )

Based on the value of coded_block_pattern, the variables CodedBlockPatternLuma and CodedBlockPatternChroma are derived as follows.

CodedBlockPatternLuma = coded_block_pattern % 16
CodedBlockPatternChroma = coded_block_pattern / 16

The meaning of CodedBlockPatternLuma and CodedBlockPatternChroma is specified in subclause S.8.4.5.

mb_qp_delta can change the value of QPY,PRk with k being the value of quality_level in the macroblock layer.  The decoded value of mb_qp_delta shall be in the range of –( 26 + QpBdOffsetY / 2) to +( 25 + QpBdOffsetY / 2 ), inclusive. mb_qp_delta shall be inferred to be equal to 0 when it is not present for any macroblock.

For each macroblock mbAddr, the values of QPY,PRk, QP'Y,PRk, QPC,PRk, and QP'C,PRk are derived as follows

- If function base_mb_sig( mbAddr ) as specified in subclause S.8.2 is equal to FALSE, the following applies

- The value of QPY,PRk is derived as

QPY,PRk = ( ( QPY,PRk,PREV + mb_qp_delta + 52 + 2 * QpBdOffsetY ) % ( 52 + QpBdOffsetY ) ) - QpBdOffsetY

where QPY,PRk,PREV is the luma quantisation parameter, QPY, of the macroblock for which the previous syntax element mb_qp_delta was decoded (and not inferred). At the beginning of the parsing process for a progressive refinement slice, QPY,PRk,PREV is initially set to SliceQPY.

- The value of QP'Y,PRk is derived as

QP'Y,PRk = QPY,PRk + QpBdOffsetY

- The values QPC,PRk and QP'C,PRk are derived is specified in subclause S.9.5.7 when replacing QPY, QP'Y, QPC, and QP'C with QPY,PRk, QP'Y,PRk, QPC,PRk, and QP'C,PRk, respectively.

- Otherwise, the following applies.

- The derivation process for base quality slices in subclause S.7.5.5 is invoked with baseQualityLevel = quality_level – 1 as input and the output is assigned to baseQualitySlice.

- With QPY,Base and QPC,Base being the value of QPY and QPC (if QualiyLevel is equal to 1) or QPY,PR(k-1) and QPC,PR(k-1) (otherwise) for the macroblock mbAddr in the slice baseQualitySlice, the following applies.

QPY,PRk = max( 0, ( QPY,Base – 6 ) )
QPC,PRk = max( 0, ( QPC,Base – 6 ) )

- The values of QP'Y,PRk and QP'C,PRk are derived as

QP'Y,PRk = QPY,PRk + QpBdOffsetY
QP'C,PRk = QPC,PRk + QpBdOffsetC

transform_size_8x8_flag has the same semantics as transform_size_8x8_flag in subclause S.8.4.5.

coded_block_flag_luma[ i ] represents the syntax element coded_block_flag for the i-th luma 4x4 block in scanning order. It specifies whether the block contains non-zero transform coefficient levels:

- If coded_block_flag_luma[ i ] is equal to 0, all transform coefficient levels LumaLevel[ i ][ j ] with j = 0..15 are equal to 0.

- Otherwise, at least one transform coefficient levels LumaLevel[ i ][ j ] with j = 0..15 is not equal to 0

coded_block_flag_chromaDC[ iCbCr ] represents the syntax element coded_block_flag for the chroma DC block of component iCbCr. It specifies whether the block contains non-zero transform coefficient levels:

- If coded_block_flag_chromaDC[ iCbCr ] is equal to 0, all transform coefficient levels ChromaDCLevel[ iCbCr ][ j ] with j = 0..NumC8x8-1 are equal to 0.

- Otherwise, at least one transform coefficient levels ChromaDCLevel[ iCbCr ][ j ] with j = 0..NumC8x8-1 is not equal to 0

coded_block_flag_chromaAC[ iCbCr ][ i ] represents the syntax element coded_block_flag for the i-th chroma AC block of component iCbCr. It specifies whether the block contains non-zero transform coefficient levels:

- If coded_block_flag_chromaAC[ iCbCr ][ i ] is equal to 0, all transform coefficient levels ChromaACLevel[ iCbCr ][ i ][ j ] with j = 0..14 are equal to 0.

- Otherwise, at least one transform coefficient levels ChromaACLevel[ iCbCr ][ i ][ j ] with j = 0..14 is not equal to 0

significant_coeff_flag has the same semantics as significant_coeff_flag in subclause S.8.4.5.

coeff_sign_flag has the same semantics as coeff_sign in subclause S.8.4.5.

coeff_abs_level_minus1 has the same semantics as coeff_abs_level_minus1 in subclause S.8.4.5.

last_significant_coeff_flag has the same semantics as last_significant_coeff_flag in subclause S.8.4.5.

coeff_refinement_flag equal to 0 specifies that the value of a transform coefficient refinement level is equal to 0.  coeff_refinement_flag equal to 1 specifies that the value of a transform coefficient refinement level is either equal to –1  or equal to 1.

coeff_refinement_direction_flag specifies the sign of a transform coefficient refinement level. If coeff_refinement_direction_flag is equal to 1, the sign of the transform coefficient refinement level is equal to the sign of its base representation; otherwise, the refinement level has the opposite sign.

# Decoding process

The specification of this section in AVC shall apply with the following modifications.

[Ed. Note(HS): Since the order of process calls for a scalable bitstream is not as clear as for standard AVC, we should add some detailed specification here, when which process is invoked.

If all of the following conditions are true, the inverse motion compensated temporal filtering process in subclause S.8.8 “Inverse motion-compensated filtering process” is invoked with CurrPic as inputs.

- All slices of the current picture CurrPic have been decoded from the bitstream

- The syntax element dependency_id of CurrPic is equal to the highest value of the dependency_id of all the activated SPS

]

## NAL unit decoding process

The specification of this subclause in AVC shall apply with the following modifications.

[Ed. Note(HS): This subclause need to be slightly changed.]

## Slice decoding process

### Decoding process for picture order count

The specification of this subclause in AVC shall apply with the following modifications.

1) Replace the following paragraphs of this subclause in AVC

Picture order counts are used to determine initial picture orderings for reference pictures in the decoding of B slices (see subclauses 8.2.4.2.3 and 8.2.4.2.4), to represent picture order differences between frames or fields for motion vector derivation in temporal direct mode (see subclause 8.4.1.2.3), for implicit mode weighted prediction in B slices (see subclause 8.4.2.3.2), and for decoder conformance checking (see subclause C.4).

with the following

Picture order counts are used to determine initial picture orderings for reference pictures in the decoding of B slices (see subclauses S.9.2.4.2.3 and S.9.2.4.2.4), EP slices (see subclause S.9.2.4.2.6 and S.9.2.4.2.7), and EB slices (see subclause S.9.2.4.2.8 and S.9.2.4.2.9), to determine the ordering of update picture lists (see subclause S.9.2.6), to represent picture order differences between frames or fields for motion vector derivation in temporal direct mode (see subclause S.9.4.1.2.3), for implicit mode weighted prediction in B and EB slices (see subclause 8.4.2.3.2), and for decoder conformance checking (see subclause C.4).

In the following of this subclause only pictures with an identical value of their variable dependency_id are considered.

NOTE – The picture order counts of pictures with a specific value of dependency_id are decoded independenly of syntax elements and variables of pictures having a different value of the variable dependency_id.

#### Decoding process for picture order count type 0

The specification of this subclause in AVC shall apply with the following modifications.

1) Replace the following paragraphs of this subclause in AVC

Input to this process is PicOrderCntMsb of the previous reference picture in decoding order as specified in this subclause.

Outputs of this process are either or both TopFieldOrderCnt or BottomFieldOrderCnt.

- The variables prevPicOrderCntMsb and prevPicOrderCntLsb are derived as follows.

- If the current picture is an IDR picture, prevPicOrderCntMsb is set equal to 0 and prevPicOrderCntLsb is set equal to 0.

- Otherwise (the current picture is not an IDR picture), the following applies.

- If the previous reference picture in decoding order included a memory_management_control_operation equal to 5, the following applies.

- If the previous reference picture in decoding order is not a bottom field, prevPicOrderCntMsb is set equal to 0 and prevPicOrderCntLsb is set equal to the value of TopFieldOrderCnt for the previous reference picture in decoding order.

- Otherwise (the previous reference picture in decoding order is a bottom field), prevPicOrderCntMsb is set equal to 0 and prevPicOrderCntLsb is set equal to 0.

- Otherwise (the previous reference picture in decoding order did not include a memory_management_control_operation equal to 5), prevPicOrderCntMsb is set equal to PicOrderCntMsb of the previous reference picture in decoding order and prevPicOrderCntLsb is set equal to the value of pic_order_cnt_lsb of the previous reference picture in decoding order.

with the following

Input to this process is PicOrderCntMsb of the picture prevRefPic as specified in this subclause, with prevRefPic being the previous reference picture in decoding order with a value of dependency_id equal to the value of variable dependency_id of the current picture.

Outputs of this process are either or both TopFieldOrderCnt or BottomFieldOrderCnt.

- The variables prevPicOrderCntMsb and prevPicOrderCntLsb are derived as follows.

- If the current picture is an IDR picture, prevPicOrderCntMsb is set equal to 0 and prevPicOrderCntLsb is set equal to 0.

- Otherwise (the current picture is not an IDR picture), the following applies.

- If the picture prevRefPic included a memory_management_control_operation equal to 5, the following applies.

- If the picture prevRefPic is not a bottom field, prevPicOrderCntMsb is set equal to 0 and prevPicOrderCntLsb is set equal to the value of TopFieldOrderCnt for the picture prevRefPic.

- Otherwise (the picture prevRefPic is a bottom field), prevPicOrderCntMsb is set equal to 0 and prevPicOrderCntLsb is set equal to 0.

- Otherwise (the picture prevRefPic did not include a memory_management_control_operation equal to 5), prevPicOrderCntMsb is set equal to PicOrderCntMsb of the picture prevRefPic and prevPicOrderCntLsb is set equal to the value of pic_order_cnt_lsb of the picture prevRefPic.

#### Decoding process for picture order count type 1

The specification of this subclause in AVC shall apply with the following modifications.

1) Replace the following paragraphs of this subclause in AVC

Input to this process is FrameNumOffset of the previous picture in decoding order as specified in this subclause.

Outputs of this process are either or both TopFieldOrderCnt or BottomFieldOrderCnt.

The values of TopFieldOrderCnt and BottomFieldOrderCnt are derived as specified in this subclause. Let prevFrameNum be equal to the frame_num of the previous picture in decoding order.

When the current picture is not an IDR picture, the variable prevFrameNumOffset is derived as follows.

- If the previous picture in decoding order included a memory_management_control_operation equal to 5, prevFrameNumOffset is set equal to 0.

- Otherwise (the previous picture in decoding order did not include a memory_management_control_operation equal to 5), prevFrameNumOffset is set equal to the value of FrameNumOffset of the previous picture in decoding order.

NOTE – When gaps_in_frame_num_value_allowed_flag is equal to 1, the previous picture in decoding order may be a "non-existing" frame inferred by the decoding process for gaps in frame_num specified in subclause 8.2.5.2.

with the following

Input to this process is FrameNumOffset of the picture prevPic as specified in this subclause, with prevPic being the previous picture in decoding order with a value of dependency_id equal to the value of variable dependency_id of the current picture.

Outputs of this process are either or both TopFieldOrderCnt or BottomFieldOrderCnt.

The values of TopFieldOrderCnt and BottomFieldOrderCnt are derived as specified in this subclause. Let prevFrameNum be equal to the frame_num of the picture prevPic.

When the current picture is not an IDR picture, the variable prevFrameNumOffset is derived as follows.

- If the picture prevPic included a memory_management_control_operation equal to 5, prevFrameNumOffset is set equal to 0.

- Otherwise (the picture prevPic did not include a memory_management_control_operation equal to 5), prevFrameNumOffset is set equal to the value of FrameNumOffset of the previous picture in decoding order.

NOTE – When gaps_in_frame_num_value_allowed_flag is equal to 1, the picture prevPic may be a "non-existing" frame inferred by the decoding process for gaps in frame_num specified in subclause S.9.2.5.2.

#### Decoding process for picture order count type 2

The specification of this subclause in AVC shall apply with the following modifications.

1) Replace the following paragraphs of this subclause in AVC

Let prevFrameNum be equal to the frame_num of the previous picture in decoding order.

When the current picture is not an IDR picture, the variable prevFrameNumOffset is derived as follows.

- If the previous picture in decoding order included a memory_management_control_operation equal to 5, prevFrameNumOffset is set equal to 0.

- Otherwise (the previous picture in decoding order did not include a memory_management_control_operation equal to 5), prevFrameNumOffset is set equal to the value of FrameNumOffset of the previous picture in decoding order.

NOTE – When gaps_in_frame_num_value_allowed_flag is equal to 1, the previous picture in decoding order may be a "non-existing" frame inferred by the decoding process for gaps in frame_num specified in subclause 8.2.5.2.

with the following

Let prevPic be the previous picture in decoding order with a value of dependency_id equal to the value of the variable dependency_id of the current picture.

Let prevFrameNum be equal to the frame_num of the picture prevPic.

When the current picture is not an IDR picture, the variable prevFrameNumOffset is derived as follows.

- If the picture prevPic included a memory_management_control_operation equal to 5, prevFrameNumOffset is set equal to 0.

- Otherwise (the picture prevPic did not include a memory_management_control_operation equal to 5), prevFrameNumOffset is set equal to the value of FrameNumOffset of the picture prevPic.

NOTE – When gaps_in_frame_num_value_allowed_flag is equal to 1, the picture prevPic may be a "non-existing" frame inferred by the decoding process for gaps in frame_num specified in subclause S.9.2.5.2.

### Decoding process for macroblock to slice group map

The specification of this subclause in AVC shall apply.

### Decoding process for slice data partitioning

The specification of this subclause in AVC shall apply.

### Decoding process for reference picture lists construction

The specification of this subclause in AVC shall be replaced with the following

This process is invoked at the beginning of decoding of each P, SP, B, EP, or EB slice.

Outputs of this process are a reference picture list RefPicList0 and, when decoding a B or EB slice, a second reference picture list RefPicList1.

Decoded reference pictures are marked as "used for short-term reference" or "used for long-term reference" as specified by the bitstream and specified in subclause S.9.2.5. Short-term decoded reference pictures are identified by the value of frame_num. Long-term decoded reference pictures are assigned a long-term frame index as specified by the bitstream and specified in subclause S.9.2.5.

Subclause S.9.2.4.1 is invoked to specify

- the assignment of variables FrameNum, FrameNumWrap, and PicNum to each of the short-term reference pictures, and

- the assignment of variable LongTermPicNum to each of the long-term reference pictures.

Reference pictures are addressed through reference indices as specified in subclause S.9.2.4.1. A reference index is an index into a list of variables PicNum and LongTermPicNum, which is called a reference picture list. When decoding a P, EP, or SP slice, there is a single reference picture list RefPicList0. When decoding a B or EB slice, there is a second independent reference picture list RefPicList1 in addition to RefPicList0.

At the beginning of decoding of each slice, reference picture list RefPicList0, and for B slices RefPicList1, are derived as follows.

- An initial reference picture list RefPicList0 and for B and EB slices RefPicList1 are derived as specified in subclause S.9.2.4.2.

- The initial reference picture list RefPicList0 and for B and EB slices RefPicList1 are modified as specified in subclause S.9.2.4.3.

The number of entries in the modified reference picture list RefPicList0 is num_ref_idx_l0_active_minus1 + 1, and for B and EB slices the number of entries in the modified reference picture list RefPicList1 is num_ref_idx_l1_active_minus1 + 1. A reference picture may appear at more than one index in the modified reference picture lists RefPicList0 or RefPicList1.

#### Decoding process for picture numbers

The specification of this subclause in AVC shall apply with the following modifications.

1) Replace the following paragraphs of this subclause in AVC

The variables FrameNum, FrameNumWrap, PicNum, LongTermFrameIdx, and LongTermPicNum are used for the initialisation process for reference picture lists in subclause 8.2.4.2, the modification process for reference picture lists in subclause 8.2.4.3, and for the decoded reference picture marking process in subclause 8.2.5.

To each short-term reference picture the variables FrameNum and FrameNumWrap are assigned as follows. First, FrameNum is set equal to the syntax element frame_num that has been decoded in the slice header(s) of the corresponding short-term reference picture. Then the variable FrameNumWrap is derived as

if( FrameNum > frame_num )
	FrameNumWrap = FrameNum – MaxFrameNum
else
	FrameNumWrap = FrameNum

where the value of frame_num used the equation is the frame_num in the slice header(s) for the current picture.

with the following

The variables FrameNum, FrameNumWrap, PicNum, LongTermFrameIdx, and LongTermPicNum are used for the initialisation process for reference picture lists in subclause S.9.2.4.2, the modification process for reference picture lists in subclause S.9.2.4.3, the construction process of update picture lists in subclause S.9.2.6, and for the decoded reference picture marking process in subclause S.9.2.5.

To each short-term reference picture the variables FrameNum and FrameNumWrap are assigned as follows. First, FrameNum is set equal to the syntax element frame_num that has been decoded in the slice header(s) of the corresponding short-term reference picture. Then the variable FrameNumWrap is derived as

if( FrameNum > baseFrameNum )
	FrameNumWrap = FrameNum – MaxFrameNum
else
	FrameNumWrap = FrameNum

where the value of baseFrameNum is derived as follows

- If this process if invoked prior to the initialisation process for reference picture lists in subclause S.9.2.4.2 or prior to the decoded reference picture marking process in subclause S.9.2.5, baseFrameNum is set equal to frame_num of the current picture CurrPic.

- Otherwise (this process if invoked prior to the construction process of update picture lists in subclause S.9.2.6), baseFrameNum is set equal to frame_num of the picture updPic, which is an input to the construction process of update picture lists in subclause S.9.2.6

#### Initialisation process for reference picture lists

The specification of this subclause in AVC shall apply with the following modifications.

1) Replace the following paragraphs of this subclause in AVC

This initialisation process is invoked when decoding a P, SP, or B slice header.

Outputs of this process are initial reference picture list RefPicList0, and when decoding a B slice, initial reference picture list RefPicList1.

with the following

This initialisation process is invoked when decoding a P, SP, B, EP, or EB slice header.

Outputs of this process are initial reference picture list RefPicList0, and when decoding a B or EB slice, initial reference picture list RefPicList1.

##### Initialisation process for the reference picture list for P and SP slices in frames

The specification of this subclause in AVC shall apply with the following modifications.

1) After the paragraph of this subclause in AVC starting with “Output of this process …”, insert the following paragraph

For the initialisation process of the reference picture list RefPicList0 in this subclause, only reference pictures with the syntax element dependency_id that is equal to the value of the syntax element dependency_id of the current picture are considered.

##### Initialisation process for the reference picture list for P and SP slices in field

The specification of this subclause in AVC shall apply with the following modifications.

1) After the paragraph of this subclause in AVC starting with “Output of this process …”, insert the following paragraph

For the initialisation process of the reference picture list RefPicList0 in this subclause, only reference pictures with the syntax element dependency_id that is equal to the value of the syntax element dependency_id of the current picture are considered.

##### Initialisation process for the reference picture list for B slices in frames

The specification of this subclause in AVC shall apply with the following modifications.

1) After the paragraph of this subclause in AVC starting with “Output of this process …”, insert the following paragraph

For the initialisation process of the reference picture lists RefPicList0 and RefPicList1 in this subclause, only reference pictures with the syntax element dependency_id that is equal to the value of the syntax element dependency_id of the current picture are considered.

##### Initialisation process for the reference picture list for B slices in fields

The specification of this subclause in AVC shall apply with the following modifications.

1) After the paragraph of this subclause in AVC starting with “Output of this process …”, insert the following paragraph

For the initialisation process of the reference picture lists RefPicList0 and RefPicList1 in this subclause, only reference pictures with the syntax element dependency_id that is equal to the value of the syntax element dependency_id of the current picture are considered.

##### Initialisation process for reference picture lists in fields

The specification of this subclause in AVC shall apply.

##### Initialisation process for the reference picture list for EP slices in frames

This initialisation process is invoked when decoding a EP slice in a coded frame.

Output of this process is the initial reference picture list RefPicList0.

For the initialisation process of the reference picture list RefPicList0 in this subclause, only reference pictures for which all of the following conditions are true are considered.

- the syntax element dependency_id of the picture is equal to the value of the syntax element dependency_id of the current picture

- the syntax element temporal_level of the picture is equal to or less than the syntax element temporal_level of the current picture . [Ed. Note(JR/HS): This conditions shall be removed in the future. It describes the current software implementation; but in future a reference list handling for enabling temporal scalability shall be enabled as an encoder issue by using appropriate reference picture list reordering commands.]

- when key_picture_flag  is equal to 1, the picture is marked as “base representation”

- when key_picture_flag  is equal to 0, the picture is not marked as “base representation”

- When this process is invoked, there shall be at least one reference frame or complementary reference field pair that is currently marked as "used for short-term reference" or "used for long-term reference".

For EP slices, the order of short-term reference pictures in the reference picture list RefPicList0 depends on output order, as given by PicOrderCnt( ).  When pic_order_cnt_type is equal to 0, reference pictures that are marked as "non-existing" as specified in subclause S.9.2.5.2 are not included in RefPicList0.

NOTE – When gaps_in_frame_num_value_allowed_flag is equal to 1, encoders should use reference picture list reordering to ensure proper operation of the decoding process (particularly when pic_order_cnt_type is equal to 0, in which case PicOrderCnt( ) is not inferred for "non-existing" frames).

The reference picture list RefPicList0 is ordered such that short-term reference frames and short-term complementary reference field pairs have lower indices than long-term reference frames and long-term complementary reference field pairs. It is ordered as follows.

- Short-term reference frames and short-term complementary reference field pairs are ordered starting with the short-term reference frame or complementary reference field pair frm0 with the largest value of PicOrderCnt( frm0 ) less than the value of PicOrderCnt( CurrPic ) and proceeding through in descending order to the short-term reference frame or complementary reference field pair frm1 that has the smallest value of PicOrderCnt( frm1 ), and then continuing with the short-term reference frame or complementary reference field pair frm2 with the smallest value of PicOrderCnt( frm2 ) greater than the value of PicOrderCnt( CurrPic ) of the current frame and proceeding through in ascending order to the short-term reference frame or complementary reference field pair frm3 that has the largest value of PicOrderCnt( frm3 ).

- The long-term reference frames and long-term complementary reference field pairs are ordered starting with the long-term reference frame or complementary reference field pair that has the lowest LongTermPicNum value and proceeding through in ascending order to the long-term reference frame or complementary reference field pair that has the highest LongTermPicNum value.

##### Initialisation process for the reference picture list for EP slices in fields

To be specified. [Ed. Note(HS): Field pictures are currently not supported in the JSVM software.]

##### Initialisation process for the reference picture list for EB slices in frames

This initialisation process is invoked when decoding a EB slice in a coded frame.

Output of this process are the initial reference picture lists RefPicList0 and RefPicList1.

For the initialisation process of the reference picture lists RefPicList0 and RefPicList1 in this subclause, only reference pictures for which all of the following conditions are true are considered.

- the syntax element dependency_id of the picture is equal to the value of the syntax element dependency_id of the current picture

- the syntax element temporal_level of the picture is equal to or less than the syntax element temporal_level of the current picture [Ed. Note(JR/HS): This conditions shall be removed in the future. It describes the current software implementation; but in future a reference list handling for enabling temporal scalability shall be enabled as an encoder issue by using appropriate reference picture list reordering commands.]

- when when key_picture_flag  is equal to 1, the picture is marked as „base representation“

- when when key_picture_flag  is equal to 0, the picture is not marked as “base representation”

- When this process is invoked, there shall be at least one reference frame or complementary reference field pair that is currently marked as "used for short-term reference" or "used for long-term reference".

For EB slices, the order of short-term reference pictures in the reference picture lists RefPicList0 and RefPicList1 depends on output order, as given by PicOrderCnt( ).  When pic_order_cnt_type is equal to 0, reference pictures that are marked as "non-existing" as specified in subclause S.9.2.5.2 are not included in either RefPicList0 or RefPicList1.

NOTE – When gaps_in_frame_num_value_allowed_flag is equal to 1, encoders should use reference picture list reordering to ensure proper operation of the decoding process (particularly when pic_order_cnt_type is equal to 0, in which case PicOrderCnt( ) is not inferred for "non-existing" frames).

The reference picture list RefPicList0 is ordered such that short-term reference frames and short-term complementary reference field pairs have lower indices than long-term reference frames and long-term complementary reference field pairs. It is ordered as follows.

- Short-term reference frames and short-term complementary reference field pairs are ordered starting with the short-term reference frame or complementary reference field pair frm0 with the largest value of PicOrderCnt( frm0 ) less than the value of PicOrderCnt( CurrPic ) and proceeding through in descending order to the short-term reference frame or complementary reference field pair frm1 that has the smallest value of PicOrderCnt( frm1 ), and then continuing with the short-term reference frame or complementary reference field pair frm2 with the smallest value of PicOrderCnt( frm2 ) greater than the value of PicOrderCnt( CurrPic ) of the current frame and proceeding through in ascending order to the short-term reference frame or complementary reference field pair frm3 that has the largest value of PicOrderCnt( frm3 ).

- The long-term reference frames and long-term complementary reference field pairs are ordered starting with the long-term reference frame or complementary reference field pair that has the lowest LongTermPicNum value and proceeding through in ascending order to the long-term reference frame or complementary reference field pair that has the highest LongTermPicNum value.

The reference picture list RefPicList1 is ordered so that short-term reference frames and short-term complementary reference field pairs have lower indices than long-term reference frames and long-term complementary reference field pairs. It is ordered as follows.

- Short-term reference frames and short-term complementary reference field pairs are ordered starting with the short-term reference frame or complementary reference field pair frm4 with the smallest value of PicOrderCnt( frm4 ) greater than the value of PicOrderCnt( CurrPic ) of the current frame and proceeding through in ascending order to the short-term reference frame or complementary reference field pair frm5 that has the largest value of PicOrderCnt( frm5 ), and then continuing with the short-term reference frame or complementary reference field pair frm6 with the largest value of PicOrderCnt( frm6 ) less than the value of PicOrderCnt( CurrPic ) of the current frame and proceeding through in descending order to the short-term reference frame or complementary reference field pair frm7 that has the smallest value of PicOrderCnt( frm7 ).

- Long-term reference frames and long-term complementary reference field pairs are ordered starting with the long-term reference frame or complementary reference field pair that has the lowest LongTermPicNum value and proceeding through in ascending order to the long-term reference frame or complementary reference field pair that has the highest LongTermPicNum value.

- When the reference picture list RefPicList1 has more than one entry and RefPicList1 is identical to the reference picture list RefPicList0, the first two entries RefPicList1[0] and RefPicList1[1] are switched.

NOTE – A non-paired reference field is not used for inter prediction of frames independent of the value of MbaffFrameFlag.

##### Initialisation process for the reference picture list for EB slices in fields

To be specified. [Ed. Note(HS): Field pictures are currently not supported in the JSVM software.]

#### Reordering process for reference picture lists

The specification of this subclause in AVC shall apply with the following additions (inserted at the end of section 8.2.4.3).

The variable ResidualPictureFlag is set equal to 0.

For each picture refPic of the reference picture list RefPicList0, the following applies.

- When the picture refPic is marked as “residual picture” or the variable NumberOfUpdateLevel of the picture refPic is greater than the variable NumberOfUpdateLevel of the current picture, the variable ResidualPictureFlag is set equal to 1.

When slice_type is equal to EB, for each picture refPic of the reference picture list RefPicList1, the following applies.

- When the picture refPic is marked as ”residual picture“ or the variable NumberOfUpdateLevel of the picture refPic is greater than the variable NumberOfUpdateLevel of the current picture, the variable ResidualPictureFlag is set to equal to 1

### Decoded reference picture marking process

The specification of this subclause in AVC shall apply with the following additions (inserted at the end of section 8.2.5).

When the variable key_picture_flag is equal to 1, the following applies

- The decoded picture is duplicated, and the duplicated version is marked as “base representation” and is not updated by following progresive refinement slices. [Ed. Note(JR) This should be completed]

When the variable ResidualPictureFlag is equal to 1, the following applies

- The picture is marked as “residual picture”

#### Sequence of operations for decoded reference picture marking process

The specification of this subclause in AVC shall apply.

#### Decoding process for gaps in frame_num

The specification of this subclause in AVC shall apply.

#### Sliding window decoded reference picture marking process

The specification of this subclause in AVC shall apply with the following modifications.

1) After the first paragraph of this subclause in AVC starting with “This process is invoked …”, insert the following paragraph

For the this process, only pictures with the syntax element dependency_id that is equal to the value of the syntax element dependency_id of the current picture are considered.

#### Adaptive memory control decoded reference picture marking process

The specification of this subclause in AVC shall apply.

##### Marking process of a short-term picture as “unused for reference”

The specification of this subclause in AVC shall apply with the following modifications.

1) After the first paragraph of this subclause in AVC starting with “This process is invoked …”, insert the following paragraph

For the this process, only pictures with the syntax element dependency_id that is equal to the value of the syntax element dependency_id of the current picture are considered.

##### Marking process of a long-term picture as “unused for reference”

The specification of this subclause in AVC shall apply with the following modifications.

1) After the first paragraph of this subclause in AVC starting with “This process is invoked …”, insert the following paragraph

For the this process, only pictures with the syntax element dependency_id that is equal to the value of the syntax element dependency_id of the current picture are considered.

##### Assignment process of a LongTermFrameIdx to a short-term reference picture

The specification of this subclause in AVC shall apply with the following modifications.

1) After the first paragraph of this subclause in AVC starting with “This process is invoked …”, insert the following paragraph

For the this process, only pictures with the syntax element dependency_id that is equal to the value of the syntax element dependency_id of the current picture are considered.

##### Decoding process for MaxLongTermFrameIdx

The specification of this subclause in AVC shall apply with the following modifications.

1) After the first paragraph of this subclause in AVC starting with “This process is invoked …”, insert the following paragraph

For the this process, only pictures with the syntax element dependency_id that is equal to the value of the syntax element dependency_id of the current picture are considered.

##### Marking process of all reference pictures as “unused for reference” and setting MaxLongTermFrameIdx to “no long-term frame indices”

The specification of this subclause in AVC shall apply with the following modifications.

1) After the first paragraph of this subclause in AVC starting with “This process is invoked …”, insert the following paragraph

For the this process, only pictures with the syntax element dependency_id that is equal to the value of the syntax element dependency_id of the current picture are considered.

##### Process for assigning a long-term frame index to the current picture

The specification of this subclause in AVC shall apply with the following modifications.

1) After the first paragraph of this subclause in AVC starting with “This process is invoked …”, insert the following paragraph

For the this process, only pictures with the syntax element dependency_id that is equal to the value of the syntax element dependency_id of the current picture are considered.

### Derivation process for update picture lists

This process is invoked before the update process of the current picture.

Inputs of this process are

- a picture updPic

- a variable updLevel

Outputs of this process are the update picture lists updPicList0 and updPicList1.

For the derivation process of the updates picture lists updPicList0 and updPicList1 in this subclause, only pictures for which all of the following conditions are true are considered.

- the syntax element dependency_id of the picture is equal to the value of the syntax element dependency_id of the picture updPic

- the syntax element temporal_level of the picture is equal to updLevel

- the variable ResidualPictureFlag of the picture is equal to 1

The order of reference pictures in the update picture lists updPicList0 and updPicList1 depends on output order, as given by PicOrderCnt( ).  When pic_order_cnt_type is equal to 0, reference pictures that are marked as "non-existing" as specified in subclause S.9.2.5.2 are not included in either updPicList0 or updPicList1.

The update picture list updPicList0 is ordered as follows.

- Reference frames and complementary reference field pairs are ordered starting with the reference frame or complementary reference field pair frm0 with the largest value of PicOrderCnt( frm0 ) less than the value of PicOrderCnt( updPic ) and proceeding through in descending order to the reference frame or complementary reference field pair frm1 that has the smallest value of PicOrderCnt( frm1 ), and then continuing with the reference frame or complementary reference field pair frm2 with the smallest value of PicOrderCnt( frm2 ) greater than the value of PicOrderCnt( updPic ) and proceeding through in ascending order to the reference frame or complementary reference field pair frm3 that has the largest value of PicOrderCnt( frm3 ).

The update picture list updPicList1 is ordered as follows.

- Reference frames and complementary reference field pairs are ordered starting with the reference frame or complementary reference field pair frm4 with the smallest value of PicOrderCnt( frm4 ) greater than the value of PicOrderCnt( updPic ) and proceeding through in ascending order to the reference frame or complementary reference field pair frm5 that has the largest value of PicOrderCnt( frm5 ), and then continuing with the reference frame or complementary reference field pair frm6 with the largest value of PicOrderCnt( frm6 ) less than the value of PicOrderCnt( updPic ) and proceeding through in descending order to the reference frame or complementary reference field pair frm7 that has the smallest value of PicOrderCnt( frm7 ).

When the number of entries in updPicList0 or updPicList1 is greater than num_ref_idx_update_l0_active[ updLevel ] or num_ref_idx_update_l1_active[ updLevel ] for the picture updPic, respectively, the extra entries past position num_ref_idx_update_l0_active[ updLevel ] or num_ref_idx_update_l1_active[ updLevel ] are discarded from the update picture list.

## Intra prediction process

The specification of this subclause in AVC shall apply with the following modifications

1) Replace this subclause in AVC with the following

This process is invoked for I and SI macroblock types.

Inputs to this process are constructed samples prior to the deblocking filter process from neighbouring macroblocks and

- for Intra_NxN (where NxN is equal to 4x4 or 8x8) prediction mode, the associated values of IntraNxNPredMode from neighbouring macroblocks

- for Intra_Base prediction mode, the constructed samples prior to the deblocking filter process from the base macroblock and its neighbouring macroblocks.

Outputs of this process are specified as follows.

- If mb_type is not equal to I_PCM, the Intra prediction samples of components of the macroblock or in case of the Intra_NxN prediction process for luma samples, the outputs are NxN luma sample arrays as part of the 16x16 luma array of prediction samples of the macroblock.

- Otherwise (mb_type is equal to I_PCM), constructed macroblock samples prior to the deblocking filter process.

The variable MvCnt is set equal to 0.

Depending on the value of mb_type the following applies.

- If mb_type is equal to I_PCM, the process specified in subclause S.9.3.5 is invoked.

- Otherwise, if the macroblock prediction mode is equal to Intra_Base, the process specified in subclause S.9.3.6 is invoked.

- Otherwise (mb_type is not equal to I_PCM and the macroblock prediction mode is not equal to Intra_Base), the following applies.

- The decoding processes for Intra prediction modes are described for the luma component as follows.

- If the macroblock prediction mode is equal to Intra_4x4, the specification in subclause S.9.3.1 applies.

- Otherwise, if the macroblock prediction mode is equal to Intra_8x8, the specification in subclause S.9.3.2 applies.

- Otherwise (the macroblock prediction mode is equal to Intra_16x16), the specification in subclause S.9.3.3 applies.

- The decoding processes for Intra prediction modes for the chroma components are described in subclause S.9.3.4. This process shall only be invoked when chroma_format_idc is not equal to 0 (monochrome).

Samples used in the Intra prediction process shall be sample values prior to alteration by any deblocking filter operation.

NOTE – For the Intra_Base prediction process, the samples values that are used as input are sample values prior to the deblocking filter process, but these samples may be modified before they are used for intra prediction by deblocking filter operations as specified in subclause S.9.3.6.

### Intra_4x4 prediction process for luma samples

The specification of this subclause in AVC shall apply.

#### Derivation process for the Intra4x4PredMode

The specification of this subclause in AVC shall apply.

#### Intra_4x4 sample prediction

The specification of this subclause in AVC shall apply with the following modifications.

1) Replace the following paragraph of this section in AVC

- Otherwise, the sample p[ x, y ] is marked as “available for Intra_4x4 prediction” and the luma sample at luma location ( xW, yW ) inside the macroblock mbAddrN is assigned to p[ x, y ].

with the following.

- Otherwise, the sample p[ x, y ] is marked as “available for Intra_4x4 prediction” and the luma sample at luma location ( xW, yW ) inside the macroblock mbAddrN that has been reconstructed with quality_level equal to 0 is assigned to p[ x, y ].

NOTE – The neighbouring samples p[ x, y ] are reconstructed without quality enhancement that are possibly present in progressive refinement slices.

### Intra_8x8 prediction process for luma samples

The specification of this subclause in AVC shall apply.

#### Derivation process for the Intra8x8PredMode

The specification of this subclause in AVC shall apply.

#### Intra_8x8 sample prediction

The specification of this subclause in AVC shall apply with the following modifications.

1) Replace the following paragraph of this section in AVC

- Otherwise, the sample p[ x, y ] is marked as “available for Intra_8x8 prediction” and the luma sample at luma location ( xW, yW ) inside the macroblock mbAddrN is assigned to p[ x, y ].

with the following.

- Otherwise, the sample p[ x, y ] is marked as “available for Intra_8x8 prediction” and the luma sample at luma location ( xW, yW ) inside the macroblock mbAddrN that has been reconstructed with quality_level equal to 0 is assigned to p[ x, y ].

NOTE – The neighbouring samples p[ x, y ] are reconstructed without quality enhancement that are possibly present in progressive refinement slices.

### Intra_16x16 prediction process for luma samples

The specification of this subclause in AVC shall apply with the following modifications.

1) Replace the following paragraph of this section in AVC

- Otherwise, the sample p[ x, y ] is marked as “available for Intra_8x8 prediction” and the luma sample at luma location ( xW, yW ) inside the macroblock mbAddrN is assigned to p[ x, y ].

with the following.

- Otherwise, the sample p[ x, y ] is marked as “available for Intra_16x16 prediction” and the luma sample at luma location ( xW, yW ) inside the macroblock mbAddrN that has been reconstructed with quality_level equal to 0 is assigned to p[ x, y ].

NOTE – The neighbouring samples p[ x, y ] are reconstructed without quality enhancement that are possibly present in progressive refinement slices.

### Intra prediction process for chroma samples

The specification of this subclause in AVC shall apply with the following modifications.

1) Replace the following paragraph of this section in AVC

- Otherwise, the sample p[ x, y ] is marked as “available for Intra chroma prediction” and the chroma sample at chroma location ( xW, yW ) inside the macroblock mbAddrN is assigned to p[ x, y ].

with the following.

- Otherwise, the sample p[ x, y ] is marked as “available for Intra chroma prediction” and the chroma sample at chroma location ( xW, yW ) inside the macroblock mbAddrN that has been reconstructed with quality_level equal to 0 is assigned to p[ x, y ].

NOTE – The neighbouring samples p[ x, y ] are reconstructed without quality enhancement that are possibly present in progressive refinement slices.

### Sample construction process for I_PCM macroblocks

The specification of this subclause in AVC shall apply.

### Intra_Base prediction process

[Ed. Note(HS): This process is currently only specified for frame_mbs_only_flag equal to 1.]

This process is invoked when the macroblock prediction mode is equal to Intra_Base.

Input to this process are constructed samples of the base picure prior to the deblocking filter process.

Outputs of this process are

- the Intra prediction luma samples for the current macroblock predL[ x, y ] with x = 0..15, y = 0..15

- when chroma_format_idc is not equal to 0, the Intra prediction chroma samples for the current macroblock predCb[ x, y ] and predCr[ x, y ] with x = 0..MbWidthC – 1, y = 0..MbHeightC – 1

The following parameters are defined based on syntax elements chroma_phase_x_plus1, chroma_phase_y_plus1, base_chroma_phase_x_plus1, base_chroma_phase_y_plus1:

ScaledBaseChromaPhaseX = chroma_phase_x_plus1 - 1

ScaledBaseChromaPhaseY = chroma_phase_y_plus1 - 1

BaseChromaPhaseX = base_chroma_phase_x_plus1 - 1

BaseChromaPhaseY = base_chroma_phase_y_plus1 - 1

If SpatialScalabilityType is equal to or less than 1, the following applies.

- The derivation process for base macroblocks in subclause S.7.5.2 is invoked with CurrMbAddr as input and the outputs are assigned to the picture basePic and array of macroblock addresses aMbAddrBase.

NOTE – The picture basePic is represented by the sample values S'L[ x, y ], S'Cb[ x, y ], and S'Cr[ x, y ] for intra macroblocks (prior to the deblocking filter process) that are reconstructed for dependency_id equal to DependencyIdBase and quality_level equal to QualityLevelBase.

- mbAddrBase is set equal to aMbAddrBase[ 0 ].

- When the syntax element constrained_intra_pred_flag is equal to 1 for the base picture basePic, the macroblock type of the macroblock mbAddrBase in the base picture basePic shall be an I macroblock type.

Otherwise ( if SpatialScalabilityType is greater than 1), the following applies.

- When the syntax element constrained_intra_pred_flag is equal to 1 for the base picture basePic, intra_base_mb(CurrMbAddr) shall be true.

NOTE – When constrained_intra_pred_flag is equal to 1, only the samples of I macroblocks of the base picture basePic need to be constructed for generating the prediction samples for macroblocks coded in Intra_Base macroblock prediction mode.

The position ( xP, yP ) of the upper-left luma sample of the current macroblock is derived by invoking the inverse macroblock scanning process in subclause S.7.4.1 with CurrMbAddr as input and the output is assigned to ( xP, yP ).

When chroma_format_idc is not equal to 0, the position ( xC, yC ) of the upper-left chroma samples of the current macroblock are derived by

xC = xP / SubWidthC
yC = yP / SubHeightC

Positions in base layer are computed as follows.

- If SpatialScalabilityType is equal to 0, the following applies.

xB = xP – ScaledBaseLeftOffset
yB = yP – ScaledBaseTopOffset

- When chroma_format_idc is not equal to 0, let ( xCB, yCB ) be computed as:

xCB = xC – ScaledBaseLeftOffsetC
yCB = yC – ScaledBaseTopOffsetC

- Otherwise if SpatialScalabilityType is equal to 1, the following applies.

xB = ( xP – ScaledBaseLeftOffset ) / 2
yB = ( yP – ScaledBaseTopOffset ) / 2

xB1 = xB + 7
yB1 = yB + 7

- When chroma_format_idc is not equal to 0, let ( xCB, yCB ) be computed as:

xCB = ( xC – ScaledBaseLeftOffsetC ) / 2
yCB = ( yC – ScaledBaseTopOffsetC ) / 2

xCB1 = xCB + BaseMbWidthC /2 – 1
yCB1 = yCB + BaseMbHeightC /2 – 1

- Otherwise (SpatialScalabilityType is greater than 1), the following applies.

- Let deltaX and deltaY be defined as follows.

deltaX = BasePicWidth – ScaledBaseWidth
deltaY = BasePicHeight – ScaledBaseHeight

- The corresponding fractional-sample position ( xf, yf ) in base layer of ( xP , yP ) is derived as follows.

xf = ( 4 * ( xP – ScaledBaseLeftOffset ) * BasePicWidth +  2 * deltaX ) // ScaledBaseWidth
yf = ( 4 * ( yP – ScaledBaseTopOffset ) * BasePicHeight + 2 * deltaY ) // ScaledBaseHeight

- the full-sample position ( xB, yB ) is derived as follows.

xB = xf >> 2
yB = yf >> 2

- The corresponding fractional-sample position ( xf1, yf1 ) in base layer of ( xP + 15 , yP + 15 ) is derived as follows.

xf1 = ( 4 * ( xP + 15 – ScaledBaseLeftOffset ) * BasePicWidth + 2 * deltaX ) // ScaledBaseWidth
yf1 = ( 4 * ( yP + 15 – ScaledBaseTopOffset ) * BasePicHeight + 2 * deltaY ) // ScaledBaseHeight

- the full-sample position ( xB1, yB1 ) is derived as follows.

xB1 = xf1 >> 2
yB1 = yf1 >> 2

- When chroma_format_idc is not equal to 0, the following applies.

- Let deltaXC and deltaYC be defined as follows

deltaXC = ( 2 + BaseChromaPhaseX) * BasePicWidthC – ( 2 + ScaledBaseChromaPhaseX ) * ScaledBaseWidthC
deltaYC = ( 2 + BaseChromaPhaseY) * BasePicHeightC – ( 2 + ScaledBaseChromaPhaseY ) * ScaledBaseHeightC

- Let ( xfC, yfC ) be defined as follows

xfC = ( 4 * ( xC – ScaledBaseLeftOffsetC ) * BasePicWidthC + deltaXC ) // ScaledBaseWidthC
yfC = ( 4 * ( yC – ScaledBaseTopOffsetC ) * BasePicHeightC + deltaYC ) // ScaledBaseHeightC

- the full-sample position ( xCB, yCB ) is derived as follows.

xCB = xfC >> 2
yCB = yfC >> 2

- Let ( xfC1, yfC1 ) be defined as follows

xfC1 = ( 4 * ( xC + MbWidthC – 1 – ScaledBaseLeftOffsetC ) * BasePicWidthC + deltaXC ) // ScaledBaseWidthC
yfC1 = ( 4 * ( yC + MbHeightC – 1 – ScaledBaseTopOffsetC ) * BasePicHeightC + deltaYC ) // ScaledBaseHeightC

- the full-sample position ( xCB1, yCB1 ) is derived as follows.

xCB1 = xfC1 >> 2
yCB1 = yfC1 >> 2

The Intra prediction luma and chroma samples for the current macroblock are derived as follows.

- If SpatialScalabilityType is equal to 0, the following applies.

- Let S'Base,L[ x, y ] be the array of constructed luma samples S'L[ x, y ] prior to the deblocking filter process of the picture basePic that has been reconstructed with dependency_id equal to DependencyIdBase and quality_level equal to QualityLevelBase.

- When chromat_format_idc is not equal to 0, let S'Base,Cb[ x, y ] and S'Base,Cr[ x, y ] be the arrays of constructed chroma samples S'Cb[ x, y ] and S'Cr[ x, y ], respectively, prior to the deblocking filter process of the picture basePic that has been reconstructed with dependency_id equal to DependencyIdBase and quality_level equal to QualityLevelBase.

- Each Intra prediction luma sample predL[ x, y ] with x = 0..15 and y = 0..15 is derived by

predL[ x, y ] = S'Base,L[ xB + x, yB + y ]

- When chroma_format_idc is not equal to 0, each Intra prediction chroma sample predCb[ x, y ] and predCr[ x, y ] with x = 0..MbWidthC – 1 and y = 0..MbHeightC – 1 is derived by

predCb[ x, y ] = S'Base,Cb[ xCB + x, yCB + y ]
predCr[ x, y ] = S'Base,Cr[ xCB + x, yCB + y ]

- Otherwise (SpatialScalabilityType is not equal to 0), the following applies.

- The deblocking filter process in subclause S.9.3.6.1 is invoked with the picture basePic that has been reconstructed with dependency_id equal to DependencyIdBase and quality_level equal to QualityLevelBase as input and the outputs are the luma sample array S'Base,L[ x, y ], and, when chroma_format_idc is not equal to 0, the chroma sample arrays S'Base,Cb[ x, y ] and S'Base,Cr[ x, y ].

- Let baseL[ x, y ] with x = –4..( xB1 –xB + 4 ), y = –4..( yB1 – yB + 4 ) be a luma sample array and, when chroma_format_idc is not equal to 0, let baseCb[ x, y ] and baseCr[ x, y ] with x = –4..( xCB1 – xCB + 4 ) and y = –4..( yCB1 – yCB + 4 ) be two chroma samples arrays.

- Each luma sample baseL[ x, y ] with x = –4..( xB1 –xB + 4 ), y = –4..( yB1 – yB + 4 ) of the luma sample array is derived as follows.

- If any of the following conditions is true, the sample baseL[ x, y ] is marked as “not existing”.

- x is less than ( –xB ) or x is greater than ( ( BasePicWidth  – xB ) – 1 )

- y is less than ( –yB ) or y is greater than ( ( BasePicWidth  – yB ) – 1 )

- Otherwise, if the syntax element constrained_intra_pred_flag is equal to 1 for the base picture basePic and the sample S'Base,L[ xB + x, yB + y ] does not represent a sample of an I macroblock, the sample baseL[ x, y ] is marked as “not available for Intra_Base prediction”.

- Otherwise, the sample baseL[ x, y ] is marked as “available for Intra_Base prediction” and derived by

baseL[ x, y ] = S'Base,L[ xB + x, yB + y ]

- When chroma_format_idc is not equal to 0, each chroma sample baseCb[ x, y ] and baseCr[ x, y ] with 
x = –4..( xCB1 – xCB + 4 ) and y = –4..( yCB1 – yCB + 4 ) of the chroma sample arrays is derived as follows.

- If any of the following conditions is true, the samples baseCb[ x, y ] and baseCr[ x, y ] are marked as “not existing”.

- x is less than ( –xCB ) or x is greater than ( ( BasePicWidthC – xCB )– 1 )

- y is less than ( –yCB ) or y is greater than ( ( BasePicHeightC – yCB ) – 1 )

- Otherwise, if the syntax element constrained_intra_pred_flag is equal to 1 for the base picture basePic and the samples S'Base,Cb[ xCB + x, yCB + y ] and S'Base,Cr[ xCB + x, yCB + y ] do not represent samples of an I macroblock, the samples baseCb[ x, y ] and baseCr[ x, y ] are marked as “not available for Intra_Base prediction”:

- Otherwise, the samples baseCb[ x, y ] and baseCr[ x, y ] are marked as “available for Intra_Base prediction” and derived by

baseCb[ x, y ] = S'Base,Cb[ xCB + x, yCB + y ]
baseCr[ x, y ] = S'Base,Cr[ xCB + x, yCB + y ]

- The construction process for not available or not existing samples in subclause S.9.3.6.2 is invoked with the sample array baseL[ x, y ] and, when chroma_format_idc is not equal to 0, the sample arrays baseCb[ x, y ] and baseCr[ x, y ] as inputs and the outputs are the sample array baseL[ x, y ] with modified sample values, and when chromat_format_idc is not equal to 0, the sample arrays baseCb[ x, y ] and baseCr[ x, y ] with modified sample values.

- If SpatialScalabilityType is equal to 1, the following applies.

- The upsampling process for Intra_Base prediction in subclause S.9.3.6.3 is invoked with the sample array baseL[ x, y ], and when chroma_format_idc is not equal to 0, the sample arrays baseCb[ x, y ] and baseCr[ x, y ] as inputs and the outputs are predL[ x, y ], and when chroma_format_idc is not equal to 0, predCb[ x, y ] and predCr[ x, y ].

- Otherwise (SpatialScalabilityType is not equal to 1), the following applies.

- The upsampling process for Intra_Base prediction in subclause S.9.3.6.4 is invoked with the sample array baseL[ x, y ], integer luma sample positions in basePic (xB, yB) and (xB1, yB1), and when chroma_format_idc is not equal to 0, the sample arrays baseCb[ x, y ] and baseCr[ x, y ], and integer chroma sample positions in basePic (xCB, yCB) and (xCB1, yCB1) as inputs and the outputs are predL[ x, y ], and when chroma_format_idc is not equal to 0, predCb[ x, y ] and predCr[ x, y ].

#### Deblocking filter process for Intra_Base prediction

Inputs to this process is a picture basePic represented by

- its syntax elements

- the luma sample array S'L[ x, y ] containing constructed luma samples prior to the deblocking filter process

- when chroma_format_idc is not equal to 0, the chroma sample arrays S'Cb[ x, y ] and S'Cr[ x, y ] containing constructed chroma samples priori to the deblocking filter process

Outputs of this process are

- the luma sample array S'L[ x, y ] with modified sample values

- when chroma_format_idc is not equal to 0, the two chroma sample arrays S'Cb[ x, y ] and S'Cr[ x, y ] with modified sample values

The specification of this subclause is identical to the specification of subclause S.9.7 with the following modifications.

[Ed. Note(HS): Instead of repeating subclause S.8.7, the subclauses S.8.7 and S.8.7.1 should be slightly modified and used for this process as well as the “normal” deblocking filter process.]

1) Replace the beginning of subclause S.9.7 inclusive of the figure and the following paragraph

For each macroblock in ascending order of mbAddr, the following applies.

with the following

For each macroblock in ascending order of mbAddr of the picture basePic, the following applies.

2) Replace the following paragraphs of subclause S.9.7

- The variable filterInternalEdgesFlag is derived as follows.

- If disable_deblocking_filter_idc for the slice that contains the macroblock mbAddr is equal to 1, the variable filterInternalEdgesFlag is set equal to 0;

- Otherwise (disable_deblocking_filter_idc for the slice that contains the macroblock mbAddr is not equal to 1), the variable filterInternalEdgesFlag is set equal to 1.

- The variable filterLeftMbEdgeFlag is derived as follows.

- If any of the following conditions is true, the variable filterLeftMbEdgeFlag is set equal to 0.

- the left vertical macroblock edge of the macroblock mbAddr represents a picture boundary

- disable_deblocking_filter_idc for the slice that contains the macroblock mbAddr is equal to 1

- disable_deblocking_filter_idc for the slice that contains the macroblock mbAddr is equal to 2 and the left vertical macroblock edge of the macroblock mbAddr represents a slice boundary

- Otherwise, the variable filterLeftMbEdgeFlag is set equal to 1.

- The variable filterTopMbEdgeFlag is derived as follows.

- If any of the following conditions is true, the variable filterTopMbEdgeFlag is set equal to 0.

- the top horizontal macroblock edge of the macroblock mbAddr represents a picture boundary

- disable_deblocking_filter_idc for the slice that contains the macroblock mbAddr is equal to 1

- disable_deblocking_filter_idc for the slice that contains the macroblock mbAddr is equal to 2 and the top horizontal macroblock edge of the macroblock mbAddr represents a slice boundary

- Otherwise, the variable filterTopMbEdgeFlag is set equal to 1.

with the following

- The variable filterInternalEdgesFlag is derived as follows.

- If any of the following conditions is true, the variable filterInternalEdgesFlag is set equal to 0.

- disable_deblocking_filter_idc for the slice that contains the macroblock mbAddr is equal to 1

- constrained_intra_pred_flag is equal to 1 and the macroblock mbAddr is coded using an Inter macroblock prediction mode.

- Otherwise, the variable filterInternalEdgesFlag is set equal to 1.

- The variable filterLeftMbEdgeFlag is derived as follows.

- If any of the following conditions is true, the variable filterLeftMbEdgeFlag is set equal to 0.

- the left vertical macroblock edge of the macroblock mbAddr represents a picture boundary

- disable_deblocking_filter_idc for the slice that contains the macroblock mbAddr is equal to 1

- disable_deblocking_filter_idc for the slice that contains the macroblock mbAddr is equal to 2 and the left vertical macroblock edge of the macroblock mbAddr represents a slice boundary

- constrained_intra_pred_flag is equal to 1 and any of the following conditions is true

- the macroblock mbAddr is coded using an Inter macroblock prediction mode

- the macroblock to the left of the macroblock mbAddr is coded using an Inter macroblock prediction mode  [Ed. Note(HS): This condition is not accurately specified. It will not work for MbAff. It should go to the subclause S.8.7.1.]

- Otherwise, the variable filterLeftMbEdgeFlag is set equal to 1.

- The variable filterTopMbEdgeFlag is derived as follows.

- If any of the following conditions is true, the variable filterTopMbEdgeFlag is set equal to 0.

- the top horizontal macroblock edge of the macroblock mbAddr represents a picture boundary

- disable_deblocking_filter_idc for the slice that contains the macroblock mbAddr is equal to 1

- disable_deblocking_filter_idc for the slice that contains the macroblock mbAddr is equal to 2 and the top horizontal macroblock edge of the macroblock mbAddr represents a slice boundary

- constrained_intra_pred_flag is equal to 1 and any of the following conditions is true

- the macroblock mbAddr is coded using an Inter macroblock prediction mode

- the macroblock above the macroblock mbAddr is coded using an Inter macroblock prediction mode  [Ed. Note(HS): This condition is not accurately specified. It will not work for MbAff. It should go to the subclause S.8.7.1.]

- Otherwise, the variable filterTopMbEdgeFlag is set equal to 1.

3) Delete the last paragraph of subclause  [Ed.Note(MW): broken ref]

Finally, the arrays S’L, S’Cb, S’Cr are assigned to the arrays SL, SCb, SCr (which represent the decoded picture), respectively.

#### Construction process for not available or not existing samples

Inputs to this process are

- a luma sample array baseL[ x, y ] with x = –8..15 and y =–8..15

- when chroma_format_idc is not equal to 0, two chroma sample arrays baseCb[ x, y ] and baseCr[ x, y ] with 
x = –MbWidthC / 2..MbWidthC – 1 and y = –MbHeightC / 2..MbHeightC – 1

Outputs of this process are

- the luma sample array baseL[ x, y ] with modified sample values

- when chroma_format_idc is not equal to 0, the two chroma sample arrays baseCb[ x, y ] and baseCr[ x, y ] with modified modified sample values

For each component Z (with Z being L, when chroma_format_idc is equal to 0, or Z being L, Cb, or Cr, otherwise), the following applies.

- The width wZ and height hZ of the block that is upsampled are derived as follows

wZ = ( Z  = =  Y  ?  8  :  ( MbWidthC  / 2 ) )
hZ  = ( Z  = =  Y  ?  8  :  ( MbHeightC / 2 ) )

- For each sample baseZ[ x, y ] with x proceeding over –1, 0, wZ – 1, wZ, and y proceeding over –1, 0, hZ – 1, hZ, the following applies.

- If the sample baseZ[ x, y ] is marked as “available for Intra_Base prediction”, the values of the sample array baseZ[ x, y ] are not modified.

- Otherwise, if the sample baseZ[ x, y ] is marked as “not existing”, the following applies.

- A location difference ( xD, yD ) is derived by

xD = ( x  = =  –1  ?  –1  :  x  = =  widthZ   ?  1  : 0 )
yD = ( y  = =  –1  ?  –1  :  y  = =  heightZ  ?  1  : 0 )

- The constant border extension process in subclause S.9.3.6.2.1 is invoked with variables wZ and hZ, the sample array baseZ[ x, y ], the location difference ( xD, yD ), and the location ( x, y ) as inputs and the output is the sample array baseZ[ x, y ] with modified sample values.

- Otherwise (the sample baseZ[ x, y ] is marked as “not available for Intra_Base prediction”), the following applies.

- A location difference ( xD, yD ) is derived by

xD = ( ( x % 16 )  !=  0  ?  –1  :  1 )
yD = ( ( y % 16 )  !=  0  ?  –1  :  1 )

- The sample array baseZ[ x, y ] is modified as follows.

- If both the sample baseZ[ x – xD, y ] and the sample baseZ[ x, y – yD ] are marked as “available for Intra_Base prediction”, the following applies.

- A variable cornerSampleAvailable is derived as follows.

- If the sample baseZ[ x – xD, y – yD ] is marked as “available for Intra_Base prediction”, the variable cornerSampleAvailable is set equal to 1.

- Otherwise, the variable cornerSampleAvailable is set equal to 0.

- The diagonal border extension process in subclause S.9.3.6.2.2 is invoked with the variables wZ and hZ, the sample array baseZ[ x, y ], the location difference ( xD, yD ), the location ( x, y ), and the variable cornerSampleAvailable as inputs and the output is the sample array baseZ[ x, y ] with modified sample values.

- Otherwise, the following applies.

- The location difference ( xD, yD ) is modified as follows.

- If the sample baseZ[ x – xD, y ] is marked as “available for Intra_Base prediction”, yD is set equal to 0.

- Otherwise, if the sample baseZ[ x, y – yD ] is marked as “available for Intra_Base prediction”, xD is set equal to 0.

- Otherwise, the location difference ( xD, yD ) is not modified.

- The constant border extension process in subclause S.9.3.6.2.1 is invoked with variables wZ and hZ, the sample array baseZ[ x, y ], the location difference ( xD, yD ), and the location ( x, y ) as inputs and the output is the sample array baseZ[ x, y ] with modified sample values.

##### Constant border extension process

Inputs to this process are

- variables wZ and hZ

- a sample array p[ x, y ] with x = –wZ..( 2 *wZ – 1 ) and y –hZ..( 2 *hZ – 1 )

- a location difference ( xD, yD )

- a location ( xO, yO )

Output of this process is the sample array p[ x, y ] with modified sample values.

Let x0, x1, y0, and y0 be variables that are derived as follows.

x0 = xO – ( xD < 0 ? wZ :    0 )
x1 = xO + ( xD < 0 ?    0 : wZ )
y0 = yO – ( yD < 0 ?  hZ :    0 )
y1 = yO + ( yD < 0 ?    0 :  hZ )

The values of the samples p[ x, y ] with x = x0..x1 and y = y0..y1 are modified as follows.

p[ x, y ] = p[ yO – yD, xO – xD ]

##### Diagonal border extension process

Inputs to this process are

- variables wZ and hZ

- a sample array p[ x, y ] with x = –wZ..( 2 *wZ – 1 ) and y –hZ..( 2 *hZ – 1 )

- a location difference ( xD, yD )

- a location ( xO, yO )

- a variable cornerSampleAvailable

Output of this process is the sample array p[ x, y ] with modified sample values.

Let x0, x1, y0, and y0 be variables that are derived as follows.

x0 = xO – ( xD < 0 ? wZ :    0 )
x1 = xO + ( xD < 0 ?    0 : wZ )
y0 = yO – ( yD < 0 ?  hZ :    0 )
y1 = yO + ( yD < 0 ?    0 :  hZ )

The values of the samples p[ x, y ] with x = x0..x1 and y = y0..y1 are modified as follows.

- Let d be a variable that is derived by

d = xD * ( x – xO )  –  yD * ( y – yO )

- The value of the modified sample p[ x, y ] is derived as follows.

- If d is greater than 0,

p[ x, y ] = ( p[ xO + xD * ( d – 2 ), yO – yD ] +
                    2 * p[ xO + xD * ( d – 1 ), yO – yD ] + p[ xO + xD * d, yO – yD ] + 2 ) >> 2

- Otherwise, if d is less than 0,

p[ x, y ] = ( p[ xO – xD, yO – yD * ( d + 2 ) ] +
                    2 * p[ xO – xD, yO – yD * ( d + 1 ) ] + p[ xO – xD, yO – yD * d ] + 2 ) >> 2

- Otherwise (d is equal to 0), if cornerSampleAvailable is equal to 1,

p[ x, y ] = ( p[ xO, yO – yD ] + 2 * p[ xO – xD, yO – yD ] + p[ xO – xD, yO ] + 2 ) >> 2

- Otherwise (d is equal to 0 and cornerSampleAvailable is equal to 0),

p[ x, y ] = ( p[ xO, yO – yD ] + p[ xO – xD, yO ] + 1 ) >> 1

#### Upsampling process for Intra_Base prediction

Inputs to this process are

- a luma sample array baseL[ x, y ] with x = –4..11 and y = –4..11

- when chroma_format_idc is not equal to 0, two chroma sample arrays baseCb[ x, y ] and baseCr[ x, y ] with 
x = –4..BaseMbWidthC / 2 + 3 and y = –4..BaseMbHeightC / 2 + 3

Outputs of this process are

- a luma sample array predL[ x, y ] with x = 0..15 and y = 0..15

- when chroma_format_idc is not equal to 0, two chroma sample arrays predCb[ x, y ] and predCr[ x, y ] with x = 0..MbWidthC – 1 and y = 0..MbHeightC – 1

The luma samples predL[ x, y ] with x = 0..15 and y = 0..15 are derived as follows.

- Let tempL[ x, y ] with x = –4..11 and y = 0..15 be a temporary luma sample array.

- Each sample tempL[ x, y ] with x = –4..11 and y = 0..15 is derived as follows.

- If ( y % 2 ) is equal to 0,

tempL[ x, y ] = 32 * baseL[ x, y >> 1 ]

- Otherwise,

tempL[ x, y ] = ( 20 * ( baseL[ x, ( y >> 1 ) ]       + baseL[ x, ( y >> 1 ) + 1 ] )  –
                             5 * ( baseL[ x, ( y >> 1 ) – 1 ] + baseL[ x, ( y >> 1 ) + 2 ] )  +
                                   ( baseL[ x, ( y >> 1 ) – 2 ] + baseL[ x, ( y >> 1 ) + 3 ] )     )

- Each sample predL[ x, y ] with x = 0..15 and y = 0..15 is derived as follows.

- If ( x % 2 ) is equal to 0,

predL[ x, y ] = Clip1Y( ( 32 * tempL[ x >> 1, y ] + 512 ) / 1024 )

- Otherwise,

predL[ x, y ] = Clip1Y( ( 20 * ( tempL[ (x >> 1), y ) ]       + tempL[ (x >> 1)+1, y ] )  –
                                         5 * ( tempL[ (x >> 1) - 1, y ]    + tempL[ (x >> 1)+2, y ] )  +
                                               ( tempL[ (x >> 1) - 2, y ]    + tempL[ (x >> 1)+3, y ] )  + 512 ) / 1024 )

When chroma_format_idc is not equal to 0, the chroma samples predC[ x, y ] (with C being Cb or Cr) with x = 0..MbWidthC – 1, y = 0..MbHeightC – 1 are derived as follows.

- Let tempC[ x, y ] with x = –4..BaseMbWidthC / 2 + 3and y = 0..MbHeightC – 1 be a temporary chroma sample array.

- Each sample tempC[ x, y ] with x = –4..BaseMbWidthC / 2 + 3 and y = 0..MbHeightC – 1 is derived as follows.

- If ( y % 2 ) is equal to 0,

tempC[ x, y ] = 32 * baseC[ x, y >> 1 ]

- Otherwise,

tempC[ x, y ] = ( 20 * ( baseC[ x, ( y >> 1 ) ]       + baseC[ x, ( y >> 1 ) + 1 ] )  –
                             5 * ( baseC[ x, ( y >> 1 ) – 1 ] + baseC[ x, ( y >> 1 ) + 2 ] )  +
                                   ( baseC[ x, ( y >> 1 ) – 2 ] + baseC[ x, ( y >> 1 ) + 3 ] )     )

- Each sample predC[ x, y ] with x = 0..MbWidthC – 1 and y = 0..MbHeightC – 1 is derived as follows.

- If ( x % 2 ) is equal to 0,

predC[ x, y ] = Clip1C( ( 32 * tempC[ x >> 1, y ] + 512 ) / 1024 )

- Otherwise,

predC[ x, y ] = Clip1C( ( 20 * ( tempC[ ( x >> 1 ) , y ]       + tempC[ ( x >> 1 ) + 1, y ] )  –
                                         5 * ( tempC[ ( x >> 1 ) - 1, y ]   + tempC[ ( x >> 1 ) + 2, y ] )  +
                                               ( tempC[ ( x >> 1 ) - 2, y ]   + tempC[ ( x >> 1 ) + 3, y ] )  + 512 ) / 1024 )

#### Extended spatial scalability upsampling process for Intra_Base prediction

Inputs to this process are

- integer luma sample positions in basePic ( xB, yB ) and ( xB1, yB1 )

- a luma sample array baseL[ x, y ] with x = –2..( xB1 – xB + 3) and y = –2..( yB1 – yB + 3)

- when chroma_format_idc is not equal to 0,

- integer chroma sample positions in basePic ( xCB, yCB ) and ( xCB1, yCB1 )

- two chroma sample arrays baseCb[ x, y ] and baseCr[ x, y ] with x = –2..( xCB1 – xCB + 3) and y = –2..( yCB1 – yCB + 3)

Outputs of this process are

- a luma sample array predL[ x, y ] with x = 0..15 and y = 0..15

- when chroma_format_idc is not equal to 0, two chroma sample arrays predCb[ x, y ] and predCr[ x, y ] with x = 0..MbWidthC – 1 and y = 0..MbHeightC – 1

The luma samples predL[ x, y ] with x = 0..15 and y = 0..15 are derived as follows.

- Let deltaX and deltaY be defined as follows.

deltaX = BasePicWidth – ScaledBaseWidth

deltaY = BasePicHeight – ScaledBaseHeight

- The corresponding fractional-sample position ( xf, yf ) in base layer is derived as follows.

xf = ( 4 * ( x + mbIdxX * 16 ) * BasePicWidth + 2 * deltaX ) // ScaledBaseWidth

yf = ( 4 * ( y + mbIdxY * 16) * BasePicHeight + 2 * deltaY ) // ScaledBaseHeight

- Let ( xInt, yInt ) and ( xFrac, yFrac) be defined as follows

xInt = ( xf >> 2 ) - xB

yInt = ( yf >> 2 ) - yB

xFrac = xf % 4

yFrac = yf % 4

- subclause S.9.4.2.2.1 is invoked with ( xInt, yInt ), ( xFrac, yFrac ) and baseL[ x, y ] as inpus corresponding respectively to ( xIntL, yIntL ), ( xFracL, yFracL ), and the luma sample array of the selected reference picture, and with output predPartLXL[ xL, yL ] is assigned to predL[ x, y ].

When chroma_format_idc is not equal to 0, the chroma samples predC[ x, y ] ( with C being Cb or Cr) with x = 0..MbWidthC – 1, y = 0..MbHeightC – 1 are derived as follows.

- Let deltaXC and deltaYC be defined as follows

deltaXC = ( 2 + BaseChromaPhaseX)*BasePicWidthC – ( 2 + ScaledBaseChromaPhaseX ) * ScaledBaseWidthC

deltaYC = ( 2 + BaseChromaPhaseY)*BasePicHeightC – ( 2 + ScaledBaseChromaPhaseY ) * ScaledBaseHeightC

- Let ( xfC, yfC ) be defined as follows

xfC = ( 4 * ( x + mbIdxX * MbWidthC) * BasePicWidthC + deltaXC ) // ScaledBaseWidthC

yfC = pyC + ( 4 * ( y + mbIdxY * MbHeightC) * BasePicHeightC + deltaYC ) // ScaledBaseHeightC

- Let ( xIntC, yIntC ) and ( xFracC, yFracC) be defined as follows

xIntC = ( xfC >> 2 ) - xCB

yIntC = ( yfC >> 2 ) - yCB

xFracC = xfC % 4

yFracC = yfC % 4

- subclause S.9.4.2.2.1 is invoked with ( xIntC, yIntC ), ( xFracC, yFracC ) and baseC[ x, y ] as inputs corresponding respectively to ( xIntL, yIntL ), ( xFracL, yFracL ), and the luma sample array of the selected reference picture, and with output predPartLXL[ xL, yL ] is assigned to predC[ x, y ].

## Inter prediction process

The specification of this subclause in AVC shall apply with the following modifications.

1) Replace the following paragraphs of this subclause in AVC

The Inter prediction process for a macroblock partition mbPartIdx and a sub-macroblock partition subMbPartIdx consists of the following ordered steps

- Derivation process for motion vector components and reference indices as specified in subclause S.9.4.1.

Inputs to this process are

- a macroblock partition mbPartIdx,

- a sub-macroblock partition subMbPartIdx.

Outputs of this process are

- luma motion vectors mvL0 and mvL1 and when chroma_format_idc is not equal to 0 (monochrome) the chroma motion vectors mvCL0 and mvCL1

- reference indices refIdxL0 and refIdxL1

- prediction list utilization flags predFlagL0 and predFlagL1

- the sub-macroblock partition motion vector count subMvCnt.

- The variable MvCnt is incremented by subMvCnt.

- Decoding process for Inter prediction samples as specified in subclause S.9.4.2.

Inputs to this process are

- a macroblock partition mbPartIdx,

- a sub-macroblock partition subMbPartIdx.

- variables specifying partition width and height for luma and chroma (if available), partWidth, partHeight, partWidthC (if available), and partHeightC (if available)

- luma motion vectors mvL0 and mvL1 and when chroma_format_idc is not equal to 0 (monochrome) the chroma motion vectors mvCL0 and mvCL1

- reference indices refIdxL0 and refIdxL1

- prediction list utilization flags predFlagL0 and predFlagL1

Outputs of this process are

- inter prediction samples (pred); which are a (partWidth)x(partHeight) array predPartL of prediction luma samples and when chroma_format_idc is not equal to 0 (monochrome) two (partWidthC)x(partHeightC) arrays predPartCr, and predPartCb of prediction chroma samples, one for each of the chroma components Cb and Cr.

with the following

The Inter prediction process for a macroblock partition mbPartIdx and a sub-macroblock partition subMbPartIdx consists of the following ordered steps

- The derivation process for motion vector components and reference indices as specified in subclause S.9.4.1 is invoked

Inputs to this process are

- a macroblock partition mbPartIdx,

- a sub-macroblock partition subMbPartIdx.

Outputs of this process are

- luma motion vectors mvL0 and mvL1 and when chroma_format_idc is not equal to 0 (monochrome) the chroma motion vectors mvCL0 and mvCL1

- reference indices refIdxL0 and refIdxL1

- prediction list utilization flags predFlagL0 and predFlagL1

- the sub-macroblock partition motion vector count subMvCnt.

- The variable MvCnt is incremented by subMvCnt.

- When ResidualPictureFlag is equal to 0 the decoding process for inter prediction samples as specified in subclause S.9.4.2 is invoked.

Inputs to this process are

- a macroblock partition mbPartIdx,

- a sub-macroblock partition subMbPartIdx.

- variables specifying partition width and height for luma and chroma (if available), partWidth, partHeight, partWidthC (if available), and partHeightC (if available)

- luma motion vectors mvL0 and mvL1 and when chroma_format_idc is not equal to 0 (monochrome) the chroma motion vectors mvCL0 and mvCL1

- reference indices refIdxL0 and refIdxL1

- prediction list utilization flags predFlagL0 and predFlagL1

Outputs of this process are

- inter prediction samples (pred); which are a (partWidth)x(partHeight) array predPartL of prediction luma samples and when chroma_format_idc is not equal to 0 (monochrome) two (partWidthC)x(partHeightC) arrays predPartCr, and predPartCb of prediction chroma samples, one for each of the chroma components Cb and Cr.

2) Replace the following paragraphs of this subclause in AVC

The macroblock prediction is formed by placing the partition or sub-macroblock partition prediction samples in their correct relative positions in the macroblock, as follows.

The variable predL[ xP + xS + x, yP + yS + y ] with x = 0 .. partWidth – 1, y = 0 .. partHeight – 1 is derived by

predL[ xP + xS + x, yP + yS + y ] = predPartL[ x, y ]

When chroma_format_idc is not equal to 0 (monochrome) the variable predC with x = 0..partWidthC – 1, y = 0..partHeightC – 1, and C in predC and predPartC being replaced by Cb or Cr is derived by

predC[ xP / SubWidthC + xS / SubWidthC + x, yP / SubHeightC + yS / SubHeightC + y ] = predPartC[ x, y ]

with the following

The macroblock prediction is formed by placing the partition or sub-macroblock partition prediction samples in their correct relative positions in the macroblock, as follows.

The variable predL[ xP + xS + x, yP + yS + y ] with x = 0..partWidth – 1, y = 0..partHeight – 1 is derived as follows.

- If ResidualPictureFlag is equal to 0

predL[ xP + xS + x, yP + yS + y ] = predPartL[ x, y ]

- Otherwise (ResidualPictureFlag is equal to 1), predL[ xP + xS + x, yP + yS + y ] is marked as not available.

When chroma_format_idc is not equal to 0 (monochrome) the variable predC with x = 0..partWidthC – 1, y = 0..partHeightC – 1, and C in predC and predPartC being replaced by Cb or Cr is derived as follows

- If ResidualPictureFlag is equal to 0

predC[ xP / SubWidthC + xS / SubWidthC + x, yP / SubHeightC + yS / SubHeightC + y ] = predPartC[ x, y ]

- Otherwise (ResidualPictureFlag is not equal to 0), 
predC[ xP / SubWidthC + xS / SubWidthC + x, yP / SubHeightC + yS / SubHeightC + y ] is marked as not available.

### Derivation process for motion vector components and reference indices

The specification of this subclause in AVC shall apply with the following modifications.

1) Replace the following paragraphs of this subclause in AVC

- Otherwise, for X being replaced by either 0 or 1 in the variables predFlagLX, mvLX, refIdxLX, and in Pred_LX and in the syntax elements ref_idx_lX and mvd_lX, and the following applies.

- The variables refIdxLX and predFlagLX are derived as follows.

- If MbPartPredMode( mb_type, mbPartIdx ) or SubMbPredMode( sub_mb_type[ mbPartIdx ] ) is equal to Pred_LX or to BiPred,

refIdxLX     = ref_idx_lX[ mbPartIdx ]
predFlagLX = 1

- Otherwise, the variables refIdxLX and predFlagLX are specified by

refIdxLX     =  –1
predFlagLX = 0

- The variable subMvCnt for sub-partition motion vector count is set equal to predFlagL0 + predFlagL1.

- When predFlagLX is equal to 1, the derivation process for luma motion vector prediction in subclause 8.4.1.3 is invoked with mbPartIdx subMbPartIdx, refIdxLX, and list suffix LX as the input and the output being mvpLX.  The luma motion vectors are derived by

mvLX[ 0 ] = mvpLX[ 0 ] + mvd_lX[ mbPartIdx ][ subMbPartIdx ][ 0 ]
mvLX[ 1 ] = mvpLX[ 1 ] + mvd_lX[ mbPartIdx ][ subMbPartIdx ][ 1 ]

with the following

- Otherwise, for X being replaced by either 0 or 1 in the variables predFlagLX, mvLX, refIdxLX, and in Pred_LX and in the syntax elements ref_idx_lX, mvd_lX and motion_prediction_flag_lX, and the following applies.

- If MbPartPredMode( mb_type, mbPartIdx ) or SubMbPredMode( sub_mb_type[ mbPartIdx ] ) is equal to Pred_LX or to BiPred, the following applies.

- If base_mode_flag is equal to 1 or base_mode_refinement_flag is equal to 1, the derivation process for base luma motion vectors and reference indices in subclause  S.9.4.1.5 is invoked with mbPartIdx, subMbPartIdx, and the list suffix LX as inputs, the outputs are assigned to the reference index refIdxLX and the luma motion vector mvLX, and the prediction utilization flag predFlagLX is set equal to 1.

predFlagLX = 1

- Otherwise, the following applies.

- The reference index refIdxLX and a luma motion vector prediction mvpLX are derived as follows.

- If motion_prediction_flag_lX[ mbPartIdx ] is equal to 1, the following applies.

- The derivation process for base reference indices in subclause S.9.4.1.5.1 is invoked with mbPartIdx and the list suffix LX as inputs and the output is assigned to refIdxLX.

- The derivation process for base luma motion vector predictions in subclause S.9.4.1.5.2 is invoked with mbPartIdx, subMbPartIdx, and the list suffix LX as inputs and the output is assigned to mvpLX.

- Otherwise, the following applies.

- The reference index refIdxLX is set equal to ref_idx_lX[ mbPartIdx ]

- refIdxLX = ref_idx_lX[ mbPartIdx ]

- The derivation process for luma motion vector prediction in subclause S.9.4.1.3 is invoked with mbPartIdx subMbPartIdx, refIdxLX, and list suffix LX as the input and the output is assigned to mvpLX

- The prediction utilization flag predFlagLX and the luma motion vector mvLX are derived by

predFlagLX = 1
mvLX[ 0 ]   = mvpLX[ 0 ] + mvd_lX[ mbPartIdx ][ subMbPartIdx ][ 0 ]
mvLX[ 1 ]   = mvpLX[ 1 ] + mvd_lX[ mbPartIdx ][ subMbPartIdx ][ 1 ]

- Otherwise, the following applies.

refIdxLX     = –1
predFlagLX = 0

#### Derivation process for luma motion vectors for skipped macroblocks in P and SP slices

The specification of this subclause in AVC shall apply.

#### Derivation process for luma motion vectors for B_Skip, B_Direct_16x16, and B_Direct_8x8

The specification of this subclause in AVC shall apply with the following modifications.

1) Replace the following paragraphs of this subclause in AVC

The derivation process depends on the value of direct_spatial_mv_pred_flag, which is present in the bitstream in the slice header syntax as specified in subclause 7.3.3, and is specified as follows.

- If direct_spatial_mv_pred_flag is equal to 1, the mode in which the outputs of this process are derived is referred to as spatial direct prediction mode.

- Otherwise (direct_spatial_mv_pred_flag is equal to 0), mode in which the outputs of this process are derived is referred to as temporal direct prediction mode.

Both spatial and temporal direct prediction mode use the co-located motion vectors and reference indices as specified in subclause 8.4.1.2.1.

The motion vectors and reference indices are derived as follows.

- If spatial direct prediction mode is used, the direct motion vector and reference index prediction mode specified in subclause 8.4.1.2.2 is used, with subMvCnt being an output.

- Otherwise (temporal direct prediction mode is used), the direct motion vector and reference index prediction mode specified in subclause 8.4.1.2.3 is used and the variable subMvCnt is derived as follows.

- If subMbPartIdx is equal to 0, subMvCnt is set equal to 2.

- Otherwise (subMbPartIdx is not equal to 0), subMvCnt is set equal to 0.

with the following

The derivation process depends on the values of direct_spatial_mv_pred_flag and slice_type, which are present in the bitstream in the slice header syntax as specified in subclause S.8.3.3, and is specified as follows.

- If direct_spatial_mv_pred_flag is equal to 1 and slice_type is not equal to EB (B slice in scalable extension), the mode in which the outputs of this process are derived is referred to as spatial direct prediction mode.

- Otherwise, if direct_spatial_mv_pred_flag is equal to 1 and slice_type is equal to EB (B slice in scalable extension), the mode in which the outputs of this process are derived is referred to as spatial direct prediction mode in scalable extension.

- Otherwise (direct_spatial_mv_pred_flag is equal to 0), the mode in which the outputs of this process are derived is referred to as temporal direct prediction mode.

The spatial direct prediction mode and the temporal direct prediction mode use the co-located motion vectors and reference indices as specified in subclause S.9.4.1.2.1.

The motion vectors and reference indices are derived as follows.

- If direct_spatial_mv_pred_flag is equal to 1 and slice_type is not equal to EB (B slice in scalable extension), the spatial direct motion vector and reference index prediction mode specified in subclause S.9.4.1.2.2 is used, with subMvCnt being an output.

- Otherwise, if direct_spatial_mv_pred_flag is equal to 1 and slice_type is equal to EB (B slice in scalable extension), the spatial direct motion vector and reference index prediction mode in scalable extension specified in subclause S.9.4.1.2.4 is used, with subMvCnt being an output.

- Otherwise (direct_spatial_mv_pred_flag is equal to 0), the direct motion vector and reference index prediction mode specified in subclause S.9.4.1.2.3 is used and the variable subMvCnt is derived as follows.

- If subMbPartIdx is equal to 0, subMvCnt is set equal to 2.

- Otherwise (subMbPartIdx is not equal to 0), subMvCnt is set equal to 0.

##### Derivation process for the co-located 4x4 sub-macroblock partitions

The specification of this subclause in AVC shall apply.

##### Derivation process for spatial direct luma motion vector and reference index prediction mode

The specification of this subclause in AVC shall apply with the following modifications.

1) Replace the following paragraph of this subclause in AVC

This process is invoked when direct_spatial_mv_pred_flag is equal to 1 and any of the following conditions is true.

with the following

This process is invoked when direct_spatial_mv_pred_flag is equal to 1, the syntax element slice_type is not equal to EB (B slice in scalable extension), and any of the following conditions is true.

##### Derivation process for temporal direct luma motion vector and reference index prediction mode

The specification of this subclause in AVC shall apply.

##### Derivation process for spatial direct luma motion vector and reference index prediction mode in scalable extension

This process is invoked when direct_spatial_mv_pred_flag is equal to 1, the syntax element slice_type is equal to EB (B slice in scalable extension), and any of the following conditions is true.

- mb_type is equal to B_Skip

- mb_type is equal to B_Direct_16x16

- sub_mb_type[ mbPartIdx ] is equal to B_Direct_8x8.

Inputs to this process are

- the macroblock partition index mbPartIdx,

- the sub-macroblock partition index subMbPartIdx.

Outputs of this process are

- the reference indices refIdxL0 and refIdxL1,

- the motion vectors mvL0 and mvL1,

- the sub-partition motion vector count subMvCnt,

- the prediction utilization flags, predFlagL0 and predFlagL1.

The reference indices refIdxL0 and refIdxL1 are derived as follows.  [Ed. Note(HS): It may be beneficial to replace the derivation process for the reference indices with a process similar to that of sec. S.8.4.1.2.1 (normal spatial direct mode).]

refIdxL0 = 0,
refIdxL1 = 0.

The motion vectors mvLX (with X being 0 or 1) are derived as follows.

- If sub_mb_type[ mbPartIdx ] is equal to B_Direct_8x8, the process specified in subclause S.9.4.1.3 is invoked with mbPartIdx, subMbPartIdx = 0, refIdxLX, and list suffix LX as inputs and the output is assigned to mvLX.

- Otherwise, the process specified in subclause S.9.4.1.3 is invoked with mbPartIdx = 0, subMbPartIdx = 0, refIdxLX, and list suffix LX as inputs and the output is assigned to mvLX.

NOTE – In the first case, the returned motion vector mvLX is identical for all four 4x4 sub-macroblock partitions of a sub-macroblock, while in the second case, the returned motion vector mvLX is identical for all sixteen 4x4 sub-macroblock partitions of a macroblock.

The prediction utilization flags predFlagLX (with X being 0 or 1) are derived as follows.

- If refIdxLX is less than 0, predFlagLX is set equal to 0.

- Otherwise (refIdxLX is equal to or greater than 0), predFlagLX is set equal to 1.

The variable subMvCnt is derived as follows.

- If subMbPartIdx is equal to 0 or direct_8x8_inference_flag is equal to 0, subMvCnt is set equal to predFlagL0 + predFlagL1.

- Otherwise (subMbPartIdx is not equal to 0 and direct_8x8_inference_flag is equal to 1), subMvCnt is set equal to 0.

#### Derivation process for luma motion vector prediction

The specification of this subclause in AVC shall apply.

##### Derivation process for median luma motion vector prediction

The specification of this subclause in AVC shall apply.

##### Derivation process for motion data of neighbouring partitions

The specification of this subclause in AVC shall apply.

#### Derivation process for chroma motion vectors

The specification of this subclause in AVC shall apply.

#### Derivation process for base luma motion vectors and reference indices

This process is invoked when any of the following conditions is true.

- base_mode_flag is equal to 1

- base_mode_refinement_flag is equal to 1

motion_prediction_flag_lX is equal to 1

Inputs to this process are

- a macroblock partition index mbPartIdx

- a sub-macroblock partition index subMbPartIdx

- a list suffix LX (the list suffix is also used as lX or ListX)

Outputs of this process are

- the reference index refIdxLX

- the luma motion vector mvLX.

The reference index refIdxLX and a luma motion vector prediction mvpLX are derived as follows.

- The derivation process for base reference indices in subclause S.9.4.1.5.1 is invoked with mbPartIdx and the list suffix LX as inputs and the output is assigned to refIdxLX.

- The derivation process for base luma motion vector predictions in subclause S.9.4.1.5.2 is invoked with mbPartIdx, subMbPartIdx, and the list suffix LX as inputs and the output is assigned to mvpLX.

The luma motion vector mvLX is derived as follows.

- If base_mode_refinement_flag is equal to 0, the luma motion vector prediction mvpLX is assigned to the luma motion vector mvLX

mvLX = mvpLX

- Otherwise (base_mode_refinement_flag is equal to 1), the luma motion vector mvLX is derived by

mvLX[ 0 ] = mvpLX[ 0 ] + mvd_ref_lX[ mbPartIdx ][ subMbPartIdx ][ 0 ]
mvLX[ 1 ] = mvpLX[ 1 ] + mvd_ref_lX[ mbPartIdx ][ subMbPartIdx ][ 1 ]

##### Derivation process for base reference indices

This process is invoked when any of the following conditions is true.

- base_mode_flag is equal to 1

- base_mode_refinement_flag is equal to 1

- motion_prediction_flag_lX is equal to 1.

Inputs to this process are

- a macroblock partition index mbPartIdx

- a list suffix LX (the list suffix is also used as lX or ListX)

Output of this process is the reference index refIdxLX.

The reference index refIdxLX is derived as follows.

- If SpatialScalabilityType is lower than 2, the following applies.

- The derivation process for base partitions in subclause S.7.5.4 is invoked with CurrMbAddr and mbPartIdx as inputs and the outputs are assigned to the picture basePic, the macroblock address mbAddrBase, and the macroblock partition index mbPartIdxBase.

- Let refPicBase be the picture that is references by the syntax element ref_idx_lX[ mbPartIdxBase ] of the base macroblock with macroblock address mbAddrBase inside the base picture basePic.

- Otherwise ( SpatialScalabilityType is greater than or equal to 2 ), the following applies.

- Let BaseRefL0[ ][ ] and BaseRefL1[ ][ ] be the reference index arrays that have been derived by the invocation of subclause S.9.4.1.6.1 with CurrMbAddr as input.

- Let refPicBase be the picture that is referenced by BaseRefLX[ mbPartIdx ][ 0 ].

- Let refPic be the picture, for which all of the following conditions are true.

- The syntax element dependency_id for the picture refPic is equal to the syntax element dependency_id of the current picture CurrPic.

- The value of PicOrderCnt( refPic ) is equal to the value of PicOrderCnt( refPicBase ).

- refIdxLX is set equal to the lowest valued reference index in the current reference index list RefPicListX that references refPic. RefPicListX shall contain a variable PicNum or LongTermPicNum that references refPic.

NOTE – The returned reference index refIdxLX is identical for all sub-macroblock partitions of a sub-macroblock.

##### Derivation process for base luma motion vector predictions

This process is invoked when any of the following conditions is true.

- base_mode_flag is equal to 1

- base_mode_refinement_flag is equal to 1

- motion_prediction_flag_lX is equal to 1.

Inputs to this process are

- a macroblock partition index mbPartIdx

- a sub-macroblock partition index subMbPartIdx

- a list suffix LX (the list suffix is also used as lX or ListX)

Output of this process is the prediction mvpLX of the luma motion vector mvLX.

A base motion vector mvBaseLX is derived as follows.

- If SpatialScalabilityType is lower than 2, the following applies.

- The derivation process for base partitions in subclause S.7.5.4 is invoked with CurrMbAddr, mbPartIdx, and subMbPartIdx as inputs and the outputs are assigned to the picture basePic, the macroblock address mbAddrBase, the macroblock partition index mbPartIdxBase, and the sub-macroblock partition index subMbPartIdxBase.

- Let mvBaseLX be the motion vector of the macroblock or sub-macroblock partition mbAddrBase\mbPartIdxBase\subMbPartIdxBase of the base picture basePic.

- Otherwise (SpatialScalabilityType is greater than or equal to 2), the following applies.

- Let BaseMvL0[ ][ ] and BaseMvL1[ ][ ] be the motion vectors arrays that have been derived by the invocation of subclause S.9.4.1.6.1 with CurrMbAddr as input.

- Let mvBaseLX[ subMbPartIdx ] be equal to the motion vector BaseMvLX[ mbPartIdx ][ subMbPartIdx ].

The derivation process for luma motion vector prediction scaling is invoked in subclause S.9.4.1.5.3 with mvBaseLX as input and mvpLX as output.

- If SpatialScalabilityType is greater than 2, the following applies.

mvScaled[ 0 ] + = 4 * ( RefScaledBaseLeftOffset - ScaledBaseLeftOffset )
mvScaled[ 1 ] + = 4 * ( RefScaledBaseTopOffset - ScaledBaseTopOffset )

where RefScaledBaseLeftOffset and RefScaledBaseTopOffset are defined as follows.

- Let refPic be the picture that is referenced by the syntax element ref_idx_lX[ mbPartIdx ] of the current macroblock. RefScaledBaseLeftOffset and RefScaledBaseTopOffset correspond to ScaledBaseLeftOffset and ScaledBaseTopOffset values of refPic.

##### Derivation process for motion vector scaling

Input to this process is a motion vector mvBase.

Output to this process is a motion vector mvScaled.

- If SpatialScalabilityType is equal to 0, mvScaled = mvBase

- Otherwise if SpatialScalabilityType is equal to 1, the following applies.

mvScaled[ 0 ] = (mvBase[ 0 ] << 1 )
mvScaled[ 1 ] = (mvBase[ 1 ] << 1 ).

- Otherwise, the following applies.

mvScaled[ 0 ] = ( mvBase[ 0 ] * ScaledBaseWidth + sign(mvBase[ 0 ] ) * BasePicWidth / 2  ) // BasePicWidth
mvScaled[ 1 ] = ( mvBase[ 1 ] * ScaledBaseHeight + sign(mvBase[ 1 ] ) * BasePicHeight / 2  ) // BasePicHeight

#### Derivation processes for Extended Spatial Scalability

##### Derivation process for base macroblock motion data

This process is invoked SpatialScalabilityType is greater than 1.

Input to this process is a macroblock address mbAddr.

Output to this process is mb_type inherited from base layer.

This process derives reference indices arrays BaseRefLX[ ][ ] and motion vectors arrays BaseMvLX[ ][ ], inherited from base layer. These arrays are indexed by i identifying a macroblock partition inside the macroblock mbAddr and j identifying a sub-macroblock partition inside the macroblock partition i of the macroblock mbAddr.

- If intra_base_mb( CurrMbAddr ) is TRUE, mb_type is set to I_NxN

- Otherwise,

- If SpatialScalabilityType is equal to 2, the following applies.

- The derivation for macroblock partition label in S.9.4.1.6.5 is invoked with mbAddr as input and the output  is assigned to the macroblock label mbLabel.

- Let nbMbPart be the number of partitions corresponding to mbLabel.

- For each partition index, indexed by mbPartIdx = 0..nbMbPart-1,

- macroblock partition motion data is derived by invoking subclause S.9.4.1.6.2, with mbAddr and mbPartIdx as inputs.

- Otherwise ( SpatialScalabilityType is greater than 2), the following applies:

- For each partition index, indexed by mbPartIdx = 0..3, the following applies.

- 8x8 blocks motion data inheritance from base layer is achieved by invoking subclause S.9.4.1.6.2, with mbAddr and mbPartIdx as inputs.

- The derivation for macroblock label in S.9.4.1.6.5 is invoked with mbAddr as input and the output  is assigned to mbLabel.

- nbMbPart is derived as the number of partitions corresponding to mbLabel.

- Let S be a variable that is derived as follows.

- If slice_type is equal to IE, S shall be equal to I.

- Otherwise, if slice_type is equal to PE, S shall be equal to P.

- Otherwise (slice_type is equal to BE), S shall be equal to B.

- Let predFlagL0 and predFlagL1 be booleans initialized to FALSE.

- If  mbLabel is equal to 8x8, mb_type shall be inferred to S_8x8

- Otherwise,

- For each partition, indexed by mbPartIdx = 0..nbMbPart-1, with X being replaced by 0 and 1,

- refIdxLX is set equal to BaseRefLX[ mbPartIdx ][ 0 ]

- If  refIdxLX is not equal to -1 predFlagLX is set to TRUE.

- If predFlagL0 and predFlagL1 are equal to TRUE, mb_type shall be inferred to S_Bi_mbLabel.

- Otherwise, if predFlagL0 is equal to TRUE,  mb_type shall be inferred to S_L0_mbLabel.

- Otherwise, mb_type shall be inferred to S_L1_mbLabel.

##### Derivation process for base macroblock partition motion

This process is invoked when SpatialScalabilityType is greater than 1.

Inputs to this process are

- a macroblock address mbAddr,

- a macroblock partition index mbPartIdx.

The derivation for sub-macroblock partitions motion data of partition mbPartIdx is derived by invoking subclause S.9.4.1.6.3, with mbAddr and mbPartIdx as inputs.

The derivation for macroblock partitions label in subclause S.9.4.1.6.4 is invoked with CurrMbAddr and mbPartIdx as inputs and the output is assigned to mbPartLabel.

Let refIdxBaseLX[ i ] mvBaseLX[ i ], for X being replaced by 0 and 1, be respectively equal to BaseRefLX[ mbPartIdx ][ i ] and BaseMvLX[ mbPartIdx ][ i ]

- If mbPartLabel is equal to INTRA,

- Let predIdx be the idx of the first neighbouring 8x8 block among the horizontal, vertical and diagonal blocks of mbPartIdx, such as predLabelPred is not equal to INTRA (predLabelPred being derived by invoking subclause S.9.4.1.6.4 with mbAddr and predIdx as inputs).

- The derivation process for macroblock partition inheritance in subclause S.9.4.1.6.2 is invoked with mbAddr, predIdx as inputs.

- Let nbSubPart be the number of sub-partitions corresponding to predLabelPred.

- For subMbPartIdx indexed by 0..nbSubPart – 1, and for X being replaced by 0 and 1,

refIdxBaseLX[ subMbPartIdx ] = BaseRefLX[ PredIdx ][ subMbPartIdx ]

mvBaseLX[ subMbPartIdx ] = BaseMvLX[ PredIdx ][ subMbPartIdx ]

- Otherwise, if mbPartLabel is equal to BLK_4x4, the following applies.

- For X being replaced by 0 and 1,

- If at least two refIdxBaseLX[ i ] with i=0..3, are different , the following applies.

- Let variable minRedIdxLX set to the minimum reference index of the refIdxBaseLX[ i ] different to -1.

- For each sub-macroblock partition, indexed by subMbPartIdx =0..3

- If refIdxBaseLX[ subMbPartIdx ] is not equal to minRefIdxLX,

- Let predIdx be the idx of the first neighbouring 4x4 block among the horizontal, vertical and diagonal blocks of subMbPartIdx, such as refIdxBaseLX[ predIdx ] = = minRefIdxLX.

- refIdxBaseLX[ subMbPartIdx ] = refIdxBaseLX [ predIdx ]

- mvBaseLX[ subMbPartIdx ] = mvBaseLX[ predIdx ]

- Otherwise, if mbPartLabel is equal to BLK_8x8,

- For X being replaced by 0 and 1,

refIdxBaseLX[ 0 ] = refIdxBaseLX[ predIdx ]

mvBaseLX[ 0 ] = mvBaseLX [ predIdx ]

- Otherwise (mbPartLabel is equal to BLK_8x4 or to BLK_4x8),

- For X being replaced by 0 and 1,

- If  refIdxBaseLX[ 0 ] is not equal to refIdxBaseLX[ 1 ]

- The derivation for macroblock class index in subclause S.9.4.1.6.2 is invoked with mbAddr as input and the output is assigned to mbClassIdx.

- Let predIdx be a variable derived as follows.

- Let mbClassIdxSuffix be the name of mbClassIdx without the first word.

- If mbClassIdx is equal to Corner_Y (with Y = 0..3 ),

- If ( mbPartLabel is equal to BLK_8x4 and mbClassIdxSuffix > 1 ) or ( mbPartLabel is equal to BLK_4x8 and mbClassIdxSuffix % 2 = = 1 ), predIdx is set to 1

- Otherwise, predIdx is set to 0

- Otherwise, predIdx is set equal to mbClassIdxSuffix

- If refIdxBaseLX[ predIdx ] is not equal to -1

refIdxBaseLX[ 1 – predIdx ] = refIdxBaseLX[ predIdx ]

mvBaseLX[ 1 – predIdx ] = refIdxBaseLX[ predIdx ]

- Otherwise,

refIdxBaseLX[ predIdx ] = refIdxBaseLX[ 1 – predIdx ]

mvBaseLX[ predIdx ] = mvBaseLX[ 1- predIdx ]

##### Derivation process for base sub-macroblock partitions motion

This process is invoked when SpatialScalabilityType is greater than 1.

Inputs to this process are

- the current macroblock address CurrMbAddr,

- a macroblock partition index mbPartIdx.

This process derives reference indices arrays BaseRefLX and motion vectors arrays BaseMvLX inherited from base layer for each sub-macroblock partitions of the partition mbPartIdx, using the following process.

If SpatialScalabilityType is equal to 2,

- The derivation of macroblock partition label in subclause S.9.4.1.6.4 is invoked with CurrMbAddr and mbPartIdx as inputs and the output is assigned to mbPartLabel.

- Let nbSubMbPart be the number of sub-partitions corresponding to mbPartLabel.

- For each sub-macroblock partition, indexed by subMbPartIdx = 0..nbSubMbPart-1

- The derivation process for base partitions in subclause S.7.5.4 is invoked with mbAddr, mbPartIdx and subMbPartIdx as inputs and the outputs are assigned to the picture basePic, the macroblock address mbAddrBase, mbPartIdxBase and subMbPartIdxBase.

- For X being replaced by 0 and 1,

- BaseRefLX[ mbPartIdx ][ subMbPartIdx ] is equal to the reference index of the sub-macroblock partition mbAddrBase\mbPartIdxBase of the base picture basePic.

- BaseMvLX[ mbPartIdx ][ subMbPartIdx ] is equal to the motion vector of the sub-macroblock partition mbAddrBase\mbPartIdxBase\subMbPartIdxBase of the base picture basePic if it exists, or of the macroblock partition mbAddrBase\mbPartIdxBase of the base picture basePic.

Otherwise ( SpatialScalabilityType is greater than 2), the following applies.

- The derivation process for base macroblocks in subclause S.7.5.2 is invoked with mbAddr as input and the output is assigned to the picture basePic and the array of macroblock address aMbAddrBase.

- The derivation process for the inverse macroblock scanning is invoked in subclause S.7.4.1with CurrMbAddr as input and the output is assigned to ( xP, yP ).

- Let ( xB0 , yB0 ) and ( xB, yB ) being derived as follows.

xB0 = ( ( xP – ScaledBaseLeftOffset ) * BasePicWidth + BasePicWidth / 2 ) // ScaledBaseWidth

yB0 = ( ( yP – ScaledBaseTopOffset ) * BasePicHeight + BasePicHeight / 2 ) // ScaledBaseHeight

- For each macroblock sub-partition, indexed by subMbPartIdx = 0..3, the following applies.

- Let ( x, y ) being derived as follows.

x = 2 *( mbPartIdx % 2 ) + ( subMbPartIdx % 2 )
y = 2 *( mbPartIdx / 2 ) + ( subMbPartIdx / 2 )

- Let ( xB, yB ) being derived as follows.

xB =( ( xP + 4*x + 2 – ScaledBaseLeftOffset )* BasePicWidth + BasePicWidth/2 ) // ScaledBaseWidth
yB = ( ( yP + 4*y + 2 – ScaledBaseTopOffset )* BasePicHeight + BasePicHeight/2 ) // ScaledBaseHeight

- Let mbAddrBase = aMbAddrBase[ 0 ] + 2*( ( yB-yB0 ) / 16) + ( ( xB-xB0 ) / 16 )

- The macroblock partition index of the macroblock partition in the macroblock mbAddrBase in the picture basePic covering the luma location ( xB%16 , yB%16 ) shall be assigned to mbPartIdxBase.

- The sub-macroblock partition index of the sub-macroblock partition inside the macroblock partition mbPartIdxBase of the macroblock mbAddrBase in the picture basePic covering the luma location ( xB % 16 , yB % 16 ) shall be assigned to subMbPartIdxBase

- If the mb_type of the macroblock mbAddrBase is one of the INTRA mode, as specified in Table 7-8, the following applies.

- BaseRefL0 [ mbPartIdx ][ subMbPartIdx ] = BaseRefL1[ mbPartIdx ][ subMbPartIdx ] = -1

- Otherwise, for X being replaced by 0 and 1, the following applies.

- BaseRefLX[ mbPartIdx][ subMbPartIdx ] is equal to the reference index of the macroblock or sub-macroblock partition mbAddrBase\mbPartIdxBase of the base picture basePic.

- BaseMvLX[ mbPartIdx ][ subMbPartIdx ] is equal to the motion vector of the macroblock or sub-macroblock partition mbAddrBase\mbPartIdxBase\subMbPartIdxBase of the base picture basePic.

##### Derivation process for macroblock partition label

This process is invoked when SpatialScalabilityType is greater than 1.

Inputs to this process are

- a macroblock address mbAddr,

- a macroblock partition index mbPartIdx.

Output of this process is the partition label mbPartLabel of the macroblock partition mbPartIdx.

- If SpatialScalabilityType is equal to 2,

- The derivation process for base macroblocks in subclause S.7.5.4 is invoked with mbAddr, mbPartIdx and 0 as inputs and the output is assigned to the picture basePic and the macroblock address mbAddrBase.

- Let mbTypeBase be set equal to the mb_type of the macroblock with adress mbAddrBase.

- Let variable mbTypeBaseSuffix be derived as follows.

- If mbTypeBase is equal to P_Skip or B_Skip or B_Direct_16x16

- If the syntax element direct_8x8_inference_flag is equal to 1, mbTypeBaseSuffix is equal to 8x8

- Otherwise, mbTypeBaseSuffix is equal to 16x16

- Otherwise, mbTypeBaseSuffix represents the last part of the name of mbTypeBase.

- If mbTypeBase is one of the INTRA mode, as specified in Table 7-8, mbPartLabel is set to INTRA

- Otherwise, if mbClassIdx is equal to Corner_X ( with X = 0..3 ), mbPartLabel is given by Table S-8-5 as a function of mbTypeBaseSuffix and mbPartIdx .

- Otherwise, if mbClassIdx is equal to Vert_X or Hori_X ( with X=0..1 ), mbPartLabel is given by table Table S-8-6 as a function of mbTypeBaseSuffix and mbPartIdx .

- Otherwise, mbPartLabel is set to BLK_8x8.

- Otherwise ( SpatialScalabilityType is greater than 2), the following applies.

- Let ( x, y ) being derived as follows.

x = 2 * ( mbPartIdx % 2 )

y = 2 * ( mbPartIdx / 2 )

- Let the function identicalMotion( subMbPartIdx1 , subMbPartIdx2 ) being defined as follows.

- If, for X being replaced by 0 and 1, the following conditions are TRUE, identicalMotion( subMbPartIdx1 , subMbPartIdx2 ) returns TRUE.

-BaseRefLX[ mbPartIdx ][ subMbPartIdx1 ] == BaseRefLX[ mbPartIdx ][ subMbPartIdx2 ]

-BaseMvLX[ mbPartIdx ][ subMbPartIdx1 ] == BaseMvLX[ mbPartIdx ][ subMbPartIdx2 ]

- Otherwise, identicalMotion( subMbPartIdx1 , subMbPartIdx2 ) returns FALSE.

- If for i= 0..3, X being replaced by 0 and 1, BaseRefLX[ mbPartIdx ][ i ] == -1, mbPartLabel is set to INTRA

- Otherwise if identicalMotion( 0 , 1 ) and identicalMotion( 2 , 3 ), the following applies.

- If identicalMotion( 0 , 2 ) mbPartLabel is set to BLK_8x8

- Otherwise, mbPartLabel is set to BLK_8x4

- Otherwise, if identicalMotion( 0 , 2 ) and identicalMotion( 1 , 3 ), the following applies mbPartLabel is set to BLK_4x8

- Otherwise, mbPartLabel = BLK_4x4

Table S-8-5– Macroblock partition label as a function of mbTypeBaseSuffix and mbPartIdx assuming mbClassIdx equal to Corner_X ( with X = 0..3 )

|  |  | mbPartIdx |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Corner_0 Corner_1 Corner_2 Corner_3 |  | 0 1 2 3 | 1 0 3 2 | 2 3 0 1 | 3 2 1 0 |
| mbTypeBasesuffix | 8x8 or 8x8ref0 | BLK_8x8 | BLK_4x8 | BLK_8x4 | BLK_4x4 |
|  | 8x16 | BLK_8x8 | BLK_4x8 | BLK_8x8 | BLK_4x8 |
|  | 16x8 | BLK_8x8 | BLK_8x8 | BLK_8x4 | BLK_8x4 |
|  | 16x16 | BLK_8x8 | BLK_8x8 | BLK_8x8 | BLK_8x8 |

Table S-8-6 – Macroblock partition label as a function of mbTypeBaseSuffix and mbPartIdx assuming mbClassIdx equal to Vert_X ( left side ) and Hori_X ( right side ) ( with X = 0..1 )

|  |  |  | mbPartIdx |  |  |  | mbPartIdx |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Vert_0 |  |  | 0 1 | 2 3 | Hori_0 |  | 0 2 | 1 3 |
| Vert_1 |  |  | 2 3 | 0 1 | Hori_1 |  | 1 3 | 0 2 |
| mbTypeBaseSuffix | 8x8 or 8x8ref0 16x8 |  | BLK_8x8 | BLK_8x4 | mbTypeBaseSuffix | 8x8 or 8x8ref0 8x16 | BLK_8x8 | BLK_4x8 |
|  | 16x16 8x16 |  | BLK_8x8 | BLK_8x8 |  | 16x16 16x8 | BLK_8x8 | BLK_8x8 |

##### Derivation process for macroblock label

This process is invoked when SpatialScalabilityType is greater than 1.

Input to this process is a macroblock address mbAddr.

Output of this process is the macroblock label mbLabel of the macroblock mbAddr.

- If SpatialScalabilityType is equal to 2,

- The derivation process for base macroblocks in subclause S.7.5.2 is invoked with mbAddr as input and the output is assigned to the picture basePic and the array of macroblock address aMbAddrBase.

- The derivation for macroblock class index in subclause S.9.4.1.6.2 is invoked with CurrMbAddr as input and the output is assigned to mbClassIdx.

- Let the function mbTypeSuffix( vmbType ) being defined as follows.

- If vmbType is equal to P_Skip or B_Skip or B_Direct_16x16

- If direct_8x8_inference_flag is equal to 1, mbTypeSuffix( vmbType ) returns 8x8

- Otherwise, mbTypeSuffix( vmbType ) returns 16x16

- Otherwise, mbTypeSuffix( vmbType ) returns the last  part of the name of vmbType.

- If mbClassIdx is equal to Corner_X ( with X = 0..3 )

- Let mbTypeBase be the mb_type of the macroblock with macroblock address aMbAddrBase[  0].

- If  mbTypeSuffix( mbTypeBase ) is equal to 16x16, mbLabel shall be inferred to be equal to 16x16

- Otherwise, mbLabel shall be inferred to be equal to 8x8

- Otherwise, if mbClassIdx is equal to Vert_X (with X=0..1)

- Let mbTypeBaseLeft and mbTypeBaseRight be respectively the mb_type of the macroblocks aMbAddrBase[ 0 ] and aMbAddrBase[ 1 ].

- If  mbTypeSuffix( mbTypeBaseLeft ) is in { 8x8, 8x8ref0, 16x8 } or mbTypeSuffix(mbTypeBaseRight) is in { 8x8, 8x8ref0, 16x8 }, mbLabel shall be inferred to be equal to 8x8

- Otherwise, if mbTypeSuffix( mbTypeBaseLef t) is equal to INTRA or mbTypeSuffix( mbTypeBaseRigh t) is equal to INTRA, mbLabel shall be inferred to be equal to 16x16

- Othersize, mbLabel shall be inferred to be equal to 8x16.

- Otherwise, if mbClassIdx is equal to Hori_X ( with X = 0..1 )

- Let mbTypeBaseUp and mbTypeBaseDown be respectively the mb_type of the macroblocks aMbAddrBase[0] and aMbAddrBase[1].

- If  mbTypeSuffix( mbTypeBaseUp ) is in {8x8, 8x8ref0, 8x16} or mbTypeSuffix( mbTypeBaseDown ) is in {8x8, 8x8ref0, 8x16}, mbLabel shall be inferred to be equal to 8x8

- Otherwise, if mbTypeSuffix( mbTypeBaseUp ) is equal to INTRA or mbTypeSuffix( mbTypeBaseDown ) is equal to INTRA, mbLabel shall be inferred to be equal to 16x16

- Othersize, mbLabel shall be inferred to be equal to 16x8.

- Otherwise, mbLabel shall be inferred to be equal to 8x8.

- Otherwise ( SpatialScalabilityType is greater than 2), the following applies.

- Let Y being replaced by either P or B, depending on slice_type.

- Let the function identicalMotion( mbPartIdx1 , mbPartIdx2 ) being defined as follows.

- If, for X being replaced by 0 and 1, the following conditions are TRUE, identicalMotion( mbPartIdx1 , mbPartIdx2 ) returns TRUE.

- - 	BaseRefLX[ mbPartIdx1 ][ 0 ] == BaseRefLX[ mbPartIdx2 ][ 0 ]

- - 	sub_mb_type[ mbPartIdx1 ] == sub_mb_type[ mbPartIdx2 ] are equal to Y_Z_8x8, with Z being replaced derived either by Bi, L0 or L1.

- - 	BaseMvLX[ mbPartIdx1 ][ 0 ] == BaseMvLX[ mbPartIdx2 ][ 0 ]

- Otherwise, identicalMotion( mbPartIdx1, mbPartIdx2 ) returns FALSE.

- If at least one of the sub_mb_type[ mbPartIdx ] with mbPartIdx = 0..3 is not equal to Y_Z_8x8, Z being either Bi, L0 or L1, mbLabel is set equal to 8x8,

- Otherwise if identicalMotion ( 0 , 1 ) and identicalMotion ( 2 , 3 )  are TRUE, the following applies.

- If identicalMotion ( 0 , 2 ) is TRUE, mbLabel is set equal to 16x16

- Otherwise, mbLabel is set equal to 16x8

- Otherwise If identicalMotion ( 0 , 2 ) and identicalMotion ( 1 , 3 ) are TRUE, mbLabel is set equal to 8x16

- Otherwise, mbLabel is set equal to 8x8.

##### Derivation process macroblock class index

This process is invoked when SpatialScalabilityType is greater than 1.

Input to this process is a macroblock address mbAddr.

Output of this process is the macroblock class index mbClassIdx of the macroblock located at mbAddr.

- If SpatialScalabilityType is equal to 2,

- Let mbIdxX and mbIdxY be derived as follows.

mbIdxX = mbAddr % PicWidthInMbs – ScaledBaseLeftOffset / 16

mbIdxY = mbAddr / PicWidthInMbs – ScaledBaseTopOffset / 16

- The mbClassIdx is derived as follows.

mbClassIdx = ( mbIdxY % 3 ) * 3 + mbIdxX % 3;

- Otherwise ( SpatialScalabilityType is greater than 2 ), the following applies.

- The position of the upper-left luma sample of the current macroblock is derived by invoking the inverse macroblock scanning process in subclause S.7.4.1 with mbAddr as input and the output is assigned to ( xP, yP ).

- Let ( xB0 , yB0 ) and ( xB1 , yB1 ) be derived as follows.

xB0 = ( ( xP – ScaledBaseLeftOffset ) * BasePicWidth + BasePicWidth / 2 )  // ScaledBaseWidth
yB0 = ( ( yP – ScaledBaseTopOffset ) * BasePicHeight + BasePicHeight / 2 )  // ScaledBaseHeight
xB1 = ( ( xP + 15 – ScaledBaseLeftOffset ) * BasePicWidth + BasePicWidth / 2 )  // ScaledBaseWidth
yB1 = ( ( yP + 15 – ScaledBaseTopOffset ) * BasePicHeight + BasePicHeight / 2 ) // ScaledBaseHeight

- If ( xB1 > xB0 + 15 ), the following applies.

- If ( yB1 > yB0 + 15 ),  	mbClassIdx = 4

- Otherwise, 	mbClassIdx = 1

- Otherwise if ( yB1 > yB0 + 15 ),  	mbClassIdx = 3.

- Otherwise , 		mbClassIdx = 0.

- Table S-8-7 gives the possible mbClassIdx values and their associated names.

Table S-8-7– Macroblock class Index

| mbClassIdx | mbClassIdx name |
| --- | --- |
| 0 | Corner_0 |
| 1 | Vert_0 |
| 2 | Corner_1 |
| 3 | Hori_0 |
| 4 | Center |
| 5 | Hori_1 |
| 6 | Corner_2 |
| 7 | Vert_1 |
| 8 | Corner_3 |

### Decoding process for Inter prediction samples

The specification of this subclause in AVC shall apply.

#### Reference picture selection process

The specification of this subclause in AVC shall apply.

#### Fractional sample interpolation process

The specification of this subclause in AVC shall apply with the following modifications

1) Replace the following paragraph

Inputs to this process are

- the current partition given by its partition index mbPartIdx and its sub-macroblock partition index subMbPartIdx,

- the width and height partWidth, partHeight of this partition in luma-sample units,

- a luma motion vector mvLX given in quarter-luma-sample units,

- a chroma motion vector mvCLX given in eighth-chroma-sample units, and

- the selected reference picture sample arrays refPicLXL, refPicLXCb, and refPicLXCb

Outputs of this process are

- a (partWidth)x(partHeight) array predPartLXL of prediction luma sample values and

- when chroma_format_idc is not equal to 0 (monochrome) two (partWidthC)x(partHeightC) arrays predPartLXCb, and predPartLXCr of prediction chroma sample values.

by the following

Inputs to this process are

- the current partition given by its partition index mbPartIdx and its sub-macroblock partition index subMbPartIdx,

- the width and height partWidth, partHeight of this partition in luma-sample units,

- a luma motion vector mvLX given in quarter-luma-sample units,

- a chroma motion vector mvCLX given in eighth-chroma-sample units, and

- the selected reference picture sample arrays refPicLXL(if available), refPicLXCb(if available), and refPicLXCb(if available)

- a flag clipFlag (if available) inticating whether the interpolated samples should be clipped or not. When clipFlag is not present is should be inferred to be equal to 1

Outputs of this process are

- when refPicLXL is available, a (partWidth)x(partHeight) array predPartLXL of prediction luma sample values and

- when chroma_format_idc is not equal to 0 (monochrome) two (partWidthC)x(partHeightC) arrays predPartLXCb, and predPartLXCr of prediction chroma sample values.

2) Replace the following paragraph

Let ( xIntL, yIntL ) be a luma location given in full-sample units and ( xFracL, yFracL ) be an offset given in quarter-sample units. These variables are used only inside this subclause for specifying general fractional-sample locations inside the reference sample arrays refPicLXL, refPicLXCb (if available), and refPicLXCr (if available).

For each luma sample location (0 <= xL < partWidth, 0 <= yL < partHeight) inside the prediction luma sample array predLXL, the corresponding predicted luma sample value predLXL[ xL, yL ] is derived as follows:

- The variables xIntL, yIntL, xFracL, and yFracL are derived by

xIntL = xAL + ( mvLX[ 0 ] >> 2 ) + xL		(8-220)
yIntL = yAL + ( mvLX[ 1 ] >> 2 ) + yL		(8-221)

xFracL = mvLX[ 0 ] & 3		(8-222)
yFracL = mvLX[ 1 ] & 3		(8-223)

- The prediction sample value predLXL[ xL, yL ] is derived by invoking the process specified in subclause S.9.4.2.2.1 with ( xIntL, yIntL ), ( xFracL, yFracL ) and refPicLXL given as input.

by the following

When refPicLXL is available, the following applies.

- Let ( xIntL, yIntL ) be a luma location given in full-sample units and ( xFracL, yFracL ) be an offset given in quarter-sample units. These variables are used only inside this subclause for specifying general fractional-sample locations inside the reference sample arrays refPicLXL, refPicLXCb (if available), and refPicLXCr (if available).

- For each luma sample location (0 <= xL < partWidth, 0 <= yL < partHeight) inside the prediction luma sample array predLXL, the corresponding predicted luma sample value predLXL[ xL, yL ] is derived as follows:

- The variables xIntL, yIntL, xFracL, and yFracL are derived by

xIntL = xAL + ( mvLX[ 0 ] >> 2 ) + xL		(S-8-1)
yIntL = yAL + ( mvLX[ 1 ] >> 2 ) + yL		(S-8-2)

xFracL = mvLX[ 0 ] & 3		(S-8-3)
yFracL = mvLX[ 1 ] & 3		(S-8-4)

- The prediction sample value predLXL[ xL, yL ] is derived by invoking the process specified in subclause S.9.4.2.2.1 with ( xIntL, yIntL ), ( xFracL, yFracL ), refPicLXL and clipFlag given as input.

##### Luma sample interpolation process

The specification of this subclause in AVC shall apply with the following modifications

1) Replace the following paragraph of the subclause in AVC

Inputs to this process are

- a luma location in full-sample units ( xIntL, yIntL ),

- a luma location offset in fractional-sample units ( xFracL, yFracL ), and

- the luma sample array of the selected reference picture refPicLXL

by the following

Inputs to this process are

- a luma location in full-sample units ( xIntL, yIntL ),

- a luma location offset in fractional-sample units ( xFracL, yFracL ), and

- the luma sample array of the selected reference picture refPicLXL

- a flag clipFlag (if available) inticating whether the interpolated samples should be clipped or not. When clipFlag is not present is should be inferred to be equal to 1

2) Replace the following equations

b = Clip1Y( ( b1 + 16 ) >> 5 ) 		(8-239)
h = Clip1Y( ( h1 + 16 ) >> 5 )		(8-240)

by the following

- if clipFlag is equal to 1,

b = Clip1Y( ( b1 + 16 ) >> 5 )		(S-8-5)
	h = Clip1Y( ( h1 + 16 ) >> 5 )		(S-8-6)

- otherwise (clipFlag is equal to 0)

b = ( ( b1 + 16 ) >> 5 ) 		(S-8-7)
	h = ( ( h1 + 16 ) >> 5 )		(S-8-8)

3) Replace the following equations

j = Clip1Y( ( j1 + 512 ) >> 10 )		(8-243)

by the following

- if clipFlag is equal to 1,

j = Clip1Y( ( j1 + 512 ) >> 10 )		(S-8-9)

- otherwise (clipFlag is equal to 0)

j = ( ( j1 + 512 ) >> 10 )		(S-8-10)

4) Replace the following equations

s  = Clip1Y( ( s1  + 16 ) >> 5 ) 		(8-244)
m = Clip1Y( ( m1 + 16 ) >> 5 )		(8-245)

by the following

- if clipFlag is equal to 1,

s  = Clip1Y( ( s1  + 16 ) >> 5 ) 		(S-8-11)
m = Clip1Y( ( m1 + 16 ) >> 5 )		(S-8-12)

- otherwise (clipFlag is equal to 0)

s  = ( ( s1  + 16 ) >> 5 ) 		(S-8-13)
m = ( ( m1 + 16 ) >> 5 )		(S-8-14)

##### Chroma sample interpolation process

The specification of this subclause in AVC shall apply.

#### Weighted sample prediction process

The specification of this subclause in AVC shall apply.

##### Default weighted sample prediction process

The specification of this subclause in AVC shall apply.

##### Weighted sample prediction process

The specification of this subclause in AVC shall apply.

## Transform coefficient decoding process and picture construction process prior to deblocking filter process

The specification of this subclause in AVC shall apply.

### Specification of transform decoding process for 4x4 luma residual blocks

Replace the specification of this subclause in AVC with the following.

This specification applies when transform_size_8x8_flag is equal to 0.

Let numProgRef be the syntax element quality_level for the current picture to be reconstructed. For quality_level equal to 0, the luma transform coefficients are represented by the variable LumaLevel. For k = 1..numProgRef, let lumaLevelk be the variable LumaLevel of the macroblock with macroblock address CurrMbAddr and quality_level equal to k.

When residual_prediction_flag is equal to 1, the macroblock residual prediction process as specified in subclause S.9.5.14 is invoked with the 16x16 array resPredL of the residual prediction luma samples as output.

When the current macroblock prediction mode is not equal to Intra_16x16, the variables LumaLevel, lumaLevelk with k = 1..numProgRef contains the levels and refinement levels for the luma transform coefficients. For a 4x4 luma block indexed by luma4x4BlkIdx = 0..15, the following ordered steps are specified.

- The inverse transform coefficient scanning process as described in subclause S.9.5.5 is invoked with LumaLevel[ luma4x4BlkIdx ] as the input and the two-dimensional array c as the output.

- For each k = 1..numProgRef, the inverse transform coefficient scanning process as described in subclause S.9.5.5 is invoked with lumaLevelk[ luma4x4BlkIdx ] as the input and the two-dimensional array ck as the output.

- The scaling and transformation process for residual 4x4 blocks as specified in subclause S.9.5.10 is invoked with c, numProgRef, and ck with k = 1..numProgRef as the input and r as the output.

- When residual_colour_transform_flag is equal to 1, the variable RY,ij is set equal to rij with i, j = 0..3 and this process is suspended until after completion of the residual colour transform process as specified in subclause S.9.5.13, after the completion of which, the variable rij is set equal to RG,ij with i, j = 0..3 and this process is continued.

- The position of the upper-left sample of a 4x4 luma block with index luma4x4BlkIdx inside the macroblock is derived by invoking the inverse 4x4 luma block scanning process in subclause S.7.4.3 with luma4x4BlkIdx as the input and the output being assigned to ( xO, yO ).

- The 4x4 array u with elements uij for i, j = 0..3 is derived as follows

- If residual_prediction_flag is equal to 0,

uij = rij

- Otherwise (residual_prediction_flag is equal to 1)

uij = resPredL[ xO + x, yO + y ] + rij

- The picture construction process prior to deblocking filter process in subclause S.9.5.12 is invoked with luma4x4BlkIdx, predL, and u as the inputs.

### Specification of transform decoding process for luma samples of Intra_16x16 macroblock prediction mode

Replace the specification of this subclause in AVC with the following.

Let numProgRef be the syntax element quality_level for the current picture to be reconstructed. For quality_level equal to 0, the luma transform coefficients for Intra_16x16 macroblocks are represented by the variables Intra16x16DCLevel and Intra16x16ACLevel. For k = 1..numProgRef, let lumaLevelk be the variable LumaLevel of the macroblock with macroblock address CurrMbAddr and quality_level equal to k.

When residual_prediction_flag is equal to 1, the macroblock residual prediction process as specified in subclause S.9.5.14 is invoked with the 16x16 array resPredL of the residual prediction luma samples as output.

When the current macroblock prediction mode is equal to Intra_16x16, the variables Intra16x16DCLevel, Intra16x16ACLevel, lumaLevelk with k = 1..numProgRef contain the levels and refinement levels for the luma transform coefficients. The transform coefficient decoding proceeds in the following ordered steps:

- The 4x4 luma DC transform coefficients of the macroblock are decoded.

  - The inverse transform coefficient scanning process as described in subclause S.9.5.5 is invoked with Intra16x16DCLevel as the input and the two-dimensional array c as the output.

  - For each k = 1..numProgRef, the following applies

- A variable Intra16x16DCLevelk, which is a list of 16 entries, is derived by

Intra16x16DCLevelk[ n ] = lumaLevelk [ n ][ 0 ]    with n = 0..15

- The inverse transform coefficient scanning process as described in subclause S.9.5.5 is invoked with Intra16x16DCLevelk as the input and the two-dimensional array ck as the output.

  - The scaling and transformation process for luma DC transform coefficients for Intra_16x16 macroblock type as specified in subclause S.9.5.8 is invoked with c, numProgRef, and ck with k = 0..numProgRef as the input and dcY as the output.

- For a 4x4 luma block indexed by luma4x4BlkIdx = 0..15, the following ordered steps are specified.

  - The variable lumaList, which is a list of 16 entries, is derived. The first entry of lumaList is the corresponding value from the array dcY. Figure S-8-1 shows the assignment of the indices of the array dcY to the luma4x4BlkIdx. The two numbers in the small squares refer to indices i and j in dcYij, and the numbers in large squares refer to luma4x4BlkIdx.

Figure S-8-1 – Assignment of the indices of dcY to luma4x4BlkIdx

The elements in lumaList with index n = 1..15 are specified as

lumaList[ n ] = Intra16x16ACLevel[ luma4x4BlkIdx ][ n - 1 ]

  - The inverse transform coefficient scanning process as described in subclause S.9.5.5 is invoked with lumaList as the input and the two-dimensional array c as the output.

  - For each k = 1..numProgRef, the following applies

- A variable lumaListk, which is a list of 16 entries, is derived by

lumaListk[ n ] = ( n  = =  0  ?  0  :  lumaLevelk[ luma4x4BlkIdx ][ n ] )   with n = 0..15

- The inverse transform coefficient scanning process as described in subclause S.9.5.5 is invoked with lumaListk as the input and the two-dimensional array ck as the output.

  - The scaling and transformation process for residual 4x4 blocks as specified in subclause S.9.5.10 is invoked with c, numProgRef, and ck with k = 1..numProgRef as the input and r as the output.

  - When residual_colour_transform_flag is equal to 1, the variable RY,ij is set equal to rij with i, j = 0..3 and this process is suspended until after completion of the residual colour transform process as specified in subclause S.9.5.13, after the completion of which, the variable rij is set equal to RG,ij with i, j = 0..3 and this process is continued.

  - The position of the upper-left sample of a 4x4 luma block with index luma4x4BlkIdx inside the macroblock is derived by invoking the inverse 4x4 luma block scanning process in subclause S.7.4.3 with luma4x4BlkIdx as the input and the output being assigned to ( xO, yO ).

  - The 4x4 array u with elements uij for i, j = 0..3 is derived as follows

- If residual_prediction_flag is equal to 0,

uij = rij

- Otherwise (residual_prediction_flag is equal to 1)

uij = resPredL[ xO + x, yO + y ] + rij

  - The picture construction process prior to deblocking filter process in subclause S.9.5.12 is invoked with luma4x4BlkIdx, predL, and u as the inputs.

### Specification of transform decoding process for 8x8 luma residual blocks

Replace the specification of this subclause in AVC with the following.

This specification applies when transform_size_8x8_flag is equal to 1.

Let numProgRef be the syntax element quality_level for the current picture to be reconstructed. For quality_level equal to 0, the luma transform coefficients are represented by the variables LumaLevel8x8. For k = 1..numProgRef, let lumaLevel8x8k be the variable LumaLevel8x8 of the macroblock with macroblock address CurrMbAddr and quality_level equal to k.

When residual_prediction_flag is equal to 1, the macroblock residual prediction process as specified in subclause S.9.5.14 is invoked with the 16x16 array resPredL of the residual prediction luma samples as output.

The variables LumaLevel8x8[ luma8x8BlkIdx ] and lumaLevel8x8k[ luma8x8BlkIdx ] with luma8x8BlkIdx = 0..3 and k = 1..numProgRef contain the levels and refinement levels for the luma transform coefficients for the luma 8x8 block with index luma8x8BlkIdx.

For an 8x8 luma block indexed by luma8x8BlkIdx = 0..3, the following ordered steps are specified.

- The inverse scanning process for 8x8 luma transform coefficients as described in subclause S.9.5.6 is invoked with LumaLevel8x8[ luma8x8BlkIdx ] as the input and the two-dimensional array c as the output.

- For each k = 1..numProgRef, the inverse scanning process for 8x8 luma transform coefficients as described in subclause S.9.5.6 is invoked with lumaLevel8x8k[ luma8x8BlkIdx ] as the input and the two-dimensional array ck as the output.

- The scaling and transformation process for residual 8x8 blocks as specified in subclause S.9.5.11 is invoked with c, numProgRef, and ck with k = 1..numProgRef as the input and r as the output.

- When residual_colour_transform_flag is equal to 1, the variable RY,ij is set equal to rij with i, j = 0..7 and this process is suspended until after completion of the residual colour transform process as specified in subclause S.9.5.13, after the completion of which, the variable rij is set equal to RG,ij with i, j = 0..7 and this process is continued.

- The position of the upper-left sample of an 8x8 luma block with index luma8x8BlkIdx inside the macroblock is derived by invoking the inverse 8x8 luma block scanning process in subclause S.7.4.4 with luma8x8BlkIdx as the input and the output being assigned to ( xO, yO ).

- The 8x8 array u with elements uij for i, j = 0..7 is derived as

- If residual_prediction_flag is equal to 0,

uij = rij

- Otherwise (residual_prediction_flag is equal to 1)

uij = resPredL[ xO + x, yO + y ] + rij

- The picture construction process prior to deblocking filter process in subclause S.9.5.12 is invoked with luma8x8BlkIdx and u as the inputs.

### Specification of transform decoding process for chroma samples

Replace the specification of this subclause in AVC with the following.

This process is invoked for each chroma component Cb and Cr seperately.

Let numProgRef be the syntax element quality_level for the current picture to be reconstructed. For quality_level equal to 0, the chroma transform coefficients are represented by the variables ChromaDCLevel and ChromaACLevel. For k = 1..numProgRef, let chromaDCLevelk and chromaACLevelk be the variables ChromaDCLevel and ChromaACLevel of the macroblock with macroblock address CurrMbAddr and quality_level equal to k.

When residual_prediction_flag is equal to 1, the macroblock residual prediction process as specified in subclause S.9.5.14 is invoked with the arrays resPredcr and resPredcb of the residual prediction luma samples as output.

For each chroma component, the variables ChromaDCLevel[ iCbCr ], ChromaACLevel[ iCbCr ] and chromaDCLevelk[ iCbCr ], chromaACLevelk[ iCbCr ], with iCbCr set equal to 0 for Cb and iCbCr set equal to 1 for Cr and k = 1..numProgRef, contain the levels and refinement levels for both components of the chroma transform coefficients.

Let the variable numChroma4x4Blks be set equal to (MbWidthC / 4) * (MbHeightC / 4).

For each chroma component, the transform decoding proceeds separately in the following ordered steps:

- The numChroma4x4Blks chroma DC transform coefficients of the 4x4 chroma blocks of the component indexed by iCbCr of the macroblock are decoded.

- Depending on the variable chroma_format_idc, the following applies.

- If chroma_format_idc is equal to 1, the following applies.

- The 2x2 array c is derived using the inverse raster scanning process applied to ChromaDCLevel as follows

c=ChromaDCLevel  iCbCr  0 ChromaDCLevel iCbCr  1 ChromaDCLevel iCbCr  2 ChromaDCLevel iCbCr  3

- For each k = 1..numProgRef, the 2x2 array ck is derived using the inverse raster scanning process applied to chromaDCLevelk as follows

ck=ChromaDCLevelk iCbCr  0 ChromaDCLevelk iCbCr  1 ChromaDCLevelk iCbCr  2 ChromaDCLevelk iCbCr  3

- Otherwise, if chroma_format_idc is equal to 2, the following applies.

- The 2x4 array c is derived using the inverse raster scanning process applied to ChromaDCLevel as follows

c=ChromaDCLevel[iCbCr][0]ChromaDCLevel[iCbCr][2]ChromaDCLevel[iCbCr][1]ChromaDCLevel[iCbCr][5]ChromaDCLevel[iCbCr][3]ChromaDCLevel[iCbCr][6]ChromaDCLevel[iCbCr][4]ChromaDCLevel[iCbCr][7]

- For each k = 1..numProgRef, the 2x4 array ck is derived using the inverse raster scanning process applied to chromaDCLevelk as follows

ck=ChromaDCLevelk[iCbCr][0]ChromaDCLevelk[iCbCr][2]ChromaDCLevelk[iCbCr][1]ChromaDCLevelk[iCbCr][5]ChromaDCLevelk[iCbCr][3]ChromaDCLevelk[iCbCr][6]ChromaDCLevelk[iCbCr][4]ChromaDCLevelk[iCbCr][7]

- Otherwise (chroma_format_idc is equal to 3), the following applies.

- The inverse scanning process for transform coefficients as specified in subclause S.9.5.5 is invoked with ChromaDCLevel[ iCbCr ] as the input and the two-dimensional 4x4 array c as the output.

- For each k = 1..numProgRef, the inverse scanning process for transform coefficients as specified in subclause S.9.5.5 is invoked with chromaDCLevelk[ iCbCr ] as the input and the two-dimensional 4x4 array ck as the output.

- The scaling and transformation process for chroma DC transform coefficients as specified in subclause S.9.5.9 is invoked with c, numProgRef, and ck with k = 1..NumProgRef as the input and dcC as the output.

- For each 4x4 chroma block indexed by chroma4x4BlkIdx = 0..numChroma4x4Blks – 1 of the component indexed by iCbCr, the following ordered steps are specified.

- The variable chromaList, which is a list of 16 entries, is derived. The first entry of chromaList is the corresponding value from the array dcC. Figure S-8-2 shows the assignment of the indices of the array dcC to the chroma4x4BlkIdx. The two numbers in the small squares refer to indices i and j in dcCij, and the numbers in large squares refer to chroma4x4BlkIdx.

a				b				c

Figure S-8-2 – Assignment of the indices of dcC to chroma4x4BlkIdx: 
(a) chroma_format_idc equal to 1, (b) chroma_format_idc equal to 2, (c) chroma_format_idc equal to 3

The elements in chromaList with index n = 1..15 are specified as

chromaList[ n ] = ChromaACLevel[ chroma4x4BlkIdx ][ n – 1 ]

- The inverse scanning process for transform coefficients as specified in subclause S.9.5.5 is invoked with chromaList as the input and the two-dimensional array c as the output.

- For each k = 0..numProgRef, the following applies

- A variable chromaListk, which is a list of 16 entries, is derived by

chromaListk[ n ] = ( n  = =  0  ?  0  :  chromaACLevelk[ chroma4x4BlkIdx ][ n – 1 ]   with n = 0..15

- The inverse scanning process for transform coefficients as specified in subclause S.9.5.5 is invoked with chromaListk as the input and the two-dimensional array ck as the output.

- The scaling and transformation process for residual 4x4 blocks as specified in subclause S.9.5.10 is invoked with c, numProgRef, and ck with k = 1..numProgRef as the input and r as the output.

- Depending on the variable chroma_format_idc, the position of the upper-left sample of a 4x4 chroma block with index chroma4x4BlkIdx inside the macroblock is derived as follows.

- If chroma_format_idc is equal to 1 or 2, the following applies.

xO = InverseRasterScan( chroma4x4BlkIdx, 4, 4, 8, 0 )

yO = InverseRasterScan( chroma4x4BlkIdx, 4, 4, 8, 1 )

- Otherwise (chroma_format_idc is equal to 3), the following applies.

xO = InverseRasterScan( chroma4x4BlkIdx / 4, 8, 8, 16, 0 ) + 
	 InverseRasterScan( chroma4x4BlkIdx % 4, 4, 4, 8, 0 )

yO =InverseRasterScan( chroma4x4BlkIdx / 4, 8, 8, 16, 1 ) + 
	 InverseRasterScan( chroma4x4BlkIdx % 4, 4, 4, 8, 1 )

- When residual_colour_transform_flag is equal to 1, the variable xO' is set equal to xO % ( 4 << transform_size_8x8_flag ), the variable yO' is set equal to yO % ( 4 << transform_size_8x8_flag ), and the following applies.

- If this process is invoked for the chroma component Cb, the variable RCb,mn is set equal to rij with i, j = 0..3, m = xO' + i, n = yO' + j, and this process is suspended until after completion of the residual colour transform process as specified in subclause S.9.5.13, after which the variable rij is set equal to RB,mn with i, j = 0..3, m = xO' + i, n = yO' + j and this process is continued.

- Otherwise (this process is invoked for the chroma component Cr), the variable RCr,mn is set equal to rij with i, j = 0..3, m = xO' + i, n = yO' + j and this process is suspended until after the completion of the residual colour transform process as specified in subclause S.9.5.13, after which the variable rij is set equal to RR,mn with i, j = 0..3, m = xO' + i, n = yO' + j and this process is continued.

- The 4x4 array u with elements uij for i, j = 0..3 is derived as

- If residual_prediction_flag is equal to 0,

uij = rij

- Otherwise (residual_prediction_flag is equal to 1)

uij = resPredC[ xO + x, yO + y ] + rij

- The picture construction process prior to deblocking filter process in subclause S.9.5.12 is invoked with chroma4x4BlkIdx, predC, and u as the inputs.

### Inverse scanning process for transform coefficients

The specification of this subclause in AVC shall apply.

### Inverse scanning process for 8x8 luma transform coefficients

The specification of this subclause in AVC shall apply.

### Derivation process for the chroma quantisation parameters and scaling function

The specification of this subclause in AVC shall apply.

### Scaling and transformation process for luma DC transform coefficients for Intra_16x16 macroblock type

The specification of this subclause in AVC shall apply with the following modifications.

1) Replace the following paragraph of the subclause in AVC

Inputs to this process are transform coefficient level values for luma DC transform coefficients of Intra_16x16 macroblocks as a 4x4 array c with elements cij, where i and j form a two-dimensional frequency index.

with the following

Inputs to the process are

- a 4x4 array c with elements cij which represents luma DC transform coefficient levels of an Intra_16x16 macroblock

- a variable numProgRef (when available)

- numProgRef 4x4 arrays ck with elements ckij with k = 1..numProgRef, which represent luma DC transform coefficient refinement levels of an Intra_16x16 macroblock

2) Replace the following paragraph of the subclause in AVC

The inverse transform for the 4x4 luma DC transform coefficients is specified by:

f=111111−1−11−1−111−11−1c00c01c02c03c10c11c12c13c20c21c22c23c30c31c32c33111111−1−11−1−111−11−1.

with the following

The inverse transform for the 4x4 luma DC transform coefficients is specified by:

f=111111−1−11−1−111−11−1c00c01c02c03c10c11c12c13c20c21c22c23c30c31c32c33111111−1−11−1−111−11−1     and

fk=111111−1−11−1−111−11−1ck00ck01ck02ck03ck10ck11ck12ck13ck20ck21ck22ck23ck30ck31ck32ck33111111−1−11−1−111−11−1     with k = 1...numProgRef

3) Before the paragraphs of the subclause in AVC

The bitstream shall not contain data that results in any element dcYij of dcY with i, j = 0..3 that exceeds the range of integer values from -2(7 + BitDepthY) to 2(7 + BitDepthY)–1, inclusive.

insert the following

When numProgRef is available and greater than 0, for each k = 1..numProgRef, the following applies.

–	If QP'Y,PRk is greater than or equal to 36, the scaled result shall be derived as

dcYij=fij+( fij*LevelScale( QP'Y,PRk%6, 0, 0 ))<<( QP'Y,PRk / 6 −6 ), with i,j=0..3.

–	Otherwise (QP'Y,PRk is less than 36), the scaled result shall be derived as

dcYij=fij+( fij∗LevelScale( QP'Y,PRk % 6, 0, 0 )+25−QP'Y,PRk/6)>>( 6 −QP'Y,PRk / 6 ), with i,j=0..3

### Scaling and transformation process for chroma DC transform coefficients

The specification of this subclause in AVC shall apply with the following modifications.

1) Replace the following paragraph of the subclause in AVC

Inputs to this process are transform coefficient level values for chroma DC transform coefficients of one chroma component of the macroblock as an (MbWidthC / 4)x(MbHeightC / 4) array c with elements cij, where i and j form a two-dimensional frequency index.

with the following

Inputs to the process are

- a (MbWidthC / 4)x(MbHeightC / 4) array c with elements cij which represents chroma DC transform coefficient levels of one chroma component of the macroblock

- a variable numProgRef (when available)

- numProgRef (MbWidthC / 4)x(MbHeightC / 4) arrays ck with elements ckij with k = 1..numProgRef, which represent chroma DC transform coefficient refinement levels of one chroma component of the macroblock

2) Replace the following paragraph of the subclause in AVC

- If chroma_format_idc is equal to 1, the inverse transform for the 2x2 chroma DC transform coefficients is specified as

f=111−1c00c01c10c11111−1

with the following

- If chroma_format_idc is equal to 1, the inverse transform for the 2x2 chroma DC transform coefficients is specified as

f=111−1c00c01c10c11111−1     and

fk=111−1ck00ck01ck10ck11111−1     with k = 1..numProgRef

3) Replace the following paragraph of the subclause in AVC

- Otherwise, if chroma_format_idc is equal to 2, the inverse transform for the 2x4 chroma DC transform coefficients is specified as

f=111111−1−11−1−111−11−1c00c01c10c11c20c21c30c31111−1     and

with the following

- Otherwise, if chroma_format_idc is equal to 2, the inverse transform for the 2x4 chroma DC transform coefficients is specified as

f=111111−1−11−1−111−11−1c00c01c10c11c20c21c30c31111−1     and

fk=111111−1−11−1−111−11−1ck00ck01ck10ck11ck20ck21ck30ck31111−1     with k = 1..numProgRef

4) Replace the following paragraph of the subclause in AVC

- Otherwise, the inverse transform for the 4x4 chroma DC transform coefficients is specified as

f=111111−1−11−1−111−11−1c00c01c02c03c10c11c12c13c20c21c22c23c30c31c32c33111111−1−11−1−111−11−1     and

with the following

- Otherwise, the inverse transform for the 4x4 chroma DC transform coefficients is specified as

f=111111−1−11−1−111−11−1c00c01c02c03c10c11c12c13c20c21c22c23c30c31c32c33111111−1−11−1−111−11−1     and

fk=111111−1−11−1−111−11−1ck00ck01ck02ck03ck10ck11ck12ck13ck20ck21ck22ck23ck30ck31ck32ck33111111−1−11−1−111−11−1     with k = 1..numProgRef

5) Before the paragraphs of the subclause in AVC

The bitstream shall not contain data that results in any element dcCij of dcC with i, j = 0..3 that exceeds the range of integer values from –2(7 + BitDepth'C) to 2(7 + BitDepth'C)–1, inclusive.

insert the following

When numProgRef is available and greater than 0, for each k = 1..numProgRef, the following applies.

- If chroma_format_idc is equal to 1, the scaled result is modified by

dcCij=fij+(( fij* LevelScale( QP'C,PRk % 6, 0, 0 ))<<( QP'C,PRk/ 6 ))>> 5, with i,j=0, 1

- If chroma_format_idc is equal to 2, the following applies.

- The variable QP'C,DC,PRk is derived as

QP'C,DC,PRk = QP'C,PRk + 3

- Depending on the value of QP'C,DC,PRk, the following applies.

- If QP'C,DC,PRk is greater than or equal to 36, the scaled result shall be modified by

dcCij=fij+( fij*LevelScale( QP'C,DC,PRk%6, 0, 0 ))<<( QP'C,DC,PRk / 6 −6 ), with i= 0..3,j=0,1

- Otherwise (QP'C,DC,PRk is less than 36), the scaled result shall be modified by

dcCij=fij+( fij∗LevelScale( QP'C,DC,PRk % 6, 0, 0 )+25−QP'C,DC,PRk/6)>>( 6 −QP'C,DC,PRk / 6 ),  with  i = 0..3,j=0,1

- Otherwise (chroma_format_idc is equal to 3), the following applies.

- If QP'C,PRk is greater than or equal to 36, the scaled result shall be modified by

dcCij=fij+( fij*LevelScale( QP'C,PRk%6, 0, 0 ))<<( QP'C,PRk / 6 −6 ), with i,j= 0..3.

- Otherwise (QP'C,PRk is less than 36), the scaled result shall be modified by

dcCij=fij+( fij∗LevelScale( QP'C,PRK % 6, 0, 0 )+25−QP'C,PRK/6)>>( 6 −QP'C,PRk / 6 ),  with  i,j = 0..3.

### Scaling and transformation process for residual 4x4 luma blocks

The specification of this subclause in AVC shall apply with the following modifications.

1) Replace the following paragraph of the subclause in AVC

Input to this process is a 4x4 array c with elements cij which is either an array relating to a residual block of the luma component or an array relating to a residual block of a chroma component.

with the following

Inputs to this process are

- a 4x4 array c with elements cij which is either an array relating to a residual block of the luma component or an array relating to a residual block of a chroma component.

- a variable numProgRef (when available)

- numProgRef 4x4 arrays ck with elements ckij with k = 1..numProgRef, which are arrays relating to residual refinement blocks of the luma component or a chroma component

2) Before the paragraph of the subclause in AVC

- Scaling of 4x4 block transform coefficient levels cij proceeds as follows.

insert the following

- For each k = 1..numProgRef, a variable qPk is derived as follows.

- If the input arrays ck relate to a luma residual block

- qPk = QP'Y,PRk

- Otherwise (the input array c relates to a chroma residual block),

- qPk = QP'C,PRk

3) Before the paragraphs of the subclause in AVC

The bitstream shall not contain data that results in any element dij of d with i, j = 0..3 that exceeds the range of integer values from -2(7 + BitDepthY) to 2(7 + BitDepthY)–1, inclusive.

insert the following

When numProgRef is available and greater than 0, for each k = 1..numProgRef, the following applies.

- If qPk is greater than or equal to 24, the scaled result shall be modified as

dij=dij+( ckij∗LevelScale( qPk%6, i, j ))<<( qP / 6 −4), with i,j=0..3.

- Otherwise (qPk is less than 24), the scaled result shall be modified as

dij=dij+( ckij∗LevelScale( qPk%6, i, j )+23−qPk6)>>( 4 −qPk / 6 ), with i,j=0..3.

### Scaling and transformation process for residual 8x8 luma blocks

The specification of this subclause in AVC shall apply with the following modifications.

1) Replace the following paragraph of the subclause in AVC

Input to this process is an 8x8 array c with elements cij which is an array relating to an 8x8 residual block of the luma component.

with the following

Inputs to this process are

- an 8x8 array c with elements cij which is an array relating to an 8x8 residual block of the luma component

- a variable numProgRef (when available)

- numProgRef 8x8 arrays ck with elements ckij with k = 1..numProgRef, which are arrays relating to 8x8 residual refinement blocks of the luma component

2) Before the paragraph of the subclause in AVC

The bitstream shall not contain data that results in any element dij of d with i, j = 0..7 that exceeds the range of integer values from -2(7 + BitDepthY) to 2(7 + BitDepthY)–1, inclusive.

insert the following

When numProgRef is available and greater than 0, for each k = 1..numProgRef, the following applies.

- If QP'Y,PRk is greater than or equal to 36, the scaled result shall be modified as

dij=dij+( ckij∗LevelScale8x8( QP'Y,PRk%6, i, j ))<<( QP'Y,PRk / 6 −6 ), with i,j=0..7.

- Otherwise (QP'Y,PRk is less than 36), the scaled result shall be modified as

dij=dij+( ckij∗LevelScale8x8( QP'Y,PRk%6, i, j )+25−QP'Y,PRk6)>>( 6 −QP'Y,PRk / 6 ), with i,j=0..7.

### Picture construction process prior to deblocking filter process

The specification of this subclause in AVC shall apply with the following modifications.

1) Replace the following paragraphs of the subclause in AVC

Inputs to this process are

- luma4x4BlkIdx or chroma4x4BlkIdx or luma8x8BlkIdx

- a sample array u with elements uij which is either a 4x4 luma block or a 4x4 chroma block or an 8x8 luma block

Outputs of this process are constructed sample blocks S' prior to the deblocking filter process.

with the following

Inputs to this process are

- luma4x4BlkIdx or chroma4x4BlkIdx or luma8x8BlkIdx

- a residual sample array u with elements uij which is the residual sample array for either a 4x4 luma block or a 4x4 chroma block or an 8x8 luma block

- when available, a prediction sample array p with elements pij which is the arrays of prediction samples for either a 4x4 luma block or a 4x4 chroma block or an 8x8 luma block

Outputs of this process are

- constructed sample blocks S' prior to the deblocking filter process or constructed residual sample blocks S' prior to both the motion-compensated prediction process and the deblocking filter process

- constructed residual sample blocks Sres

2) Replace the following paragraphs of the subclause in AVC

- Depending on the variable MbaffFrameFlag and the current macroblock, the following applies.

- If MbaffFrameFlag is equal to 1 and the current macroblock is a field macroblock

S'L[ xP + xO + j, yP + 2 * ( yO + i ) ] = uij   with i, j = 0..nE  1

- Otherwise (MbaffFrameFlag is equal to 0 or the current macroblock is a frame macroblock),

S'L[ xP + xO + j, yP + yO + i ] = uij   with i, j = 0..nE  1

with the following

- Depending on the variable MbaffFrameFlag and the current macroblock, the following applies.

- If MbaffFrameFlag is equal to 1 and the current macroblock is a field macroblock, the following applies

- If the array p is available as input to this process, the following applies

S'L[ xP + xO + j, yP + 2 * ( yO + i ) ] = Clip1Y( uij + pij )       with i, j = 0..nE  1
Sres,L[ xP + xO + j, yP + 2 * ( yO + i ) ] = uij                              with i, j = 0..nE  1

When qpprime_y_zero_transform_bypass_flag is equal to 1 and QP'Y is equal to 0, the bitstream shall not contain data that result in a value of S'L[ xP + xO + j, yP + 2 * ( yO + i ) ] that is not equal to uij + pij.

- Otherwise (the array p is no input to this process), the following applies

S'L[ xP + xO + j, yP + 2 * ( yO + i ) ] = uij                              with i, j = 0..nE  1
Sres,L[ xP + xO + j, yP + 2 * ( yO + i ) ] = uij                              with i, j = 0..nE  1

- Otherwise (MbaffFrameFlag is equal to 0 or the current macroblock is a frame macroblock), the following applies

- If the array p is available as input to this process, the following applies

S'L[ xP + xO + j, yP + yO + i ] = Clip1Y( uij + pij )       with i, j = 0..nE  1
Sres,L[ xP + xO + j, yP + yO + i ] = uij                              with i, j = 0..nE  1

When qpprime_y_zero_transform_bypass_flag is equal to 1 and QP'Y is equal to 0, the bitstream shall not contain data that result in a value of S'L[ xP + xO + j, yP + yO + i ] that is not equal to uij + pij.

- Otherwise (the array p is no input to this process), the following applies

S'L[ xP + xO + j, yP + yO + i ] = uij                              with i, j = 0..nE  1
Sres,L[ xP + xO + j, yP + yO + i ] = uij                              with i, j = 0..nE  1

3) Replace the following paragraphs of the subclause in AVC

- The subscript C in the variable S'C is replaced with Cb for the Cb chroma component and with Cr for the Cr chroma component.

with the following

- The subscript C in the variables S'C and Sres,C is replaced with Cb for the Cb chroma component and with Cr for the Cr chroma component.

4) Replace the following paragraphs of the subclause in AVC

- Depending on the variable MbaffFrameFlag and the current macroblock, the following applies.

- If MbaffFrameFlag is equal to 1 and the current macroblock is a field macroblock

S'C[ ( xP / subWidthC ) + xO + j, ( ( yP + SubHeightC – 1 ) / SubHeightC ) + 2 * ( yO + i ) ] = uij
		with i, j = 0..3

- Otherwise (MbaffFrameFlag is equal to 0 or the current macroblock is a frame macroblock),

S'C[ ( xP/ subWidthC ) + xO + j, ( yP / SubHeightC ) + yO + i ] = uij       with i, j = 0..3

with the following

- Depending on the variable MbaffFrameFlag and the current macroblock, the following applies.

- If MbaffFrameFlag is equal to 1 and the current macroblock is a field macroblock, the following applies

- Let xC and yC be two variables that are derived as follows.

xC = xP / SubWidthC
yC = ( yP + SubHeight – 1 ) / SubHeightC

- If the array p is available as input to this process, the following applies

S'C[ xC + xO + j, yC + 2 * ( yO + i )] = 	Clip1Y( uij + pij )       with i, j = 0..3
Sres,C[ xC + xO + j, yC + 2 * ( yO + i )] = uij 	                            with i, j = 0..3

- Otherwise (the array p is no input to this process), the following applies

S'C[ xC + xO + j, yC + 2 * ( yO + i )] = uij                              with i, j = 0..3
Sres,C[ xC + xO + j, yC + 2 * ( yO + i )] = uij 	                             with i, j = 0..3

- Otherwise (MbaffFrameFlag is equal to 0 or the current macroblock is a frame macroblock), the following applies

- Let xC and yC be two variables that are derived as follows.

xC = xP / SubWidthC
yC = yP / SubHeightC

- If the array p is available as input to this process, the following applies

S'C[ xC + xO + j, yC + yO + i] = Clip1Y( uij + pij )       with i, j = 0..3
Sres,C[ xC + xO + j, yC + yO + i] = uij                              with i, j = 0..3

- Otherwise (the array p is no input to this process), the following applies

S'C[ xC + xO + j, yC + yO + i] = uij                              with i, j = 0..3
Sres,C[ xC + xO + j, yC + yO + i] = uij                              with i, j = 0..3

5) Add behind the last paragraph of the subclause in AVC

NOTE – The residual sample arrays Sres,L, Sres,Cb, and Sres,Cr are only required for the residual prediction process of the picture with an indentical value of picture order count and the variables DependencyIdBase and QualityLevelBase equal to dependency_id and quality_level, respectively, of the current picture. When the decoding of a picture with a different value of picture order count is started, the residual sample arrays Sres,L, Sres,Cb, and Sres,Cr of the current picture are not required anymore.

### Residual colour transform process

The specification of this subclause in AVC shall apply.

### Residual prediction process

[Ed. Note(MW): This process is currently only specified for frame_mbs_only_flag equal to 1.]

This process is invoked when residual_prediction_flag is equal to 1.

Output of this process are

- an array resPredL[x, y], with x = 0..15 and y = 0..15, of the residual prediction luma samples for the current macroblock.

- when chroma_format_idc is not equal to 0, two arrays resPredCr[x, y] and resPredCb[x, y], with x = 0..MbWidthC - 1 and y = 0..MbHeightC - 1, of the residual prediction chroma samples for the current macroblock, one for each of the chroma components Cb and Cr.

The derivation process for base macroblocks as specified in subclause S.7.5.2 is invoked with CurrMbAddr as input and the base macroblock address array aMbAddrBase and the base picture basePic as output.

NOTE – The residual sample arrays Sres,L, Sres,Cb , and Sres,Cr that are used in the subclause correspond to the reconstructed residual samples of the base picture basePic that has been reconstructed for dependency_id equal to DependencyIdBase and quality_level equal to QualityLevelBase.

Let SBase,res,L[ x, y ] be the array of reconstructed luma residual samples Sres,L[ x, y ] of the picture basePic that has been reconstructed with dependency_id equal to DependencyIdBase and quality_level equal to QualityLevelBase.

When chroma_format_idc is not equal to 0, let SBase,res,Cb[ x, y ] and SBase,res,Cr[ x, y ] be the arrays of reconstructed chroma residual samples Sres,Cb[ x, y ] and Sres,Cr[ x, y ], respectively, of the picture basePic that has been reconstructed with dependency_id equal to DependencyIdBase and quality_level equal to QualityLevelBase.

The position of the upper-left luma sample of the current macroblock CurrMbAddr is derived by invoking the inverse macroblock scanning process in subclause S.7.4.1 with CurrMbAddr as input and the output being assigned to ( xP, yP ).

When chroma_format_idc is not equal to 0, the position ( xC, yC ) of the upper-left chroma samples of the current macroblock are derived by

xC = xP / SubWidthC
yC = yP / SubHeightC

The following parameters are defined based on syntax elements chroma_phase_x_plus1, chroma_phase_y_plus1, base_chroma_phase_x_plus1, base_chroma_phase_y_plus1:

ScaledBaseChromaPhaseX = chroma_phase_x_plus1 - 1

ScaledBaseChromaPhaseY = chroma_phase_y_plus1 - 1

BaseChromaPhaseX = base_chroma_phase_x_plus1 - 1

BaseChromaPhaseY = base_chroma_phase_y_plus1 - 1

The residual prediction samples resPredL[x, y] with x, y = 0..15 are derived as follows.

- If SpatialScalabilityType is equal to 0, the following applies.

resPredL[ x, y ] = SBase,res,L[ xP – ScaledBaseLeftOffset  + x, yP – ScaledBaseTopOffset + y ]

- Otherwise, if SpatialScalabilityType is equal to 1,

- Let aMbAddrBase[0] be assigned to currMbAddrBase

- Let ( xB, yB ) be the following

xB = ( xP – ScaledBaseLeftOffset ) / 2
yB = ( yP – ScaledBaseTopOffset ) / 2

- Let base8x8Flag be equal to the value of transform_size_8x8_flag for the base macroblock currMbAddrBase of the base picture basePic, and the following applies

- If base8x8Flag is equal to 0, for each location difference ( xD, yD ), with xD = 0..1 and yD = 0..1, the following applies.

- A 4x4 array r[x, y] with x, y = 0..3  is derived by

r[ x, y ] = SBase,res,L[ xB + 4 * xD + x, yB + 4 * yD + y ]    with x, y = 0..3

- The upsampling process as specified in S.9.5.14.1 is invoked with blkSize = 4 and r as input and rU as output.

- The residual prediction samples resPredL are derived by

resPredL[ 8 * xD + x, 8 * yD + y ] = rU[ x, y ]     with x, y = 0..7

- Otherwise (base8x8Flag is equal to 1), the following applies.

- An 8x8 array r[x, y] with x, y = 0..7 is derived by

r[ x, y ] = SBase,res,L[ xB + x, yB + y ]    with x, y = 0..7

- The upsampling process as specified in S.9.5.14.1 is invoked with blkSize = 8 and r as input and rU as output.

- The residual prediction samples resPredL are derived by

resPredL[ x, y ] = rU[ x, y ]     with x, y = 0..15

- Otherwise (SpatialScalabilityType is greater than 1), the following applies.

- Let deltaX and deltaY be defined as follows.

deltaX = BasePicWidth – ScaledBaseWidth
deltaY = BasePicHeight – ScaledBaseHeight

- Let arrays basePosX4[x] and basePosY4[y] with x = 0..15 and y = 0..15 be derived as follows.

basePosX4[x] = ( 4 * ( x + xP – ScaledBaseLeftOffset ) * BasePicWidth + 2 * deltaX ) // ScaledBaseWidth
basePosY4[y] = ( 4 * ( y + yP – ScaledBaseTopOffset ) * BasePicHeight + 2 * deltaY ) // ScaledBaseHeight

- the full-sample position ( xB, yB ) is derived as follows.

xB = basePosX4[0] >> 2
yB = basePosY4[0] >> 2

- the full-sample position ( xB1, yB1 ) is derived as follows.

xB1 = (basePosX4[15] + 3 ) >> 2
yB1 = (basePosY4[15] + 3 ) >> 2

- An array TransBlkBase[x, y] with x = 0..xB1 – xB and y = 0..yB1 – yB is derived as follows.

- A basePic macroblock address TempMbAddrBase is derived as

TempMbAddrBase = ( y + yB ) / 16 * BasePicWidthInMbs + ( x + xB ) / 16

- Let base8x8Flag be equal to the value of transform_size_8x8_flag for the macroblock TempMbAddrBase of the basePic

- If transform_size_8x8_flag is 0

TransBlkBase[x, y] = ( ( ( ( y + yB ) / 16 ) % 2 ) * 2 + ( ( ( x + xB ) / 16 ) % 2 ) ) * 16 +
                                         ( ( ( y + yB ) / 4 ) % 4 ) * 4 + ( ( ( x + xB ) / 4 ) % 4 )

- Otherwise (transform_size_8x8_flag is 1)

TransBlkBase[x, y] = ( ( ( ( y + yB ) / 16 ) % 2 ) * 2 + ( ( ( x + xB ) / 16 ) % 2 ) ) * 16 +
                                         ( ( ( y + yB ) / 8 ) % 2 ) * 2 + ( ( ( x + xB ) / 8 ) % 2 )

- An 8x8 array r[x, y] with x = x = 0..xB1 – xB and y = 0..yB1 – yB is derived by

r[ x, y ] = SBase,res,L[ xB + x, yB + y ]

- The upsampling process as specified in S.9.5.14.2 is invoked with ( xB, yB ), ( 16, 16 ), basePosX4, basePosY4, ( xB1 – xB + 1, yB1 – yB + 1 ), TransBlkBase, BitDepthY, and r as input and resPredL[x, y] with x, y = 0..15 as output.

When chroma_format_idc is not equal to 0, residual prediction samples resPredC[x, y], with x = 0..MbWidthC – 1, y = 0..MbHeightC - 1, and  with C being replaced by Cb, Cr are derived as follows

- If SpatialScalabilityType is equal to 0, the following applies.

resPredC[ x, y ] = SBase,res,C[ xC xC – ScaledBaseLeftOffsetC + x, yC – ScaledBaseTopOffsetC + y ]

- Otherwise if SpatialScalabilityType is equal to 1, the following applies.

- Let ( xCB, yCB ) be the following

xCB = ( xC – ScaledBaseTopOffsetC ) / 2
yCB = ( yC – ScaledBaseLeftOffsetC ) / 2

- For each location difference ( xD, yD ), with xD = 0..( ( MbWidthC / 8 ) – 1 ) and yD = 0.. ( ( MbHeightC / 8 ) – 1 ),.

- A 4x4 array r[x, y] with x, y = 0..3  is derived by

r[ x, y ] = SBase,res,C[ xCB + 4 * xD + x, yCB + 4 * yD + y ]

- The upsampling process as specified in S.9.5.14.1 is invoked with blkSize = 4 and r as input and rU as output.

- The residual prediction samples resPredL[x, y] with x = 0..MbWidthC – 1, y = 0..MbHeightC – 1  are derived by

resPredC[ 8 * xD + x, 8 * yD + y ] = rU[ x, y ]     with x, y = 0..7

- Otherwise (SpatialScalabilityType is greater than 1), the following applies.

- Let deltaXC and deltaYC be defined as follows.

deltaXC = ( 2 + BaseChromaPhaseX) * BasePicWidthC – ( 2 + ScaledBaseChromaPhaseX ) * ScaledBaseWidthC
deltaYC = ( 2 + BaseChromaPhaseY) * BasePicHeightC – ( 2 + ScaledBaseChromaPhaseY ) * ScaledBaseHeightC

- Let arrays basePosCX4[x] and basePosCY4[y] with x = 0..MbWidthC – 1 and y = 0..MbHeightC – 1 be derived as follows.

basePosCX4[x] = ( 4 * ( x + xC – ScaledBaseLeftOffsetC ) * BasePicWidthC + deltaXC ) // ScaledBaseWidthC
basePosCY4[y] = ( 4 * ( y + yC – ScaledBaseTopOffsetC ) * BasePicHeightC + deltaYC ) // ScaledBaseHeightC

- the full-sample position ( xCB, yCB ) is derived as follows.

xCB = basePosCX4[0] >> 2
yCB = basePosCY4[0] >> 2

- the full-sample position ( xCB1, yCB1 ) is derived as follows.

xCB1 = ( basePosCX4[ MbWidthC – 1 ] + 3 ) >> 2
yCB1 = ( basePosCY4[ MbHeightC –1 ] + 3 ) >> 2

- An array TransBlkBaseC[x, y] with x = 0..xCB1 – xCB and y = 0..yCB1 – yCB is derived as follows.

BaseMbIdxX = ( x + xCB ) / MbWidthC

BaseMbIdxY = ( y + yCB ) / MbHeightC

TransBlkBaseC[x, y] = ( ( BaseMbIdxY % 2 ) * 2 + ( BaseMbIdxX % 2 ) ) * 16 +
                                      ( ( ( y + yCB – BaseMbIdxY * MbHeightC ) / 8 ) % 2 ) * 2 +
                                      ( ( ( x + xCB – BaseMbIdxX * MbWidthC ) / 8 ) % 2 )

- An array r[x, y] with x = 0..xCB1 – xCB and y = 0..yCB1 – yCB is derived by

r[ x, y ] = SBase,res,C[ ( xCB + x, yCB + y ]

- The upsampling process as specified in S.9.5.14.2 is invoked with ( xCB, yCB ), (MbWidthC, MbHeightC ), basePosCX4, basePosCY4, ( xCB1 – xCB + 1, yCB1 – yCB + 1 ), TransBlkBaseC, BitDepthC, and r as input and resPredC[x, y] with x = 0..MbWidthC – 1, y = 0..MbHeightC – 1 as output.

#### Upsampling process for residual prediction

Input to this process are

- a variable blkSize

- a NxN array resBase with N = blkSize.

Output of this process is an MxM array resInterp with M = 2 * blkSize.

The array resInterp is derived by the following ordered steps.

- Step 1

resInterp[ 2 * x, 2 * y ] = resBase[ x, y ] with x, y = 0..blkSize - 1

- Step 2

resInterp[ 2 * x, 2 * y + 1 ]  = ( resBase[ x, y ] + resBase[ x, y + 1 ] + 1 ) >> 1 with x = 0..blkSize – 1 and y = 0..blkSize - 2

- Step 3

resInterp[ 2 * x + 1, y ] = ( resInterp[ 2 * x, y ] + resInterp[ 2 * x + 2, y ] + 1 ) >> 1 with x = 0..blkSize - 2 and y = 0..2 * blkSize - 2

- Step 4

resInterp[ x, 2 * blkSize - 1 ] = resInterp[ x, 2 * blkSize - 2 ] with x = 0..2 * blkSize - 2

- Step 5

resInterp[ 2 * blkSize – 1, y ] = resInterp[ 2 * blkSize – 2, y ] with y = 0..2 * blkSize – 1

#### Extended spatial scalability upsampling process for residual prediction

Input to this process are

- two variables ( x0, y0 ) represent an integer position offset

- two variables ( M, N ) representing the output block size

- an array posX[x] with x = 0..M – 1

- an array posY[y] with y = 0..N – 1

- two variables ( m, n ) representing the base block size

- an array TransBlkIdx[x, y] with x = 0..m – 1 and y = 0..n – 1

- a variable BitDepth to control the output data range

- an array resBase[x, y] with x = 0..m –1 and y = 0..n – 1.

Output of this process is an array resInterp[x, y] with x = 0..M – 1 and y = 0..N – 1.

Each output sample resInterp[ x, y ] with x = 0..M – 1 and y = 0..N – 1 is derived as follows.

- Let r6[i, j] with i, j = -2..3 be derived as follows.

- let (x1, y1) be derived as

x1 = ( posX[ x ] >> 2 ) – x0
y1 = ( posy[ y ] >> 2 ) – y0

- let r6[0, 0] be derived as

r6[0, 0] = r[x1, y1]

- let r6[0, j] with j = -2..3 be derived as

r6[0, -1] = ( y1 > 0 && TransBlkIdx[x1, y1] == TransBlkIdx[x1, y1 – 1] ) ? r[x1, y1 – 1] : r6[0, 0]
r6[0, -2] = ( y1 > 1 && TransBlkIdx[x1, y1] == TransBlkIdx[x1, y1 – 2] ) ? r[x1, y1 – 2] : r6[0, -1]
r6[0, 1] = ( y1 < (n-1) && TransBlkIdx[x1, y1] == TransBlkIdx[x1, y1 + 1] ) ? r[x1, y1 + 1] : r6[0, 0]
r6[0, 2] = ( y1 < (n-2) && TransBlkIdx[x1, y1] == TransBlkIdx[x1, y1 + 2] ) ? r[x1, y1 + 2] : r6[0, 1]
r6[0, 3] = ( y1 < (n-3) && TransBlkIdx[x1, y1] == TransBlkIdx[x1, y1 + 3] ) ? r[x1, y1 + 3] : r6[0, 2]

- let r6[i, j] with i, j = -2..3 be derived as

r6[-1, j] = ( x1 > 0 && TransBlkIdx[x1, y1] == TransBlkIdx[x1-1, y1] ) ? r[x1-1, y1] : r6[0, j]
r6[-2, j] = ( x1 > 1 && TransBlkIdx[x1, y1] == TransBlkIdx[x1-2, y1] ) ? r[x1-2, y1] : r6[-1, j]
r6[1, j] = ( x1 < (m-1) && TransBlkIdx[x1, y1] == TransBlkIdx[x1+1, y1] ) ? r[x1+1, y1] : r6[0, j]
r6[2, j] = ( x1 < (m-2) && TransBlkIdx[x1, y1] == TransBlkIdx[x1+2, y1] ) ? r[x1+2, y1] : r6[1, j]
r6[3, j] = ( x1 < (m-3) && TransBlkIdx[x1, y1] == TransBlkIdx[x1+3, y1] ) ? r[x1+3, y1] : r6[2, j]

- let (x2, y2) be derived as

x2 = posX[ x ] % 4
y2 = posy[ y ] % 4

- subclause S.9.4.2.2.1 is invoked with ( 2, 2 ), ( x2, y2 ), r6[ i, j ] and a flag clipFlag equal to 0 as inpus corresponding respectively to ( xIntL, yIntL ), ( xFracL, yFracL ), and the luma sample array of the selected reference picture, and with output predPartLXL[ xL, yL ] is assigned to rU.

- let resInterp[ x, y ] be derived as

resInterp[ x, y ] = Clip3( 1 – ( 1 << BitDepth ), ( 1 << BitDepth ) – 1, rU )

## Decoding process for P macroblocks in SP slices or SI macroblocks

The specification of this subclause in AVC shall apply.

## Deblocking filter process

The specification of this subclause in AVC shall apply.

### Filtering process for block edges

The specification of this subclause in AVC shall apply.

### Filtering process for a set of samples across a horizontal or vertical block edge

The specification of this subclause in AVC shall apply.

#### Derivation process for the luma content dependent boundary filtering strength

The specification of this subclause in AVC shall apply with the following modifications.

1) Replace the following paragraph of the subclause in AVC

The variable bS is derived as follows

- If the block edge is also a macroblock edge and any of the following conditions are true, a value of bS equal to 4 shall be the output:

- the samples p0 and q0 are both in frame macroblocks and either or both of the samples p0 or q0 is in a macroblock coded using an Intra macroblock prediction mode

with the following

The variable bS is derived as follows

- If the samples p0 and q0 are both in macroblocks coded using the I_BL macroblock prediction mode, a value of bS equal to 1 shall be the output:

- Otherwise, if the block edge is also a macroblock edge and any of the following conditions are true, a value of bS equal to 4 shall be the output:

- the samples p0 and q0 are both in frame macroblocks and either or both of the samples p0 or q0 is in a macroblock coded using an Intra macroblock prediction mode

2) Replace the following paragraph of the subclause in AVC

- Otherwise, if the following condition is true, a value of bS equal to 2 shall be the output:

- the luma block containing sample p0 or the luma block containing sample q0 contains non-zero transform coefficient levels

with the following

- Otherwise, if one of the following conditions are true, a value of bS equal to 2 shall be the output:

- the luma block containing sample p0 or the luma block containing sample q0 contains non-zero transform coefficient levels

- the syntax element nal_unit_type is equal to 20 and residual_prediction_flag is equal to 1 for the luma block containing sample p0 or the luma block containing sample q0 and the prediction array resPredX as derived in subclause S.9.5.14 contains non-zero samples, with X indicating the applicable component L, Cb, or Cr.

## Inverse motion-compensated filtering process

[Ed. Note(HS): This process is currently only specified for frame_mbs_only_flag equal to 1.]

Inputs to this process are all pictures not marked as “unused for reference”, all pictures marked as "residual" and the current picture CurrPic.

Outputs of this process are modified version of the input pictures.

The inverse motion compensated filtering process proceeds in the following steps.

- Let CurTemporalLevel[pic], MaxActivePrdL0[pic], MaxActivePrdL1[pic], CompleteFlag[pic], CurActivePrdL0[pic], CurActivePrdL1[pic], MaxActiveUpdL0[pic][], MaxActiveUpdL1[pic][], CurActiveUpdL0[pic][], and CurActiveUpdL1[pic][] be a set of variables of picture pic. Every decoded picture pic will have this set of variables. They are initialized as follows:

- If CurrPic is marked as “residual picture”, the following applies:

- The variable CurTemporalLevel[CurrPic] is set equal to the value of the syntax element temporal_level for the picture CurrPic minus 1.

CurTemporalLevel[CurrPic] =  temporal_level - 1

- The variable CurActivePrdLX[CurrPic] (with X being 0 or 1) is set equal to 0.

- The variable MaxActivePrdLX[CurrPic] (with X being 0 or 1) are set equal to set equal to 1 plus the maximum value of the syntax element num_ref_idx_lX_active_minus1 of all slices inside the picture CurrPic

- Otherwise (CurrPic is not marked as “residual picture”), the following applies:

- The variable CurTemporalLevel[CurrPic] is set equal to the value of the syntax element temporal_level for the picture CurrPic.

CurTemporalLevel[CurrPic] =  temporal_level

- The variables MaxActivePrdLX[CurrPic] and CurActivePrdLX[CurrPic] (with X being 0 or 1) are set equal to 0.

- Let MaxActiveUpdLX[CurrPic][] and CurActiveUpdLX[CurrPic][] (with X being 0 or 1) be 1-dimensional array of scalar values with dimension decomposition_stages. All entries of MaxActiveUpdLX[CurrPic][] and CurActiveUpdLX[CurrPic][] (with X being 0 or 1) are initially set to 0. For updLevel in the range from temporal_level to decomposition_stages-1, MaxActiveUpdLX[CurrPic][updLevel] (with X being 0 or 1) is set equal to the maximum value of the syntax element num_ref_idx_update_lX_active[updLevel ] of all slices inside the picture CurrPic.

- The variable CompleteFlag[CurrPic] is set equal to 0.

- Depending on the marking of CurrPic, the following applies

- If the picture CurrPic is marked as “residual picture”, the “check for inverse prediction process” in subclause S.9.8.1 is invoked with CurrPic as input.

- Otherwise (the picture CurrPic is not marked as “residual picture”), the “check for inverse update process” in subclause S.9.8.2 is invoked with CurrPic as input.

### Check for inverse prediction process

Inputs to this process are:

- A picture prdPic that is to be checked in this process if it can be inverse predicted.

- A picture updPic that is inverse updated prior to invoking this process. If updPic is not available it indicates that no picture is inverse updated prior to invoking this process and updPic is marked as “non-existing”.

Outputs of this process are zero or more modified pictures (including prdPic), which replace the inputs version.

When CurTemporalLevel[prdPic] is greater or equal to decomposition_stages the following applies:

- The variable CompleteFlag[prdPic] is set to 1

- The rest of the inverse prediction process is not executed.

For each picture refPic in the reference picture lists RefPicListX (with X being 0 or 1) of prdPic, the following applies:

- When CurTemporalLevel[refPic] is equal to CurTemporalLevel[prdPic] and CompleteFlag[prdPic] is equal to 0, the following applies:

- Let Y be a template variable that is derived by Y = 1 – X.

- The variable CurActiveUpdLY[refPic][CurTemporalLevel[refPic]] is incremented by 1,

CurActiveUpdLY[refPic][CurTemporalLevel[refPic]] = CurActiveUpdLY[refPic][CurTemporalLevel[refPic]] + 1

- The “check for inverse update process” in subclause S.9.8.2 is invoked with refPic and prdPic as input.

- When CompleteFlag[prdPic] is equal to 1 and updPic is marked as “non-existing” the following applies:

- The variable CurActivePrdLX[prdPic] is incremented by 1,

CurActivePrdLX[prdPic] = CurActivePrdLX[prdPic] + 1

- When all of the following conditions are true,

- CompleteFlag[prdPic] is equal to 0

- updPic is marked as “non-existing” or updPic is equal to refPic

- CurTemporalLevel[refPic] is greather than CurTemporalLevel[prdPic]

the following applies:

- The variable CurActivePrdLX[prdPic] is incremented by 1,

CurActivePrdLX[prdPic] = CurActivePrdLX[prdPic] + 1

When all of the following conditions are true for picture prdPic,

- CurActivePrdL0[prdPic] is equal to MaxActivePrdL0[prdPic]

- CurActivePrdL1[prdPic] is equal to MaxActivePrdL1[prdPic]

The following applies:

- The inverse prediction step is performed on prdPic. The “motion compensated prediction process” in subclause S.9.8.4 is invoked with prdPic as input and the output is a modified picture prdPic, which replaces the input version.

- The marking “residual picture” is removed for the picture prdPic.

- The variable CurTemporalLevel[prdPic] is incremented by 1,

CurTemporalLevel[prdPic] = CurTemporalLevel[prdPic] + 1

- When CurTemporalLevel[prdPic] is equal to decomposition_stages the following applies

- The variable CompleteFlag[prdPic] is set equal to 1.

- Otherwise (CurTemporalLevel[prdPic] is not equal to decomposition_stages) the following applies:

- The “check for inverse update process” in subclause S.9.8.2 is invoked with prdPic as input

### Check for inverse update process

Inputs to this process are

- A picture updPic that is to be checked in this process if it can be inverse updated.

- A picture prdPic that is checked for inverse prediction prior to invoking this process. If prdPic is is not available it is marked as “non-existing” and it indicates that no picture is checked for inverse prediction prior to invoking this process.

Output of this process is possibly the modified picture updPic, which replace the input version.

When CurTemporalLevel[updPic] is greater or equal to decomposition_stages the following applies:

- The variable CompleteFlag[prdPic] is set to 1

- The rest of the inverse update process is not executed.

When all of the following conditions are true for picture updPic,

- updPic is not marked as “residual picture”

- CompleteFlag[prdPic] is equal to 0

- CurTemporalLevel[updPic] is less than decomposition_stages

- CurActiveUpdL0[updPic][CurTemporalLevel[updPic]] is equal to MaxActiveUpdL0[updPic][CurTemporalLevel[updPic]]

- CurActiveUpdL1[updPic][CurTemporalLevel[updPic]] is equal to MaxActiveUpdL1[updPic][CurTemporalLevel[updPic]]

The following applies:

- The inverse update step is performed on updPic. The “motion compensated update process” in subclause S.9.8.3 is invoked with updPic and CurTemporalLevel[updPic] as inputs and the output is a modified picture updPic, which replaces the input version.

- The variable CurTemporalLevel[updPic] is incremented by 1,

CurTemporalLevel[updPic] = CurTemporalLevel[updPic] + 1

- The variable NumberOfUpdateLevel of the picture updPic is decremented  by 1,

NumberOfUpdateLevel = NumberOfUpdateLevel - 1

- For each picture refPic in the update picture lists updPicListX (with X being 0 or 1) of updPic, the following applies:

- When prdPic is marked as “non-existing” or prdPic is not equal to refPic, the “check for inverse prediction process” in subclause S.9.8.1 is invoked with refPic and updPic as inputs.

- The “check for inverse update process” in subclause S.9.8.2 is invoked with updPic as input.

### Motion-compensated update process

[Ed. Note(HS): This process is currently only specified for frame_mbs_only_flag equal to 1.]

Inputs to this process are

- a picture updPic

- a variable updLevel

Output of this process is a modified picture updPic, which replaces the input version.

Depending of the value of low_complexity_update_flag, the following applies.

- If low_complexity_update_flag is equal to 1, the low-complexity motion-compensated update process in subclause S.9.8.3.3 is invoked with updPic and updLevel as input, the output is the modified picture updPic, and the current motion-compensated update process is suspended.

- Otherwise (low_complexity_update_flag is equal to 0), the specification in the remainder of this subclause applies.

The derivation process for update picture lists in subclause S.9.2.6 is invoked with updPic and updLevel as inputs and the outputs are the update picture lists updPicList0 and updPicList1.

The derivation process for update data in subclause S.9.8.3.1 is invoked with updPic, updLevel, updPicList0, and updPicList1 as inputs and the outputs are a variable mbType for each macroblock of updPic, a variable subMbType[ ] for each each sub-macroblock of each macroblock of updPic (where applicable), variables updFlagLX[ ], updRefIdxLX[ ] (with LX being 0 or 1) for each macroblock partition of each macroblock of updPic (where applicable), variables updMvLX[ ][ ], updMvCLX[ ][ ] (with LX being 0 or 1) for each macroblock or sub-macroblock partition of each macroblock of updPic (where applicable), and a variable numConLX[ ] (with X being 0 or 1) for each 4x4 luma block of each macroblock of updPic (where applicable).

For each macroblock mbAddr of the update picture updPic for which mbType is not equal to I_NxN, the following applies.

- Let updMbLXL[ x, y ] (with X being 0 or 1) be a 16x16 array of samples, and when chroma_format_idc is not equal to 0, let updMbLXCb[ x, y ] and updMbLXCr[ x, y ] (with X being 0 or 1) be two (MbWidthC)x(MbHeightC) arrays of samples.

- Let updMbWL[ x, y ] (with X being 0 or 1) be a 16x16 array of samples, and when chroma_format_idc is not equal to 0, let updMbWCb[ x, y ] and updMbWCr[ x, y ] (with X being 0 or 1) be two (MbWidthC)x(MbHeightC) arrays of samples.

- For each macroblock partition or sub-macroblock partition mbPartIdx\subMbPartIdx of the macroblock mbAddr, the following applies.

- The variables partWidth and partHeight are derived as follows.

- If mbType is not equal to P_8x8 or P_8x8ref0 or B_8x8,

partWidth  = MbPartWidth( mbType )
partHeight = MbPartHeight( mbType )

- Otherwise,

partWidth  = SubMbPartWidth( subMbType[ mbPartIdx ] )
partHeight = SubMbPartHeight( subMbType[ mbPartIdx ] )

- When chroma_format_idc is not equal to 0, the variables partWidthC and partHeightC are derived as follows.

partWidthC  = partWidth  / SubWidthC
partHeightC = partHeight / SubHeightC

- When updFlagLX[mbPartIdx] (with X being 0 or 1) is equal to 1, the following applies.

- The update picture selection process in subclause S.9.8.3.2 is invoked with updPicListX and updRefIdxLX as inputs and the outputs are an arrays of luma samples residualPicLXL and, when chroma_format_idc is not equal to 0, two arrays of chroma samples residualPicLXCb and residualPicLXCr.

- The fractional sample interpolation process in subclause S.9.4.2.2 is invoked with mbPartIdx, subMbPartIdx, partWidth, partHeight, updMvLX, updMvCLX, residualPicLXL, a flag clipFlag equal to 0 and, when chroma_format_idc is not equal to 0, residualPicLXCb and residualPicLXCr as inputs and the outputs are the (partWidth)x(partHeight) array of luma samples predPartLXL, and, when chroma_format_idc is not equal to 0, two (partWidthC)x(partHeightC) arrays of chroma samples predPartLXCb and predPartLXCr.

- The inverse sub-macroblock partition scanning process in subclause S.7.4.2.2 is invoked with mbPartIdx and subMbPartIdx as inputs and the output is assigned to the luma location ( xP, yP ).

- When chroma_format_idc is not equal to 0, a chroma location ( xC, yC ) is derived by

xC = xP / SubWidthC
yC = yP / SubHeightC

- Each sample updMbLXL[ x, y ] with x = xP..xP + partWidth – 1 and y = yP..yP + partHeight – 1 of the 16x16 array updMbLXL[ x, y ] is modified by

updMbLXL[ x, y ] = predPartLXL[ x – xP, y – yP ]

- When chroma_format_idc is not equal to 0, each sample updMbLXC[ x, y ] (with C being Cb or Cr) with x = xC..xC + partWidthC – 1 and y = yC..yC + partHeightC – 1 of the (MbWidthC)x(MbHeightC) array updMbLXC[ x, y ] is modified by

updMbLXC[ x, y ] = predPartLXC[ x – xC, y – yC ]

- For each 4x4 luma block of the macroblock specified by the luma 4x4 block index luma4x4BlkIdx, the following applies.

- The inverse 4x4 luma block scanning process in subclause S.7.4.3 is invoked with luma4x4BlkIdx as input and the output is assigned to the luma location ( xP, yP ).

- Let mbPartIdx be the macroblock partition index of the macroblock partition that covers the luma location ( xP, yP ).

- When chroma_format_idc is not equal to 0, a chroma location ( xC, yC ) is derived by

xC = xP / SubWidthC
yC = yP / SubHeightC

- When updFlagLX[mbPartIdx] (with X being 0 or 1) is equal to 1, a weighting factor wX is derived as follows.

- An energy measure eX is derived by

- The weighting factor wX is derived by

wX = max( 0, min( 8, numConLX[ luma4x4BlkIdx ] ) )  *  max( 0, min( 16, 20 – eX ) )

- Each sample updMbWL[ x, y ] with x = xP..xP + 3 and y = yP..yP + 3 of the 16x16 array updMbWL[ x, y ] is modified as follows. [Ed. Note(HS): The following does not take into account the prediction weights of the corresponding motion compensated prediction steps.]

- If updFlagL0[mbPartIdx] is equal to 1 and updFlagL1[mbPartIdx] is equal to 1,

updMbWL[ x, y ] = ( ( w0 * updMbL0L[ x, y ] ) / 128 + ( w1 * updMbL1L[ x, y ] ) / 128 + 1 ) >> 1

- Otherwise, if updFlagL0[mbPartIdx] is equal to 1,

updMbWL[ x, y ] = ( ( w0 * updMbL0L[ x, y ] ) / 128 + 1 ) >> 1

- Otherwise,

updMbWL[ x, y ] = ( ( w1 * updMbL1L[ x, y ] ) / 128 + 1 ) >>1

- When chroma_format_idc is not equal to 1, each sample updMbWC[ x, y ] (with C being Cb or Cr) with x = xC..xC + ( 4 / SubWidthC ) – 1 and y = yC..yC + ( 4 / SubHeightC ) – 1 of the (MbWidthC)x(MbHeightC) array updMbWC[ x, y ] is modified as follows. [Ed. Note(HS): The following does not take into account the prediction weights of the corresponding motion compensated prediction steps.]

- If updFlagL0[mbPartIdx] is equal to 1 and updFlagL1[mbPartIdx] is equal to 1,

updMbWC[ x, y ] = ( ( w0 * updMbL0C[ x, y ] ) / 128 + ( w1 * updMbL1C[ x, y ] ) / 128 + 1 ) >> 1

- Otherwise, if updFlagL0[mbPartIdx] is equal to 1,

updMbWC[ x, y ] = ( ( w0 * updMbL0C[ x, y ] ) / 128 + 1 ) >> 1

- Otherwise,

updMbWC[ x, y ] = ( ( w1 * updMbL1C[ x, y ] ) / 128 + 1 ) >> 1

- The picture updPic, which is stored in the decoded picture buffer, is modified as follows.

- Let S'L[ x, y], S'Cb[ x, y], and S'Cr[ x, y] represent the luma and chroma samples arrays of the picture updPic.

- The inverse macroblock scanning process in subclause S.7.4.1 is invoked with mbAddr as input and the output is assigned to the luma location ( xP, yP ).

- When chroma_format_idc is not equal to 0, a chroma location ( xC, yC ) is derived by

xC = xP / SubWidthC
yC = yP / SubHeightC

- Each sample S'L[ x, y] with x = xP..xP + 15 and y = yP..yP + 15 is modified by

S'L[ x, y] = Clip1Y( S'L[ x, y] – ( updMbWL[ x – xP, y – yP ] >> 1 ) )

- When chroma_format_idc is not equal to 0, each sample S'C[ x, y] (with C being Cb or Cr) with x = xC..xC + MbWidthC – 1 and y = yC..yC + MbHeightC – 1 is modified by

S'C[ x, y] = Clip1Y( S'C[ x, y] – ( updMbWC[ x – xP, y – yP ] >> 1 ) )

#### Derivation process of update data

[Ed. Note(HS): This process is currently only specified for frame_mbs_only_flag equal to 1.]

Inputs to this process are

- a picture updPic

- a variable updLevel

- the update picture lists updPicList0 and updPicList1

Output of this process are

- a variable mbType for each macroblock of the picture updPic

- variables subMbType[ mbPartIdx ] for each sub-macroblock of each macroblock of the picture updPic (where applicable)

- the variables updFlagL0[ mbPartIdx ], updFlagL1[ mbPartIdx ], updRefIdxL0[ mbPartIdx ], and updRefIdxL1[ mbPartIdx ] for each macroblock partition of each macroblock of the picture updPic (where applicable)

- the luma motion vectors updMvL0[ mbPartIdx ][ subMbPartIdx ] and updMvL1[ mbPartIdx ][ subMbPartIdx ] and the chroma motion vector updMvCL0[ mbPartIdx ][ subMbPartIdx ] and updMvCL1[ mbPartIdx ][ subMbPartIdx ] for each macroblock or sub-macroblock partition of each macroblock of the picture updPic (where applicable)

- variables numConL0[ luma4x4BlkIdx] and numConL1[ luma4x4BlkIdx] for each 4x4 luma block of each macroblock of the picture updPic (where applicable)

- when low_complexity_update_flag is equal to 1, the rounded integer-sample luma motion vectors updMvIntL0[ mbPartIdx ][ subMbPartIdx ] and updMvIntL1[ mbPartIdx ][ subMbPartIdx ] (where applicable)

Let maxActiveLX (with X being 0 or 1) be a variable that is set equal to the maximum value of the syntax element num_ref_idx_update_active_lX[ updLevel ] of all slices inside the picture updPic.

Let mvUpdateLX[ r, x, ,y ] (with X being 0 or 1) be a 3-dimensional array of motion vectors and let numConnectedLX[ r, x, y ] (with X being 0 or 1) be a 3-dimensional array of scalar values. These arrays have the dimensions ( maxActiveLX )x( PicWidthInMbs / 4 )x( PicHeightInMbs / 4 ). Both motion vector components of all entries of mvUpdateLX[ r, x, ,y ] are initially set to 0, and all entries of numConnectedLX[ r, x, y ] are initially set to 0.

The arrays mvUpdateLX[ r, x, ,y ] and numConnectedLX[ r, x, y ] are modified as specified in the following. This modification process proceeds over all pictures pic of the update picture list updPicListLX (with X being 0 or 1) in increasing order of the reference index; for each picture pic, it proceeds over all macroblocks mbAddr in scanning order; for each macroblock mbAddr, it proceeds over all 4x4 luma blocks luma4x4BlkIdx in scanning order; and for each 4x4 luma block inside each macroblock of each picture, the following applies.

- When the macroblock mbAddr of the picture pic was not coded using an intra macroblock prediction mode, the following applies.

- Let mbPartIdx and subMbPartIdx be the macroblock and sub-macroblock partition index, respectively, that identify the macroblock or sub-macroblock partition inside the macroblock mbAddr in the picture pic, which covers the 4x4 luma block luma4x4BlkIdx.

- Let Y be a template variable that is derived by Y = 1 – X.

- Let r be the reference index RefIdxLY[ mbPartIdx ][ subMbPartIdx ] of the macroblock mbAddr in the picture pic.

- Let refPic be the picture that is referenced by r using the reference picture list RefPicListY that was derived for the slice, which contains the macroblock mbAddr inside the picture pic.

- When the picture refPic is equal to the picture updPic, the following applies.

- Let refMV be the motion vector MvLY[ mbPartIdx ][ subMbPartIdx ] of the macroblock mbAddr in the picture pic.

- Let ( x, y ) be a luma location that is derived as follows.

- The inverse macroblock scanning process in subclause S.7.4.1 is invoked with mbAddr as input and the output is a luma location ( xO, yO ).

- The inverse 4x4 luma block scanning process in subclause S.7.4.3 is invoked with luma4x4BlkIdx as input and the output is a luma location ( xP, yP ).

- The luma location ( x, y ) is derived by

x = xO + xP + ( ( refMv + 2 ) >> 2 ) 	(S-8-15)

y = yO + yP + ( ( refMv + 2 ) >> 2 ) 	(S-8-16)

- For each luma location difference ( xD, yD ) of the ordered set { ( 0, 0 ), ( 4, 0 ), ( 0, 4 ), ( 4, 4 ) }, the following applies.

- Let ( xB, yB ) be a luma block coordinate that is derived by

xB = ( x + xD ) >> 2		(S-8-17)

yB = ( y + yD ) >> 2		(S-8-18)

- Let numSamples be a variable that is derived by

numSamples = ( xD > 0  ?  x % 4  :  4 – ( x % 4 ) )  *  ( yD > 0  ?  y % 4  :  4 – ( y % 4 ) ) 			(S-8-19)

- When xB is greater or equal to 0, yB is greater or equal to 0, xB is less than ( 4 * PicWidthInMbs ), and yB is less than ( 4 * PicHeightInMbs ), the following applies.

- If mvUpdateLX[ r, xB, yB ][ 0 ] is equal to ( –refMv[ 0 ] ) and mvUpdateLX[ r, xB, yB ][ 1 ] is equal to ( –refMv[ 1 ] ), the following applies

numConnectedLX[ r, xB, yB ] += numSamples

- Otherwise, the following applies

numConnectedLX[ r, xB, yB ] = numSamples 	(S-8-20)

mvUpdateLX[ r, xB, yB ][ 0 ]  = –refMv[ 0 ] 	(S-8-21)

mvUpdateLX[ r, xB, yB ][ 1 ]  = –refMv[ 1 ] 	(S-8-22)

For each macroblock mbAddr of the picture updPic, the following applies.

- The inverse macroblock scanning process in subclause S.7.4.1 is invoked with mbAddr as input and the output is the luma location ( xO, yO ).

- The variable mbType is initially set equal to B_8x8.

- For each of the 4 macroblock partitions or sub-macroblocks specified by mbPartIdx or luma8x8BlkIdx, the following applies.

- The inverse 8x8 luma block scanning process in subclause S.7.4.4 is invoked with luma8x8BlkIdx as input and the output is the luma location ( xP, yP ).

- Let numRefIdxActiveLX (with X being 0 or 1) be the num_ref_idx_update_active_lX[ updLevel ] of the slice that contains the macroblock mbAddr inside the picture updPic.

- The reference indices updRefIdxLX[ mbPart ] and update flags updFlagLX[ mbPart ] (with X being replaced by 0 or 1) are derived by the following pseudo-code

updRefIdxLX[ mbPartIdx ] = –1
numMaxSamples                  =  0
for( r = 0; r < numRefIdxActiveLX; r++ )  {
    numSamples = ( numConnectedLX[ r, ( xO + xP ) / 4 ,       ( yO + yP ) / 4 ] +
                               numConnectedLX[ r, ( xO + xP ) / 4 + 1,  ( yO + yP ) / 4 ] +
                               numConnectedLX[ r, ( xO + xP ) / 4,        ( yO + yP ) / 4 + 1 ] +
                               numConnectedLX[ r, ( xO + xP ) / 4 + 1,  ( yO + yP ) / 4 + 1 ] )
    if( numSamples > numMaxSamples )  {
        updRefIdxLX[ mbPartIdx ] = r
        numMaxSamples                  = numSamples
    }
}

updFlagLX[ mbPart ] = ( updRefIdxLX >= 0  ?  1  :  0 ) 	(S-8-23)

- Depending on the values of updFlagL0 and updFlagL1), the sub-macroblock type subMbType[ mbPartIdx ] is derived as follows

- If updFlagL0 is equal to 1 and updFlagL1 is equal to 1, subMbType[ mbPartIdx ] is set equal to B_Bi_4x4.

- Otherwise, if updFlagL0 is equal to 1, subMbType[ mbPartIdx ] is set equal to B_L0_4x4.

- Otherwise, if updFlagL1 is equal to 1, subMbType[ mbPartIdx ] is set equal to B_L1_4x4.

- Otherwise (updFlagL0 is equal to 0 and updFlagL1 is equal to 0), subMbType[ mbPartIdx ] is marked as “undefined”.

- When any of the variables subMbType[ mbPartIdx ] for the macroblock mbAddr is marked as “undefined”, the variable mbType is set equal to I_NxN.

- When mbType is not equal to I_NxN, for each sub-macroblock partition or 4x4 luma block specified by mbPartIdx and subMbPartIdx or luma4x4BlkIdx, the following applies.

- The inverse 4x4 luma block scanning process in subclause S.7.4.3 is invoked with luma4x4BlkIdx as input and the output is the luma location ( xP, yP ).

- When updFlagLX[ mbPartIdx ] (with X being 0 or 1) is equal to 1, the following applies.

- The components of the motion vector updMvLX[ mbPartIdx ][ subMbPartIdx ] are derived by

updMvLX[ mbPartIdx ][ subMbPartIdx ][ 0 ] =
                 mvUpdateLX[ [ updRefIdxLX[ mbPartIdx ], ( xO + xP ) / 4, ( yO + yP ) / 4 ][ 0 ] 	(S-8-24)
updMvLX[ mbPartIdx ][ subMbPartIdx ][ 1 ] =
                 mvUpdateLX[ [ updRefIdxLX[ mbPartIdx ], ( xO + xP ) / 4, ( yO + yP ) / 4 ][ 1 ] 	(S-8-25)

- When low_complexity_update_flag is equal to 1, the components of the rounded integer-sample motion vector updMvLX’[ mbPartIdx ][ subMbPartIdx ] are derived by:

updMvIntLX[ mbPartIdx ][ subMbPartIdx ][ 0 ] =
		( ( updMvLX[ mbPartIdx ][ subMbPartIdx ][ 0 ] + 1 ) >> 2 ) 	(S-8-26)
updMvIntLX[ mbPartIdx ][ subMbPartIdx ][ 1 ] =
		( ( updMvLX[ mbPartIdx ][ subMbPartIdx ][ 1 ] + 1 ) >> 2 ) 	(S-8-27)

- The derivation process for chroma motion vectors in subclause S.9.4.1.4 is invoked with updMvLX[ mbPartIdx ][ subMbPartIdx ] and updRefIdxLX[ mbPartIdx ] as inputs and the output is assigned to the chroma motion vector updMvCLX[ mbPartIdx ][ subMbPartIdx ].

NOTE – Due to the weighting process specified in subclause S.9.8.3, sub-macroblock partitions or 4x4 luma blocks, for which the corresponding variable numConnectedLX[ ] is less than or equal to 8, are not updated using a picture of list X. Thus, in an efficient implementation, the motion vectors updMvLX and updMvCLX for these sub-macroblock partitions don’t need to be derived and no motion compensation process needs to be performed for the list X prediction.

- The variables numConLX[ luma4x4BlkIdx ] (with X being 0 or 1) are derived by

numConLX[ luma4x4BlkIdx ] =
                   numConnectedLX[ [ updRefIdxLX[ mbPartIdx ], ( xO + xP ) / 4, ( yO + yP ) / 4 ] 	(S-8-28)

NOTE – For an efficient implementation, it is possible to further summarize the 4x4 sub-macroblock and 8x8 macroblock partitions based on the reference indices, motion vectors, and the variables numConLX in order to obtain larger partitions for the motion compensation process. Such a summarisation will lead to numerical identical decoding results as the process that is described in this subclause, but the complexity of the motion compensated update process can be reduced. [Ed: Note(HS): That does not seem to be true for the simplified update process.]

#### Update picture selection process

[Ed. Note(HS): This process is currently only specified for frame_mbs_only_flag equal to 1.]

Inputs to this process are

- an update picture list updPicListX

- a reference index updIdxLX

Outputs of this process are

- an (PicWidthInSamplesL)x(PicHeightInSamplesL) array of luma samples residualPicLXL

- when chroma_format_idc is not equal to 0, two (PicWidthInSamplesC)x(PicHeightInSamplesC) arrays of chroma samples residualPicLXCb and residualPicLXCr

Let refPic be the picture that is referred by PicNum = updPicListX[ updIdxLX ].

Let refPicLXL be the (PicWidthInSamplesL)x(PicHeightInSamplesL) arrays of luma samples of the picture refPic.

Each sample residualPicLXL[ x, y ] of the array of luma samples residualPicLXL with x = 0.. PicWidthInSamplesL and y = 0.. PicHeightInSamplesL is derived as follows.

- If the luma sample refPicLXL[ x, y ] in the picture refPic represents a sample of a macroblock coded in Intra macroblock prediction mode,

residualPicLXL[ x, y ] = 0 		(S-8-29)

- Otherwise,

residualPicLXL[ x, y ] = refPicLXL[ x, y ] 		(S-8-30)

When chroma_format_idc is not equal to 0, the following applies.

- Let refPicLXCb and refPicLXCr be the (PicWidthInSamplesC)x(PicHeightInSamplesC) arrays of chroma samples of the picture refPic.

- Each sample residualPicLXC[ x, y ] (with Z being Cb or Cr) of the array of chroma samples residualPicLXC with x = 0.. PicWidthInSamplesC and y = 0.. PicHeightInSamplesC is derived as follows.

- If the chroma sample refPicLXC[ x, y ] in the picture refPic represents a sample of a macroblock coded in Intra macroblock prediction mode,

residualPicLXC[ x, y ] = 0 		(S-8-31)

- Otherwise,

residualPicLXC[ x, y ] = refPicLXC[ x, y ] 		(S-8-32)

#### Low-complexity motion-compensated update process

[Ed. Note(HS): This process is currently only specified for frame_mbs_only_flag equal to 1.]

Inputs to this process are

- a picture updPic

- a variable updLevel

Output of this process is a modified picture updPic, which replaces the input version.

The derivation process for update picture lists in subclause S.9.2.6 is invoked with updPic and updLevel as inputs and the outputs are the update picture lists updPicList0 and updPicList1.

The derivation process for update data in subclause S.9.8.3.1 is invoked with updPic, updLevel, updPicList0, and updPicList1 as inputs and the outputs are a variable mbType for each macroblock of updPic, a variable subMbType[ ] for each each sub-macroblock of each macroblock of updPic (where applicable), variables updFlagLX[ ], updRefIdxLX[ ] (with LX being 0 or 1) for each macroblock partition of each macroblock of updPic (where applicable), variables updMvLX[ ][ ], updMvCLX[ ][ ] (with LX being 0 or 1) for each macroblock or sub-macroblock partition of each macroblock of updPic (where applicable), and a variable numConLX[ ] (with X being 0 or 1) for each 4x4 luma block of each macroblock of updPic (where applicable), and the variables updMvIntLX[ ][ ] (with LX being 0 or 1) for each macroblock or sub-macroblock partition of each macroblock of updPic (where applicable).

For each macroblock mbAddr of the update picture updPic for which mbType is not equal to I_NxN, the following applies.

- Let updMbLXL[ x, y ] (with X being 0 or 1) be a 16x16 array of samples, and when chroma_format_idc is not equal to 0, let updMbLXCb[ x, y ] and updMbLXCr[ x, y ] (with X being 0 or 1) be two (MbWidthC)x(MbHeightC) arrays of samples.

- Let updMbWL[ x, y ] (with X being 0 or 1) be a 16x16 array of samples, and when chroma_format_idc is not equal to 0, let updMbWCb[ x, y ] and updMbWCr[ x, y ] (with X being 0 or 1) be two (MbWidthC)x(MbHeightC) arrays of samples.

- For each 4x4 luma block of the macroblock specified by the luma 4x4 block index luma4x4BlkIdx, the following applies.

NOTE – Due to the formulation of the derivation process for update data in subclause S.9.8.3.1, the motion vectors updMvLX, updMvCLX are specified for macroblock sub-partitions that are identical with 4x4 luma blocks.

- The inverse 4x4 luma block scanning process in subclause S.7.4.3 is invoked with luma4x4BlkIdx as input and the output is assigned to the luma location ( xP, yP ).

- Let mbPartIdx\subMbPartIdx be the macroblock partition and subpartition index of the macroblock partition that covers the luma location ( xP, yP ). mbPartIdx\subMbPartIdx and variables partWidth and partHeight are derived by

mbPartIdx = luma4x4BlkIdx / 4 		(S-8-33)
subMbPartIdx = luma4x4BlkIdx % 4 		(S-8-34)

partWidth  = 4 		(S-8-35)
partHeight = 4 		(S-8-36)

- When chroma_format_idc is not equal to 0, a chroma location ( xC, yC ) and variables partWidthC and partHeightC are derived by

xC = xP / SubWidthC 		(S-8-37)
yC = yP / SubHeightC 		(S-8-38)

partWidthC  = partWidth  / SubWidthC 		(S-8-39)
partHeightC = partHeight / SubHeightC 		(S-8-40)

- When updFlagLX[mbPartIdx] (with X being 0 or 1) is equal to 1, the following applies.

- The update picture selection process in subclause S.9.8.3.2 is invoked with updPicListX and updRefIdxLX as inputs and the outputs are an arrays of luma samples residualPicLXL and, when chroma_format_idc is not equal to 0, two arrays of chroma samples residualPicLXCb and residualPicLXCr.

- The integer luma sample motion compensation process in subclause S.9.8.3.3.1 is invoked with mbPartIdx, subMbPartIdx, partWidth, partHeight, updMvIntLX, and residualPicXL as inputs, and the output is the (partWidth)x(partHeight) array of luma samples predPartLXL.

- A weighting factor wX is derived by

wX = max( 0, min( 8, numConLX[ luma4x4BlkIdx ] ) )  *  max( 0, min( 16, 20 – eX ) ) 	(S-8-41)

with

eX=128+x=03y=03predPartLXL[x,y]>>(BitDepth−8)2 	(S-8-42)

- The low-complexity fractional sample interpolation process in subclause S.9.8.3.3.2 is invoked with mbPartIdx, subMbPartIdx, partWidth, partHeight, updMvLX, updMvCLX, wX, and residualPicLXL as inputs and the output is the (partWidth)x(partHeight) array of luma samples predPartLXL

- When chroma_format_idc is not equal to 0, the fractional sample interpolation process in subclause S.9.4.2.2 is invoked with mbPartIdx, subMbPartIdx, partWidth, partHeight, updMvLX, updMvCLX, residualPicLXCb, residualPicLXCr, and clipFlag equal to 0 as inputs and the outputs are two (partWidthC)x(partHeightC) arrays of chroma samples predPartLXCb and predPartLXCr.

- Each sample updMbLXL[ x, y ] with x = xP..xP + partWidth – 1 and y = yP..yP + partHeight – 1 of the 16x16 array updMbLXL[ x, y ] is modified by

updMbLXL[ x, y ] = predPartLXL[ x – xP, y – yP ] 	(S-8-43)

- When chroma_format_idc is not equal to 0, each sample updMbLXC[ x, y ] (with C being Cb or Cr) with x = xC..xC + partWidthC – 1 and y = yC..yC + partHeightC – 1 of the (MbWidthC)x(MbHeightC) array updMbLXC[ x, y ] is modified by

updMbLXC[ x, y ] = predPartLXC[ x – xC, y – yC ] 	(S-8-44)

- Each sample updMbWL[ x, y ] with x = xP..xP + 3 and y = yP..yP + 3 of the 16x16 array updMbWL[ x, y ] is derived as follows. [Ed. Note(HS): The following does not take into account the prediction weights of the corresponding motion compensated prediction steps.]

- If updFlagL0[mbPartIdx] is equal to 1 and updFlagL1[mbPartIdx] is equal to 1,

updMbWL[ x, y ] = ( ( w0 * updMbL0L[ x, y ] ) / 128 + ( w1 * updMbL1L[ x, y ] ) / 128 + 1 ) >> 1 
			(S-8-45)

- Otherwise, if updFlagL0[mbPartIdx] is equal to 1,

updMbWL[ x, y ] = ( ( w0 * updMbL0L[ x, y ] ) / 128 + 1 ) >> 1 	(S-8-46)

- Otherwise,

updMbWL[ x, y ] = ( ( w1 * updMbL1L[ x, y ] ) / 128 + 1 ) >>1 	(S-8-47)

- When chroma_format_idc is not equal to 1, each sample updMbWC[ x, y ] (with C being Cb or Cr) with x = xC..xC + ( 4 / SubWidthC ) – 1 and y = yC..yC + ( 4 / SubHeightC ) – 1 of the (MbWidthC)x(MbHeightC) array updMbWC[ x, y ] is modified as follows. [Ed. Note(HS): The following does not take into account the prediction weights of the corresponding motion compensated prediction steps.]

- If updFlagL0[mbPartIdx] is equal to 1 and updFlagL1[mbPartIdx] is equal to 1,

updMbWC[ x, y ] = ( ( w0 * updMbL0C[ x, y ] ) / 128 + ( w1 * updMbL1C[ x, y ] ) / 128 + 1 ) >> 1 	(S-8-48)

- Otherwise, if updFlagL0[mbPartIdx] is equal to 1,

updMbWC[ x, y ] = ( ( w0 * updMbL0C[ x, y ] ) / 128 + 1 ) >> 1 	(S-8-49)

- Otherwise,

updMbWC[ x, y ] = ( ( w1 * updMbL1C[ x, y ] ) / 128 + 1 ) >> 1 	(S-8-50)

- The picture updPic, which is stored in the decoded picture buffer, is modified as follows.

- Let S'L[ x, y], S'Cb[ x, y], and S'Cr[ x, y] represent the luma and chroma samples arrays of the picture updPic.

- The inverse macroblock scanning process in subclause S.7.4.1 is invoked with mbAddr as input and the output is assigned to the luma location ( xP, yP ).

- When chroma_format_idc is not equal to 0, a chroma location ( xC, yC ) is derived by

xC = xP / SubWidthC 		(S-8-51)
yC = yP / SubHeightC 		(S-8-52)

- Each sample S'L[ x, y] with x = xP..xP + 15 and y = yP..yP + 15 is modified by

S'L[ x, y] = Clip1Y( S'L[ x, y] – ( updMbWL[ x – xP, y – yP ] >> 1 ) ) 	(S-8-53)

- When chroma_format_idc is not equal to 0, each sample S'C[ x, y] (with C being Cb or Cr) with x = xC..xC + MbWidthC – 1 and y = yC..yC + MbHeightC – 1 is modified by

S'C[ x, y] = Clip1Y( S'C[ x, y] – ( updMbWC[ x – xP, y – yP ] >> 1 ) ) 	(S-8-54)

##### Integer-luma-sample motion compensation process

Inputs to this process are

- the current partition given by its partition index mbPartIdx and its sub-macroblock partition index subMbPartIdx,

- the width and height partWidth, partHeight of this partition in luma-sample units,

- an integer-sample luma motion vector mvLX given in integer-luma-sample units,

- the selected reference picture sample array refPicLXL

Outputs of this process are a (partWidth)x(partHeight) array predPartLXL of prediction luma sample values

Let ( xAL, yAL ) be the location given in full-sample units of the upper-left luma sample of the current partition given by mbPartIdx\subMbPartIdx relative to the upper-left luma sample location of the given two-dimensional array of luma samples.

The variable refPicHeightEffectiveL, which is the height of the effective reference picture luma array, is derived as follows.

- If MbaffFrameFlag is equal to 0 or mb_field_decoding_flag is equal to 0, refPicHeightEffectiveL is set equal to PicHeightInSamplesL.

- Otherwise (MbaffFrameFlag is equal to 1 and mb_field_decoding_flag is equal to 1), refPicHeightEffectiveL is set equal to PicHeightInSamplesL / 2.

For each luma sample location (0 <= xL < partWidth, 0 <= yL < partHeight) inside the prediction luma sample array predPartLXL, the corresponding prediction luma sample value predPartLXL[ xL, yL ] is derived as follows:

- The location ( xZL, yZL ) inside the given two-dimensional array refPicLXL of luma samples is derived by

xZL = Clip3( 0, PicWidthInSamplesL – 1, xAL + xL + mvLX[ 0 ] ) 	(S-8-55)
yZL = Clip3( 0, refPicHeightEffectiveL – 1, yAL + yL + mvLX[ 1 ] ) 	(S-8-56)

- The prediction luma sample value predPartLXL[ xL, yL ] is set equal to the luma sample at full-sample location ( xZL, yZL ) inside the given two-dimensional array refPicLXL of luma samples.

##### Low-complexity fractional sample interpolation process for luma samples

Inputs to this process are

- the current partition given by its partition index mbPartIdx and its sub-macroblock partition index subMbPartIdx,

- the width and height partWidth, partHeight of this partition in luma-sample units,

- a luma motion vector mvLX given in quarter-luma-sample units,

- a weighting factor wX

- the selected reference picture sample array refPicLXL

Outputs of this process are a (partWidth)x(partHeight) array predPartLXL of prediction luma sample values

Let ( xAL, yAL ) be the location given in full-sample units of the upper-left luma sample of the current partition given by mbPartIdx\subMbPartIdx relative to the upper-left luma sample location of the given two-dimensional array of luma samples.

Let ( xIntL, yIntL ) be a luma location given in full-sample units and ( xFracL, yFracL ) be an offset given in quarter-sample units. These variables are used only inside this subclause for specifying general fractional-sample locations inside the reference sample array refPicLXL.

For each luma sample location (0 <= xL < partWidth, 0 <= yL < partHeight) inside the prediction luma sample array predPartLXL, the corresponding prediction luma sample value predPartLXL[ xL, yL ] is derived as follows:

- The variables xIntL, yIntL, xFracL, and yFracL are derived by

xIntL = xAL + ( mvLX[ 0 ] >> 2 ) + xL 		(S-8-57)
yIntL = yAL + ( mvLX[ 1 ] >> 2 ) + yL 		(S-8-58)

xFracL = mvLX[ 0 ] & 3 		(S-8-59)
yFracL = mvLX[ 1 ] & 3 		(S-8-60)

- The prediction luma sample value predPartLXL[ xL, yL ] is derived as follows.

- If wX is greater than 64, the prediction sample value predPartLXL[ xL, yL ] is derived by invoking the luma sample interpolation process with 4-tap filter in subclause S.9.8.3.3.2.1 with ( xIntL, yIntL ), ( xFracL, yFracL ) and refPicLXL given as input.

- Otherwise (wX is less than or equal to 64), the prediction sample value predPartLXL[ xL, yL ] is derived by invoking the luma sample interpolation process with bi-linear filter in subclause S.9.8.3.3.2.2 with ( xIntL, yIntL ), ( xFracL, yFracL ) and refPicLXL given as input.

###### Luma sample interpolation process with a 4-tap filter

Inputs to this process are

- a luma location in full-sample units ( xIntL, yIntL ),

- a luma location offset in fractional-sample units ( xFracL, yFracL ), and

- the luma sample array of the selected reference picture refPicLXL

Output of this process is a predicted luma sample value predPartLXL[ xL, yL ].

The variable refPicHeightEffectiveL, which is the height of the effective reference picture luma array, is derived as follows.

- If MbaffFrameFlag is equal to 0 or mb_field_decoding_flag is equal to 0, refPicHeightEffectiveL is set equal to PicHeightInSamplesL.

- Otherwise (MbaffFrameFlag is equal to 1 and mb_field_decoding_flag is equal to 1), refPicHeightEffectiveL is set equal to PicHeightInSamplesL / 2.

In  Figure S-8-3, the positions labelled with upper-case letters within shaded blocks represent luma samples at full-sample locations inside the given two-dimensional array refPicLXL of luma samples. These samples may be used for generating the predicted luma sample value predPartLXL[ xL, yL ]. The locations ( xZL, yZL ) for each of the corresponding luma samples Z, where Z may be A, B, C, D, E, F, G, H, I, J, K, or L, inside the given array refPicLXL of luma samples are derived as follows:

xZL = Clip3( 0, PicWidthInSamplesL – 1, xIntL + xDZL ) 	(S-8-61)
yZL = Clip3( 0, refPicHeightEffectiveL – 1, yIntL + yDZL ) 	(S-8-62)

Figure S-8-3 – Integer samples (shaded blocks with upper-case letters) and fractional sample positions (un-shaded blocks with lower-case letters) for quarter sample luma interpolation

Table S-8-8 specifies ( xDZL, yDZL ) for different replacements of Z.

Table S-8-8 – Differential full-sample luma locations

| Z | A | B | C | D | E | F | G | H | I | J | K | L |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| xDZL | 0 | 1 | -1 | 0 | 1 | 2 | -1 | 0 | 1 | 2 | 0 | 1 |
| yDZL | -1 | -1 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 2 | 2 |

Given the luma samples ‘A’ to ‘L’ at full-sample locations ( xAL, yAL ) to ( xLL, yLL ), the luma samples ‘a’ to ‘s’ at fractional sample positions are derived by applying a 4-tap filters with tap values that depend on the sample positions. The process for each fractional position is described below.

- The samples at half sample positions labelled b shall be derived by first calculating intermediate values denoted as b1 by applying the 4-tap filters ( -2, 10, 10, -2 ) to the nearest integer position samples in the horizontal direction. The samples at quarter sample positions labelled a and c shall be derived by first calculating intermediate values denoted as a1 and c1 by applying the 4-tap filters ( -2, 14, 5, -1 ) and (-1, 5, 14, -2 ), respectively, to the nearest integer position samples in the horizontal direction:

a1 = (-2 * C + 14 * D + 5 * E – F ) 		(S-8-63)
b1 = (-2 * C + 10 * D + 10 * E – 2 * F ) 		(S-8-64)
e1 = (-C + 5 * D + 14 * E – 2 * F ) 		(S-8-65)

- The final prediction values a, b, and c shall be derived using:

a = ( a1 + 8 ) >> 4  		(S-8-66)
b = ( b1 + 8 ) >> 4  		(S-8-67)
c = ( c1 + 8 ) >> 4 		(S-8-68)

- The samples at half sample positions labelled h shall be derived by first calculating intermediate values denoted as b1 by applying the 4-tap filters ( -2, 10, 10, -2 ) to the nearest integer position samples in the vertical direction. The samples at quarter sample positions labelled d and m shall be derived by first calculating intermediate values denoted as d1 and m1 by applying the 4-tap filters ( -2, 14, 5, -1 ) and (-1, 5, 14, -2 ), respectively, to the nearest integer position samples in the vertical direction:

d1 = (-2 * A + 14 * D + 5 * H – K )
h1 = (-2 * A + 10 * D + 10 * H – 2 * K )
m1 = (-A + 5 * D + 14 * H – 2 * H )

- The final prediction values d, h, and m shall be derived using:

d = ( d1 + 8 ) >> 4  		(S-8-69)
h = ( h1 + 8 ) >> 4  		(S-8-70)
m = ( m1 + 8 ) >> 4 		(S-8-71)

- The samples at sample position labelled as e, f, g, i, j, k, n, p, and q shall be derived by first calculating intermediate values denoted as e1, f1, g1, i1, j1, k1, n1, p1, and q1, respectively, by applying the 6-tap filter to the intermediate values of the closest quarter sample positions in either the horizontal direction

e1 = (-2 * dd + 14 * d1 + 5 * ee – ff ) 		(S-8-72)
f1 = (-2 * dd + 10 * d1 + 10 * ee – 2 * ff ) 		(S-8-73)
g1 = (-dd + 5 * d1 + 14 * ee – 2 * ff ) 		(S-8-74)

i1 = (-2 * gg + 14 * h1 + 5 * ii – jj ) 		(S-8-75)
j1 = (-2 * gg + 10 * h1 + 10 * ii – 2 * jj ) 		(S-8-76)
k1 = (-gg + 5 * h1 + 14 * ii – 2 * jj ) 		(S-8-77)

n1 = (-2 * jj + 14 * m1 + 5 * kk – mm ) 		(S-8-78)
p1 = (-2 * jj + 10 * m1 + 10 * kk – 2 * mm ) 		(S-8-79)
q1 = (-jj + 5 * m1 + 14 * kk – 2 * mm ) 		(S-8-80)

or vertical direction

e1 = (-2 * aa + 14 * a1 + 5 * nn – rr ) 		(S-8-81)
i1 = (-2 * aa + 10 * a1 + 10 * nn – 2 * rr ) 		(S-8-82)
n1 = (-aa + 5 * a1 + 14 * nn – 2 * rr ) 		(S-8-83)

f1 = (-2 * bb + 14 * b1 + 5 * pp – ss ) 		(S-8-84)
j1 = (-2 * bb + 10 * b1 + 10 * pp – 2 * ss ) 		(S-8-85)
p1 = (-bb + 5 * b1 + 14 * pp – 2 * ss ) 		(S-8-86)

g1 = (-2 * cc + 14 * c1 + 5 * qq – tt ) 		(S-8-87)
k1 = (-2 * cc + 10 * c1 + 10 * qq – 2 * tt ) 		(S-8-88)
q1 = (-cc + 5 * c1 + 14 * qq – 2 * tt ) 		(S-8-89)

because these yield an equal result, where intermediate values denoted as aa, bb, cc, nn, pp, qq, rr, ss, and tt shall be derived by applying the 4-tap filters horizontally in the same manner as the derivation of a1, b1, and c1; and intermediate values denoted as dd, ee, ff, gg, hh, ii, jj, kk, and mm shall be derived by applying the 4-tap filters vertically in the same manner as the derivation of d1, h1, and m1;. The final prediction values z with z equal to e, f, g, i, j, k, n, p, and q shall be derived using:

z = ( z1 + 128 ) >> 8

The luma location offset in fractional-sample units ( xFracL, yFracL ) specifies which of the generated luma samples at full-sample and fractional-sample locations is assigned to the predicted luma sample value predPartLXL[ xL, yL ]. This assignment is done according to Table S-8-9. The value of predPartLXL[ xL, yL ] shall be the output.

Table S-8-9 – Assignment of the luma prediction sample predPartLXL[ xL, yL ]

| xFracL | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| yFracL | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 |
| predPartLXL[ xL, yL ] | D | d | h | m | a | e | i | n | b | f | j | p | c | g | k | q |

###### Luma sample interpolation process with bi-linear filter

Inputs to this process are

- a luma location in full-sample units ( xIntL, yIntL ),

- a luma location offset in fractional-sample units ( xFracL, yFracL ), and

- the luma sample array of the selected reference picture refPicLXL

Output of this process is a predicted luma sample value predPartLXL[ xL, yL ].

The variable refPicHeightEffectiveL, which is the height of the effective reference picture luma array, is derived as follows.

- If MbaffFrameFlag is equal to 0 or mb_field_decoding_flag is equal to 0, refPicHeightEffectiveL is set equal to PicHeightInSamplesL.

- Otherwise (MbaffFrameFlag is equal to 1 and mb_field_decoding_flag is equal to 1), refPicHeightEffectiveL is set equal to PicHeightInSamplesL / 2.

The sample coordinates specified in the following equations are used for generating the predicted luma sample value predPartLXL[ xL, yL ].

xAL = Clip3( 0, PicWidthInSamplesL – 1, xIntL ) 		(S-8-90)
xBL = Clip3( 0, PicWidthInSamplesL – 1, xIntL + 1 ) 		(S-8-91)
xCL = Clip3( 0, PicWidthInSamplesL – 1, xIntL ) 		(S-8-92)
xDL = Clip3( 0, PicWidthInSamplesL – 1, xIntL + 1 ) 		(S-8-93)

yAL = Clip3( 0, refPicHeightEffectiveL – 1, yIntL ) 		(S-8-94)
yBL = Clip3( 0, refPicHeightEffectiveL – 1, yIntL ) 		(S-8-95)
yCL = Clip3( 0, refPicHeightEffectiveL – 1, yIntL + 1 ) 	(S-8-96)
yDL = Clip3( 0, refPicHeightEffectiveL – 1, yIntL + 1 ) 	(S-8-97)

Given the luma samples A, B, C, and D at full-sample locations specified by ( xAL, yAL ) to ( xDL, yDL ), the predicted luma sample value predPartLXL[ xL, yL ] is derived as follows:

predPartLXL[ xL, yL ] = ( ( 4 – xFracL ) * ( 4 – yFracL ) * A + xFracC * ( 4 – yFracC ) * B +
		                         ( 4 – xFracL ) * yFracL * C            + xFracC * yFracC * D            + 8 ) >> 4 	(S-8-98)

### Motion-compensated prediction process

Input to this process is a picture predPic.

Output of this process is a modified picture predPic, which replaces the input version.

For each macroblock mbAddr in scanning order of the picture predPic, the following applies.

- When the macroblock mbAddr is not coded using an Intra macroblock prediction mode, for each macroblock or sub-macroblock partition mbPartIdx\subMbPartIdx of the macroblock mbAddr, the following applies.

- The variables partWidth and partHeight are derived as follows.

- If mbType is not equal to P_8x8 or P_8x8ref0 or B_8x8,

partWidth  = MbPartWidth( mbType ) 		(S-8-99)
partHeight = MbPartHeight( mbType ) 		(S-8-100)

- Otherwise,

partWidth  = SubMbPartWidth( subMbType[ mbPartIdx ] ) 	(S-8-101)
partHeight = SubMbPartHeight( subMbType[ mbPartIdx ] ) 	(S-8-102)

- When chroma_format_idc is not equal to 0, the variables partWidthC and partHeightC are derived as follows.

partWidthC  = partWidth  / SubWidthC 		(S-8-103)
partHeightC = partHeight / SubHeightC 		(S-8-104)

- Let mvL0, mvL1, refIdxL0, refIdxL1, predFlagL0, and predFlagL1 be the variables MvL0[ mbPartIdx ][ subMbPartIdx ], MvL1[ mbPartIdx ][ subMbPartIdx ], RefIdxL0[ mbPartIdx ], RefIdxL1[ mbPartIdx], PredFlagL0[ mbPartIdx ], and PredFlagL1[ mbPartIdx ], respectively, that have been derived by the invocation of subclause S.9.4.1 during the decoding of the picture predPic.

- When chroma_format_idc is not equal to 0, the derivation process for chroma motion vectors in subclause S.9.4.1.4 is invoked with mvLX and refIdxLX (with X being 0 or 1) as input and the output is the chroma motion vector mvCLX.

- The decoding process for Inter prediction samples in subclause S.9.4.2 is invoked with the variables mbPartIdx, subMbPartIdx, partWidth, partHeight, partWidthC, and partWidthC (when available), the motion vectors mvL0, mvL1, mvCL0, and mvCL1 (when available), the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 as inputs and the outputs are a (partWidth)x(partHeight) array predPartL of prediction luma samples and when chroma_format_idc is not equal to 0 (monochrome) two (partWidthC)x(partHeightC) arrays predPartCr, and predPartCb of prediction chroma samples, one for each of the chroma components Cb and Cr.

- The picture predPic, which is stored in the decoded picture buffer, is modified as follows.

- Let S'L[ x, y], S'Cb[ x, y], and S'Cr[ x, y] represent the luma and chroma samples arrays of the picture predPic.

- The inverse macroblock scanning process in subclause S.7.4.1 is invoked with mbAddr as input and the output is assigned to the luma location ( xO, yO ).

- The inverse sub-macroblock partition scanning process in subclause S.7.4.2.2 is invoked with mbPartIdx and subMbPartIdx as inputs and the output is assigned to the luma location ( xP, yP ).

- A luma location ( xL, yL ) is derived by

xL = xO + xP		(S-8-105)
yL = yO + yP		(S-8-106)

- When chroma_format_idc is not equal to 0, a chroma location ( xC, yC ) is derived by

xC = xL / SubWidthC		(S-8-107)
yC = yL / SubHeightC		(S-8-108)

- Each sample S'L[ x, y] with x = xL..( xL + predWidth – 1 ) and y = yL..( yL + predHeight – 1 ) is modified by

S'L[ x, y] = Clip1Y( S'L[ x, y] + predPartL[ x – xL, y – yL ] ) 	(S-8-109)

- When chroma_format_idc is not equal to 0, each sample S'C[ x, y] (with C being Cb or Cr) with x = xC..( xC + predWidthC – 1 ) and y = yC..( yC + predHeightC – 1 ) is modified by

S'C[ x, y] = Clip1Y( S'C[ x, y] + predPartC[ x – xC, y – yC ] ) 	(S-8-110)

After modifying the luma and chroma samples arrays S'L[ x, y], S'Cb[ x, y], and S'Cr[ x, y] of the picture predPic, the deblocking filter process specified in subclause S.9.7 is invoked for the picture predPic.

# Parsing process

The specification of this subclause in AVC shall apply.

## Parsing process for Exp-Golomb codes

The specification of this subclause in AVC shall apply.

## CAVLC parsing process for transform coefficient levels

The specification of this subclause in AVC shall apply.

## CABAC parsing process for slice data

The specification of this subclause in AVC shall apply..

### Initialisation process

The specification of this subclause in AVC shall apply.

#### Initialisation process for context variables

The specification of this subclause in AVC shall apply with the following modifications

Add Table S-9-10 to table 9-11

Table S-9-10– Association of ctxIdx and syntax elements for each slice type in the initialisation process

|  | Syntax element | Table | Slice type |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PR | EI | EP | EB |
| macroblock_layer_in_scalable_extension( ) | base_mode_flag | Table S-9-11 |  | 460-462 | 460-462 | 460-462 |
|  | base_mode_refinement_flag | Table S-9-11 |  |  | 463 | 463 |
|  | intra_base_flag | Table S-9-11 |  | 464 | 464 | 464 |
| mb_pred_in_scalable_extension( ) and sub_mb_pred_in_scalable_extension( ) | motion_prediction_flag_l0 | Table S-9-11 |  |  | 465 | 465 |
|  | motion_prediction_flag_l1 | Table S-9-11 |  |  | 465 | 465 |
|  | mvd_ref_l0 | Table S-9-11 |  |  | 466 | 466 |
|  | mvd_ref_l1 | Table S-9-11 |  |  | 466 | 466 |
| residual_in_scalable_extension( ) | residual_prediction_flag | Table S-9-11 |  |  | 467 | 467 |
|  | transform_size_8x8_flag | Table 9-16 | 399-401 | 399-401 | 399-401 | 399-401 |
|  | coded_block_flag_luma[] | Table 9-18 | 85-104 | 85-104 | 85-104 | 85-104 |
|  | coded_block_flag_chromaDC | Table 9-18 | 85-104 | 85-104 | 85-104 | 85-104 |
|  | coded_block_flag_chromaAC | Table 9-18 | 85-104 | 85-104 | 85-104 | 85-104 |
| luma_coefficient() and chromaDC_coefficient() and chromaAC_coefficient() | coded_block_pattern_bit[] | Table 9-18 | 73-76 | 73-76 | 73-76 | 73-76 |
|  | mb_qp_delta | Table 9-17 | 60-63 | 60-63 | 60-63 | 60-63 |
|  | last_significant_coeff_flag | Table 9-19 Table 9-22 Table 9-24 Table 9-24 | 166-226 338-398 417-425 451-459 | 166-226 338-398 417-425 451-459 | 166-226 338-398 417-425 451-459 | 166-226 338-398 417-425 451-459 |
|  | significant_coeff_flag | Table 9-20 Table 9-23 Table 9-24 Table 9-24 Table S-9-12 | 105-165 277-337 402-416 436-450 470-481 | 105-165 277-337 402-416 436-450 | 105-165 277-337 402-416 436-450 | 105-165 277-337 402-416 436-450 |
|  | coeff_abs_level_minus1 | Table 9-21 Table 9-24 | 227-275 426-435 | 227-275 426-435 | 227-275 426-435 | 227-275 426-435 |
| luma_coefficient_ref ( ) and chromaDC_coefficient_ref () and chromaAC_coefficient_ref () | coeff_refinement_flag | Table S-9-11 | 468 |  |  |  |
|  | coeff_refinement_direction_flag | Table S-9-11 | 469 |  |  |  |

Replace the following paragraphs of this subclause in AVC

NOTE – ctxIdx equal to 276 is associated with the end_of_slice_flag and the bin of mb_type, which specifies the I_PCM macroblock type. The decoding process specified in subclause S.10.3.3.2.4 applies to ctxIdx equal to 276. This decoding process, however, may also be implemented by using the decoding process specified in subclause S.10.3.3.2.1. In this case, the initial values associated with ctxIdx equal to 276 are specified to be pStateIdx = 63 and valMPS = 0, where pStateIdx = 63 represents a non-adapting probability state.

with the following

NOTE – ctxIdx equal to 276 is associated with the end_of_slice_flag, end_of_progressive_refinement_slice_flag and the bin of mb_type, which specifies the I_PCM macroblock type. The decoding process specified in subclause S.10.3.3.2.4 applies to ctxIdx equal to 276. This decoding process, however, may also be implemented by using the decoding process specified in subclause S.10.3.3.2.1. In this case, the initial values associated with ctxIdx equal to 276 are specified to be pStateIdx = 63 and valMPS = 0, where pStateIdx = 63 represents a non-adapting probability state.

Add the following tables

Table S-9-11 – Values of variables m and n for ctxIdx from 460 to 469 [Ed. Note(HS): The initialization values currently represent a simple uniform distribution. Suitable initialization values still need to be determined.]

| Initialisation variables | ctxIdx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 460 | 461 | 462 | 463 | 464 | 465 | 466 | 467 | 468 | 469 |
| m | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| n | 64 | 64 | 64 | 64 | 40 | 64 | 64 | 64 | 64 | 64 |

Table S-9-12 – Values of variables m and n for ctxIdx from 470 to 481 [Ed. Note(HS): The initialization values are just copied from the corresponding context with levelListIdx-1. Suitable initialization values still need to be determined.]

| ctxIdx | I or SI base layer slices [Ed. Note(HS): That needs to be changed] |  | Value of cabac_init_idc |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 |  | 1 |  | 2 |  |
|  | m | n | m | n | m | n | m | n |
| 470 | -2 | 78 | -2 | 76 | -46 | 127 | -1 | 74 |
| 471 | 14 | 62 | -7 | 100 | -44 | 127 | -5 | 90 |
| 472 | 18 | 59 | 13 | 68 | 18 | 55 | 13 | 91 |
| 473 | 0 | 95 | 9 | 63 | 7 | 61 | 8 | 77 |
| 474 | 12 | 72 | 9 | 69 | 0 | 89 | -22 | 127 |
| 475 | -11 | 74 | -20 | 63 | 2 | 59 | -11 | 68 |
| 476 | 3 | 68 | -5 | 91 | -8 | 95 | -10 | 99 |
| 477 | 9 | 64 | 17 | 54 | 7 | 69 | 15 | 57 |
| 478 | -1 | 83 | 6 | 64 | -2 | 75 | 2 | 70 |
| 479 | 2 | 95 | 6 | 70 | 14 | 60 | 12 | 70 |
| 480 | 12 | 70 | 26 | 43 | 18 | 50 | 25 | 42 |
| 481 | 2 | 59 | 2 | 59 | 2 | 58 | -11 | 68 |

#### Initialisation process for the arithmetic decoding engine

The specification of this subclause in AVC shall apply

### Binarization process

The specification of this subclause in AVC shall apply with the following addition

Replace the following paragraphs of this subclause in AVC

The possible values of the context index ctxIdx are in the range 0 to 459, inclusive. The value assigned to ctxIdxOffset specifies the lower value of the range of ctxIdx assigned to the corresponding binarization or binarization part of a syntax element.

ctxIdx = ctxIdxOffset = 276 is assigned to the syntax element end_of_slice_flag and the bin of mb_type, which specifies the I_PCM macroblock type as further specified in subclause S.10.3.3.1. For parsing the value of the corresponding bin from the bitstream, the arithmetic decoding process for decisions before termination (DecodeTerminate) as specified in subclause S.10.3.3.2.4 shall be applied.

with the following

The possible values of the context index ctxIdx are in the range 0 to 469, inclusive. The value assigned to ctxIdxOffset specifies the lower value of the range of ctxIdx assigned to the corresponding binarization or binarization part of a syntax element.

ctxIdx = ctxIdxOffset = 276 is assigned to the syntax element end_of_slice_flag, end_of_progressive_refinement_slice_flag and the bin of mb_type, which specifies the I_PCM macroblock type as further specified in subclause S.10.3.3.1. For parsing the value of the corresponding bin from the bitstream, the arithmetic decoding process for decisions before termination (DecodeTerminate) as specified in subclause S.10.3.3.2.4 shall be applied.

Add Table S-9-13 to Table 9-25

Table S-9-13 – Syntax elements and associated types of binarization, maxBinIdxCtx, and ctxIdxOffset

| Syntax element | Type of binarization | maxBinIdxCtx | ctxIdxOffset |
| --- | --- | --- | --- |
| base_mode_flag | FL, cMax=1 | 0 | 460 |
| base_mode_refinement_flag | FL, cMax=1 | 0 | 463 |
| intra_base_flag | FL, cMax=1 | 0 | 464 |
| motion_prediction_flag_l0, motion_prediction_flag_l1 | FL, cMax=1 | 0 | 465 |
| mvd_ref_l0, mvd_ref_l1 | STU, cMax=1 | prefix: 0 suffix: na | prefix: 466 suffix: na |
| residual_prediction_flag | FL, cMax=1 | 0 | 467 |
| cycle_start_symbol | FL, cMax=1 | na | na,(uses DecodeBypass) |
| cycle_start_symbol_ext | FL, cMax=1 | na | na,(uses DecodeBypass) |
| end_of_progressive_refinement_slice_flag | FL, cMax=1 | 0 | 276 |
| coded_block_pattern_bit | FL, cMax=1 | 3 | 73 |
| coeff_refinement_flag | FL, cMax=1 | 0 | 468 |
| coeff_refinement_direction_flag | FL, cMax=1 | 0 | 469 |

#### Unary (U) binarization process

The specification of this subclause in AVC shall apply

#### Truncated unary (TU) binarization process

The specification of this subclause in AVC shall apply

#### Concatenated unary/ k-th order Exp-Golomb (UEGk) binarization process

The specification of this subclause in AVC shall apply

#### Fixed-length (FL) binarization process

The specification of this subclause in AVC shall apply

#### Binarization process for macroblock type and sub-macroblock type

The specification of this subclause in AVC shall apply

#### Binarization process for coded block pattern

The specification of this subclause in AVC shall apply

#### Binarization process for mb_qp_delta

The specification of this subclause in AVC shall apply

#### Signed truncated unary (STU) binarization proces

Input to this process is a request for a STU binarization for a syntax element and cMax.

Output of this process is the STU binarization of the syntax element.

A STU bin string is a concatenation of a prefix bit string and (when present) a suffix bit string. The prefix of the binarization is specified by invoking the TU binarization process for the prefix part Abs( synElVal ) of a syntax element value synElVal as specified in subclause S.10.3.2.2.

The STU bin string is derived as follows.

- If the prefix bit string is equal to the bit string that consists of a single bit with value equal to 0, the bin string of a syntax element having value synElVal consists only of a prefix bit string,

- Otherwise, the bin string of the STU suffix part of a syntax element value synElVal consists of a single bit which is specified by bs = ( ( synElVal > 0 ) ?  0 : 1 ).

### Decoding process flow

The specification of this subclause in AVC shall apply

#### Derivation process for ctxIdx

The specification of this subclause in AVC shall be replaced by the following specification

Inputs to this process are binIdx, maxBinIdxCtx and ctxIdxOffset.

Output of this process is ctxIdx.

Table S-9-14 and Table 9-30 show the assignment of ctxIdx increments (ctxIdxInc) to binIdx for all ctxIdxOffset values except those related to the syntax elements coded_block_flag, coded_block_flag_luma, coded_block_flag_chromaAC, coded_block_flag_chromaDC, significant_coeff_flag, last_significant_coeff_flag, and coeff_abs_level_minus1.

The ctxIdx to be used with a specific binIdx is specified by first determining the ctxIdxOffset associated with the given bin string or part thereof. The ctxIdx is determined as follows.

- If the ctxIdxOffset is listed in Table S-9-14 and Table 9-30, the ctxIdx for a binIdx is the sum of ctxIdxOffset and ctxIdxInc, which is found in Table S-9-14 and Table 9-30. When more than one value is listed in Table S-9-14 and Table 9-30   for a binIdx, the assignment process for ctxIdxInc for that binIdx is further specified in the subclauses given in parenthesis of the corresponding table entry.

- Otherwise (ctxIdxOffset is not listed in Table S-9-14 and Table 9-30), the ctxIdx is specified to be the sum of the following terms: ctxIdxOffset and ctxIdxBlockCatOffset(ctxBlockCat) as specified in Table 9-31 and ctxIdxInc(ctxBlockCat). Subclause S.10.3.3.1.3 specifies which ctxBlockCat is used. Subclause S.10.3.3.1.1.9 specifies the assignment of ctxIdxInc(ctxBlockCat) for coded_block_flag, coded_block_flag_luma, coded_block_flag_chromaAC and coded_block_flag_chromaDC, and subclause S.10.3.3.1.3 specifies the assignment of ctxIdxInc(ctxBlockCat) for significant_coeff_flag, last_significant_coeff_flag, and coeff_abs_level_minus1.

All bins with binIdx greater than maxBinIdxCtx are parsed using ctxIdx assigned to maxBinIdxCtx.

All entries in Table S-9-14 and Table 9-30 labelled with “na” correspond to values of binIdx that do not occur for the corresponding ctxIdxOffset.

ctxIdx = 276 is assigned to the binIdx of mb_type indicating the I_PCM mode. For parsing the value of the corresponding bins from the bitstream, the arithmetic decoding process for decisions before termination as specified in subclause S.10.3.3.2.4 shall be applied.

Add the content of Table S-9-14 to Table 9-30

Table S-9-14 – Assignment of ctxIdxInc to binIdx for all ctxIdxOffset values except those related to the syntax elements coded_block_flag, significant_coeff_flag, last_significant_coeff_flag, and coeff_abs_level_minus1

| ctxIdxOffset | binIdx |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | >= 6 |
| 460 | 0,1,2 (subclause S.10.3.3.1.1.11) | na | na | na | na | na | na |

##### Assignment process of ctxIdxInc using neighbouring syntax elements

The specification of this subclause in AVC shall apply with the following modifications

Subclause S.10.3.3.1.1.1 specifies the derivation process of ctxIdxInc for the syntax element mb_skip_flag.

Subclause S.10.3.3.1.1.2 specifies the derivation process of ctxIdxInc for the syntax element mb_field_decoding_flag.

Subclause S.10.3.3.1.1.3 specifies the derivation process of ctxIdxInc for the syntax element mb_type.

Subclause S.10.3.3.1.1.4 specifies the derivation process of ctxIdxInc for the syntax element coded_block_pattern and codec_block_pattern_bit.

Subclause S.10.3.3.1.1.5 specifies the derivation process of ctxIdxInc for the syntax element mb_qp_delta.

Subclause S.10.3.3.1.1.6 specifies the derivation process of ctxIdxInc for the syntax elements ref_idx_l0 and ref_idx_l1.

Subclause S.10.3.3.1.1.7 specifies the derivation process of ctxIdxInc for the syntax elements mvd_l0 and mvd_l1.

Subclause S.10.3.3.1.1.8 specifies the derivation process of ctxIdxInc for the syntax element intra_chroma_pred_mode.

Subclause S.10.3.3.1.1.9 specifies the derivation process of ctxIdxInc for the syntax element coded_block_flag.

Subclause S.10.3.3.1.1.10 specifies the derivation process of ctxIdxInc for the syntax element transform_size_8x8_flag.

Subclause S.10.3.3.1.1.11 specifies the derivation process of ctxIdxInc for the syntax element base_mode_flag.

###### Derivation process of ctxIdxInc for the syntax element mb_skip_flag

The specification of this subclause in AVC shall apply

###### Derivation process of ctxIdxInc for the syntax element mb_field_decoding_flag

The specification of this subclause in AVC shall apply

###### Derivation process of ctxIdxInc for the syntax element mb_type

The specification of this subclause in AVC shall apply

###### Derivation process of ctxIdxInc for the syntax element coded_block_pattern and codec_block_pattern_bit

The specification of this subclause in AVC shall apply with the following modifications

###### Derivation process of ctxIdxInc for the syntax element mb_qp_delta

The specification of this subclause in AVC shall apply

###### Derivation process of ctxIdxInc for the syntax elements ref_idx_l0 and ref_idx_l1

The specification of this subclause in AVC shall apply

###### Derivation process of ctxIdxInc for the syntax elements mvd_l0 and mvd_l1

The specification of this subclause in AVC shall apply

###### Derivation process of ctxIdxInc for the syntax element intra_chroma_pred_mode

The specification of this subclause in AVC shall apply

###### Derivation process of ctxIdxInc for the syntax element coded_block_flag, coded_block_flag_luma, coded_block_flag_chromaAC and coded_block_flag_chromaDC

The specification of this subclause in AVC shall apply

###### Derivation process of ctxIdxInc for the syntax element transform_size_8x8_flag

The specification of this subclause in AVC shall apply

###### Derivation process of ctxIdxInc for the syntax element base_mode_flag

Output of this process is ctxIdxInc.

When MbaffFrameFlag is equal to 1 and mb_field_decoding_flag has not been decoded (yet) for the current macroblock pair with top macroblock address 2 * ( CurrMbAddr / 2 ), the inference rule for the syntax element mb_field_decoding_flag as specified in subclause S.7.4.4 shall be applied.

The derivation process for neighbouring macroblocks specified in subclause S.6.4.8.1 is invoked and the output is assigned to mbAddrA and mbAddrB.

Let the variable condTermFlagN (with N being either A or B) be derived as follows.

- If mbAddrN is not available or base_mode_flag for the macroblock mbAddrN is equal to 1, condTermFlagN is set equal to 0.

- Otherwise (mbAddrN is available and base_mode_flag for the macroblock mbAddrN is equal to 0), condTermFlagN is set equal to 1.

The variable ctxIdxInc is derived by

ctxIdxInc = condTermFlagA + condTermFlagB

##### Assignment process of ctxIdxInc using prior decoded bin values

The specification of this subclause in AVC shall apply

##### Assignment process of ctxIdxInc for syntax elements significant_coeff_flag, last_significant_coeff_flag, and coeff_abs_level_minus1

The specification of this subclause in AVC shall applywith the following modifications

Replace the following paragraphs of this subclause in AVC

For the syntax elements significant_coeff_flag and last_significant_coeff_flag in blocks with ctxBlockCat < 5 and ctxBlockCat != 3, the variable ctxIdxInc is derived by

ctxIdxInc = levelListIdx		(9-15)

where levelListIdx ranges from 0 to maxNumCoeff  2, inclusive.

For the syntax elements significant_coeff_flag and last_significant_coeff_flag in blocks with ctxBlockCat  = =  3, the variable ctxIdxInc is derived by

ctxIdxInc = Min( levelListIdx / NumC8x8, 2 )		(9-16)

where levelListIdx ranges from 0 to 4 * NumC8x8  2, inclusive.

For the syntax elements significant_coeff_flag and last_significant_coeff_flag in 8x8 luma blocks with ctxBlockCat  = =  5, Table 9-34 contains the specification of ctxIdxInc for the given values of levelListIdx, where levelListIdx ranges from 0 to 62, inclusive.

with the following

If any of the following conditions is true, Table S-9-15 contains the specification of ctxIdxInc for the given values of levelListIdx.

- ctxBlockCat < 5, ctxBlockCat != 3, and levelListIdx = = maxNumCoeff – 1

- ctxBlockCat = = 3 and levelListIdx = = 4 * NumC8x8 – 1

- ctxBlockCat = = 5 and levelListIdx = = 63

Otherwise, the following applies.

- For the syntax elements significant_coeff_flag and last_significant_coeff_flag in blocks with ctxBlockCat < 5 and ctxBlockCat != 3, the variable ctxIdxInc is derived by

ctxIdxInc = levelListIdx		(9-15)

where levelListIdx ranges from 0 to maxNumCoeff  2, inclusive.

- For the syntax elements significant_coeff_flag and last_significant_coeff_flag in blocks with ctxBlockCat  = =  3, the variable ctxIdxInc is derived by

ctxIdxInc = Min( levelListIdx / NumC8x8, 2 )		(9-16)

where levelListIdx ranges from 0 to 4 * NumC8x8  2, inclusive.

- For the syntax elements significant_coeff_flag and last_significant_coeff_flag in 8x8 luma blocks with ctxBlockCat  = =  5, Table 9-34 contains the specification of ctxIdxInc for the given values of levelListIdx, where levelListIdx ranges from 0 to 62, inclusive.

Table S-9-15 – Determination of ctxIdxInc for special values of levelListIdx for the syntax element significant_coeff_flag depending on field_pic_flag, mb_field_decoding_flag, and ctxBlockCat

| field_pic_flag ¦ ¦ mb_field_decoding_flag | ctxBlockCat | ctxIdxInc |
| --- | --- | --- |
| 0 | 0 | 365 |
| 0 | 1 | 351 |
| 0 | 2 | 338 |
| 0 | 3 | 324 |
| 0 | 4 | 322 |
| 0 | 5 | 73 |
| 1 | 0 | 199 |
| 1 | 1 | 185 |
| 1 | 2 | 172 |
| 1 | 3 | 158 |
| 1 | 4 | 156 |
| 1 | 5 | 45 |

Replace the following paragraphs of this subclause in AVC

Let numDecodAbsLevelEq1 denotes the accumulated number of decoded transform coefficient levels with absolute value equal to 1, and let numDecodAbsLevelGt1 denotes the accumulated number of decoded transform coefficient levels with absolute value greater than 1. Both numbers are related to the same transform coefficient block, where the current decoding process takes place.

with the following

Let the variables numDecodAbsLevelEq1 and numDecodAbsLevelGt1 be derived as follows.

- If slice_type is not equal to PR, numDecodAbsLevelEq1 denotes the accumulated number of decoded transform coefficient levels with absolute value equal to 1, and numDecodAbsLevelGt1 denotes the accumulated number of decoded transform coefficient levels with absolute value greater than 1. Both numbers are related to the same transform coefficient block, where the current decoding process takes place.

- Otherwise (slice_type is equal to PR), numDecodAbsLevelEq1 and numDecodAbsLevelGt1 are set to zero.

#### Arithmetic decoding process

The specification of this subclause in AVC shall apply

##### Arithmetic decoding process for a binary decision

The specification of this subclause in AVC shall apply

##### Renormalization process in the arithmetic decoding engine

The specification of this subclause in AVC shall apply

##### Bypass decoding process for binary decisions

The specification of this subclause in AVC shall apply

##### Decoding process for binary decisions before termination

The specification of this subclause in AVC shall apply