updated 2006-01-23

IA32

Intel 32-bit Processors

(includes 80486 and all Pentium varieties)

Assembly Language Programming

This describes user-level programming in 32-bit mode, as supported by The Assembler and The Emulator. It does not (yet) describe operations normally considered to be within the domain of operating systems, such as virtual memory and paging, interrupts, segment descriptors, and getting the processor into 32-bit mode in the first place. The Emulator by default sets up all of these features before starting a program, and The Assembler by default assumes that all of these features will have been set up in advance.

An assembly language program consists of a sequence of instructions. Most of those instructions are implemented directly by the IA32 CPU; the assembler simply translates them from a human readable form (e.g. INC EBX) into their directly executable binary form (e.g. 01000011) which is stored in the output file. Other instructions are Directives, which direct the assembler to take some special action, which may or may not directly result in additional bytes in the output file.

The input to the assembler is a simple text file, with a name normally ending in “.asm”. This file is processed line-by-line, and the output generated is normally written to an Image File, with the same name but ending in “.bin” instead.  An image file is the simplest possible form for a directly executable file.  It consists of nothing but the bytes produced by processing the input file; there is no special formatting.  The Emulator will load those bytes, unprocessed, into memory starting at address 0x100000, then begin execution by jumping to that same address.

The IA32 Architecture

Memory addresses range between 0 and 0xFFFFFFFF (0 to 232-1), but which memory locations are actually accessible depends upon the amount of physical memory (RAM SIMs and DIMMs) installed, and the virtual memory and paging schemes implemented by the operating system and CPU or emulator.

The CPU also provides a number of special registers, which are simply very fast memory locations, and flags, which are single bit yes/no condition indicators.  There are no general-purpose registers.  Under IA32, every register has special purposes and restrictions.  This makes the generation of efficient executable code a very expensive goal, which is almost never achieved.  The generally available registers are EAX, EBX, ECX, EDX, ESI, EDI, ESP, EBP, and EIP.  All of these are 32-bit integer registers.  EAX and EDX are always lost when a division operation is performed. The numerator of a division must be stored in EAX.  All stack operations and function calls require ESP to be used as a stack pointer. Multi-byte memory transfers always result in the loss of the contents of EDI, ESI, and ECX, and require that the parameters (destination, source, and size) are stored in those registers.  Loop instructions require that the count is stored in ECX.  EIP must always contain the address of the next instruction to be executed, and it can not be directly accessed by programs.

There are a large number of one bit flags, those useful in general programming are OF, SF, ZF,  CF, and occasionally DF.  Most instructions do not modify the flags at all.  These four generally useful flags are usually modified only by certain arithmetic operations: OF indicates that overflow occurred (the result did not fit in the number of bits, usually 32, available); CF indicates that Carry or Borrow occurred (really only meaningful in addition and subtraction); SF indicates that the result was negative (S for Sign); ZF indicates that the result was Zero.  The DF flag controls the “direction” of the string instructions (MOVSB, etc.): if DF is 1, string instructions work backwards. It is strongly advised to keep DF=0 and forget that it exists.  PF is rarely useful: some instructions set it to indicate that the least significant 8 bits of the result produced has an even number of 1’s in it.  AF is only used to assist with BCD operations.

The flags are collected together into one large Virtual Register known as EFLAGS.  EFLAGS is a 32-bit register. Not all of the bits are currently assigned a meaning, and only a few of the meaningful bits are relevant to general programming. Their positions in the register are as follows: (bit 0 is the least significant)

| 3 | 3 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | O | D |  |  | S | Z |  | A |  | P |  | C |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | * | * | * | 0 | * | * | * | F | F | * | * | F | F | 0 | F | 0 | F | 1 | F |

Those marked with a 0 must always be 0, those marked with a 1 must always be 1. Those marked with a * play a vital role in system operations, and must not be modified by normal (non- operating-system) programs.

An instruction consists of an operation code, usually a mnemonic name hinting at what the instruction does when executed (e.g. ADD, HLT), followed by some number (maybe zero) of operands.  The operands indicate what values are to be operated on and where the results are to be stored.  Operands may refer to numeric constants, the contents of registers, and the contents of memory locations.  For example, “ADD EAX, 73” means “add the number 73 to the number stored in the EAX register, saving the result back in EAX”.  “ADD  [123], ECX”  means “add the number stored in register ECX to the number stored in memory at address 123, saving the result back in memory at address 123”.  “IMUL  EDI, [123], 52” means multiply the number stored in memory at address 123 by the fixed number 52, saving the result in the EDI register.  “HLT” means “Halt the CPU”.  The IA32 instruction set is appallingly irregular.  For instance, you can multiply any register by anything, and you can multiply any memory contents by any register or constant, but only EAX can be divided by anything.  Multiplications may have two or three operands, but additions and subtractions may only have two. The list is almost endless.

Modes

The IA32 processors can operate in a number of different modes.  They may be set to operate with 16 or 32 bit addresses, and independently of the address size, they may be set to operate with 16 or 32 bit addresses.  Independently of address and operand size, the processor may be running in what Intel calls “real mode”, “protected mode”, “virtual 8086 mode”, and a couple of others.  Most of these combinations of modes are pointless.  The only mode that is useful for general programming is 32-bit addresses + 32-bit operands + protected mode; I call this combination “32-bit mode”.

When an IA32 processor starts up, it is automatically and unavoidably in the 16-bit addresses + 16-bit operands + real mode configuration. I refer to this as “16-bit mode”.  Almost the only time there is any point in programming in 16-bit mode is when writing the small part of an operating system that is executed when the processor starts up (or “reboots”).  The sole task of code written in 16-bit mode should be to get the processor into 32-bit mode so proper programs can be run. Although The Assembler and The Emulator both fully support both 16-bit and 32-bit mode,  16-bit mode is not of interest to a normal programmer, and is not discussed further after the end of this subsection.

The address size and the operand size are completely different concerns.  The instruction “ADD [123], ECX” could be used in both 16 and 32 bit address mode;  the only address in memory that it accesses is 123, and 123 fits perfectly happily in 16 bits.  However, it can only be used in 32-bit operand mode, because its job is to add together two 32-bit values, regardless of how those values were found.  The instruction “ADD [123], CX” can also be used with any address size, but only with 16 bit operands.  The instruction “ADD [WORD 123], ECX” clearly uses 32 bit operands (ECX is a strictly 32 bit register), but is explicitly using 16 bit addresses.

These distinctions may seem confusing and pointless, and they generally are. Everyone would be much better off if processors automatically started in 32-bit address + 32-bit operand mode, and the 16-bit versions just didn’t exist. The only slight disadvantage would be that software written explicitly for 25-year-old processors that did not have any 32-bit modes would not work. A small price to pay.  In 16-bit mode, everything behaves differently.  Operands are interpreted differently, and segmentation (which controls how memory is accessed) is controlled differently. Just stay away from 16 bit programming whenever possible. It has no real adavantages.

Numbers

The Assembler accepts numbers written in any of the following forms:

Decimal	just type the number	e.g. 1243, -7

Hexadecimal	preceded by “0x”	e.g. 0x3EA7F

Binary	preceded by “0b”	e.g. 0b1001001101

ASCII	enclosed in single quotes	e.g. ‘A’

Named Constant	just type the name	e.g. cat

Named Memory Address	just type the name	e.g. cat

Computed Expression	enclose in parentheses	e.g. (cat+1), ((x*16)+y)

Current Location	use the ‘$’ symbol	e.g. $, ($+100)

Up to four ASCII characters may appear inside the single quotes. ‘AB’ is equivalent to ((‘A’*256)+’B’),  ‘XYZ’ is equivalent to ((((‘X’*256)+’Y’)*256)+’Z’), etc.

Named constants are defined using the EQU directive.

Named memory addresses are defined using Labelling.

In computed expressions, the parentheses are not optional. Every group of two operands separated by an operator must be enclosed in their own pair of parentheses.

The values of numbers written in any of these formats are assumed to be 32 bit integers.

Labelling and Memory Locations

The Assembler automatically maintains a Current Location Pointer. This is initially set to 0x100000, but may be modified.  Every time a byte of output is produced (either by translating an instruction to executable form, or as the result of obeying a directive), the current location pointer is incremented.  Thus, assuming that the program is loaded into memory starting at address 0x100000 before execution, the current location pointer during assembly has the same value as the EIP register will have when execution reaches the same point in the program.

A location in memory is given a name by putting a label in the assembly language program. Any instruction or directive may be preceded by any number of label definitions; these definitions may be on the same line of text or on a separate line before.  A label may consist of any combination of letters, digits and the five characters ‘$’, ‘.’, ‘_’, ‘!’, and ‘?’, so long as it begins with a letter and does not already mean something to the assembler.  Labels are not case sensitive, “cat” is the same label as both “Cat” and “CAT”.  A label definition simply consists of a label name followed by a colon.  Examples:

JMP   P$7

loop:   INC   EBX

Here:   ADD   EAX, 31

L42:

p$7:    JMP   LOOP

When The Assembler encounters a label definition, it treats it as a simple named constant definition. The value associated with the name is set permanently to the value of the current location pointer.

Unlike C and C++, The Assembler does not require definition before use, so the above example (referring to the label p$7 before it is defined) is perfectly valid.  Assemblers are almost invariably Two Pass affairs: the entire process of assembly is performed twice, the first phase is just to find out where all the labels will be, only the second phase produces real output.  If the assembler finds that a label’s location changes from one phase to the next, that is usually the result of an error in the assembler itself, and is reported as a Phase Error.

As a practical example, this shows how an in-memory variable is typically used. It defines variables called “val” and “form”; val is initially set to 107. The program computes 25×val2+12, and saves the result in the variable form.

IMUL  EAX, [val], 25

IMUL  EAX, [val]

ADD   EAX, 12

MOV   [form], EAX

HLT

val:    DD    107

form:   DD    0

“DD” is an assembly directive, not an IA32 executable instruction.  It directs the assembler to deposit a number (17 and 0 in the example) in memory at the current location, formatted as a 32 bit (i.e. four byte) integer.  Intel calls 4 byte values Double Words, or Dwords;  “DD” stands for “Deposit Dword”.  Although variables may be defined anywhere in an assembly language program, it is important to be sure that they will not be accidentally executed.  DD deposits bytes in the output file indistinguishable from the bytes that result from translating real instructions. The number 107 is in fact the executable code for the IMUL instruction, so if the HLT instruction had not already halted the processor, the variable “val” would be accidentally executed after the end of the intentional program, and would perform an extra unintended multiplication.  Execution of a program naturally starts at its beginning, so variables are usually put out of the way at the end.

Constant Definitions

Constants are defined by the following form:

name EQU value

This is not listed as a normal directive because it does not exactly follow the normal syntax for a directive: the code, EQU, does not come first.  The “name” may be anything that would be acceptable as a label name (but this is not a label definition, there is no colon after the name). “EQU” simply stands for “Equals”, or “is defined to be”, and the “Value” may be given in any of the formats for a number given above.

Named constants have exactly the same status as defined labels; the two kinds of name are in fact not distinguishable by the assembler.  There is still no define-before-use rule, but The Assembler is strictly two-pass.  Every named constant must be fully defined before it is used for the first time in the second pass.  That is quite a complicated rule, but easily illustrated by a few examples.  There is no cause for worrying about this rule, just don’t try any silly “clever” tricks, and nothing will go wrong.

EQU just defines a name that the assembler will know about. It does not result in any bytes being produced in the output file.  A label definition such as “xyz:” is equivalent to “xyz EQU $”.

Example 1:	year    EQU   1976

MOV   EAX, year

This is perfectly straight-forward, year is fully defined before it is ever used.

Example 2:	        MOV   EAX, year

year    EQU   1976

This is also acceptable, year is fully defined by the end of the first pass, so its value may be used anywhere.

Example 3:	        MOV   EAX, year

num     EQU   76

year    EQU   (1900+num)

This is also acceptable, for the same reason.

Example 4:	year    EQU   (1900+num)

num     EQU   76

MOV   EAX, year

This is also acceptable, but verging cleverness. Num is used before it is defined, but it is fully defined after one pass through the input file. Therefore year is properly defined during the second pass, and can be used after its definition in the MOV instruction.

Example 5:	        MOV   EAX, year

year    EQU   (1900+num)

num     EQU   76

This does not work. Num is only defined at the end of the first pass, so year’s definition is not complete until it is reached during the second pass, which is after it is needed for the MOV instruction. There is no third pass to handle similar cases of folly.

Comments

Any characters on an input line following a semi-colon (except inside quotes) are completely ignored by the assembler.  Example:

count: DB    0    ;  number of elephants stored here

Directives

Some directives cause new bytes to be written to the output file, some just instruct the assembler to do something.  Every assembler provides its own menagerie of directives, they are not defined by IA32;  The Assembler follows the most common conventions wherever possible, but sometimes convention is just too ill-considered.  The directives listed here are only those useful in normal programming.  Special directives for setting up segment descriptors, interrupt handlers, and so on, exist but are not described here.

In the following descriptions, <num> represents any number written in any of the formats for numbers described above.

ORG   <num>

Sets the initial value of the Current Location Pointer.  Short for “ORiGin”, it is used to define the starting address of the program: where in memory it will be loaded for execution.  It should not be fooled around with, because operating systems usually have a very fixed idea of where programs should be loaded, and may not stand for changes.  This directive may only be used before any bytes have been generated in the output file.

LOC   <num>

Sets the value of the Current Location Pointer.  May only be used if the number given is greater than or equal to the current value of the Current Location Pointer, it can not be used to erase bytes already deposited in the output file.  If the assembler is producing a .bin file, this directive simply outputs enough zero bytes to reach the required location.  Careless use can produce an over-sized output file.

SPACE <num>

Increments the Current Location Pointer by the given number.  If the assembler is producing a .bin file, this directive simply outputs a number of zero bytes. “SPACE n” is equivalent to “LOC ($+n)”.

ALIGN <num>

Increments the Current Location Pointer until it is a multiple of the number given.  Most instructions run faster if their memory operands are properly aligned (e.g. four-byte values should begin at addresses that are multiples of 4).  A few instructions do not work if they access unaligned memory locations.

ALIGN <num1>, <num2>

Increments the Current Location Pointer until it equivalent modulo <num1> to <num2>. This is a very rare but still existent alignment requirement

DB <val1>

DB <val1>, <val2>

DB <val1>, <val2>, <val3>

“Deposit Byte”. The operands are written to the output file as single byte integers.

There is no limit to the number of <val> elements, except that no single directive or instruction may produce more than 1024 bytes of output.

The operands may be numbers or strings. Strings are enclosed in double quotes, and may be of any length; each byte of the string is deposited into the output file.

For example, DB “cat”, 0  is equivalent to  DB ‘c’, ‘a’, ‘t’, 0

DW <num1>

DW <num1>, <num2>

DW <num1>, <num2>, <num3>

“Deposit Word”. The operands are written to the output file as two-byte integers.

There is no limit to the number of <val> elements, except that no single directive or instruction may produce more than 1024 bytes of output.

DD <num1>

DD <num1>, <num2>

DD <num1>, <num2>, <num3>

“Deposit Dword”. The operands are written to the output file as four-byte integers.

There is no limit to the number of <val> elements, except that no single directive or instruction may produce more than 1024 bytes of output.

END

Provides a logical end-of-file. The remainder of the input file is not processed.

DATECODE

Deposits 12 bytes in the output file, representing the date and time at which assembly began, as ASCII codes in the format YYYYMMDDHHMM.

BITS 16

BITS 32

Changes the default mode. Future instructions will be assembled in 16-bit or 32-bit operand mode, according to the operand.

INCLUDE “<name>”

Identical to the C/C++ compiler directive #include.  The indicated file, whose name must be enclosed in double quotes, is assembled exactly as though its contents appeared verbatim at this point in the input file.  The extension “.asm” is assumed if none appears in the string.

USING “<name>”

Reads special definitions from the indicated file. The extension “.def” is assumed if none appears in the string. This is not the same as INCLUDE: a .def file only contains numeric constant definitions, and even those are not in the EQU form. The file must be a sequence of “name = value” entries, one per line.  A .def file is normally used to set up the addresses of special functions or variables that are set up before execution by the operating system. For example, when assembling a program for running under The Operating System, the directive USING “system” is normally included.  This provides the addresses of the $printf, $inchar, etc operating system functions.

Other directives not described here:  SECTION,  SEGDESC,  INTGATE,  DQ,  DT.

The Standard Skeleton

Programs to be run by The Emulator in its default operating mode should normally begin with the following two directives:

USING “system”

ORG   $code

The standard file “system.def” defines certain special addresses, including $code, which is set to 0x100000. The ORG directive therefore tells the assembler to assume that the code it generates will be loaded into memory starting at address 0x100000.  The last executable instruction should normally be followed by a RET instruction (if you are sure that the program will not mishandle the stack) or a HLT instruction to ensure that the program does stop when it reaches the end. No special directives are needed at the end of the file.

The minimal program is

USING “system”

ORG   $code

RET

The “hello world” program is

USING “system”

ORG   $code

PUSH  str

PUSH  4

CALL  [$printf]

RET

str: DB “Get lost, matey\n”, 0

The Stack

IA32 processors provide a special-purpose register, ESP, which behaves as a stack pointer. The Emulator by default initialises the stack pointer to a sensible value before program execution begins; in other execution environments, one of the first actions of any program should be to set the stack pointer appropriately.

IA32 stacks grow downwards. ESP should be set initially to hold the address of the end of a region of free memory.  Every time a value is saved on the stack (i.e. Pushed), ESP is decremented by the number of bytes that object occupies. Every time a value is removed from the stack (i.e. Popped), ESP is incremented by the number of bytes that object occupied.  Stack alignment is inviolable: in 32-bit mode, every push operation pushes four bytes regardless of the natural size of the object, and of course every pop operation pops four bytes. In 16-bit mode, every stack operation pushes or pops exactly two bytes.

In 32-bit mode, a push operation is equivalent to first subtracting 4 from ESP, then saving the pushed value in four bytes memory at the address given by ESP.  A pop operation is equivalent to first reading four bytes of memory from the address stored in ESP, then adding 4 to ESP.

There is no general mechanism for ensuring that the stack does not grow into memory already in use for something else. It is the normal duty of the operating system to provide such protection through the paging, segmentation, or virtual memory system. Typically, the memory addresses below the stack (those that are at risk of being over-run) are protected against all accesses, so an over-grown stack crashes the program.

Certain instructions implicitly access the stack and modify ESP. The PUSH and POP instructions do not mention ESP, but still modify it.  When a function or procedure is called, the return address is automatically pushed on the stack.  It is normal, but not required, to pass parameters to functions by pushing them onto the stack immediately before the CALL instruction.  The RET instruction implements a return from a function by popping the return address, jumping back to it, and popping the appropriate number of parameters to return the stack to its pre-call condition.

Operands

Only a few instructions, such as “HLT”, are totally self contained. Most need to specify parameters or operands.  Operands may describe fixed numbers (constants), the addresses of memory locations (either with label names or less directly), and CPU registers.  The IA32 processors accept a large number of different operand forms, which are described here.  It is essential to remember that IA32 is extremely irregular: operand forms are not interchangeable. Certain instructions can only work on certain operand forms.  The operand forms listed here are grouped together in named sections (such as “reg”, “reg8”, “regmem”, “const”), which are referred to in the instruction descriptions.  For example, the description of the ADD instruction includes the form “ADD reg, const”.  That means that there is a form of the ADD instruction which accepts a “reg” form as its first operand and a “const” form as its second;  there is no form of ADD which accepts a “reg” and a “reg8” as its two operands.

const

Any number written in any of the number formats described above.  If a label name is used as an operand, it represents the numeric address associated with that label, not the contents of that memory location.  Jump instructions generally specify their destinations as labels in the form of a const.

reg

Any of EAX, EBX, ECX, EDX, ESI, EDI, EBP, or ESP.

When a register name appears as an operand, it indicates that the contents of that register are to be accessed. Registers do not reside in memory (they are part of the CPU), so they do not have addresses, so this use is not completely inconsistent with the const group’s syntax.

mem

Accessing the contents of memory, as a 32-bit value.

There are many forms for this group, at the end of this section.

Whenever the contents of four consecutive memory locations are to be accessed (either reading or writing) as a 32 bit value, the operand is specified using the mem syntax. Square brackets may be understood as meaning “Contents of”.

The opening square bracket may be preceded by the word DWORD to disambiguate an instruction if mem16 or mem8 would also be allowed in the same context.

memreg

Any operand in either the mem group or the reg group.

reg8

Any of AL, BL, CL, DL, AH, BH, CH, or DH.

These are effectively eight-bit registers, but have no independent existence.  AL is simply a name for the least significant eight bits of EAX: changing AL changes EAX, changing EAX usually changes AL.  AH is similarly just a name for the second least significant eight bits of EAX.  BL and BH are parts of EBX,  CL and CH are parts of ECX,  DL and DH are parts of EDX.  There are no eight-bit accessible components of ESI, EDI, ESP, or EBP.

It is very rare that there is any point in using 8-bit values, except when working with strings.

mem8

Accessing the contents of memory, as an 8-bit value.

It is very rare that there is any point in using 8-bit values, except when working with strings.

The syntax is exactly the same as for the mem group; the opening square bracket may be preceded by the word BYTE to disambiguate an instruction if mem or mem16 would also be allowed in the same context.

memreg8

Any operand in either the mem8 group or the reg8 group.

reg16

Any of AX, BX, CX, DX, SI, or DI.

These are effectively sixteen-bit registers, but have no independent existence.  AX is simply a name for the least significant sixteen bits of EAX: changing AX changes EAX, changing EAX usually changes AX.  BX is part of EBX,  CX is part of ECX,  DX is part of EDX,  SI is part of ESI,  DI is part of EDI.  There are no sixteen-bit accessible components of ESP or EBP.

It is very rare that there is any point in using 16-bit values. It actually slows down execution when the processor is in 32-bit mode.

mem16

Accessing the contents of memory, as a 16-bit value.

It is very rare that there is any point in using 16-bit values. It actually slows down execution when the processor is in 32-bit mode.

The syntax is exactly the same as for the mem and mem8 groups; the opening square bracket may be preceded by the word WORD to disambiguate an instruction if mem or mem8 would also be allowed in the same context.

memreg16

Any operand in either the mem16 group or the reg16 group.

mem group forms

[name]

[number]

This form is used if the address of the memory location is known at assembly time. This generally means global variables. Examples:

MOV   EAX, [0x103F7]   ; save EAX in four bytes starting at address 0x103F7

INC   BYTE [count]        ; add one to the single byte global variable

[reg]

This form is used if the address of the memory location has been stored in a register. For example, if EBX contains the value 412000, then

MOV   EAX, [EBX]

copies the four-byte value stored in memory locations 412000 to 412003 into the EAX register.  EAX is a 32 bit register, so all operands are 32 bit.  There are only a very few instructions that take mixed-size operands, and they are all specially indicated in the descriptions.  All normal instructions require all operands to be the same size, so in this example there is no need to write DWORD [EBX].  Similarly,

MOV   AX, [EBX]

copies the two-byte value stored in memory locations 412000 to 412001 into the least significant 16 bits of the EAX register (the other 16 bits remain unmodified).  AX is a 16 bit register, so all operands are 16 bit.  In this example there is no need to write WORD [EBX].   Also, in the instruction

INC   [EBX]

the four-byte value stored in memory locations 412000 to 412003 is incremented. This is because the default operand size is 32 bits (unless you have used the BITS 16 directive). Ambiguous cases use the default operand size.

INC   WORD [EBX]

Increments the two-byte value stored in memory locations 412000 to 412001.

The following two instructions are wrong and will be rejected by the assembler:

INC   [AX]

INC   WORD [AX]

Only 32 bit registers should appear inside the square brackets. Memory addresses are 32 bit values even if the values stored in them are not.

This discussion about operand sizes applies equally to all forms in the mem, mem8, and mem16 operand groups.

[reg+num]

[reg-num]

[reg + BYTE num]

The address of the memory location to be accessed is calculated by adding the given number to the contents of the given register. For example, if EBX contains the value 412000 and ESP contains the value 0xFFFFFF8A, then

MOV   [EBX+345], ECX

INC   BYTE [ESP-0x2018]

will write the value of the ECX register into memory at addresses 412345 to 412348, then increment the single byte stored in memory at location 0xFFFFDF74.

[reg+reg]

The registers may not both be ESP.

The address of the memory location to be accessed is calculated by adding together the two given registers. For example, if EBX contains the value 412000 and ESI contains the value 1208, then

MOV   EBX, [EBX+ESI]

will read the four-byte value stored in memory locations 413208 to 413211, and store it in the register EBX.  Of course, the change in EBX does not affect the address calculation: it is too late.

[reg+reg+num]

[reg+reg-num]

[reg + reg + BYTE num]

The registers may not both be ESP.

This should be obvious by now. The two registers and the number are added together, the sum of those three quantities gives the address of the memory location to be accessed.

[reg*scale]

Scale must be 1, 2, 4, or 8, and must be a constant number.

The register may not be ESP.

The address to be used is calculated by multiplying the value of the register by the scale.

This form is not of much use.

[reg1+reg2*scale]

Scale must be 1, 2, 4, or 8, and must be a constant number.

The register reg2 may not be ESP.

The meaning of this form should be obvious, even though it is not very useful

[reg*scale+num]

[reg*scale-num]

[reg*scale + BYTE num]

Scale must be 1, 2, 4, or 8, and must be a constant number.

The register may not be ESP.

The meaning of this form should be obvious given the previous descriptions.

It is primarily used for accessing elements of global arrays.  If A is the address of the beginning of an array of four-byte integers, and A[x] is to be accessed, then the value of x should be calculated and stored in a register (say EDX), then the operand [EDX*4+A] refers directly to A[x]. The allowable scales are 1, 2, 4, and 8 because these are the most common element sizes, corresponding to chars, short ints, ints, and doubles, respectively. Having an operand form that includes the scaling in its address calculation simplifies the programming required for array accesses, and speeds up programs.

[reg1+reg2*scale+num]

[reg1+reg2*scale-num]

[reg1+reg2*scale + BYTE num]

Scale must be 1, 2, 4, or 8, and must be a constant number.

The register reg2 may not be ESP.

The meaning of this form should be obvious given the previous descriptions.

It is primarily used for accessing elements of local arrays.  If an array is stored in memory at an address that is only known relative to the value in some register (for example, the address of array A may be (EBP-96); this may seem strange, but is actually a very common arrangement), then to access A[x], the value of x is again computed in a register (say EDX) again. Then the operand [EBP+EDX*4-96] refers directly to A[x].

Instructions

Only those instructions that are useful in general programming are described here.

Key: instruction mnemonics and other fixed forms appear in capital letters, they should be typed exactly as they appear here (except of course that the capitalisation is irrelevant).  Operands appear in the form name:kind, where name is just a name used to refer to the operand in the following description, and kind is one of the operand group names listed above. For example, “IMUL dest:reg, src:const” would represent an instruction called IMUL that has two operands, the first must be a 32-bit register, and is referred to by the name “dest”, and the second must be a numeric constant, and is referred to by the name “src”.

AAA	(not covered)	Used in performing BCD (Binary Coded Decimal) additions.

AAD	(not covered)	Used in performing BCD (Binary Coded Decimal) divisions.

AAM	(not covered)	Used in performing BCD (Binary Coded Decimal) multiplications.

AAS	(not covered)	Used in performing BCD (Binary Coded Decimal) subtractions.

ADC	Add, including carry.

The ADC instructions are exactly the same as the ADD instructions (q.v.), except that the result is incremented by 1 if the carry flag CF was 1 when the instruction started.  The ADC instruction is used in programming multi(>4)-byte additions.

ADD  dest:reg, src:regmem

ADD  dest:regmem, src:const

ADD  dest:regmem, src:reg

32-bit integer addition. Dest and src are read, the two values are added, and the result is written back to dest.  The flags CF, OF, ZF, and SF are all modified.  CF indicates carry: the 33rd bit of the result would have been 1; OF indicates overflow: basically the result is incorrect, either the two inputs were negative but the result is positive, or the two inputs were positive but the result is negative; ZF indicates that the 32-bit result was zero; SF indicates that the result was negative (i.e. bit 32 of the result was 1).

ADD  dest:reg16, src:regmem16

ADD  dest:regmem16, src:const

ADD  dest:regmem16, src:reg16

16-bit integer addition. Exactly the same as the 32-bit version, except that only 2 bytes are read from each operand, only a 16 bit addition is performed, and only 2 bytes of the destination are overwritten with the result. The flags are all set according to the 16 bit result: CF is set if bit 17 would have been 1, SF is set if bit 16 is 1, etc.

ADD  dest:reg8, src:regmem8

ADD  dest:regmem8, src:const

ADD  dest:regmem8, src:reg8

8-bit integer addition. Exactly the same as the 32-bit version, except that only 1 byte is read from each operand, only an 8 bit addition is performed, and only 1 byte of the destination is overwritten with the result. The flags are all set according to the 8 bit result: CF is set if bit 9 would have been 1, SF is set if bit 8 is 1, etc.

AND  dest:reg, src:regmem

AND  dest:regmem, src:const

AND  dest:regmem, src:reg

32-bit bitwise logical and. Dest and src are read, the two values are anded together, and the result is written back to dest.  The flags CF, OF, ZF, and SF are all modified.  CF and OF are both set to zero; ZF indicates that the 32-bit result was zero; SF indicates that the result was negative (i.e. bit 32 of the result was 1). This is exactly the same as the ADD instruction except for the operation that is performed.

AND  dest:reg16, src:regmem16

AND  dest:regmem16, src:const

AND  dest:regmem16, src:reg16

16-bit bitwise logical and. Exactly the same as the 16-bit ADD, except that an AND operation is performed. Flags are set as above, for the 16 bit result.

AND  dest:reg8, src:regmem8

AND  dest:regmem8, src:const

AND  dest:regmem8, src:reg8

8-bit bitwise logical and. Exactly the same as the 8-bit ADD, except that an AND operation is performed. Flags are set as above, for the 8 bit result.

ARPL	(not covered)	For systems programming only.

BOUND  idx:reg, bounds:mem

Used to check that the value idx (typically an array index) lies between two precomputed limits. Two consecutive four-byte values are read from memory, starting from the location indicated by bounds; if the value of idx is less than the first value or greater than the second one, an interrupt (INT 5) is triggered.  If it is in range, execution continues normally.  The flags are not modified.

BOUND  idx:reg16, bounds:mem16

Exactly the same as the 32-bit version, except that two two-byte values are read from memory, and only a 16-bit comparison is performed.

BSF  dest:reg, val:regmem

“Bit Scan Forwards”.  The number of trailing zeros in the operand val is counted; the result, which will be between 0 and 31 inclusive, is stored in dest.  Exception: if the value of val is zero, the result is undefined. Only the ZF flag is modified: it is set to 1 if val was zero.

Alternatively, this can be thought of as finding the bit-index of the last, or least-significant non-zero bit; a bit index of zero refers to the least significant bit.

Examples:	val=8 => dest=3 and ZF=0

val=0x123000 => dest=12 and ZF=0

val=0 => dest unknown and ZF=1

BSF  dest:reg16, val:regmem16

“Bit Scan Forwards”.  16-bit version.

BSR  dest:reg, val:regmem

“Bit Scan Reverse”.  Almost identical to BSF, except that it finds the index of the first, or most significant non-zero bit.  Only the ZF flag is modified: it is set to 1 if val was zero.

Alternatively, this can be thought of as computing thirty-two minus the number of leading zeros.

Examples:	val=8 => dest=3 and ZF=0

val=0x123000 => dest=20 and ZF=0

val=0 => dest unknown and ZF=1

BSR  dest:reg16, val:regmem16

“Bit Scan Reverse”.  16-bit version.

BSWAP  opd:reg

“Byte order Swap”.  The value in the register is converted between big-endian and little-endian encoding: its first and fourth bytes are swapped, and its second and third bytes are swapped. The flags are not modified.

BT  val:regmem, bitnum:reg

BT  val:regmem, bitnum:const

“Bit Test”.  The single bit indicated by bitnum of the value val is extracted and stored in the carry flag CF.  Bits are numbered surprisingly sensibly: a bitnum of 0 refers to the least significant bit, 31 the most. Neither val not bitnum are modified. Only the CF clag is modified.  The conditional jump instructions JC and JNC, the conditional set instructions SETC and SETNC, and the special shift instructions RCL and RCR may be used to inspect or access that bit in CF.  Example:

BT   ECX, 3

JC   wasset

jumps to the label “wasset” if bit three of ECX (the bit whose weight is 8) is on.

BT  val:regmem16, bitnum:reg16

BT  val:regmem16, bitnum:const

“Bit Test”. A sixteen bit version of the above.

BTC  val:regmem, bitnum:reg

BTC  val:regmem, bitnum:const

BTC  val:regmem16, bitnum:reg16

BTC  val:regmem16, bitnum:const

“Bit Test and Complement”.  Performs exactly the same function as BT, except that the bit examined is flipped (1=>0, and 0=>1) after the test is performed (so CF is not flipped). Val is modified, exactly one bit is changed, and CF is set accordingly.

BTR  val:regmem, bitnum:reg

BTR  val:regmem, bitnum:const

BTR  val:regmem16, bitnum:reg16

BTR  val:regmem16, bitnum:const

“Bit Test and Reset”.  Performs exactly the same function as BT, except that the bit examined is set to zero after the test is performed. Val is modified, at most one bit being changed, and CF is set accordingly.

BTS  val:regmem, bitnum:reg

BTS  val:regmem, bitnum:const

BTS  val:regmem16, bitnum:reg16

BTS  val:regmem16, bitnum:const

“Bit Test and Set”.  Performs exactly the same function as BT, except that the bit examined is set to one after the test is performed. Val is modified, at most one bit being changed, and CF is set accordingly.

CALL  fn:const

CALL  fn:regmem

CALL  fn:regmem16

Function or procedure call.  The current value of EIP is pushed onto the stack, then EIP is set to the value of fn.  In the most common usage fn is simply a label, in which case CALL is simply an unconditional jump that first saves the return address.  When the regmem form is used, the value of the operand gives the address of the function. So if EAX=123, then “CALL EAX” causes a jump to location 123, and “CALL [EAX]” causes a jump to the location whose address is stored in the four bytes stored in memory at locations 123 to 126. It should be remembered that all jumps to constant addresses are relative jumps: “CALL 123” adds 123 to the current value of EIP, whereas the other forms are absolute.  All three of these instruction forms are called by Intel “Near calls”.

CALL  seg:const : addr:const

This is what Intel refers to as a “Far call”.  First, CS (the code segment register) is pushed, then EIP is pushed, then CS is set to the value of the first constant seg, then EIP is set to the value of the second constant addr.  This is an absolute (non-relative) call to a function in a different segment. It is of no use in normal programming.

CALL FAR  dest:mem

CALL FAR  dest:mem16

This is another “Far call”.  First, CS (the code segment register) is pushed, then EIP is pushed, then a two byte value is read from the indicated memory location and stored in CS, then a four byte value is read from the following memory locations and stored in EIP.  This is an absolute (non-relative) call to a function in a different segment. It is of no use in normal programming.  In the 16-bit version, the second value read from memory (and stored in EIP) is also only a two-byte value.

CBW

“Convert Byte to Word”.  The eighth bit of EAX is copied 8 times into the second-least significant byte of EAX. This has the effect of setting AX equal to AL, preserving the sign bit.  The instruction has no operand, it can only work on EAX.

CDQ

“Convert Dword to Quadword”.  The sign bit of EAX is copied 32 times over all 32 bits of EDX. This has the effect of setting the 64 bit value in EDX:EAX equal to EAX, preserving the sign bit.  The instruction has no operand, it can only work on EAX and EDX.

CLC

“Clear Carry”.  The flag CF is set to zero.

CLD

“Clear Direction”.  The flag DF is set to zero.

CLI

“Clear Interrupt enable”.  Interrupts will be ignored until re-enabled (see STI).

CLTS	(not covered)	Clear “task switched” flag, not used in normal programming.

CMC

“CoMplement Carry”.  The carry flag CF is inverted.

CMP  dest:reg, src:regmem

CMP  dest:regmem, src:const

CMP  dest:regmem, src:reg

CMP  dest:reg16, src:regmem16

CMP  dest:regmem16, src:const

CMP  dest:regmem16, src:reg16

CMP  dest:reg8, src:regmem8

CMP  dest:regmem8, src:const

CMP  dest:regmem8, src:reg8

“Compare”.  These instructions behave exactly the same way as the “ADD” instructions (q.v.), except that instead of adding the two operands, they are compared. The flags are set to represent the result of the comparison, and neither operand is modified.  The four flags OF, SF, ZF, and CF are all modified.  ZF is set to one if the two operands are equal;  SF is set to one if the first operand (dest) is less than the second (src).  The CF and OF flags are set if adding dest and (minus src) would cause them to be set.  The values of the flags are not normally examined directly after a CMP, but are instead used to control a conditional jump. For example:

CMP   EAX, EBX

JL    ABC

will cause a jump to label ABC is the value in EAX is less than the value in EBX.

CMPSB

“Compare String of Bytes”.  Equivalent to “CMP  BYTE [ESI], [EDI]”; after the comparison, ESI and EDI are both incremented by 1.  The comparison sets the flags, but the two increments do not.  May be used with the “REP” prefix (q.v.).  If DF is 1, the increments are replaced by decrements.

CMPSW

“Compare String of Words”.  Equivalent to “CMP  WORD [ESI], [EDI]”; after the comparison, ESI and EDI are both incremented by 2.  The comparison sets the flags, but the two increments do not.  May be used with the “REP” prefix (q.v.).  If DF is 1, the increments are replaced by decrements.

CMPSD

“Compare String of Dwords”.  Equivalent to “CMP  [ESI], [EDI]”; after the comparison, ESI and EDI are both incremented by 4.  The comparison sets the flags, but the two increments do not.  May be used with the “REP” prefix (q.v.).  If DF is 1, the increments are replaced by decrements.

CMPXCHG  dest:regmem, src:reg

If  EAX and the value of dest are equal, then the flag ZF is set to 1, and dest is overwritten with src.  Otherwise, the flag ZF is set to 0, and EAX is set equal to the value of dest.

CMPXCHG  dest:regmem16, src:reg16

If  AX and the value of dest are equal, then the flag ZF is set to 1, and dest is overwritten with src.  Otherwise, the flag ZF is set to 0, and AX is set equal to the value of dest.

CMPXCHG  dest:regmem8, src:reg8

If  AL and the value of dest are equal, then the flag ZF is set to 1, and dest is overwritten with src.  Otherwise, the flag ZF is set to 0, and AL is set equal to the value of dest.

CWD

“Convert Word to Dword”.  The sixteenth bit of EAX is copied 16 times into the least significant 16 bits of EDX. This has the effect of setting the 32 bit value in DX:AX equal to AX, preserving the sign bit.  The instruction has no operand, it can only work on EAX and EDX.

CWDE

“Convert Word to Dword”.  The sixteenth bit of EAX is copied 16 times into the most significant 16 bits of EAX. This has the effect of setting EAX equal to AX, preserving the sign bit.  The instruction has no operand, it can only work on EAX.

DAA	(not covered)	Used in performing BCD (Binary Coded Decimal) additions.

DAS	(not covered)	Used in performing BCD (Binary Coded Decimal) subtractions.

DEC  dest:regmem

“Decrement”.  The value of dest is reduced by 1.  The flags SF, ZF, CF, and OF are set exactly as they would be if an ADD instruction had been used to add -1 to dest.

DEC  dest:regmem16

A sixteen bit version of the above.

DEC  dest:regmem8

An eight bit version of the above.

DIV  EAX, src:regmem

The 64 bit value in EDX:EAX is divided by the 32-bit value of src.  The result is stored in EAX, and the remainder is stored in EDX.  This is an unsigned division: the 64th bit of EDX:EAX is not treated as a sign bit, just part of the positive numerator.  The flags SF, ZF, CF, and OF are modified by this instruction, but their values are unpredictable (i.e. it ruins them). To perform a 32 bit division, first set EDX to zero; do not use the sixteen bit version of this instruction, as it only provides a 16 bit divisor.  For a normal signed divide, see IDIV.

DIV  AX, src:regmem16

The 32 bit value in DX:AX is divided by the 16-bit value of src.  The result is stored in AX, and the remainder is stored in DX.  This is an unsigned division: the 32nd bit of DX:AX is not treated as a sign bit, just part of the positive numerator.  The flags SF, ZF, CF, and OF are modified by this instruction, but their values are unpredictable (i.e. it ruins them). For a normal signed divide, see IDIV.

DIV  AL, src:regmem8

The 16 bit value in AX is divided by the 8-bit value of src.  The result is stored in AL, and the remainder is stored in AH.  This is an unsigned division: the 16th bit of AX is not treated as a sign bit, just part of the positive numerator.  The flags SF, ZF, CF, and OF are modified by this instruction, but their values are unpredictable (i.e. it ruins them). For a normal signed divide, see IDIV.

ENTER	(not covered)

F***

The floating point instructions, all instructions with names beginning with the letter F, are not (yet) included in this description.

HLT

Halt the processor. This is not a very strong halt, if an interrupt is caused by any external event, it will still be processed.  A true halt is achieved by using the two instruction sequence of CLI followed by HLT.

IDIV  EAX, src:regmem

The 64 bit value in EDX:EAX is divided by the 32-bit value of src.  The result is stored in EAX, and the remainder is stored in EDX.  This is a normal signed division: the 64th bit of EDX:EAX is treated as a sign bit.  The flags SF, ZF, CF, and OF are modified by this instruction, but their values are unpredictable (i.e. it ruins them). To perform a 32 bit division, first set extend the sign of EAX into EDX, using the CDQ instruction, which is provided for exactly that purpose.

IDIV  AX, src:regmem16

The 32 bit value in DX:AX is divided by the 16-bit value of src.  The result is stored in AX, and the remainder is stored in DX.  This is a normal signed division: the 32nd bit of DX:EX is treated as a sign bit.  The flags SF, ZF, CF, and OF are modified by this instruction, but their values are unpredictable (i.e. it ruins them). The CWD instruction is used to extend the sign of AX into DX before executing this instruction.

IDIV  AL, src:regmem8

The 16 bit value in AX is divided by the 8-bit value of src.  The result is stored in AL, and the remainder is stored in AH.  This is a normal signed division: the 16th bit of AX is treated as a sign bit.  The flags SF, ZF, CF, and OF are modified by this instruction, but their values are unpredictable (i.e. it ruins them).

IMUL  src:regmem

The values of EAX and src are multiplied together to produce a 64 bit result. EDX is overwritten with the most significant 32 bits of the result, and EAX is overwritten with the least significant 32 bits. The flags SF, ZF, CF, and OF are modified but in an unpredictable or useless way.

IMUL  dest:reg, src:regmem

IMUL  dest:reg, src:const

The values of dest and src are multiplied together and dest is overwritten with the result. The flags SF and ZF are modified but in an unpredictable way. The CF and OF are never set to 1, but are both set to zero if the result does not require more than 32 bits.

IMUL  dest:reg, src1:regmem, src2:const

The values of src1 and src2 are multiplied together and dest is overwritten with the result. The flags SF and ZF are modified but in an unpredictable way. The CF and OF are never set to 1, but are both set to zero if the result does not require more than 32 bits.

IMUL  dest:reg16, src:regmem16

IMUL  dest:reg16, src:const

IMUL  dest:reg16, src1:regmem16, src2:const

The sixteen bit version of the above.

IMUL  src:regmem16

The values of AX and src are multiplied together to produce a 32 bit result. DX is overwritten with the most significant 16 bits of the result, and AX is overwritten with the least significant 16 bits. The flags SF, ZF, CF, and OF are modified but in an unpredictable or useless way.

IMUL  src:regmem8

The values of AL and src are multiplied together to produce a 16 bit result. AX is overwritten with the result. The flags SF, ZF, CF, and OF are modified but in an unpredictable or useless way.

IN	(not covered)	Load value from external i/o port.

INC  dest:regmem

“Increment”.  The value of dest is increased by 1.  The flags SF, ZF, CF, and OF are set exactly as they would be if an ADD instruction had been used to add 1 to dest.

INC  dest:regmem16

A sixteen bit version of the above.

INC  dest:regmem8

An eight bit version of the above.

INSB	(not covered)	Load string of byte values from external i/o port.

INSW	(not covered)	Load string of word values from external i/o port.

INSD	(not covered)	Load string of dword values from external i/o port.

INT	(not covered)	Cause an interrupt.

INTO	(not covered)	Cause an interrupt if flag OF is 1.

INVD	(not covered)	Clear cache.

INVLPG	(not covered)	Invalidate a paging cache entry.

IRETD

“Return from end of interrupt handler”.  This function must be used instead of the normal RET instruction at the end of an interrupt handling routine. As well as the normal popping of the return address from the stack,  it restores other values (flags and CS) that are also automatically pushed when an interrupt is caught.

IRET

Version of IRETD only to be used in 16-bit mode.

J***  addr:const

J***  BYTE addr:const

Conditional Jump instructions.  The flags are examined, and if a condition defined by the complete instruction name is satisfied, a jump to the address addr occurs. If the condition is not met, execution continues with the next instruction.  All jumps are relative.  If you specify a label, The Assembler replaces it with the number that would have to be added to EIP to reach that location.  If you are certain that that number will fit in one byte (i.e. it is between -128 and +127 inclusive) then the word BYTE instruct the assembler to produce a more compact form of the instruction.  When the word BYTE is used, these are called “Short” jumps; otherwise they are perfectly normal “Near” jumps.

In the following table, the various forms of conditional jump are listed. Along with the instruction name and a phrase hinting at its meaning, the condition under which a jump occurs is given in two forms: first as a numeric relation based on the assumption that the instruction CMP A, B was just executed, then as the actual requirement on the flag values.

JA	Above	A>B, unsigned	CF=0 and ZF=0

JAE	Above or Equal	A>=B, unsigned	CF=0

JB	Below	A<B, unsigned	CF=1

JBE	Below or Equal	A<=B, unsigned	CF=1 or ZF=1

JC	Carry	Carry	CF=1

JCXZ	CX zero	CX=0, 16 bit	CX=0 (flags ignored)

JECXZ	ECX zero	ECX=0	ECX=0 (flags ignored)

JE	Equal	A=B	ZF=1

JZ	Zero	A=B	ZF=1

JG	Greater	A>B	ZF=0 and SF=OF

JGE	Greater or Equal	A>=B	SF=OF

JL	Less	A<B	SF≠OF

JLE	Less or Equal	A<=B	ZF=1 or SF≠OF

JNA	Not Above	A<=B, unsigned	CF=1 or ZF=1

JNB	Not Below	A>=B, unsigned	CF=0

JNC	No Carry	No Carry	CF=0

JNE	Not Equal	A≠B	ZF=0

JNG	Not Greater	A<=B	ZF=1 or SF≠OF

JNL	Not Less	A>=B	SF=OF

JNL	No Overflow	No Overflow	OF=0

JNS	No Sign	A-B not negative	SF=0

JNZ	Not Zero	A≠B	ZF=0

JO	Overflow	Overflow	OF=1

JS	Sign	A-B negative	SF=1

JZ	Zero	A=B	ZF=1

The flags are not modified by any conditional jump; neither is anything else except EIP.

JP	(not covered)

JNP	(not covered)

JPE	(not covered)

JPO	(not covered)

JMP  addr:const

JMP  addr:regmem

JMP  addr:regmem16

Unconditional Jump.  EIP is set to the value of addr.    When the regmem form is used, the value of the operand gives the address. So if EAX=123, then “JMP EAX” causes a jump to location 123, and “JMP [EAX]” causes a jump to the location whose address is stored in the four bytes stored in memory at locations 123 to 126. It should be remembered that all jumps to constant addresses are relative jumps: “JMP 123” adds 123 to the current value of EIP, whereas the other forms are absolute.  All three of these instruction forms are called by Intel “Near jumps”.

JMP BYTE addr:const

Short Jump.  Exactly the same as the “Near jump” above, used only when the reaching the destination requires only a one byte value to be added to EIP.

JMP  seg:const : addr:const

This is what Intel refers to as a “Far Jump”.  First, CS is set to the value of the first constant seg, then EIP is set to the value of the second constant addr.  This is an absolute (non-relative) jump to code in a different segment. It is of no use in normal programming.

JMP FAR  dest:mem

JMP FAR  dest:mem16

This is another “Far Jump”.  First, a two byte value is read from the indicated memory location and stored in CS, then a four byte value is read from the following memory locations and stored in EIP.  This is an absolute (non-relative) jump to code in a different segment. It is of no use in normal programming.  In the 16-bit version, the second value read from memory (and stored in EIP) is also only a two-byte value.

LAHF

Load the flags into AH (the second least-significant eight bits of EAX), the rest of EAX is unmodified.  SF goes into the most significant of the eight bits, then ZF in the next, then five useless or undefined bits, and finally CF goes into the least significant of the eight bits. OF is for some reason (or not) left out.

LAR	(not covered)

LEA  dest:reg, addr:mem

LEA  dest:reg16, addr:mem

“Load Effective Address”.  The address of the memory location that addr refers to is calculated and stored in dest.  The memory location itself is not accessed.  “LEA  EAX, [ABC]”  is equivalent to  “MOV EAX, ABC”,  but the instruction is actually useful when more complex addresses are provided.  For example,  “LEA  EAX, [EBX+10]” is equivalent to what the non-existent instruction  “ADD  EAX, EBX, 10”  would do if it existed.

LEAVE	(not covered)

LGDT	(not covered)

LIDT	(not covered)

LGS	(not covered)

LSS	(not covered)

LDS	(not covered)

LES	(not covered)

LFS	(not covered)

LLDT	(not covered)

LMSW	(not covered)

LOCK	(not covered)

LODSB

“Load String of Bytes”.  Equivalent to “MOV  AL, [ESI]”, after which ESI is incremented by 1.  The flags are not modified.  May be used with the “REP” prefix (q.v.), but not usefully.  If DF is 1, the increment is replaced by a decrement.

LODSW

“Load String of Words”.  Equivalent to “MOV  AX, [ESI]”, after which ESI is incremented by 2.  The flags are not modified.  May be used with the “REP” prefix (q.v.), but not usefully.  If DF is 1, the increment is replaced by a decrement.

LODSD

“Load String of Dwords”.  Equivalent to “MOV  EAX, [ESI]”, after which ESI is incremented by 4.  The flags are not modified.  May be used with the “REP” prefix (q.v.), but not usefully.  If DF is 1, the increment is replaced by a decrement.

LOOP  addr:const

Decrement ECX, then if the result is not zero, jump to the given address.  This is a stunted instruction that can only perform a Short Jump. That means that the address given must be a label, and it must be within 128 bytes of this instruction.

LOOPE  addr:const

LOOPZ  addr:const

The same as LOOP, except that the jump only occurs if ECX is not zero and the ZF flag is 1.

LOOPNE  addr:const

LOOPNZ  addr:const

The same as LOOP, except that the jump only occurs if ECX is not zero and the ZF flag is 0.

LSL	(not covered)

LTR	(not covered)

MOV  dest:reg, src:reg

MOV  dest:reg, src:mem

MOV  dest:reg, src:const

MOV  dest:mem, src:reg

MOV  dest:mem, src:const

“Move”.  Dest is overwritten with the value of src.  The flags are not modified.

Both operands may be 8, 16, or 32-bit, but must be of the same size.

MOVSB

“Move String of Bytes”.  Equivalent to “MOV BYTE [EDI], [ESI]”, after which ESI and EDI are both incremented by 1.  The flags are not modified.  May be used with the “REP” prefix (q.v.).  If DF is 1, the increments are replaced by decrements.

MOVSW

“Move String of Words”.  Equivalent to “MOV WORD [EDI], [ESI]”, after which ESI and EDI are both incremented by 2.  The flags are not modified.  May be used with the “REP” prefix (q.v.).  If DF is 1, the increments are replaced by decrements.

MOVSD

“Move String of Dwords”.  Equivalent to “MOV  [EDI], [ESI]”, after which ESI and EDI are both incremented by 4.  The flags are not modified.  May be used with the “REP” prefix (q.v.).  If DF is 1, the increments are replaced by decrements.

MOVSX  dest:reg, src:reg16

MOVSX  dest:reg, src:reg8

MOVSX  dest:reg, WORD src:mem16

MOVSX  dest:reg, BYTE src:mem8

MOVSX  dest:reg16, src:regmem8

“Move with Sign Extension”.  This is a move that does not require the operands to be the same size. The destination must be larger than the source.  The sign of the number moved is preserved when the size is changed.

MOVZX  dest:reg, src:reg16

MOVZX  dest:reg, src:reg8

MOVZX  dest:reg, WORD src:mem16

MOVZX  dest:reg, BYTE src:mem8

MOVZX  dest:reg16, src:regmem8

“Move with Zero Extension”.  This is a move that does not require the operands to be the same size. The destination must be larger than the source.  The number is assumed to be unsigned, so the extra bits are filled with zeros.

MUL  EAX, src:regmem

“Unsigned Multiply”.  The 32 bit value in EAX is multiplied by src to produce a 64 bit result. The most significant 32 bits are stored in EDX, and the least significant 32 bits are stored in EAX.  The flags SF and ZF are left in an unpredictable state;  CF and OF are both set to 0 if the most significant half of the result (i.e. the value stored in EDX) is zero, or to 1 otherwise.

MUL  AX, src:regmem16

“Unsigned Multiply”.  The 16 bit value in AX is multiplied by src to produce a 32 bit result. The most significant 16 bits are stored in DX, and the least significant 16 bits are stored in AX.  The flags SF and ZF are left in an unpredictable state;  CF and OF are both set to 0 if the most significant half of the result (i.e. the value stored in DX) is zero, or to 1 otherwise.

MUL  AL, src:regmem8

“Unsigned Multiply”.  The 8 bit value in AL is multiplied by src to produce a 16 bit result which is stored in AX.  The flags SF and ZF are left in an unpredictable state;  CF and OF are both set to 0 if the most significant half of the result (i.e. the value stored in AH) is zero, or to 1 otherwise.

NEG  dest:regmem

NEG  dest:regmem16

NEG  dest:regmem8

“Negate”.  The value of dest is read, multiplied by -1, and the result is stored back into dest. The flag CF is set to 1 if the original value of dest is 0, or 1 otherwise.  SF indicates whether the result is negative,  ZF indicates whether the result is zero,  OF indicates that overflow occurred (i.e. the original value was the maximal negative number, which has no positive form using the two’s complement representstion).

NOP

“No Operation”.  Nothing happens. Execution continues as normal with the next instruction.

NOT  dest:regmem

NOT  dest:regmem16

NOT  dest:regmem8

Logical Negation.  The value of dest is read, every individual bit is inverted, and the result is stored back into dest. The flags are not modified.

OR  dest:reg, src:regmem

OR  dest:regmem, src:const

OR  dest:regmem, src:reg

32-bit bitwise logical or. Dest and src are read, the two values are orred together, and the result is written back to dest.  The flags CF, OF, ZF, and SF are all modified.  CF and OF are both set to zero; ZF indicates that the 32-bit result was zero; SF indicates that the result was negative (i.e. bit 32 of the result was 1). This is exactly the same as the ADD instruction except for the operation that is performed.

OR  dest:reg16, src:regmem16

OR  dest:regmem16, src:const

OR  dest:regmem16, src:reg16

16-bit bitwise logical or. Exactly the same as the 16-bit ADD, except that an OR operation is performed. Flags are set as above, for the 16 bit result.

OR  dest:reg8, src:regmem8

OR  dest:regmem8, src:const

OR  dest:regmem8, src:reg8

8-bit bitwise logical or. Exactly the same as the 8-bit ADD, except that an OR operation is performed. Flags are set as above, for the 8 bit result.

OUT	(not covered)	Send value to external i/o port.

OUTSB	(not covered)	Send string of bytes to external i/o port.

OUTSW	(not covered)	Send string of words to external i/o port.

OUTSD	(not covered)	Send string of dwords to external i/o port.

POP  dest:regmem

“Remove from Stack”.  Dest is overwritten with the value stored in memory at the location indicated by the Stack Pointer ESP (or SP in 16-bit mode).  The stack pointer in then Incremented by 4 (or 2 is 16-bit mode). The flags are not modified.

POPA

“Pop All 16-bit Registers”.  The 16-bit registers are popped from the stack, according to the following scheme:

DI	=	[ESP]

SI	=	[ESP+2]

BP	=	[ESP+4]

BX	=	[ESP+8]

DX	=	[ESP+10]

CX	=	[ESP+12]

AX	=	[ESP+14]

ESP	=	ESP+16

The flags are not modified.

POPAD

“Pop All Registers”.  The 32-bit registers are popped from the stack, according to the following scheme:

EDI	=	[ESP]

ESI	=	[ESP+4]

EBP	=	[ESP+8]

EBX	=	[ESP+12]

EDX	=	[ESP+20]

ECX	=	[ESP+24]

EAX	=	[ESP+28]

ESP	=	ESP+32

The flags are not modified.

POPF

“Pop FLAGS”.  A two-byte integer is popped from the stack (i.e. copied from [ESP], before ESP is incremented by 2). This two-byte integer over-writes the least significant 16 bits of the EFLAGS register.

POPFD

“Pop EFLAGS”.  A four-byte integer is popped from the stack (i.e. copied from [ESP], before ESP is incremented by 4). This integer completely over-writes the EFLAGS register, setting or clearing all flags.

PUSH  src:regmem

PUSH  src:const

“Save on Stack”.  The stack pointer ESP is decremented by 4 (or 2 in 16-bit mode), then the value of src is stored in the memory location indicated by the new value of ESP.

PUSHA

“Push All 16-bit Registers”.  The 16-bit registers are saved on the stack, according to the following scheme:

[ESP-2]	=	AX

[ESP-4]	=	CX

[ESP-6]	=	DX

[ESP-8]	=	BX

[ESP-10]	=	LeastSignificant16Bits(ESP)

[ESP-12]	=	BP

[ESP-14]	=	SI

[ESP-16]	=	DI

ESP	=	ESP-16

The flags are not modified.

PUSHAD

“Push All Registers”.  The registers are saved on the stack, according to the following scheme:

[ESP-4]	=	EAX

[ESP-8]	=	ECX

[ESP-12]	=	EDX

[ESP-16]	=	EBX

[ESP-20]	=	ESP

[ESP-24]	=	EBP

[ESP-28]	=	ESI

[ESP-32]	=	EDI

ESP	=	ESP-32

The flags are not modified.

PUSHF

“Push FLAGS”.  The least significant two bytes of the EFLAGS register are saved on the stack (i.e. ESP is decremented by 2, then the value is stored in the memory location indicated by the new value of ESP). The flags are not modified.

PUSHFD

“Push EFLAGS”.  The EFLAGS register is saved on the stack (i.e. ESP is decremented by 4, then EFLAGS is stored in the memory location indicated by the new value of ESP). The flags are not modified.

RCL	(not covered)	Rotate Left through CF.

RCR	(not covered)	Rotate Right through CF.

ROL	(not covered)	Rotate Left.

ROR	(not covered)	Rotate Right.

REP*	(not covered)	Various repeat prefixes.

RET

RET  num:const

“Return from Function”.  If num is not provided, it is taken to be zero.  This performs the opposite action to that of CALL. The value of EIP is popped from the stack, then num additional bytes are popped from the stack and ignored. The next instruction to be executed will be fetched from the address that was popped into EIP. The flags are not modified.

RETF

RETF  num:const

“Far Return from Function”.  If num is not provided, it is taken to be zero.  This performs the opposite action to that of CALL FAR. The value of EIP is popped from the stack, then the value of CS (the code segment register) is popped from the stack, then num additional bytes are popped from the stack and ignored. The next instruction to be executed will be fetched from the address that was popped into EIP. This instruction is not used in normal programming.  The flags are not modified.

SAHF

“Store AH into FLAGS”.  Bits 15 to 8 of EAX over-write bits 7 to 0 of EFLAGS.

SAL	(not covered)	Arithmetic Shift Left.

SAR	(not covered)	Arithmetic Shift Right.

SHL	(not covered)	Shift Left.

SHR	(not covered)	Shift Right.

STI

“Set Interrupt enable”.  Enables Interrupt processing. Until STI is performed, no interrupts generated outside the CPU will be detected. This instruction is not used in normal programming. (see CLI).

SUB  dest:reg, src:regmem

SUB  dest:regmem, src:const

SUB  dest:regmem, src:reg

32-bit integer subtraction. Dest and src are read, src is subtracted from dest, and the result is written back to dest.  The flags CF, OF, ZF, and SF are all modified.  CF indicates carry: the 33rd bit of the result would have been 1; OF indicates overflow: basically the result is incorrect; ZF indicates that the 32-bit result was zero; SF indicates that the result was negative (i.e. src was greater then dest).

SUB  dest:reg16, src:regmem16

SUB  dest:regmem16, src:const

SUB  dest:regmem16, src:reg16

16-bit integer subtraction. Exactly the same as the 32-bit version, except that only 2 bytes are read from each operand, only a 16 bit subtraction is performed, and only 2 bytes of the destination are overwritten with the result. The flags are all set according to the 16 bit result: CF is set if bit 17 would have been 1, SF is set if bit 16 is 1, etc.

SUB  dest:reg8, src:regmem8

SUB  dest:regmem8, src:const

SUB  dest:regmem8, src:reg8

8-bit integer subtraction. Exactly the same as the 32-bit version, except that only 1 byte is read from each operand, only an 8 bit subtraction is performed, and only 1 byte of the destination is overwritten with the result. The flags are all set according to the 8 bit result: CF is set if bit 9 would have been 1, SF is set if bit 8 is 1, etc.

XCHG  aaa:reg, bbb:reg

XCHG  aaa:reg, bbb:mem

XCHG  aaa:mem,  bbb:reg

“Exchange or Swap”.  The values aaa and bbb are read, swapped, then written back to their original locations. The operands may be 8, 16, or 32 bit integers, but in any one instruction both must be the same size.  The flags are not modified.

XOR  *, *

“Exclusive Or”.  This instruction is exactly the same as the regular OR instruction in every way, except that an Exclusive Or operation is performed on the operands instead of the regular inclusive or.