Options for The Assembler

Normal Use

assem filename

Input file is filename, “.asm” added if there are no dots in it.  Output file is same name with “.bin” replacing the “.asm”.

assem -

The input is read from standard input instead of from a file.

A large number of filenames (or dashes) may be given on the command line.  Each is processed in turn, the output being assembled into a single output file.  The default name for the output file is taken from the first input file.

Optional additions

-bin

Produce complete unformatted executable file.

This is required if the file is to be written directly to disc for execution.

-o=filename

or

-o filename

Changes the name of the output file.

-l

A complete listing is produced, repeating each line of the input file, following it with the output it produced showing addresses, in hexadecimal.

-l=filename

The listing is written to a file.

-L

or

-L=filename

Same as “-l”, but a different listing format is used.

-report label

After the assembly process is complete, the final value of the label is displayed.

On Rabbit, The Assembler and The Emulator are installed in /usr/local/bin, which should normally be in every user’s path. On other systems, who knows: ask the administrator.  Neither The Assembler nor The Emulator require and special privileges, so if they are not installed, it doesn’t matter.  Just put them anywhere convenient.

Options for The Emulator

Normal Use

emu

Enters interactive setup mode, in which all parameters and initial states, including registers, processor modes, and input files may be selected before anything happens.

emu -load filename

The named file, assumed to be in the simple “.bin” format (actually a lack of format), is loaded into memory at the default starting address (0x100000, but this may be changed in interactive setup).  Interactive setup mode is entered before anything happens.

emu -load filename -go

The named file is loaded into memory and execution is started automatically. No user interaction is required.

Additional Command-line options

-capture

All output from the running program is captured and stored in a file called emu.out.

-quiet

No additional informative output is produced apart from the output from the running program and required responses to interactive commands.

Interactive Commands

After interactive setup mode has been dispensed with (just press enter), The Emulator enters single step mode.  Every time you press ENTER, one instruction is executed. The instruction itself is printed. Any memory locations accessed are printed before the instruction is executed, anything that was modified is also printed afterwards.  After execution of each instruction, the values of all registers and the current stack frame (between EBP and ESP) are printed, and the next instruction to be executed is printed.  Instead of just pressing ENTER, a command may be entered at any time. This does not interfere with execution.

Numbers

When commands accept numeric or address operands, they may be typed in decimal or in the C notation for hexadecimal: “0x” immediately followed by a sequence of hexadecimal digits.

exit

Exit.

load filename

The contents of the file, whatever it is: text, executable code, anything, is loaded into memory at the program start address (usually 0x100000).

load filename address

The contents of the file is loaded into memory at the given address.

show address

The contents of a few bytes of memory starting at the given address is displayed.

show

The next few bytes, immediately following those most recently showed, are displayed.

enter address

Type bytes in hexadecimal, separated by spaces.  They are stored in memory starting at the address given.

enter

Same as enter as above, but the first memory address written is the last operand given to a show command, or the address following the last one written by an enter command, whichever happened most recently.

break address

Set a breakpoint at the given address.  Whenever execution reaches a breakpoint, the program stops, returning the emulator to interactive command mode.

break

Report the currently active breakpoint address.

r

Display the values of all the registers.

run address

Start execution from the given address.  Execution is automatic, not single-stepping.  Automatic execution continues until a HLT instruction is executed, or a RET when the stack is empty, or when something goes wrong (usually an exception), or if you are running under unix and type control-C.

run

Run as above, but starting from the instruction that would have been executed next if you just pressed ENTER to single step.

step

Execute just one instruction, reporting all operands and results. The same is pressing ENTER.